**************************************************************************

*   Genius LAN GE2000II series card NDIS drivers for WFW311     V100R00  *

**************************************************************************

Date: 07/22/1994

To install the GE2000II series card NDIS 2.0.1 and/or NDIS 3.0 driver for

Microsoft Windows for Workgroups, fllow the instruction given below:

1. Insert GE2000II driver diskette in the system's fllopy drive.

2. In the Windows for Workgroups 3.11 Program Manager, double click on the

Windows Setup icon from the MAIN window.

3. While in the Windows Setup dialog box, select the Change Network Settings

from the Options menu to install driver from GE2000II driver diskette.

4. From the Network Setup dialog box, select the "Drivers..." button to

install the GE2000II NDIS 2.0.1 and NDIS 3.0 drivers.

5. Then, in the Network Drivers dialog box, select Add Adapter button to

install the GE2000II device drivers.

6. Now, in the Add Network Adapter window, choose the "Unlisted or Updated

Network Adapter" from the menu and then select OK.

7. The Install driver dialog box will appear. Enter the GE2000II driver

diskette and specify the WFW311 path to install the PCnet drivers. For

example,

A:\WFW311

Then select OK.

8. The Gniue LAN GE2000 Ethernet Adapter will appear under the Network

Adapters list. Simply select OK to continue.

9. Then, the user will be asked a series of questions for the correct

Interrupt channel, I/O base address...,etc.

10. Then, select OK to exit. Windows for Workgroups 3.11 will ask the user to

reboot the machine once installation is complete.