GENSURF

General Surface Modeling Program

Version 1.00

Graphics Software Technologies

P.O. Box 3064

Orange, CA  92391

Copyright (c) 1992-1993 by Graphics Software Technologies

All Rights Reserved

REGISTRATION

Shareware is a term for software that can be freely copied and

shared. The term describes copyrighted software which the author

supports and encourages people to copy and share.

The concept is based upon these principles:

People need to try programs to see if they are useful.

Software authors can be supported directly by users.

Copying and networking of programs can be encouraged.

We encourage  you to register your 500 point version of GENSURF

for $39.00 or to order the 10,000 point version for $89.00.

Registration has a number of benefits to you:

You   will  receive the most current version of the program

if the one you register is not the most current.

You will receive a current manual if the one you registered

is not the most current.

You will encourage us to develop more useful programs.

1.   Introduction

GENSURF is a complete GENeral SURFace modeling and contouring

package that allows you to input contour data from the keyboard

into a text file. The program creates triangulated irregular

networks (TIN), contours and 3D meshes directly in the AutoCAD

drawing.

Version 1.00

500 points maximum

3D TIN as AutoCAD faces

Processes 1000 TIN's in 5 seconds (486/50)

Straight contour lines

20 x 20 mesh

Documentation on diskette

Version 2.00

10,000 points maximum

3D TIN as AutoCAD faces

Processes 20,000 TIN's in 40 seconds (486/50)

Straight contour lines

Rounded contour lines

Contour lines that honor the data

400 x 400 mesh maximum

Documentation on diskette

In order to keep costs to a minimum we do not provide direct

technical support.

2.   Installation

Make a directory named GENSURF. Copy all the files from the

GENSURF diskette to the GENSURF directory by using the DOS

COPY command.

The files you should have are:

GENSURF.EXP

GENSURF.LSP

GENSURF.MNU

POINTS1.DAT

POINTS1.PAR

POINTS2.DAT

POINTS2.PAR

POINTS3.DAT

POINTS3.PAR

POINTS4.DAT

POINTS4.PAR

GENSURF.DOC

3.  Starting the Program

Start AutoCAD as you would for any drawing. Call the drawing file

POINTS1. Then load the special menu called GENSURF.MNU. To start

GENSURF just pick the SETUP choice in the GENSURF menu. This will

load the program and initialize it. You can now process the

POINTS1.DAT data file, make triangles, make contours, make meshes,

and plot any of them.

Although the data file can have any name and extension, it is

that the drawing name you start with is the same as the data file.

The data file should have the extension of ".dat". The only problem

with having a different data file name than the drawing and a

different extension is that the EDIT DATA FILE menu command will

not work correctly. You can still edit any ASCII data file from

AutoCAD by using the shell command and giving the command line as

the text editor followed by the data file name.

GENSURF creates several work files. These files are:

dwgname.BIN

dwgname.PAR

dwgname.TIN

dwgname.MSH

GENSURF creates the following layers:

gs-point

gs-tri

gs-cont

gs-index

gs-mesh

3.   AutoCAD Menu

Setup                 this installs the Contour program and

lisp

-------------------

Read Points XYZ       this will prompt for the point data

file name in an ASCII X, Y, Z format

Edit Point File       this will place the points data

file in the text editor for editing

Edit Parameters       this will place the parameter file

in the text editor for editing

-------------------

Plot Points           this plots the points in the drawing

Plot Points Fast      this plots ghosted point on the screen

-------------------

Process Triangles     this process the TIN

Plot Triangles        this will plot the triangles in the

drawing

Plot Triangles Fast   this will plot a ghosted TIN

-------------------

Process Contours      this process the contour lines

Plot Quick Contours   this plots the contour polygons as

straight lines

Plot Contours Fast    this plots ghosted contour lines

-------------------

Process Mesh          this will make a 3D mesh

3D Mesh Faces         this will plot colored 3D mesh faces

-------------------

ACAD Menu             this places you back in the regular

AutoCAD menu

SETUP

The setup menu item runs a shall lisp program which installs

the GENSURF ADS program.

READ POINTS XYZ

This menu will display the copyright notice and request the points

file name. It then reads all the data for further processing. The

points are in an ASCII file with one X, Y, and Z point triplet per

line.

EDIT POINT FILE

This menu item causes the program to place you in the text

editor with the points file. You can them edit any

coordinate.

EDIT PARAMETER FILE

This menu item places you in the text editor with the

parameter file which may then be modified.

PLOT POINTS

This will plot the points in the drawing. The points are on

a layer named

PLOT POINTS FAST

This plots the points using an AutoCAD GRDRAW command. It is

very fast since the points do not become a part of the

drawing. Any command which causes a redraw will erase this

image.

PROCESS TRIANGLES

This create the TIN.

PLOT TRIANGLES

This will plot the triangles in the drawing. The triangles

are on a layer named

PLOT TRIANGLES FAST

This plots the triangles using an AutoCAD GRDRAW command. It

is very fast since the triangles do not become a part of the

drawing. Any command which causes a redraw will erase this

image.

PROCESS CONTOURS

This will create the connected contour lines. The lines are

stored as polygons.

PLOT QUICK CONTOURS

This will plot the contour polylines in the AutoCAD drawing.

They are drawn as straight lines with no rounding. The lines

honor all data points. The polylines could be smoothed using

an AutoCAD command.

PLOT CONTOURS FAST

This plots the contour polygons using an AutoCAD GRDRAW

command. It is very fast since the are not  part of the

drawing. Any command which causes a redraw will erase this

image.

PROCESS MESH

This creates the mesh

PLOT MESH XY

This plots a mesh in the AutoCAD drawing. Each line in the X

direction is a single polygon as is each line in the Y

direction.

4.   Data File

The points data file is a file containing the X, Y, Z

information for each point of data to be used in the

calculations of the contour map. The program will prompt for

the data file name. The name may end with any extension.

GENSURF expects the data to be stored in an ASCII text file.

This file may be created from a text editor, spreadsheet,

output from a data collector, or entered directly from the

DOS editor.

The X, Y, Z data are entered in free format. That means they

can be located anywhere on the line as long as they are

separated by at least one space.

460.    0.    150.

540.    0.    150.

480.  300.    152.

560.  280.    152.

620.  700.    155.

705.  700.    155.

465.  100.    150.5

545.  100.    150.5

470.  160.    151.

550.  160.    151.

475.  240.    151.5

555.  240.    151.5

530.  440.    153.

615.  440.    153.

570.  560.    154.

655.  560.    154.

590.  610.    154.5

675.  610.    154.5

510.  380.    152.5

595.  380.    152.5

5.   Parameter File Format

0.00  =    easting (x) minimum

1000.00  =    easting (x) maximum

0.00  =    northing (y) minimum

1000.00  =    northing (y) maximum

0.00  =    minimum elevation

250.00  =    maximum elevation

5.0   =    contour interval

50.0   =    heavy contour interval

1.50  =    elevation exaggeration

Contour Parameter File .PAR

The Parameter File contains the program parameters that determine

ranges of data for calculations, plotting conditions, and mesh

information.

Each line contains a text description of the parameter followed

by an equal sign and then followed by the corresponding data.

Changes can only be made to the values to the right of the equal

sign (=). You cannot add, delete, or move any lines in this file.

A minimum of one space must precede and follow the equal sign.

Not all values are checked by CONTOUR/386, therefore, mistakes

may cause unusual behavior.

Listed below is the information file, with example data and an

explanation of the use of each parameter.

min x calc         = 10000.00

This value determines the left most boundary of data

that will be read in and used for calculations.

max x calc         = 18000.00

This value determines the right most boundary of data

that will be read in and used for calculations.

min y calc         = 20000.00

This value determines the bottom boundary of data that

will be read in and used for calculations.

max y calc         = 27500.00

This value determines the upper boundary of data that

will be read in and used for calculations.

min z calc         = 50.00

This value determines the lowest elevation of data that

will be read in and used for calculations. This number

is usually an integer and used as the starting

elevation for plotting contour lines.

max z calc         = 85.00

This value determines the highest elevation of data

that will be read in and used for calculations.

cont interval      = 2.0

This value determines the elevation interval of

calculated and plotted contour lines. In the example, a

contour line will be plotted every 2 feet.

cont heavy int     = 10.0

This value is the annotated contour line interval. For

example, for a contour interval of 2 feet, a heavy line

will be drawn every 10 feet or each fifth interval when

cont heavy int is set to 10.0. A value of 0.0 indicates

no heavy contour will be drawn.

plot z scale factor = 0.0

When plotting in 3D, it is often necessary to

exaggerate the elevations in order make them look

appropriate. The plot z scale factor allows you to

exaggerate the elevation scale. There are three values

available for selection:

< 0.0  automatic z factoring

= 0.0  uses actual elevation

> 0.0  uses the factor as a multiplication factor

7.   EDITOR

GENSURF makes use of the DOS editor EDIT. The purpose of the

editor is to quickly and easily edit GENSURF data files.

When you have completed your editing by either quitting or

saving a file and exiting, you will be back in AutoCAD. If

you save the file before returning to the program, the

editor automatically saves a backup file with the extension

".BAK".

If you have made changes to your data file then you will

need to load the points file again by chosing that option in

the menu.

8.   THEORY AND CONCEPTS

Data Selection

The GENSURF program reads all of the data to create a

contour map. It is possible to use only a subset of the data

for creating a map. There are two situations where this may

be required:

1.   There is more data in the file than you want to use to

make your map.

2.   You may want a map containing only certain contour

intervals.

There are two methods of data selection. One method is to

define the window of included data by entering the

appropriate values in the .PAR file for:

x min calc

x max calc

y min calc

y max calc

The second method is to define the range of acceptable

elevations. If a point contains an elevation outside this

range, it will not be included in the calculations.

Creating the TIN

GENSURF is based upon the method and algorithms for

Triangulated Irregular Network or TIN's. GENSURF generates

true Delauney triangulation.

The program determines the best way to connect the points to

form Delauney triangles. It analyzes these triangles in a

fast, single pass procedure.

Interpolated Contour Strings

Once the triangles are established, the program locates the

point of crossing of the specified contour interval for each

leg of the triangle.

The program will then connect all of the located points of

the same contour values for each triangle. These lines are

referred to as elements.

Next, the program will sort all of the elements by

elevation, taking into account the adjoining of the points

creating an AutoCAD polyline.

The polylines may then be displayed by by using the Quick

method.

Display Methods

There is one method for displaying the contour lines that

have been calculated.

The QUICK method displays the contour lines as straight

lines between the interpolated points along each triangle.

This method is useful for quickly checking changes made in

the map, as well as producing final maps which have a lot of

data points.

The FAST method will display the line using GRDRAW. This

method allows you to see the lines without having them

actually become part of the drawing.

GENSURF REGISTRATION FORM

Name________________________________________________

Company_____________________________________________

Address_____________________________________________

City, State, Zip____________________________________

Daytime telephone___________________________________

To  register,  send  $25.00  for the 500 point version or

$89.00 for the 10,000 point version (money order, or personal

check  -  California residents add 7.75% sales tax) to:

Graphics Software Technologies

P.O. Box 3064

Orange, CA 92665

Likes, dislikes, features wanted, problems?  List below:

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________