Serial File Copy

Version 2.1

Copyright (C) 1989 & 1990

By MarshallSoft

All rights are reserved.

MarshallSoft

PO Box 4543

Huntsville, AL. 35815

(205) 881-4630

page 1

TABLE OF CONTENTS

Chapter                          Page

-------                          ----

1.0  Introduction.....................  3

2.0  Summary of SFC Features..........  3

3.0  Distribution Files...............  4

4.0  Purchasing a Null Modem Cable....  4

5.0  Building a Null Modem Cable......  5

6.0  Running SFC......................  7

6.1  Startup Menu................  7

6.2  Startup Problems............  8

6.3  Status Window...............  9

6.4  Command Menu................ 10

6.5  Terminating SFC............. 11

7.0  Cloning SFC...................... 12

8.0  Support Programs................. 13

8.1  CLONE, GETSFC, and PUTSFC... 13

8.2  LOOPBACK.................... 13

8.3  TERMINAL.................... 13

8.4  MODEM....................... 13

8.5  PORTEXAM.................... 13

8.6  GETFILE and PUTFILE......... 14

8.7  CHECKCRC.................... 14

8.8  CRYPT....................... 14

8.9  SPACE....................... 14

9.0  Performance...................... 15

9.1  Test 1...................... 15

9.2  Test 2...................... 15

9.3  Test 3...................... 15

9.4  Other Tests..................15

10.0  CRC Calculations................. 16

11.0  User Registration................ 16

12.0  Revision History................. 17

13.0  Warranty......................... 18

14.0  Final Comments................... 18

page 2

1.0 Introduction

The  Serial  File  Copy  program  SFC  is  designed  to allow the

transfer of disk files ( including entire directories  which  may

themselves   contain   directories  )  between  any  two  IBM  PC

compatible computers using standard RS232 serial ports.  The  SFC

program  is  particularly  useful in copying files between IBM PC

compatibles with 5.25" disks and those with 3.5"  disks  such  as

the  IBM  PS/2  and  the  various  IBM PC compatible laptops. The

computers must both be IBM PC  compatible  (  this  includes  the

TANDY 1000 ) with 256 KB or more of RAM.

Both  computers ( between which you want to transfer files ) must

have RS232 serial ports configured for use with  a  modem.  COM1,

COM2,  COM3  or  COM4 may be used. Baud rate of 1200, 2400, 4800,

9600, 19200, or 38400 may be selected. Baud rates  of  57600  and

115200 are available in the registered version.

There  are only two differences between the shareware version and

the registered version of the SFC program. The first is that  the

registered  version  can  run  at  baud rates of 57600 and 115200

baud. The second is that the registered version does not have the

opening shareware screen.

2.0 Summary of SFC Features

o Supports COM1, COM2, COM3 and COM4.

o Supports baud rates from 1200 to 115200.

o Select groups of files with DOS wildcards ( eg: *.DOC )

o Select entire directories ( recursively ).

o Complete disk & transfer status on screen.

o On line help provided for all SFC commands.

o Option to send only files with a later timestamp.

o Load DOS Shell from within SFC.

o Full 16 bit CRC checksum is used.

o Bad file packets are automatically retransmitted.

o Change logged drive and path from within SFC.

o Display directories from within SFC.

o File date & time preserved after copying.

o Supports command line parameters.

o Supports batch file lists ( lists of files to send ).

o Option to send or skip any file from selected group.

o Can print files on remote computer.

page 3

3.0 Distribution Files

If a previous version of SFC is used  to  copy  the  distribution

files  be  sure  to rename the older version ( eg: rename sfc.exe

sfc_old.exe ) so that there  is  no  attempt  to  run  the  older

version  with the newer version. The following files should be on

the SFC distribution disk:

SFC.EXE  --  The executable Serial File Copy ( SFC ) program.

SFC.DOC  --  This documentation file.

SFC.HLP  --  File required for on-line help ( Type F1 for help ).

SFC.QRC  --  Quick reference card.

SFC.INV  --  Invoice ready to print.

SFCSW.EXE -  Displays shareware screen for SFC.

Registered users are also provided  with  the  following  support

programs.  None  are  necessary  in  order  to  run  SFC, but are

provided as a "Thank You" for registering. For a  description  of

these  programs  see  the  sections  on  "Support  Programs"  and

"Cloning SFC".

CLONE.COM    -- 'Clones' SFC on the target computer.

GETSFC.COM   -- Created by CLONE on the target computer.

PUTSFC.COM   -- Sends SFC to GETSFC on the target computer.

LOOPBACK.COM -- Performs loopback testing on specified port.

TERMINAL.COM -- Communications program with XMODEM & scripts.

TERMINAL.DOC -- TERMINAL documentation file.

MODEM.EXE    -- Controls and displays modem control status.

PORTEXAM.EXE -- Displays all information about specified port.

GETFILE.COM  -- Small file transfer program used with PUTFILE.

PUTFILE.COM  -- Small file transfer program used with GETFILE.

CHECKCRC.EXE -- Computes the CRC for any file.

CRYPT.EXE    -- Encrypts and decrypts any file.

SPACE.COM    -- Displays free disk space and allocation units.

4.0 Purchasing a Null Modem Cable

A 'null modem cable' is necessary to connect  the  two  computers

together using the RS232 ports ( the port used to talk to a modem

).  A  null  modem cable can be purchased ready for use or can be

assembled by connecting a 'null modem adapter' to a normal  RS232

modem cable.

Null  modem  adapters  and  cables  can be purchased from several

sources such as Radio Shack and most  electronics  parts  houses.

"Computer  Shopper"  regularly  carries  advertisements  for null

modem adapters and cables. Be advised that  not  all  null  modem

adapters  and cables connect all the lines that may be necessary.

Although all should work for SFC some may not work for the  MSDOS

CTTY command which is used by the CLONE program.

Read  the  following section on building a null modem cable for a

discussion on cable requirements.

page 4

5.0 Building a Null Modem Cable

Building the null modem cable is easy. The only  tricky  part  is

being  absolutely  positive  that you have identified the correct

pins. Refer to the diagrams below as necessary. The pin numbering

is shown for the front of each connector, where the pins ( male )

or holes ( female ) are located. When viewed from the back side (

where the wires are soldered ) the pins  and  holes  run  in  the

opposite direction. Soldering is recommended, although solderless

connectors which use crimp pins can be used instead. You can also

solder each wire to a crimp pin as its much easier than soldering

the wire directly onto the connector.

Male DB 25                       Male DB 9

*****************************************    *****************

*  1  2  3  4  5  6  7  8  9 10 11 12 13  *  *  1  2  3  4  5  *

*  14 15 16 17 18 19 20 21 22 23 24 25  *    *   6  7  8  9  *

***************************************      ***************

Female DB 25                     Female DB 9

*****************************************    *****************

* 13 12 11 10  9  8  7  6  5  4  3  2  1  *  *  5  4  3  2  1  *

*   25 24 23 22 21 20 19 18 17 16 15 14 *    *   9  8  7  6  *

***************************************      ***************

DB 25   DB 9    Pin Definition

=====   ====    ==============

7       5      Signal Ground ( SG )

2       3      Transmit Data ( TX )

3       2      Receive Data ( RX )

20       4      Data Terminal Ready ( DTR )

6       6      Data Set Ready ( DSR )

4       7      Request to Send ( RTS )

5       8      Clear to Send ( CTS )

8       1      Carrier Detect ( CD )

SFC  itself  requires only signal ground (SG), transmit (TX), and

receive (RX) lines in order to operate.  The  first  step  is  to

determine  the  type  of  connectors  required. Examine the RS232

connectors  on  your  computers.  You  will  need   to   purchase

connectors  that mate with the computers connectors. For example,

lets say that you have a laptop with a male DB 9 connector and  a

desktop  with a female DB 25 connector. Then you need a female DB

9 and a male DB 25 connector for your cable. IBM ATs and  laptops

require  a  female  DB  9  to  mate  while most XT class machines

require a female DB 25 to mate.

The  DB  25  and  DB  9  connectors  can  be  purchased  at  most

electronics  parts  houses.  Radio  Shack also stocks both solder

type and solderless DB connectors.

Next you will need about 6 feet of ordinary  4-wire  solid  phone

cable,  although  any  3-wire  cable  of  24  to  20 gauge can be

page 5

substituted. If the CLONE program is to be used then 5 wires  are

required.  Up  to 50 feet of cable can be used. Shielded cable is

even better but not necessary. Cable can be purchased by the foot

at most stores that sell it.

Once the connectors and cable have been purchased you  are  ready

to  assemble  your  null  modem  cable.  First connect the signal

ground ( SG ) line  straight  between  connectors.  Then  connect

Transmit Data ( TX ) from one connector to Receive Data ( RX ) on

the  other.  If you wish to use the SFC cloning program CLONE you

must also connect Data Terminal  Ready  (  DTR  )  from  the  end

connected to the computer running CLONE to Data Set Ready ( DSR )

on  the  other  end.  Some  computers ( like my TANDY 3000 ) also

require you to connect Request To Send  (  RTS  )  from  the  end

connected  to the computer running CLONE to Clear To Send ( CTS )

on the other end.

You then should have either 3 ( or 5 in  order  to  use  CLONE  )

lines  connected  between connectors. Once completed, test the TX

and RX connections by running the  TERMINAL  (  or  any  terminal

emulator  )  program on both sides. Whatever is typed on one side

is  displayed  on  the  other.  Finally,   test   the   remaining

connections by cloning SFC from one computer to the other.

Refer  to the Pin Definition Table above in order to identify the

correct pins for your connector. When viewing the connector  side

of  both  the  DB  9 and DB 25 with the wide part at the top, the

pins are numbered left to right, top to bottom.  Most  connectors

have  the  pin  numbers  on  the  connector  itself but are often

difficult to read because they are very small.

If  you  have  never  soldered  before,  you  should  use  either

solderless connectors ( you crimp the wires on ) or find a friend

who  has  some  experience  soldering.  Remember  that  soldering

doesn't have to be pretty in order to  work.  Practice  a  little

first  with  some spare wire if you are out of practice. When you

have completed construction of your null modem cable, verify that

the correct pins have been connected. If you cannot  get  SFC  to

operate, then most likely you made a mistake in constructing your

cable.

page 6

6.0 Running SFC

Connect  your  PC compatible computers together with a null modem

cable between the RS232 ports ( the port used to talk to a  modem

).  Be  sure  that you are using a proper null modem cable rather

than a standard cable.

Normally, SFC can be started on both computers by typing 'SFC' on

each. This is referred  to  as  the  AUTOSYNC  startup.  On  some

computers,  AUTOSYNC will not work well. In this case, choose one

computer to be the MASTER and the other the SLAVE.  It  makes  no

difference which is which. Start SFC on the MASTER by typing "SFC

M"  followed  by  the ENTER key. Start SFC on the SLAVE by typing

"SFC S" followed by the ENTER key.

Additionally, both  the  COM  port  and  the  baud  rate  can  be

specified  on the command line by typing "SFC X Y Z" where X is A

for AUTOSYNC, M for MASTER or S for SLAVE; Y is 1 for COM1  to  4

for COM4; and Z is the baud rate ( 1200 to 115200 ). For example,

"SFC  M  2  57600" will start up SFC on COM2 at 57600 baud as the

MASTER, and "SFC S 1" will start up SFC on COM1  at  the  default

baud  rate as the SLAVE. If AUTOSYNC is specified on one side, it

must also be specified on the other. Any errors  on  the  command

line are ignored and defaults are used.

6.1 Startup Menu

After  startup,  a  menu  will  be displayed at the bottom of the

screen with 4 options. Use up and down arrow  keys  to  move  the

menu  bar  -- use ENTER to make selection. Note that on-line help

is available by typing function key 1 ( F1 ).

CONNECT: Attempts to connect to the other computer. The  baud

rate must be the same on both computers.

SET BAUD: Allows the user to select the baud rate. 57600 baud

and 115200 baud are available only in the registered version.

Note  that  some  4.77 MHZ PC compatibles won't run at 115200

baud.

SET PORT: Allows the user to select the communications  port,

COM1  to  COM4.  Of  course, hardware must exist for the port

selected.

SET PRINTER: Allows the user to select the printer port, PRN1

or PRN2 for printing by the remote system.

QUIT: Quits and returns to MSDOS.

Set the port and/or baud rate on both computers if the default  (

COM1  at  38400  in  shareware  version  and 115200 in registered

version ) is not acceptable. Then select the CONNECT option.  SFC

will now attempt connection to the other computer.

page 7

6.2 Startup Problems

If  the  message "Connection completed" is not displayed then the

computers are not talking to each other. The problem may  be  one

of the following:

(1)  You  must  start  SFC  on  both computers! SFC should be

started on both computers within one minute of each other.

(2) The wrong port is selected. For example, if SFC  is  told

to use COM1 but COM2 is actually connected to the cable.

(3)  You  may  have  selected different baud rates. They must

match! The default baud  rate  is  38400  for  the  shareware

version and 115200 for the registered version.

(4)  The  selected  baud rate is too fast. Run at the fastest

processor speed possible ( for  multi  speed  machines  )  or

select a slower baud rate.

(5)  You do not have a null modem cable connecting the two. A

normal cable ( without a null modem adapter ) will not  work.

Test the null modem cable by running TERMINAL or any terminal

emulation program. Whatever is typed on one side is displayed

on the other. If this does not happen then you have a problem

with your null modem cable.

(6)  One of the computers may be configured as DCE ( in order

to talk to a serial printer ) instead of DTE (  in  order  to

talk  to a modem ). In this case, a normal cable ( instead of

a null modem cable ) will work.

(7)  Your  serial  port  is  defective.  This  is   unlikely.

Registered users can run LOOPBACK to test the port.

(8)  COM1  &  COM2 work but COM3 & COM4 don't. There are some

differences in the way COM3 and COM4 are set up among various

IBM compatibles. Use COM1 or COM2  instead.  (  MSDOS  should

place  the  address  of  the COM3 port at 40:0004 and COM4 at

40:0006 ).

(9) Your PC is not IBM hardware compatible! For example,  the

TANDY  2000  is  not  hardware compatible with respect to the

serial ports.

If this is the first attempt to run SFC then use the manual mode:

Type "SFC M" on one computer and "SFC S" on  the  other.  If  you

still  cannot  get  communication  between  computers, registered

users can call  the  technical  support  telephone  number  (205)

881-4630  after  5:00  PM  CST  Monday through Friday or Saturday

after 2:00 PM CST for assistance.

page 8

6.3 Status Window

The "Status" window contains information having to  do  with  the

status of SFC. The definition of each field follows:

PORT: The Communications port ( COM1 to COM4 ) is displayed.

BAUD: The baud rate ( 1200 to 115200 ) is displayed.

UNIT:  #  bytes  per  allocation unit is displayed. Space for

disk files is always allocated as a multiple of this value.

DRIVE: The currently logged drive is displayed.

PATH: The current directory path is displayed.

MODE: Runtime mode is displayed.

"ready"   -- Displayed when ready to accept keyboard input.

"MENU"    -- Displayed after having selected a menu command.

"send"    -- Displayed while sending files.

"receive" -- Displayed while receiving files.

FREE: Free disk space in bytes is displayed.

Three runtime options ( ASK, DIR, CHK )  are  also  displayed  as

either "On" or "Off". All options can be toggled between "On" and

"Off"  by  use  of  the "Toggle Options" selection of the command

menu ( See section 6.4 ).

ASK: Asks the user for permission before sending  each  file.

Normally  off.  Typically  selected  when  using wildcards in

specifying a range of files for which  you  wish  to  exclude

some files.

DIR: Enables sending of directories ( recursively ). Normally

on, toggle this off if you wish to ignore directories.

CHK: Send only files with a later date & time if file already

exists.  Normally  off,  toggle  this  on if you want to do a

backup of more recently modified  files.  "...skipped..."  is

displayed  if  the  file  is  not  copied  because of the CHK

option.

Notes: (1) All options need only be set on the sending  computer.

(2)  Be sure that the clocks on your two computers are set to the

same date and time ( to within a few seconds )  if  you  plan  on

using the CHK option.

page 9

6.4 Command Menu

The  Command  Menu contains the choices the user has once SFC has

connected. Use the up and down arrow keys to move  the  menu  bar

and  ENTER  to  make  your  selection. Once one of the choices is

selected, the user can return without SFC performing the selected

function by typing the ESC key. Type  the  F1  function  key  for

help.

QUIT: Quits SFC and returns to MSDOS.

SET  CURRENT DRIVE: Changes the currently selected drive. The

user can "log onto" any legal DOS drive ( A, B,  etc.  ).  If

you  change  diskettes  then  you  must always select the SET

DRIVE option before continuing.

SET CURRENT PATH: Changes the currently selected  path.  When

this option is selected, a "Enter Path:" prompt is displayed.

The user then edits the current path which is displayed.

All  paths  must  start  with a backslash ( full path name ).

When the user is satisfied, typing ENTER will save the  path.

Then  MSDOS  will  attempt  to  change the "current" path. If

successful, the new path will be displayed  in  the  "Status"

window.

DIRECTORY: Displays a directory of files requested. When this

option  is selected, a "Enter files" prompt is displayed. The

user then types the file or files ( using the MSDOS wildcards

* and ? ). Filenames can be edited similarly to paths.

SEND FILE(S): Sends files to the other computer.  No  special

action  is  required  on  the receiving side except that both

sides must have previously selected CONNECT. The user will be

prompted for the filename in the same way as when "Directory"

was selected including the use of the MSDOS wildcards  *  and

?.  If you specify that a directory is to be sent and the DIR

option is set to "On", then all files  in  that  directory  (

including  other  directories  )  will  be  sent. In this way

entire directory structures can be copied. The date and  time

for normal files ( but not directories ) will be preserved.

An  indirect  file  can  also  be  specified by prefixing the

filename by the '@' character. An indirect file is a standard

ASCII text file containing the list of files to be sent.  The

indirect  file must have been previously prepared and contain

one filename ( wildcards accepted ) per line. Indirect  files

are  typically  used  to  specify  a  list  of files that are

transferred on a regular basis.

page 10

REMOTE PRINT: Sends the specified file to the remote (  other

)  computer  for  printing.  No  wildcards  are  allowed. The

printing can be canceled at any time by typing any key before

the normal end of the print job.

TOGGLE OPTIONS: Allows the user to toggle each of  the  three

runtime options ( ASK, DIR, and CHK ). The current setting is

always  displayed in the status area. For a definition of the

runtime options, see SFC Status Window above.

DOS SHELL: Loads the DOS shell, allowing the user to  execute

DOS  commands.  The  user must be very careful not to execute

any command that changes the serial  ports  or  modifies  the

screen   environment.   Please  note  that  many  application

programs do change the screen environment.  It  is  safer  to

stay  with  the  standard  DOS  commands  unless a particular

executable has been previously tested.

When entering text in response to any of the above commands, type

ESC to return back to the main menu without selecting. The editor

starts in "Overstrike" mode ( each typed character  overlays  the

previous character ), but can be switched to "Insert" mode ( each

typed   character   is  inserted  at  the  cursor,  shifting  all

characters to the right over one space) by typing the INSERT  key

(  which toggles between Overstrike and Insert mode ). The DELETE

key deletes the character underneath  the  cursor.  The  END  key

moves  the  cursor  to  the end of the string, while the HOME key

moves the cursor to the start of the string.

6.5 Terminating SFC

A series of files being transmitted can be terminated  (  without

aborting the system ) by typing ESC during transmission. The user

will  be  returned  to  the  SFC menu after the completion of the

current file transmission.

The user may break out of SFC at any time and return to MSDOS  by

typing  "CTRL"  and  "BREAK"  simultaneously. This should only be

used when the program hangs for some  reason  and  SFC  does  not

respond  to  the  keyboard.  Always try to use the menu selection

QUIT to quit SFC and return to MSDOS rather than Control Break.

SFC is bi-directional ( files can be sent in both  directions  ).

However, do not attempt to send files from one computer while you

are editing a path or filename on the other.

page 11

7.0 Cloning SFC

Registered  users  are  provided  with a 'cloning' capability for

SFC. This enables the user to  get  a  copy  of  SFC  on  another

computer via a null modem cable. The only requirement is that the

receiving  (  target  ) computer have the standard MSDOS programs

MODE and DEBUG available in the currently logged directory. Also,

Data Set Ready ( DSR ) and ( for some computers ) Clear To Send (

CTS ) must be set to high on the target computer before the  CTTY

command will work. See the section on null modem cables.

We  first  assume  that the SFC programs are all available on one

computer ( we'll call it the source computer ) and that the other

computer ( we'll call it the target ) has only the standard MSDOS

programs DEBUG and MODE. In addition, we'll assume  that  a  null

modem  is  connected between the two using port COM1 on both. The

task is to get SFC on the target computer.

The first step is to boot the target  computer  and  then  verify

that  both  DEBUG and MODE are in the currently logged directory.

Then type "MODE COM1:1200,N,8,2" to set the  baud  rate,  parity,

and stop bits on the target computer.

The  second  step  is  to force DSR and CTS to high on the target

computer. This is required by the MSDOS CTTY command. If you have

a null modem cable in which  DTR  from  the  source  computer  is

connected  to DSR on the target and RTS on the source computer is

connected to CTS on the target computer (  as  described  in  the

cable  construction  section  ) then the CLONE program will force

DSR and CTS high on the target computer.

The third ( optional ) step is to test step  2  by  typing  'CTTY

COM1'   on   the   target   computer  followed  by  typing  'MODE

COM1:1200,N,8,2' and then 'TERMINAL' on the source computer. Once

done, all commands typed on the keyboard of the  source  computer

will  be  executed by the target computer. Type "CTTY CON" on the

source computer. If  control  is  then  returned  to  the  target

computer  then  we  are  ready  for  the next step. If the target

computer does  not  respond  to  commands  typed  on  the  source

computer  then  either  the cable is not right or DSR and CTS are

not high on the target computer. Remember to type ESC to get  out

of the TERMINAL program.

The  fourth  step  is  to  type  'CLONE'  on  the source computer

followed by typing 'CTTY COM1' on the target computer. CLONE will

create the program GETSFC on the target computer. It takes  about

a  minute  to run. Once done, CLONE breaks the connection between

the computers. Do a directory on  the  target  computer  and  you

should now see the program GETSFC.COM.

The  fifth  and  last  step  is to first run GETSFC on the target

computer followed by running PUTSFC on the  source  computer.  Be

sure that SFC.EXE is in the same directory as PUTSFC. The process

takes  about  a minute and results in SFC.EXE being copied to the

target computer.

page 12

8.0 Support Programs

8.1 CLONE, PUTSFC, and PUTSFC.

Don't have SFC  on  the  target  computer  ?  The  CLONE  program

provides  a way to 'clone' SFC onto another computer. The program

PUTSFC is used to send a copy of SFC to GETSFC which  is  created

by  CLONE  on  the target computer. With CLONE you only need your

null modem cable to connect to computers with  a  different  disk

format.  See  the  section  entitled "CLONING SFC" for a complete

discussion.

8.2 LOOPBACK

If you suspect that your serial port may be defective,  then  use

LOOPBACK  to test the internal circuitry. The serial ports have a

built in 'loop  back'  test  capability  which  is  used  by  the

LOOPBACK  program.  For  example,  to  test  the  COM1 port, type

'LOOPBACK 1'

8.3 TERMINAL

Looking for a small no frills communications program with  XMODEM

file  transfer  protocol and a script command language ? TERMINAL

may be what you are looking for. Use TERMINAL to  download  files

from  bulletin boards or to help test null modem cables. TERMINAL

is documented in the TERMINAL.DOC file.

8.4 MODEM

The MODEM program is used to turn the modem control lines DTR and

RTS on and off, and to display DTR, DSR, RTS, and CTS. MODEM  can

help  you  test  out  your null modem cable. For example, to work

with COM3, type 'MODEM 3'. A menu will be displayed providing all

options.

8.5 PORTEXAM

The PORTEXAM program will tell you all you ever  wanted  to  know

about  the  status of any serial port. It reports the serial port

address, the interrupt vector, the location of the ISR code,  the

status  of  all  modem control and modem status lines, and if the

serial interrupt is enabled.  Very  useful  as  a  cable  testing

program and for advanced users. For example, to examine COM1 type

'PORTEXAM 1'.

page 13

8.6 GETFILE and PUTFILE

GETFILE  and PUTFILE are very small ( and fast to load ) programs

designed to send only a few  files  between  computers  when  the

capabilities  of SFC are not needed. They run at 38400 baud using

COM1 and do checksum data verification. Run GETFILE first on  the

target  computer  followed  by  "PUTFILE  filename" on the source

computer  where  'filename'  is  the  name  of  the  file  to  be

transferred.

8.7 CHECKCRC

The  CHECKCRC  program  is  used to compute the Cyclic Redundancy

Check ( CRC ) of any file ( sort of a 16-bit  file  'fingerprint'

).  For  example, to compute the CRC of all files ending with the

extension .DOC type 'CHECKCRC *.DOC'. Use the CHECKCRC program to

verify that two copies of the same file are indeed the same file.

8.8 CRYPT

Need to protect your data ? The CRYPT program is used to  encrypt

and  decrypt  any  file.  To use, type 'CRYPT filename1 filename2

key' and  the  file  'filename1'  will  be  encryted  to  produce

'filename2'  using  the  numeric  key  'key'.  If  'key'  is  not

specified then you will be prompted for it. The same key performs

both encryption  and  decryption.  For  example  'CRYPT  MYFILE.C

MYFILE.X  123' will encrypt MYFILE.C using the key 123 and create

file 'MYFILE.X'. To decrypt, repeat the process  using  the  same

key: 'CRYPT MYFILE.X MYFILE.C 123'.

8.9 SPACE

Need  a  quick and easy way to determine how much free disk space

is available ? Use the SPACE program  to  display  the  remaining

free  disk  space  and  the  allocation unit size of the specfied

disk. For example, type 'SPACE C' to display free space on  drive

C.

page 14

9.0 Performance

The  following benchmark results were obtained using a Tandy 1000

( 4.77 MHZ), a Tandy 1400LT ( 4.77 & 9.16 MHZ ), and a Tandy 3000

( 10 MHZ ) computer. Note that the 'standard' IBM PC runs at 4.77

MHZ. Virtual disks ( ramdisks ) were used  in  order  to  measure

performance  as independent of disk speed as possible. The use of

hard disks will result in a little slower times than shown  while

the  use  of  floppy disks will result in slower times still. The

use of faster computers will result in faster times  than  shown.

Three  files  were  used:  file A is 10,000 bytes long, file B is

50,000 bytes and file C is 100,000 bytes long. All times  are  in

seconds  and  are  from start to finish including ramdisk access.

Your results should be very similiar.

9.1 Test 1 ( 10 MHZ Tandy 3000 to 4.77 MHZ Tandy 1000 )

baud rate   File A   File B   File C

---------   ------   ------   ------

38400       4.0     16.5     32.3

115200       2.2      9.8     14.7

9.2 Test 2 ( 10 MHZ Tandy 3000 to 4.77 MHZ Tandy 1400LT )

baud rate   File A   File B   File C

---------   ------   ------   ------

38400       3.3     15.3     30.3

115200       1.6      6.7     12.7

9.3 Test 3 ( 9.16 MHZ Tandy 1400LT to 10 MHZ Tandy 3000 )

baud rate   File A   File B   File C

---------   ------   ------   ------

115200       1.5      6.1     12.1

9.4 Other Tests

Serial File Copy  has  been  benchmarked  against  several  other

shareware  programs. All that we have tried ( 3 ) are slower when

using the same baud rate. One program took almost twice  as  long

to transfer files. If anyone finds a faster file transfer utility

than SFC please write or call!

page 15

10.0 CRC Calculations

A  full  16 bit Cyclic Redundancy Check ( CRC ) using the CCITT (

The  Consultative  Committee  in  International  Telegraphy   and

Telephony  )  polynomial  is used by SFC. The CRC of each file is

computed before transmission by the sender and  sent  along  with

the file being transmitted. The receiver then computes the CRC of

the  received  file  and compares it to the CRC sent. If the CRCs

match, then the file is considered to be correct and  a  "OK"  is

displayed.  CRC  calculations  are  considered  to  be  the  best

available method for  verifing  data  integrity.  They  are  much

superior to checksums and detect 99.99 percent of data errors. If

an  error  is detected, the offending packet ( part of the file )

will be re-transmitted up to 3 times.

11.0 User Registration

The shareware  version  of  SFC  is  provided  so  that  you  may

personally  determine  the usefulness of SFC for yourself. If you

can use SFC please register your use with us.

MarshallSoft

P.O. Box 4543

Huntsville, AL.

35815-4543

Please pay by check.  Payment  must  accompany  purchase  orders.

Print  the file SFC.INV if an invoice is needed. Two packages are

available. The "Personal Package" is available for  $15  postpaid

and includes:

o High Speed Version - 115200 baud. No shareware screens.

o All 12 support programs - See 'Support Programs' section.

o Notices of upgrades and related products.

o A dozen of my favorite public domain & shareware utilities.

The "Business Package" is available for $25 postpaid and includes all

of the Personal Package plus:

o Telephone support for 1 year.

o 1/2 off all future releases of SFC.

o Backbone bound printed Users Manual.

The registered user will receive the latest version by return mail. A

720 KB 3.5" diskette is provided ( unless a 5.25" is requested )

page 16

12.0 Revision History

Version 1.0 -- 10 April 1989 -- Original release

Version 1.1 -- 30 July 1989

Added ASK option.

Added CHK option.

Moved DIR option from separate menu item to options menu.

Fixed bug in disk file size reporting.

Changed units of FREE from kilobytes to bytes.

Changed internal parameters for easier synchronization.

Increased PATH length from 58 to a full 64 characters.

SLAVE side must start SFC with "SFC /S".

Could not generate backslash on Tandy 1000.

Version 1.2 -- 16 October 1989 ( no shareware version )

Added a new AUTOSYNC mode as well as manual startup mode.

Added help file support.

Greatly expanded documention file.

Version 1.3 -- 1 November 1989 ( no shareware version )

Fixed problem with port COM2.

Support the port and baud rate specified on command line.

Simplified operation of AUTOSYNC.

Added indirect file ( '@' file ) support.

Added support for COM3 and COM4.

Version 2.0 -- 10 November 1989

Internal change to allow 4.77 MHZ PCs to run at 115200.

Allows transfer of read only files.

Version 2.1 -- 16 February 1990

Added DOS Shell menu option.

Modified AUTOSYNC for increased reliability.

Increased buffer size to 2000 bytes for 57600 & 115200 baud.

Provide cloning program and other supporting programs.

Added ability to do remote printing.

'NO RESPONSE' error does not abort system.

page 17

13.0 Warranty

The  user  of this software assumes all liability for its use. In

no case shall MarshallSoft be liable for any  damages,  including

any  incidental  or  consequential damages. The prudent user will

keep multiple backups of all important programs and data.

14.0 Final Comments

An update to SFC is planned about  twice  a  year  for  the  next

several  years.  It  is amazing that as one gets into programming

something like SFC new ideas  are  constantly  occurring.  Future

upgrades  will add new features, make SFC easier to use, make SFC

faster,   add   new   supporting   programs,   or   enhance   the

documentation.

If  you  find  any errors or have any suggestions for improvement

please write or call (205) 881-4630  after  5:00  PM  CST  Monday

through Friday or after 2:00 PM Saturday.

Consulting   services  for  serial  communications  projects  are

available. If you have a project in mind,  write  or  give  me  a

call.  I may very likely already have something close to what you

need.

Thanks for trying out SERIAL FILE COPY. I hope that SFC will meet

your expectations. --- Mike Marshall ( Author ).

page 18

#