��������������������������������������������������������������������

X10EC X10 Event Control

��������������������������������������������������������������������

Version 1.00

December 1993

Copyright 1993 � D.Huras � All Rights Reserved

�����������������������������������������������X10EC.DOC v1.00������

CONTENTS

��������������������������������������������������������������������

1. DESCRIPTION               14. SAVE OPTIONS

2. REQUIREMENTS              15. INTELLIGENT EDIT

3. WARRANTY                  16. UNDOCUMENTED CP290 COMMANDS

4. FEATURE OVERVIEW          17. LAST UPLOAD DATE/TIME

5. QUICK START INSTRUCTIONS  18. CP290 ICON STORAGE

6. INSTALLATION INSTRUCTIONS 19. RECYCLE PROCESS

7. COMMAND LINE ARGUMENTS    20. COMPATIBILITY ISSUES

8. PROGRAM OVERVIEW          21. PROBLEM RESOLUTION

9. KEYBOARD USAGE            22. REGISTRATION INFORMATION

10. START UP OPTIONS          23. UPGRADE INFORMATION

11. COMMUNICATION PORTS       24. OTHER HOME AUTOMATION SOFTWARE

12. SCREEN DISPLAY OPTIONS    25. RELEASE HISTORY

13. SUN TIME PARAMETERS       26. ACKNOWLEDGMENTS

1. DESCRIPTION

��������������������������������������������������������������������

X10EC is an Event Scheduler for the X10 CP290 Computer Interface(*1)

with many distinguishing features.  X10EC is a full-screen DOS

program that uses a simple point & click user interface.  (A Mouse

though not required is highly recommended).  X10EC is designed to be

robust software that is processor speed independent; however, the

overhead required for the user interface is significant and a 286 or

above processor is recommended.  This program is shareware and you

are invited to evaluate it for 30 days.  If you continue to use the

program after the 30-day evaluation period you are asked to register

it.  The author retains all rights to the program.

(*1) The CP290 Computer Interface is a device manufactured by X-10

that can be connected to a microcomputer serial port.  The CP290

also plugs into an electrical outlet and it is through this

connection that the CP290 imposes X-10 commands onto the powerline.

X-10 commands use the power line as a carrier; other X-10 devices

connected to the power line will respond to these commands (ie.

switch ON/OFF lights, appliances).

2. REQUIREMENTS

��������������������������������������������������������������������

� An IBM-Compatible PC with at least 1 serial port.

� An X10 CP290 Computer Interface attached to a serial port.

� A Microsoft-Compatible Mouse is highly recommended

� A Colour adapter/display is recommended although the program

should run with a monochrome display/adapter.  If you

experience problems with the default colors, then try the /BW

command line argument which will set everything to Black &

White.

The program requires approximately 500K available conventional memory

to run.  The program will attempt to run if less than 500K is

available; however, depending on the number of Events/Devices

loaded and the number of modeless Windows opened, an 'Out of Memory'

error may occur.  X10EC will forewarn you of a low memory situation

during program startup.  You can reduce the memory requirements

somewhat by foregoing the online Help Sub-system (See STARTUP

OPTIONS).

3. WARRANTY

��������������������������������������������������������������������

X10EC is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of X10EC.

4. FEATURE OVERVIEW

��������������������������������������������������������������������

� User-Friendly point & click DOS character-based Interface

� Most program functions can be controlled using a mouse; minimal

keyboard intervention is required.

� Supports Standard CommPorts 1 thru 4 and a User Defined CommPort

that can support any IRQ between 1 & 15 (inclusive) and any

Base I/O Address.

� SUPPORTS * 5 * UNDOCUMENTED CP290 Commands

� Compatible with the X10.DAT file format used by X10.EXE

(X10.DAT files can be read, created, updated and written)

� Supports X10.EXE 'Frozen' Events

� Supports Relative Times (Sunrise/Sunset +/- Offsets)

� Stores ALL EVENT information (including Sunrise/Sunset Offsets

and Last Upload Date/Time) within the CP290

� EVENTS can be programmed without touching the keyboard

(using a mouse)

� EVENTS can be sorted using one of 3 sort criteria's

� Flexible EVENT Scheduling; Program EVENTS from a DEVICE List

Window -OR- an EVENT List Window

� 'Intelligent' Edit (referential integrity) automatically

updates EVENTS when you change a DEVICE House/Unit address

� Merge Device Descriptions from any number of X10.DAT

files into one X10.DAT file

� Runs in batch mode to recycle Sunrise/Sunset times

� Detects and exploits EGA/VGA 43/50 line mode

� A comprehensive on-line Help sub-system that can be disabled

at the user's option to decrease program load time.

� The program WILL load without a CP290 Interface attached, however

all attempts at serial communications will result in errors; the

program will attempt to trap all such errors.

5. QUICK START INSTRUCTIONS

��������������������������������������������������������������������

These instructions are targetted for experienced users who wish

to get up & running quickly without reading this entire document:

1) Copy X10EC.EXE & X10EC.HLP to a sub-directory of your choice;

preferably NOT the same sub-directory where X10.EXE resides.

2) If you have a relevant X10.DAT file, then copy it to the

sub-directory where you have installed X10EC.EXE; if you don't

have an X10.DAT file X10EC can build one for you.

3) If your CP290 is on CommPort 1, then run X10EC without any

command line arguments; otherwise use the /Cn argument where

n = CommPort (for non-standard IRQ/Base_IO_Address set up see

COMMUNICATION PORTS section).

4) a) X10.DAT File AVAILABLE

X10EC will load the Devices and Events from your X10.DAT

file and display them in the DEVICES and EVENTS Windows

respectively.  If the EVENTS in your X10.DAT file are not

up to date, then 'Download Events' using the 'Transfer'

menu item.  When the Events have been downloaded they

will be matched against your DEVICES; any Events for

house/unit codes that are not defined as a DEVICE will

show up as 'Unnamed Device' - click on 'Name Device' to

Name these Devices.

b) X10.DAT File NOT AVAILABLE

X10EC will load and attempt to load Devices & Events from

an X10.DAT file (that doesn't exist); each load attempt

will abort, click on OK to continue.  You will see an

empty EVENTS window, use the 'Download Events' menu item

under 'Transfer' to populate your EVENTS Window. Now

scroll through the EVENT List and click on 'Name Device'

for each 'Unnamed Device'; type a Description and/or

Location for each.

5. SAVE your X10.DAT file; press CTRL-S.

6. EDIT existing DEVICES and/or EVENTS by double-clicking

the list item your wish to change. (or click on Modify once

you've highlighted the list item).  You can apply as many edits

as you wish and experiment with the program. NO CHANGES WILL

BE UPLOADED TO YOUR CP290 UNTIL YOU EXPLICITLY UPLOAD THEM

USING THE TRANSFER MENU.  Similarly, no changes will be

written to your X10.DAT File until you explicitly use the

SAVE Menu.  X10EC will prompt you to save your X10.DAT File

if any changes were made to the Devices or Events when you

Exit the program UNLESS you use the Unconditional Exit which

quickly Exits the Program without prompting you to save

anything.

6. INSTALLATION INSTRUCTIONS

��������������������������������������������������������������������

There are no special installation instructions; simply copy the

program to a sub-directory of your choice and run it.  The program

will look for 3 files in the directory it is run from.  However,

the program will run with or without the following 3 files:

1) X10EC.INI

The X10EC.INI file is a text file containing all program

parameters; there is no need to create this file yourself

since the program will generate one whenever you select the

'Save Options' menu item. If an X10EC.INI file is not found

(which will be the case the first time X10EC run), X10EC

will revert to program defaults.  A sample X10EC.INI file

(generated by X10EC.EXE) is shown below.

2) X10EC.HLP

X10EC retrieves the on-line help text from this file. If the

file is not found then X10EC will continue loading however

the help feature will be disabled. If you wish to reduce

X10EC's memory requirements, you can uncheck 'Load Help'

from the Startup Options screen, or use the /NH command

line argument.

3) X10.DAT

The default X10 Startup File is X10.DAT; X10EC will attempt

to retrieve Device Descriptions and Events from this file

in order to populate the DEVICES and EVENTS Windows respectively.

An X10.DAT file is a file format used by the X10 program

supplied by X10 with the CP290 interface.  From the Startup

Options screen you can instruct X10EC to look for the Startup

X10 File on any drive/sub-directory.  Conversely, you can

instruct X10EC to NOT retrieve Devices and/or Events from

an X10.DAT file upon Program Startup; in this case X10EC

will open empty DEVICES and EVENTS Windows.

;

; X10EC.INI File Created: 12-12-1993 07:43:57

; All Lines prefixed with ; are ignored

; ---------------------------------------------------------

;

; CommPort=1,2,3 OR 4  DEFAULT=1

;

[Communications]

CommPort=1

;

[Startup]

LoadDevices=YES

LoadEvents=FILE

X10File=F:\X10\X10.DAT

UseCP290IconStorage=YES

LoadHelp=YES

ConfirmDeletes=YES

AutoRefreshDeviceNames=YES

AutoRefreshEventCounts=YES

RefreshClocks=5

LinesPerPage=60

UseBoxChars=NO

;

[Display]

ThreeD=YES

Shadows=YES

Graphics=YES

;

; Colour Values

; -------------

; 0=Black  4=Red       8=Gray          12=Bright Red

; 1=Blue   5=Magenta   9=Bright Blue   13=Pink

; 2=Green  6=Brown    10=Bright Green  14=Yellow

; 3=Cyan   7=White    11=Bright Cyan   15=Bright White

;

BackColor=7

ForeColor=0

HiLiteKey=14

DisabledItem=8

MenuBackColor=1

MenuForeColor=7

MenuSelectedBackColor=0

MenuSelectedForeColor=7

LabelBackColor=9

LabelForeColor=11

HiliteText1ForeColor=4

HiliteText2ForeColor=1

HiliteText3ForeColor=14

HelpTitleBackColor=5

HelpTitleForeColor=15

StatusTitleBackColor=4

StatusTitleForeColor=15

WarningTitleBackColor=12

WarningTitleForeColor=15

;

[SunParms]

LatitudeDegrees=43

LatitudeMinutes=39

LatitudeNS=N

LongitudeDegrees=79

LongitudeMinutes=56

LongitudeEW=W

GMTOffSetHours=5

GMTOffSetMinutes=0

GMTOffSetPlusMinus=+

;

; End of X10EC.INI (07:43:57)

This X10EC.FILE was created using the 'Save Options' menu item.

7. COMMAND LINE ARGUMENTS

��������������������������������������������������������������������

DEFAULTS:  If the program is run without any command line arguments

it will default to COMMPORT 1.

The following command line arguments can be entered in any order:

/Annnn(n) � Base I/O Address applicable to User Defined CommPort

(Must be specified as a decimal number)

If /I agrument is present, but /A is not; then Base

I/O Address will default to 1016 (3F8 Hex).

/BW  � Use Monochrome colours (Black & White)

/Cn  � COMMPORT where n = 1,2,3 & 4

/F:[d]\filename � X10.DAT Startup File Specification

where [d] = drivespec eg. /F:C:\X10\X10.DAT

/In  � IRQ applicable to User Defined CommPort where n = 1 - 15

If /A argument is present, but /I is not; then IRQ

will default to 4.

/NH  � Do Not Load HELP Sub-System

/NH- � Load HELP Sub-System (Override INI directive)

/RECYCLE � Perform Recycle Process (can abbreviate as /R)

COMMAND LINE OPTIONS OVERRIDE X10EC.INI SETTINGS

8. PROGRAM OVERVIEW

��������������������������������������������������������������������

X10EC has been designed to be flexible so that the user controls the

program rather than the program controlling the user.  There are a

variety of options that let the user determine how the program will

respond in certain instances including Program Startup.  The DEVICES

and EVENTS Windows are the 'cornerstones' of the program but they do

not limit the user to performing Device related activities in the

DEVICES Window or Event related activities in the EVENTS Window.

For example, if you spot an EVENT in the Events Window and notice

that there is no Device Description for it, you can simply highlight

the Event and click on the 'Name Device' button; an 'Insert Device'

window will pop up with the House and Unit code preset from the

highlighted Event.  Once you have entered the Device Location and/or

Description it will be propagated to all Events in the EVENTS Window

that have targetted that particular house/unit code.  Similarly,

while in the DEVICES Window (which provides you a Count of how many

Events have been scheduled for each Device), you may notice that a

particular Device has no Events scheduled.  In this case, you can

simply highlight the Device and click the 'Schedule Event' button

and an 'Insert Event' window will pop up with the Device Description

already filled in.

When X10EC loads, it looks at the Startup Options to determine if

Devices and/or Events are to be loaded.  Devices can only be loaded

from an 'X10.DAT' file whereas Events can be loaded from either the

same X10.DAT file or downloaded from the CP290.  Loading Events from

the file is faster than downloading them, but you may want to ensure

you're always working with current information by waiting for the

download.  (Also, if you've RECYCLE'd your Events you may want to

see what the Relative Times were for the last RECYCLE iteration; in

this case you should download the Events).  If you have defined

devices across multiple X10.DAT files, you can merge them all

without disturbing the currently loaded Events.  Use the 'Retrieve

Devices ONLY' menu item; X10EC will ask you if you want to replace

the loaded Devices with the Devices from the File OR if you want to

append the Devices from the File; select Append to merge the

Devices.  Note that any time X10EC loads Devices it will discard

duplicate descriptions; it will also sort Devices by House/Unit code

in support of the referential integrity features offered by X10EC.

The Event Counts displayed in the DEVICES Window refer to

single-unit Events, Group and Global Events are not included in

these counts.

Once you've loaded Devices/Events, you can update them freely.  The

Insert, Modify, & Delete buttons will behave as expected. To use the

Modify or Delete button you must have highlighted an entry from the

List box.  Instead of clicking the Modify button you can double

click the list item (or press the spacebar).  The Clone button is

similar to the Modify button since it will present a change form

just as if you've clicked the Modify button.  The difference is a

new entry will be created.  This is handy if you're programming

ON/OFF times; Insert the ON Event, then clone the ON Event, change

the action to OFF and set the Time.

In the Events Window you can Freeze or Secure Events from the list

box.  If you Freeze an Event, an asterisk will be displayed beside

the Event Number.  If you Secure an Event, a '�' will precede the

time.  (Secured Events will occur at +/- 30 minutes from the set

time).  You can easily swap between Windows by clicking the '#' key.

The 'Window' menu item allows you to manage the location and size of

the EVENTS and DEVICES windows.  If your display is set for

43/50 line mode, you may find that the Split,Horizontal provides you

with the optimal display.  The Unsplit command reverts both Windows

back to their startup state.

You can print an Event or Device listing at anytime using 'Print

Report' under the 'File' menu.  Each time you print a report you

have the option of printing directly to your printer or to a file.

The lines per report page can be set from the 'Start Up' options

screen.

Once you're finished updating Event/Devices you can save them to

your designated X10 Startup File (which will appear on the File

Menu), or to any other filename (use 'Save AS').  You can also

upload your Events to the CP290.  X10EC gives you 2 choices, you can

upload the modified Events ONLY, or ALL Events. CAUTION:  ONLY USE

THE 'UPDATES ONLY' option if you are sure that the currently loaded

Events are based on the Events loaded in the CP290 before you

applied your Updates.  If you do not explicitly Save and/or Upload

the program will prompt you to before exiting; UNLESS YOU EXIT USING

THE 'UNCONDITIONAL EXIT' which quickly exits the program without

prompting you to save/upload anything.

9. KEYBOARD USAGE

��������������������������������������������������������������������

Use the TAB/Shift-TAB keys to move among the various controls. The

Up/Down arrows can be used to move up and down List Boxes; the

Spacebar can be used to select the current List Box item.  The Enter

key can be used to check/uncheck a check box, or invoke a command

button.  The ESCAPE key will exit the current function or the

program if no other windows are visible.  Most Command Buttons

and controls have quick access keys which you can identify by

the highlighted character in the button/control name; press the

ALT key + the quick access key to invoke the action.

The menu-bar can be activated by pressing the ALT key, you can use

the cursors keys to navigate the menus, or press the highlighted

character to immediately invoke the desired menu item.  Many of the

menu items have short-cut keys.  The following list shows ALL menu

items along with the associated short-cut key if one is defined.

ĴFILE Menu��

New

Devices                 �

Events                  �

Open X10 File

Retrieve Devices/Events � Ctrl-R

Retrieve Devices ONLY   �

Retrieve Events ONLY    �

Save X10 File             � Ctrl-S

Save X10 File AS          � Ctrl-A

Print Report

Print Device Listing    �

Print Event Listing     � Ctrl-P

Exit                      � Ctrl-X

Unconditional Exit        � Ctrl-Q

ĴWINDOW Menu���

Devices                   � Ctrl-D

Events                    � Ctrl-E

Split

Horizontal              � Ctrl-H

Vertical                � Ctrl-V

Swap                    � Ctrl-W

UnSplit                   �

ĴACTIONS Menu��

Refresh Updated

Devices Name            � F2

Event Counts            � F3

Sort Events By

Location,Description    � Shift-F1

Housecode,UnitCode      � Shift-F2

Day,Time                � Shift-F3

ReCycle Sun Times         � F4

Show Sun Times            � Ctrl-T

ĴTRANSFER Menu��

Upload Events TO CP290

All Events              � Ctrl-U

Updated Events          �

Download Events FROM CP290�

Clear Events              �

ĴOPTIONS Menu��

Communications            � Ctrl-C

Display                   �

Startup                   � Ctrl-B

Sunrise/Sunset Parms      �

Save Options              � Ctrl-O

ĴHELP MENU��

Help Information          � F1

About                     � Ctrl-F1

10. STARTUP OPTIONS

��������������������������������������������������������������������

DEVICES & EVENTS

����������������

The DEVICES Window and EVENTS Window are always opened each time you

start X10EC.  You can setup X10EC to automatically populate these

Windows by clicking the appropriate option buttons.  The only source

to populate the DEVICES Window is an X10.DAT file; the EVENTS Window

can be populated from either an X10.DAT file or a CP290 download.

Only one X10.DAT startup file can be identified and it will source

both the DEVICES Window and the EVENTS Window (if applicable).

GENERAL OPTIONS

���������������

General options can be modified at any time while X10EC is executing

and the effect will be immediate.

SUNRISE/SUNSET SUPPORT:

Uncheck if you DO NOT want X10EC to use the ICON memory area of

the CP290; in this case the Relative Time feature will be

disabled.

LOAD HELP SUBSYSTEM:

Uncheck to save memory and speed up program load time.

CONFIRM DELETE REQUESTS:

Uncheck to delete without confirmation, You may want to turn

off the confirmation if you're doing many deletes,

it's a good idea to leave it checked just in case.

AUTOREFRESH DEVICE NAMES

On slower machines it could become a nuisance to wait for the

program to refresh the Device Names in the EVENT Window

whenever a Device Name is modified. You may want to Refresh 'ON

DEMAND' instead by using F2.

AUTOREFRESH EVENT COUNTS:

See comments re: AUTOREFRESH DEVICE NAMES;Uncheck to Refresh

'ON DEMAND' (F3).

CP290 CLOCK REFRESH:

The PC Time in the screen header is refreshed every minute; the

CP290 Time in the screen header can also be refreshed

periodically by setting the refresh rate greater than 0.

NOTE:

����

THE FIELD WIDTH IS ONLY 1 CHARACTER AND THE COMPILER DEFAULTS

TO KEYBOARD 'INSERT MODE'; PRESS THE INSERT KEY TO TOGGLE THE

THE KEYBOARD TO 'TYPEOVER MODE' TO CHANGE REFRESH VALUE.

LINES PER REPORT PAGE

Valid Range is 10-99. (see previous comments re: keyboard

INSERT MODE).

PRINT IBM BOX CHARACTERS:

If your printer is set up to print the IBM ASCII character set

you can improve the appearance of the DEVICE & EVENT printed

reports by checking this box

11. COMMUNICATION PORTS

��������������������������������������������������������������������

X10EC supports "pseudo standard" CommPorts 1 thru 4 using the

following settings:

������������������������������������Ŀ

�          �      � Base I/O Address �

� CommPort � IRQ  ������������������Ĵ

�          �      � Decimal �  HEX.  �

������������������������������������Ĵ

�    1     �   4  �  1016   �  3F8   �

������������������������������������Ĵ

�    2     �   3  �   760   �  2F8   �

������������������������������������Ĵ

�    3     �   4  �  1000   �  3E8   �

������������������������������������Ĵ

�    4     �   3  �   744   �  2E8   �

��������������������������������������

If one of these CommPort settings matches your hardware

configuration simply select the appropriate CommPort from the

Communications Options screen.  (You could also start X10EC with the

/C command line argument or use the COMMPORT= X10EC.INI parameter).

You can define your own CommPort by selecting User Defined CommPort

from the Communications Options screen and setting the IRQ interrupt

and Base I/O Address.  (You could also use the /I and /A Command

Line Args or IRQ= & Base_IO_Address= X10EC.INI file directives).

IRQ can be set to any value between 1 and 15 (inclusive).  Base I/O

Address can be set to any value greater than zero.  CAUTION: your

hardware may "lock up" if incorrect settings are used.  Once you

have enabled communications with the CP290, then use the 'Save

Options' menu item to save the information to the X10EC.INI file.

THE BASE I/O ADDRESS MUST BE SPECIFIED IN DECIMAL NOTATION.

IBM PS/2 CommPort Parameters are shown below:

������������������������������������Ŀ

�   PS/2   �      � Base I/O Address �

� CommPort � IRQ  ������������������Ĵ

�          �      � Decimal �  HEX.  �

������������������������������������Ĵ

�    1     �   4  �  1016   �  3F8   �

�    2     �   3  �   760   �  2F8   �

�    3     �   3  � 12832   � 3220   �

�    4     �   3  � 12840   � 3228   �

�    5     �   3  � 16928   � 4220   �

�    6     �   3  � 16936   � 4228   �

�    7     �   3  � 21024   � 5220   �

�    8     �   3  � 21032   � 5228   �

��������������������������������������

A User Defined CommPort must be used for

PS/2 CommPorts 3-8.

12. SCREEN DISPLAY OPTIONS

��������������������������������������������������������������������

This feature allows the user to choose their own screen colours.  A

list of screen elements is presented on the left side of the screen

and all of the available Foreground and Background (when applicable)

colours are displayed next to the screen elements.  To change a

colour, simply select the screen element you wish to change and

click on the preferred Foreground/Background colours.  The Colour

defaults can be restored by clicking on the Colour button, similarly

Black & White colours can be selected by clicking on the Monochrome

button.  Note: some colour combinations will render the screen

unreadable, if this happens press the [Esc] key to exit the dialogue

and restore the colours to their previous setting.  Once you are

satisfied with your colour selections the 'Save Options' command can

be used to store them so that X10EC always starts up with your

selected colours.

The colours for message boxes cannot be changed (always black

on white;compiler limitation).

There are 3 additional program options that can be set:

THREE-D

The program will attempt to render the screen controls in 3D.

If you choose a bright screen background, the 3D effect will

not be evident and you may wish to turn off THREE-D.  It is

also recommended that you turn off THREE-D for Monochrome

Monitors.

WINDOW SHADOW

The program will paint shadows underneath and to the right of

loaded screen windows; this helps to define the boundaries of

pop-up windows.  It is recommended that you turn the WINDOW

SHADOW off for Monochrome Monitors.

GRAPHIC GAUGES

The Graphic Gauges checkbox determines whether or not graphical

EGA/VGA characters are used to construct the sliding percentage

complete gauge.  If this causes problems with your screen

display, then uncheck the box; the ASCII character '�' 219 will

be used to paint the gauges instead.

13. SUN TIME PARAMETERS

��������������������������������������������������������������������

This screen under the 'Options' menu allows you to set the

Longitude, Latitude & GMT Offset parameters specific to your

location.  These parameters are used to calculate your local Sunrise

and Sunset times.  Sample Longitude & Latitude parameters for a

limited selection of cities can be set by using the combo box (click

on the down arrow or press F4 to expand the list box).  THE GMT

OFFSET MUST BE EXPLICITLY SET; it can vary depending on whether

Daylight Savings Time is in effect or not.  eg. for the Eastern Time

Zone the GMT Offset is +4 when Daylight Savings Time is in effect

and +5 when it is not.  The calculated Sunrise/Sunset times will be

accurate to within a minute or two.

You can view the Sun Times for the upcoming 7 days by using the

'Show Sun Times' option under the 'Actions' menu.

Note: when you set a Relative Time and want to see what Absolute

time the Program will upload to the CP290, then use the 'ReCycle

Sun Times' option under 'Actions' and then double-click the Event.

The time is shown in the Absolute Time frame.  Note: X10EC will

always 'ReCycle' the Sun Times prior to an upload.

When X10EC encounters a Relative Time it determines the Absolute

Time to upload as follows:

1) It looks at the 'Day(s) of Week' the Event is scheduled,

finds the next Date the Event will run and uses the Sunrise/

Sunset Times for that Day.

2) If the Event is to run today, it will compare the current time

to the Sunrise/Sunset +/- Offset Time.  If the time has already

past, then it will use the Sunrise/Sunset time for one week

from today.

3) X10EC will not allow the +/- Offset Time to cause the Event to

span to the previous day or the next day; X10EC will only

negatively Offset a time to 12:00 AM on the scheduled day, it

will only positively Offset a time to 11:59 PM.

14. SAVE OPTIONS

��������������������������������������������������������������������

This menu option saves all of the current program settings to the

X10EC.INI file.  If an X10EC.INI file exists, the program will

display a Warning Message and confirm that you wish to overwrite the

existing X10EC.INI file before proceeding.  The X10EC.INI file that

X10EC generates is a plain ASCII file that you can edit using your

favourite editor.  X10EC will notify you of any errors within the

X10EC.INI by displaying the offending line along with a message

indicating it will be ignored.  Note that the X10EC.INI file is

parsed before command line arguments and therefore will be

overridden by contradicting command line arguments.

15. INTELLIGENT EDIT

��������������������������������������������������������������������

X10EC offers some degree of referential integrity when you're

editing Device Names:

� If you modify a Device Address (ie. House and/or Unit Code)

X10EC can optionally examine all of the scheduled Events and

update* any that target the Device to reflect the new

house/unit code.

� If you Delete a Device Description X10EC can optionally

examine all of the scheduled Events and purge* any that are

specifically targetted for the deleted Device.

� If you change a Device Name; any Events which target the

Device will be updated to reflect the new Device Name the

next time the Event list gets focus. (IF AUTOREFRESH Device

Names is checked - otherwise you can press F2 at any time to

refresh Device Names).

�����

* Group and Global Events are left intact; only single unit

Events will be purged/modified.

16. UNDOCUMENTED CP290 COMMANDS

��������������������������������������������������������������������

X10EC supports the following undocumented CP290 commands:

Fade OFF   � DIMS Light(s) to 0 then turns Unit(s) OFF

(Can target multiple units for this command)

X-Light ON � Exclusive Light ON - Issues an ALL LIGHTS OFF

then Brightens selected Unit(s)

(Can target multiple units for this command)

All Lights ON  � All Lights on selected House Code set ON

All Lights OFF � All Lights on selected House Code set OFF

All Units OFF  � All Units on selected House Code set OFF

NOTE: Storing Events with these undocumented commands MAY CAUSE

problems with some CP290 Event Schedulers.  These commands

will be transparent to X10.EXE; however, X10.EXE can

only display ON/OFF/DIM.  If X10.EXE encounters one of

these undocumented commands it will display it as blank.

17. LAST UPLOAD DATE/TIME

��������������������������������������������������������������������

If 'Sunrise/Sunset' support is enabled, X10EC will store the Last

Upload Date/Time in the CP290 Interface.  When Events are uploaded

to the CP290, the Last Upload Date/Time is updated.  Each time

Events are downloaded from the CP290, the Last Upload Date/Time will

be displayed in the Screen Header (left margin, 3rd line).  X10EC

will also extract the Last Upload Date/Time from X10.DAT files when

they are retrieved.

18. CP290 ICON STORAGE

��������������������������������������������������������������������

The CP290 also has memory set aside for Icon storage.  The DOS

character-based software does not display any graphical Icons hence

this storage area is unused when the CP290 is used with the X10.EXE

program.  X10EC uses this memory area to store Sunrise/Sunset (see

Sun Time Parameters) information applicable to the scheduled Events

as well as the 'Last Upload' Date/Time and other information.

Hence, all Event Information including Sunrise/Sunset Offset

information is stored within the device.  This means whenever you

download Event Information from the CP290 the Sunrise/Sunset offset

information is also downloaded; you do not have to 'match' or

'associate' the downloaded events with a data file in order to

translate the 'Absolute' Time of Day back to a 'Relative'

(Sunrise/Sunset +/- Offset) Time.  This also provides a complete

backup to the Event Information stored in an X10.DAT file.

You can elect not to use the CP290 Icon Storage area by unchecking

the 'Support Sunrise/Sunset' box on the 'Start Up' options screen.

(This will also disable support for the 'Last Upload Date/Time'

information in the screen header).

19. RECYCLE PROCESS

��������������������������������������������������������������������

The RECYCLE Process runs X10EC in Batch Mode and updates any Events

that have been scheduled with Relative Times (ie.  Sunset/Sunrise

+/- Offsets).  This allows you to run X10EC from a Batch File weekly

to ensure your Sunset/Sunrise times are current.  X10EC will perform

the following steps when run with the /RECYCLE Command Line

Argument:

� Calculate Sunset/Sunrise Times for the upcoming 7 Days

� Download Event Information from the CP290

� Download Sunset/Sunrise Offset Information from the CP290

� Update Events with Sunset/Sunrise relative times

� Upload any updated Events to the CP290

� Backup the old X10EC.LOG to X10EC_B.LOG

� Log all activity to X10EC.LOG

Note that the RECYCLE process DOES NOT read EVENT INFORMATION from

the X10 Startup File, this is to ensure that only currently

scheduled Events as stored in the CP290 are updated.  (There is no

guarantee that the current X10 Startup File as defined in the

X10EC.INI reflects the contents of the CP290 Event Memory.)

Consequently you may wish to set up X10EC to always download Events

from the CP290 upon startup rather than retrieving Events from the

X10.DAT file.

20. COMPATIBILITY ISSUES

��������������������������������������������������������������������

FREEZE

������

X10EC is file compatible with X10.EXE but there are some limitations

associated with partially 'Frozen' Group Events.  X10.EXE allows you

to partially freeze Group Events, that is some units within the

group can be frozen whereas other remain scheduled Events.  X10EC

does not support this level of granularity for Frozen Events; it

considers any Event either Frozen or Thawed (ie. Not Frozen) and

this applies to single unit Events and multiple unit Events alike.

If a multiple unit Event is Frozen by X10EC then all units defined

to the Event will be Frozen.

X10.EXE relies solely on the X10.DAT file to decipher Frozen Events

that it 'Freezes'.  Conversely X10EC.EXE can decipher Frozen Events

that it 'Freezes' from either an Event Download or an X10.DAT file.

IMPACT

������

If Events have been FROZEN by X10.EXE, then X10EC.EXE will behave as

follows:

Event Source Event Type   Freeze  X10EC Interpretation

������������ �����������  ������  ������������������������������

X10.DAT File Single Unit  Frozen  No impact: will interpret

X10.DAT File Multi-Unit   Partial X10EC will capture unFrozen

units only and treat as a

Frozen Event.

X10.DAT File Multi-Unit   Frozen  No impact: will interpret

CP290 D/L    Single Unit  Frozen  X10EC will NOT capture unit;

will treat as Frozen Event

CP290 D/L    Multi-Unit   Partial X10EC will capture unFrozen

units only and treat as an

unFrozen Event

CP290 D/L    Multi-Unit   Frozen  X10EC will NOT capture units;

will treat as Frozen Event

RECOMMENDATION

��������������

If this all seems confusing, just be sure to unFreeze any Events you

have Frozen with X10.EXE before using X10EC.EXE.  Any Events Frozen

by X10EC will be translated correctly by X10.EXE if you decide to

fallback to X10.EXE.

FILE COMPATIBILITY

������������������

X10.EXE will read and interpret the X10.DAT files created by X10EC

that include X10EC extensions.  Of course, the extensions added by

X10EC will be transparent to X10.EXE, however, THEY WILL BE LOST IF

X10.EXE OVERWRITES the file.    (Extensions include Sunrise/Sunset

information and Last Upload Date/Time).  For this reason, it is

recommended that you create an X10.DAT file for the exclusive use of

X10.EXE rather than sharing X10.DAT files among the two programs.

21. PROBLEM RESOLUTION

��������������������������������������������������������������������

Generally, if your CP290 works with the X-10 supplied software then

you should have no problems running X10EC.  The program will load if

you run it without any arguments (thereby defaulting to COMMPORT 1)

and your CP290 is actually attached to COMMPORT 2.  If this is the

case X10EC will report 'Timed Out waiting for a response' when it

attempts to retrieve the CP290 Time, Day of Week and Base HouseCode.

You can toggle to the correct COMMPORT once the program is running

by using the Options, Communications menu.  The Save Options menu

option will save all settings to the X10EC.INI which obviates the

need for command line arguments.

If you run X10EC with a modem attached to the COMMPORT, you may get

unpredictable results since the modem will echo everything back to

X10EC.

If you use a Serial Mouse, this may cause a conflict if X10EC

attempts to communicate with a CP290 on the same CommPort.  You may

want to start X10EC with the /C command line argument to ensure it

does not default to CommPort 1 if your mouse is also on CommPort 1.

If you suspect that your CP290 is failing, you should try using the

diagnostic/debug features provided by X10DC.

In some cases X10EC may report communication errors (ie.  time-outs,

unmatched checksums, etc).  This may occur when scheduled Events

collide with CP290 Uploads/Downloads that are in progress.  (The

CP290 transmits onto the serial port line whenever a scheduled event

occurs and when you press the rocker buttons on the CP290).  X10EC

attempts to minimize collisions by discarding unexpected serial

input, however the possibility still exists.  You should just

re-invoke the interrupted action to recover from this type of

problem.

The open architecture of the PC compatible operating environment and

the near infinite combinations of hardware and software complicate

the compatibility of applications software.  While every effort has

been made to ensure that X10EC is compatible with as many

hardware/software combinations as possible, there may be operating

environments in which X10EC will not run properly.

Problems can be reported to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

FAX: 905-873-6429

CIS: 72767,472

Internet: david.huras@canrem.com

22. REGISTRATION INFORMATION

��������������������������������������������������������������������

The registration fee for X10EC is $25. (US Dollars for US shipping

addresses, CDN Dollars for Canadian shipping addresses).  X10EC is

currently supplied as functioning software without built-in delays

or obtrusive "nag" screens prompting you to register.  The Relative

Time feature (ie. Sunrise/Sunset support) is the only feature not

supported in the shareware version of X10EC.  A SIGNIFICANT amount

of time and effort went into this program; please register the

program if you intend to use it beyond the 30-day evaluation period.

Registered Users will be shipped a printed manual and a 3�" diskette

(if you would prefer a 5�" diskette please advise) containing the

registered version of the program.  Registration Benefits:

� Full support for the Relative Time feature which allows you

to schedule Events relative to your local Sunset/Sunrise time

offset by up to +/- 255 minutes.

� BONUS PROGRAM: Registered Users will also receive a bonus

program; X10DAT reads an X10.DAT file and prepares a report

listing all installed devices and scheduled events including

"frozen" events and multiple unit events. See the X10DAT.RPT

file for a report sample.

� DISCOUNTED REGISTRATION FEE for X10DC; X-10 Direct Control.

Registered Users can register X10DC for $15; the regular

registration fee is $20.

Complete the order form in the REGISTER.FRM file and send with

your payment to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

Alternatively, you may wish to register using CompuServe Shareware

Registration; you can register X10EC, X10DC or BOTH using CompuServe

Shareware Registration.  The Registration Fee will be charged to

your CompuServe billing account and the latest version of the

software will be shipped within one business day. GO SWREG while on

CompuServe to use Shareware Registration.

Program Title         Program Name    Program ID#   Charge

�������������������   ������������    �����������   �������

X-10 Event Control       X10EC            1774       $25.00

X-10 Direct Control      X10DC             620       $20.00

X-10 Control Bundle      X10CB            1775       $40.00

(X10DC + X10EC)

Shareware Registration (SWREG) will prompt you through the

registration process; you can register programs by referring to them

by program name or ID #.  SWREG will display a description of the

program and prompt you to confirm the registration order; you have

the opportunity to abandon the registration process at any time.

23. UPGRADE INFORMATION

��������������������������������������������������������������������

��������������������������������������������������������������ͻ

�REGISTERED USERS of prior versions of X10EC can upgrade to the�

�current version at ANY TIME by simply sending a diskette      �

�containing their registered version of the software and $2    �

�for postage to the address above.                             �

��������������������������������������������������������������ͼ

24. OTHER HOME AUTOMATION SOFTWARE by David Huras

��������������������������������������������������������������������

Program   Description                                     Status

-------   ------------------------------------------   --------------

X10DC     X10 Direct Control - CP290 Point & Click     Version 1.50

Initial Release: February '93                December '93

X10EC     X10 Event Control - CP290 Point & Click      Version 1.00

Interface Program to manage Event Data.      December '93

Includes Sunrise/Sunset Time Offsets.

Initial Release: December '93

IRDC/     INFRA-RED Direct Control/Command Line Ctrl   Version 1.01

IRDQ      A program which sends instructions to a      October '93

ONE FOR ALL (6 or 12) remote control. The

remote then emits I/R signals.

IRDCW     INFRA-RED Direct Control for WINDOWS         Version 1.00

Development

��������������������������������������������������������������������

Please register this software if you use it regularly; by registering

you reinforce my commitment to ongoing development.

��������������������������������������������������������������������

25. RELEASE HISTORY � X10EC Shareware/Registered Version

��������������������������������������������������������������������

Date      Version     Changes

------   ----------   ----------------------------------------------

DEC'93    1.01        � Fixed problem whereby Hilite3 assumed Hilite2

colour when Screen Colours dialogue cancelled

� Fixed CP290 Day field width problem

(occurs on Wednesday only)

DEC'93    1.00        First Public Release

DEC'93    1.00 �.02   � Added Upload prompt when EVENTS window

unloaded (and Events updated)

� Fixed IRQ/Base_IO_Address default when

only 1 present

� Clear CP290 on Upload ALL Events (faster

than clearing individually)

� Added WARNING TITLE color to Save Options

prompt

� Fixed TITLE when Retrieving Devices ONLY

DEC'93    1.00 �.01   � Added CLS to Program Exit (To Clear

Screen)

� Added UnSplit Menu Item:  EVENTS/DEVICES

Windows revert to Startup Size

� CP290 Clock Refresh defaulted to 1 Minute

(match PC Clock Refresh)

� /F Switch corrected to bypass colon (":")

� Short-Cut Key pass thru: added work-around

code to block short-cut keys when modal

form displayed (compiler bug).

� Changed 'X10.DAT File' reference to 'X10

File' on Startup Options

� Corrected 'Screen Colors' problem whereby

Text Hilights 1,2,3 would revert to black

when dialogue was Cancel'd

� Added SPACEBAR as keyboard equivalent to

DoubleClick in EVENTS/DEVICES list boxes

NOV'93    1.00 �.00   Initial Beta Version

26. ACKNOWLEDGMENTS

��������������������������������������������������������������������

Special Thanks to Gary Soles, Beta Tester.

���������������������END OF X10EC.DOC�������������������������������