Hello fellow divers,

This is version 2.2 of the Nitrogen Absorption Program for DOS, a

program that simulates dives on your PC, enabling you to experiment with

various dive profiles to see how they affect nitrogen absorption. Lest

the personal injury lawyers salivate too much, let me state

unequivocably that NAP is NOT a dive planning device. It is simply

a dive experimenting tool and you should never use NAP as a replacement

for your dive tables.NAP operates a little bit differently than tables,

especially when dealing with repetitive dives, and will give you

different numbers than your tables as a little experimenting will

show.

NAP simulates the absorption of nitrogen by using the well-known

Haldanian model (see "Deeper into Diving" by John Lippmann for the

details). NAP can simulate several tables such as the US Navy tables

and also has a table that is similar to the PADI RDP. (The different

tables can be selected under "Model" in the main menu bar.)

NAP can simulate several dive types- single depth, multiple depth,

and repetitive. Selecting "Repetitive" will allow you to do repetitive,

single depth dives. If you want to do repetitive, multilevel dives,

select "Multi-level" and supply the information for that dive. Unlike

"Single depth" and "Repetive" dives, "Multi-level" does NOT reset the

tissue loading when it is selected, so you can simulate a multi-level

dive, do a surface interval, do another multi-level, ... as many times

as you like. So you must remember to manually clear the tissues if

you want to start a new multi-level dive by selecting "Clear Tissues"

under "File."

NAP has several ways of doing a surface interval. "Real Time" does

exactly that, it computes nitrogen outgassing in real time. "Delta Time"

is like "Real Time" except that it runs at a pace you specify. "Discrete"

allows you to enter a surface interval, and then shows you the tissue

loading at the end of the surface interval.

NAP allows you to work either in feet or meters, by selecting the

desired unit under "Units." Nitrox can also be simulated by choosing

it from the "Gases" menu item.

Pressing F1 causes a simulation to be paused and F2 causes the

simulation to toggle between a fast screen update mode, and one that

runs more slowly so you can watch what happens. The program defaults

to the slower mode on startup. F10 exits the program.

NAP for DOS is a shareware program, so if you use it you should

register it. A version for Windows is also available. There are many

things that I would like to add to NAP such as altitude diving and

decompression calculations, but the rather dismal registration

response for NAP (one in the year or so that NAP has been out) has

pretty much dampened my interest in doing anything else with it.

I really thought that a program like this would be more popular,

and that is why I spent a considerable amount of time writing it.

If you have any comments or questions about NAP, you can reach me at

Dirk Terrell

1010 NW 36th Ave.

Gainesville, FL 32609  (probably will change after summer 1994)

terrell@astro.ufl.edu

d.terrell on GEnie

The registration fee for NAP is $20 U.S.