Using the NRDIAG Diagnostics Utility

--------------------------------------

(C) Copyright 1993 Artisoft, Inc.

(Edition 1)

ABOUT NRDIAG

============

NRDIAG is the NodeRunner Micro Channel diagnostic program. This

utility is also included as part of the NRSETUP program as the option

titled "NodeRunner Diagnostics."  With NRDIAG, you can check for

problems with your adapter or network connections.  NRDIAG tests network

communications at the hardware level (adapters, cables, T-connectors,

etc.).  This allows you to isolate the hardware from the software for

more precise network diagnostics.

Run NRDIAG after installing the adapter in the computer, but BEFORE

running the network software.

NOTE:  Make sure no NodeRunner adapter drivers are loaded into memory

when you run NRDIAG.  If you are using DOS V5.0, you can do this

by typing MEM/C at the DOS prompt and viewing the list of

programs loaded. If the NodeRunner driver is running, exit any

applications, power down your PC, restart the PC and do not let

the NodeRunner drivers run from your startup batch file.

1.  When you are sure that the network drivers are not running, insert

your NodeRunner adapter diskette into a disk drive on your computer.

2.  For NodeRunner ISA adapters, type NRSETUP from the drive letter in

which you installed the NodeRunner SI adapter diskette and then choose

NodeRunner Diagnostics from the menu that appears.  For NodeRunner

Micro Channel adapters, type NRDIAG.  For example:

B:NRSETUP    (For NodeRunner ISA adapters)

B:NRDIAG     (For NodeRunner Microchannel adapters)

3.  At the prompt, type in a name for the computer you are testing.  The

name must be unique on the network.   If you are testing many

computers, keep a list of each computer's name for future reference.

4.  Repeat steps 1 - 3 on all computers to be tested in your network. Be

certain to give each computer a unique name.

5.  When NRDIAG is running on all of the computers whose adapters and

connections you want to test, make the following checks:

o  Be certain that all of the computers you are testing show up in

the window on the right side of NRDIAG's screen with the name

you assigned them in step 3.  If a computer is running NRDIAG

but its name does not appear in this window, check all adapter

and cable connections for that computer.  Also check the cables

going to and from that computer.

o  Check the following fields in the NodeRunner Statistics window

on each computer you are testing:

Good Transmits:   The number of successful transmissions

by this adapter.

Good Receives:    The number of data packets successfully

received.  A low number in this field

usually indicates bad cables or

connections.

Collisions:       These occur when two or more computers

attempt to access the network

simultaneously.  Generally you will see

some collisions during regular network

use, but an unusually high number of

collisions can indicate cable problems.

Transmit errors:  The number of unsuccessful transmissions.  A

high number in this field usually

indicates a faulty cable or adapter.

Receive errors:   The number of unsuccessful receives. A

high number in this field usually

indicates a faulty cable or adapter.

Buffer overruns:  The number of times the computer's buffers

have been exhausted.  A high number in

this field usually indicates especially

heavy network traffic from a source other

than NRDIAG.

Compare the figures in the NodeRunner Statistics window on each

computer you are testing.  If any of the fields on a computer show a

significantly different value than is displayed on the other

computers in the network, there may be a problem with the adapter,

the cable or their connections at that computer.

Note:   If one of your computers displays values in the NodeRunner

Statistics window that are approximately twice as high as the

values on other computers in the network, it is possible that

you have given two computers the same name.  Be certain that

each computer has a unique name before assuming that there is a

problem with the hardware.

o  The Diagnostic Results window gives you a report of all the

diagnostic tests performed on your adapter.