******************************  TEXT MODE  *******************************

ASMLIB Text mode video subroutines (C) Copyright 1991 - 1993 Douglas Herr

All rights reserved

Screen writing in TEXT mode

ASMLIB subroutines write directly to the video memory buffer, except

where noted.  ASMLIB detects the default display type and screen

dimensions.  ASMLIB assumes that CGA equipment requires "snow" control

and behaves accordingly.  In text modes, ASMLIB does not distinguish

between CGA and MCGA equipment.

Multiple pages of video memory are available except with MDA video adapters.

Pages supported by ASMLIB are shown below:

video system   screen size    ASMLIB support   BIOS support

-------------  -----------    --------------   ---------------

MDA            80 x 25        page  0          page  0

CGA, MCGA      80 x 25        pages 0 - 3      pages 0 - 3

CGA, EGA, VGA  40 x 25        pages 0 - 7      pages 0 - 7

EGA, VGA       80 x 25        pages 0 - 7      pages 0 - 7

EGA            80 x 43        pages 0 - 3      pages 0 - 3

VGA            80 x 50        pages 0 - 3      pages 0 - 3

EEGA          132 x 25        pages 0 - 3      pages 0 - 3

EEGA          132 x 44        pages 0 & 1      pages 0 & 1

PEGA          132 x 25        pages 0 - 3      pages 0 - 3

PEGA          132 x 43        pages 0 & 1      pages 0 & 1

HGC, HGC+, InC 80 x 25        pages 0 - 15     page  0

HGC+, InC      90 x 25        pages 0 - 13     page  0

HGC+, InC      80 x 43        page  0          page  0

HGC+, InC      90 x 43        page  0          page  0

ASMLIB subroutines assume that rows are the vertical dimension and

columns are the horizontal dimension of a text-mode screen.  The upper

left corner of the screen is at row=0 and col=0

ANSICOLOR:   determines color attribute used by ANSI device driver.

See also IsANSI.

Source:      anscolor.asm

Call with:   no parameters

Returns:     AH = color attribute

Uses:        AX

Example:

call  ansicolor         ; get ANSI screen color in AH

BLINKOFF:    disable blinking attriutes; enable bright background attributes

BLINKON:     enable blinking attriutes; disable bright background attributes

Source:      blink.asm

Call with:   no parameters

Returns:     nothing

Uses:        nothing; all registers and flags are saved

Supports:    All ASMLIB text modes

Example:

call  blinkoff             ; turn blinking attributes into bright

; background attributes

call  blinkon              ; turn bright background attributes into

; blinking attributes

CRTINFO:     central control for ASMLIB Text subroutines; determines

active video system, screen size, page offset.

Source:      crtinfo.asm

Call with:   AH = 0 for page 0 information

AH = -1 to calculate active page address

Returns:     AL = screen rows

CH = screen columns

BX:[SI] = video buffer address

DX = 3DAh if CGA or MCGA, 0 if not CGA

DX is used to monitor screen retrace, to avoid "snow"

on older CGA systems.  CRTINFO does not distinguish between

CGA and MCGA equipment; MCGA does not require snow control.

You may use GetCRT to determine if an MCGA is installed

and turn ASMLIB's snow control off with SnowOFF if the system

has an MCGA.

Uses:        ES, AX, BX, CX, DX, SI, flags

Supports:    all ASMLIB text modes

CURSORCOLOR: set cursor color and enable color palette (InColor only)

Source:      curscolor.asm ($herc.asm)

Call with:   AH = color attribute (1 - 15)

Returns:     if CF = 0, no error

if CF = 1, InColor card not installed

Uses:        Carry Flag

Supports:    Hercules InColor card

Example:

extrn   cursorcolor:proc

.code

.

.

.

mov   ah,14        ; bright yellow cursor

call  cursorcolor  ; enable InColor palette, set cursor color

CURSOROFF:   make hardware cursor invisible at present location

Source:      cursor.asm

Call with:   no parameters

Returns:     nothing

Uses:        nothing

Supports:    CGA, MCGA, MDA, HGC, HGC+, InC, EGA, PEGA, EEGA, VGA: text mode

Example:     call  cursoroff

CURSORON:    move cursor to (row, col) and update cursor shape

Cursor shape is underscore if INSERT is off, larger block

if INSERT is on.  See also UCursorON.

Source:      cursor.asm (crtinfo.asm)

Call with:   DH = row offset from top of screen

DL = column offset from left side

Returns:     nothing

Uses:        AX

Supports:    CGA, MCGA, MDA, HGC, HGC+, InC, EGA, PEGA, EEGA, VGA: text mode

Example:     mov    dh,row        ; vertical coordinate on text screen

mov    dl,column     ; horizontal coordinate

call   cursoron

GETSCREEN:   save a screen page to a memory buffer

PUTSCREEN:   copy a saved screen page to the video buffer

Source:      screen.asm (crtinfo.asm, $wcopy.asm)

Call with:   DS:[SI] pointing to memory buffer

Returns:     nothing

Uses:        nothing; all flags and registers are saved

Supports:    All ASMLIB text modes; includes CGA snow control

Example:     see ScreenMem for example

HSCROLL:     scroll a portion of the screen left or right.

Source:      hscroll.asm (a$wdata.asm, $wcopy.asm, a$putw.asm, $exit.asm)

Call with:   DS:[BX] pointing to corner coordinates

AH = color attribute for cleared columns

CX = number of columns to scroll

scrolls left if CX > 0, scrolls right if CX < 0

clears area if CX = 0

Returns:     nothing

Uses:        nothing

Supports:    All ASMLIB text modes; includes CGA snow control

Example:

.data

wdata        dw     10          ; row0

dw     10          ; column0

dw     20          ; row1

dw     40          ; column1

.code

.

.

.

lea    bx,wdata    ; DS:[BX] points to window coordinates

mov    ah,color    ; AH = color attribute for cleared columns

mov    cx,columns

call   hscroll     ; to clear entire window, columns = 0

PAINT:       change all color attributes on a text-mode screen

without re-printing the text

Source:      paint.asm (crtinfo.asm, a$paint.asm)

Call with:   AH = new color attribute

Returns:     nothing

Uses:        nothing

Supports:    All ASMLIB text modes; includes CGA snow control

Example:     mov   ah,newcolor

call  paint

RECOLOR:     change selected color attributes on a text-mode screen

Replaces oldcolor attribute with newcolor, without

re-printing the text.

Source:      recolor.asm (crtinfo.asm, a$recolr.asm)

Call with:   AL = color to replace

AH = new color attribute

Returns:     nothing

Uses:        nothing

Supports:    All ASMLIB text modes; includes CGA snow control

Example:     mov   al,olcolor

mov   ah,newcolor

call  recolor

SCREENMEM:   calculate bytes required to save screen page

Used with GetScreen and PutScreen.

Source:      smem.asm (crtinfo.asm)

Call with:   no parameters

Returns:     AX = number of bytes required

Uses:        AX

Example:     call  screenmem

call  halloc             ; this example uses near heap memory

jc    drat

mov   saved_pointer, bx

mov   si,bx

call  getscreen          ; save the screen

.

.

.

mov   si,saved_pointer

call  putscreen          ; restore the initial screen

SHOWTPAGE:   change video page shown on screen

Source:      tpage.asm (crtinfo.asm, $herc.asm)

Call with:   BL = page number

Returns:     AX = error code

0 = no error

-1 = bad page number

Uses:        AX

Supports:    All ASMLIB text modes (except MDA equipment)

Example:     mov    bl,1               ; show page 1

call   showtpage

SNOWOFF:     disables CGA snow control

SNOWON:      enables CGA snow control (ASMLIB default)

Source:      crtinfo.asm

Call with:   no parameters

Returns:     nothing

Uses:        no registers or flags

Example:

include asm.inc

extrn  snowoff:proc

extrn  getcrt:proc

.code

.

.

.

call   getcrt

cmp    ax,2             ; ASMLIB doesn't disable snow control

; on MCGA systems

jne    continue

call   snowoff          ; MCGA doesn't need snow control

continue:

STR2VBUF:    copies a string to video memory buffer, with selected

color attribute; includes snow control for CGA systems

Source:      str2vbuf.asm

Call with:   CX = maximum number of characters to print (will also stop

at NUL character)

DX = 0 to disable snow control

= 3DAh to enable CGA snow control

(if DX = 3DAh on systems without a color monitor,

the computer will jam up solid)

DS:[SI] points to first character of string

ES:[DI] points to video buffer

AH = color attribute

DF = 0 (clear direction flag with CLD)

Returns:     ES:[DI] points to video buffer following string

if AL = 0

DS:[SI] points to byte following string's NUL terminator

if AL <> 0

DS:[SI] points to next byte in string after the last one

printed on the screen

Uses:        AL, CX, DI, SI, flags

TCENTER:     prints a string on the screen, centered horizontally

TNCENTER:    prints CX bytes of a string on the screen, centered horizontally

Source:      tcenter.asm (crtinfo.asm, str2vbuf.asm, strlen.asm)

Call with:   DS:[SI] pointing to the string

DH = row to print the string on

AH = color attribute

(tncenter only) CX = number of bytes to print

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

assume  ds:@data

mov     ax,@data

mov     ds,ax       ; the string is in DGROUP

lea     si,string   ; DS:[SI] points to the string

mov     dh,row

mov     ah,attr     ; color attribute

call    tcenter     ; center the string on row in DH

TCLEAR:      clear a text-mode screen with specified color attribute

Source:      tclear.asm (crtinfo.asm, a$putw.asm)

Call with:   AH = color attribute

Returns:     nothing

Uses:        AL

Supports:    all ASMLIB text modes; includes CGA snow control

Example:     mov    ah,color

call   tclear

TCOPY:       copies one page of text-mode video memory to another page

Source:      tcopy.asm (crtinfo.asm, tpage.asm, $herc.asm)

Call with:   BH = frompage, BL = topage

Returns:     if CF = 0, all OK

if CF = 1 and AH = -1, frompage number is out-of-bounds

if CF = 1 and AL = -1, topage number is out-of-bounds

Uses:        AX, CF

Supports:    all ASMLIB text modes, pages 0 - 15; includes CGA snow control

Example:     mov    bh,0               ; frompage

mov    bl,1               ; copy from page 0 to page 1

call   tcopy

jc     oops               ; oh yuck - an MDA

TFILL:       fill a text-mode screen with specified character and color

Source:      tclear.asm (crtinfo.asm, a$putw.asm)

Call with:   AH = color attribute, AL = fill character

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:     mov    ah,color

mov    al,char              ; AL = character to fill screen

call   tfill

TGETCHR:     read character and attribute from text screen

Source:      tgetchr.asm

Call with:   DH = screen row

DL = screen column

Returns:     AH = color atttribute

AL = ASCII character code

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

include asm.inc

extrn tgetchr:proc

.data

position label word

column   db 13

row      db 18

.code

; program fragment assumes DS:@data

.

.

.

mov   dx,position

call  tgetchr         ; returns with AL = ASCII character code

;  and with AH = color attribute

TLOAD:       loads a screen image disk file saved by TSave

Source:      tsave.asm (crtinfo.asm)

Call with:   DS:[DX] = address of ASCIIZ filename

Returns:     if CF = 1, AX = MS-DOS file I/O error code

if CF = 0, no error

Uses:        AX, CF

Supports:    all ASMLIB text modes; no CGA snow control

see also TSave

Example:

extrn tload:proc

.data

filename  db 'tscreen.bin',0

.code

.

.

.

lea   dx,filename

call  tload

TPAGE:       changes active page and shows new active page

Source:      tpage.asm (crtinfo.asm, $herc.asm)

Call with:   BL = page number

for ASMLIB subroutines writing directly to video memory.

See also UseTPage and ShowTPage.  ASMLIB's active page is

not affected by INT  10h mode changes.

Page -1 may be used on two-monitor systems, directs ASMLIB

video output to Monochrome screen (except cursor).

Returns:     AX = error code

if AX = 0, no error

if AX = -1, bad page number

if AX = 1, page may be written to but cannot be shown

Uses:        AX

Supports:    text-mode screens, all row/column configurations

Example:     mov    bl,page

call   tpage

TPRINT:      print ASCIIZ string directly to video buffer.

String must be terminated by a NUL character.

Source:      tprint.asm (crtinfo.asm, str2vbuf.asm)

Call with:   DS:[SI] pointing to the string

DH = screen row, DL = screen column

AH = color attribute

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:     assume  ds:@data

mov     ax,@data

mov     ds,ax       ; the string is in DGROUP

lea     si,string   ; DS:[SI] points to the string

mov     dh,row

mov     dl,column

mov     ah,attr     ; color attribute

call    tprint

TPRINTCE:    prints an ASCIIZ string on the screen at specified position

and specified color, and clears from the end of the string

to the right edge of the screen

Source:      tprintce.asm (crtinfo.asm, str2vbuf.asm, a$putw.asm)

Call with:   DS:[SI] pointing to the string

DH = screen row, DL = screen column

AH = color attribute

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:     assume  ds:@data

mov     ax,@data

mov     ds,ax       ; the string is in DGROUP

lea     si,string   ; DS:[SI] points to the string

mov     dh,row

mov     dl,column

mov     ah,attr     ; color attribute

call    tprintce

TPRINTL:     print ASCIIZ string directly to video buffer, lower case

Source:      tprintl.asm (crtinfo.asm)

TPRINTU:     print ASCIIZ string directly to video buffer, upper case

Source:      tprintu.asm (crtinfo.asm)

String must be terminated by a NUL character.  Similar to TPrint,

but characters a - z are TPrinted as upper case A - Z (TPrintU)

or characters A - Z are TPrinted as lower case a - z (TPrintL).

The string itself is not altered.

Call with:   DS:[SI] pointing to the string

DH = screen row, DL = screen column, AH = color attribute

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:     assume  ds:@data

mov     ax,@data

mov     ds,ax       ; the string is in DGROUP

lea     si,string   ; DS:[SI] points to the string

mov     dh,row

mov     dl,column

mov     ah,attr     ; color attribute

call    tprintu

TSAVE:       saves a screen image as a disk file; does not compress the image

Source:      tsave.asm (crtinfo.asm)

Call with:   DS:[DX] = address of ASCIIZ filename

Returns:     if CF = 1, AX = MS-DOS file I/O error code

if CF = 0, no error

Uses:        AX, CF

Supports:    All ASMLIB text modes; no CGA snow control

see also TLOAD

Example:

extrn tsave:proc

.data

filename  db 'tscreen.bin',0

.code

.

.

.

lea   dx,filename

call  tsave

UCURSORON:   similar to CursorON, but the underscore cursor is used regardles

of the state of the INSERT toggle.  See also CursorON.

Source:      cursor.asm (crtinfo.asm)

Call with:   DH = row offset from top of screen

DL = column offset from left side

Returns:     nothing

Uses:        AX

Supports:    text mode, all pages, all row/column configurations

Example:     mov    dh,row

mov    dl,col

call   ucursoron

USETPAGE:    establish ASMLIB's active page

Source:      tpage.asm (crtinfo.asm, $herc.asm)

Call with:   BL = page number

ASMLIB text-mode subroutines can be directed to any video

page.  ASMLIB's active page is not affected by INT  10h

mode changes.  UseTPage does not change the page shown

on the screen.  See also TPage and ShowTPage.

If newpage = -1, ASMLIB video output is directed to a

Monochrome screen

Returns:     AX = error code

if AX = -1, bad page number

Uses:        AX, ES

Supports:    All ASMLIB text modes

Example:     mov     bl,page

call    usetpage

VSCROLL:     scroll a portion of the screen up or down.

Source:      vscroll.asm (a$wdata.asm, $wcopy.asm, a$putw.asm, $exit.asm)

Call with:   DS:[BX] pointing to corners of scroll area

AH = color attribute, CX = number of rows to scroll

Scrolls up if CX > 0, scrolls down if CX < 0, clears area

if CX = 0

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

.data

wdata        dw     10               ; row0

dw     10               ; column0

; (row0, column0) = upper left corner

dw     20               ; row1

dw     40               ; column1

; (row1, column1) = lower right corner

.code

.

.

.

lea    bx,wdata         ; DS:[BX] points to window coordinates

mov    ah,color

mov    cx,3             ; scroll up 3 rows

call   vscroll

WCLEAR:      clear a rectangular portion of a text-mode screen

Source:      wclear.asm (a$wdata.asm, crtinfo.asm, a$putw.asm)

Call with:   DS:[BX] pointing to box corner data

AH = color attribute

Returns:     nothing

Uses:        AL, flags

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

.data

wdata        dw     10               ; row0

dw     10               ; column0

dw     20               ; row1

dw     40               ; column1

.code

.

.

.

lea    bx,wdata         ; DS:[BX] points to window coordinates

mov    ah,color

call   wclear           ; clear a box on the screen with corners

; at (row0, column0), (row1, column0),

; (row0, column1) and (row1, column1)

WFILL:       fill a rectangular protion of a text-mode screen with

specified character and attribute; variation of WClear

Source:      wclear.asm (a$wata.asm, crtinfo.asm, a$wclr.asm)

Call with:   DS:[BX] pointing to box corner data

AH = color attribute

AL = fill character

Returns:     nothing

Uses:        flags

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

.data

wdata        dw     10               ; row0:    top of fill area

dw     10               ; column0: left side of fill area

dw     20               ; row1:    last row of fill area

dw     40               ; column1: right side of fill area

.code

.

.

.

lea    bx,wdata         ; DS:[BX] points to window coordinates

mov    ah,color

mov    al,''           ; use ASCII 254 for boundary

call   wframe           ; draw a box on the screen with corners

; at (row0, column0), (row1, column0),

; (row0, column1) and (row1, column1)

WSIZE:       calculate the memory required to save a screen window

Source:      wsize.asm

Call with:   DS:[BX] pointing to row & column coordinates

The row and column coordinates define the corners of the

window.

Returns:     AX = bytes required

Uses:        AX

Supports:    All ASMLIB text modes

Example:

.data

wdata        dw     10               ; row0

dw     10               ; column0

dw     20               ; row1

dw     40               ; column1

.code

.

.

.

lea    bx,wdata         ; DS:[BX] points to window coordinates

call   wsize            ; returns AX = bytes needed to save window

jc     oops

call   halloc           ; allocate space from near heap

jc     drat

WPAINT:      replace all color attributes in a window of the screen

Source:      wpaint.asm (a$wdata.asm, crtinfo.asm, a$paint.asm, $exit.asm)

Call with:   DS:[BX] pointing to window coordinates

AH = new color attribute

Returns:     nothing

Uses:        nothing; all registers and flags are saved

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

.data

wdata        dw     10               ; row0

dw     10               ; column0

dw     20               ; row1

dw     40               ; column1

.code

.

.

.

lea    bx,wdata         ; DS:[BX] points to window coordinates

mov    ah,color         ; AH = new color attribute

call   wpaint

WPRINT:      print ASCIIZ string directly to the video buffer in a

window of screen, with word wrap.  String must be terminated

by a NUL character.  WPrint clears any portion of the window

not occupied by the string.

Source:      WPRINT.ASM (crtinfo.asm, str2vbuf.asm, strlen.asm, a$wdata.asm,

a$putw.asm)

Call with:   DS:[SI] pointing to the string

DS:[BX] pointing to window corners

AH = color attribute

note that the string and the window corner data must be in

the same segment

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

extrn   wframe:proc, wprint:proc

.data

wdata   dw 10,10,20,30          ; window corner data: upper left at row 10,

; column 10; lower right at row 20, column 30

string  db "The quick brown fox jumped over the lazy dog's back",0

.code

assume  ds:@data

mov     ax,@data

mov     ds,ax       ; the string is in DGROUP

lea     si,string   ; DS:[SI] points to the string

lea     bx,wdata    ; DS:[BX] points to corner data

mov     ah,12       ; bright red!!

call    wprint

dec     wdata       ; stretch the window borders a bit

dec     wdata+2     ; so I can draw a frame around the silly text

inc     wdata+4

inc     wdata+6

mov     al,-1       ; double-line border

mov     ah,14       ; bright yellow

call    wframe

WRECOLOR:    change selected color attributes in a window of the screen

Source:      wrecolor.asm (a$wdata.asm, crtinfo.asm, a$recolr.asm, $exit.asm)

Call with:   DS:[BX] pointing to window corner data

AL = color attribute to be replaced

AH = new color attribute

Returns:     nothing

Uses:        nothing; all registers and flags are saved

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

.data

wdata        dw     10               ; row0

dw     10               ; column0

dw     20               ; row1

dw     40               ; column1

.code

.

.

.

lea    bx,wdata         ; DS:[BX] points to window coordinates

mov    ah,newcolor      ; AH = new color attribute

mov    al,oldcolor      ; AL = color to be replaced

call   wrecolor         ; replace oldcolor with newcolor

; within the window

WRESTORE:    restore a portion of a text-mode screen from a buffer

WSAVE:       save a portion of a text-mode screen to a buffer

(handy when using pop-up windows on the screen)

Source:      wsave.asm (a$wdata.asm, crtinfo.asm, $wcopy.asm)

Call with:   ES:[DI] pointing to memory buffer

DS:[BX] pointing to corner data

The memory buffer must be sized to hold the entire window.

See wsize.

Returns:     nothing

Uses:        nothing

Supports:    all ASMLIB text modes; includes CGA snow control

Example:

.data

wdata    dw 10               ; row0

dw 10               ; column0

dw 20               ; row1

dw 40               ; column1

buffer   dw 2000h            ; oversize

.code

.

.

.

; save a part of the screen

mov    ax,@data

mov    es,ax

mov    ds,ax

assume  ds:@data, es:@data

lea    bx,wdata         ; DS:[BX] points to window coordinates

lea    di,es:buffer     ; point to buffer

call   wsave

.

.

.

; sometime later ...

; restore the window saved earlier

mov    ax,@data

mov    es,ax

mov    ds,ax

assume  ds:@data, es:@data

lea    bx,wdata         ; DS:[BX] points to window coordinates

lea    di,es:buffer     ; point to buffer

call   wrestore