ID:QM Quarterdeck Memory Driver Interface

Quarterdeck Developers Note

The Quarterdeck Memory Driver interface is available in version 4.2x and above of

QEMM.  It allows the developer(s) of third party applications to determine if a

Quarterdeck Memory Driver (QDMEM) is present, which driver (QEMM, VIDRAM etc.) is

present, and it will enable the third party developer to determine where the high

memory chain begins.

QEMM permits this through the INT 2F Resident Program Communication Interface

(RPCI).  For the rest of this documentation, the term 'driver' will normally refer

to a program that uses RPCI irrespective of its method of implementation (device

driver or TSR).  If you are unfamiliar with RPCI or INT 2FH please refer to

Appendix B for information on using this specification to communicate with a QDMEM

driver.

In order to communicate with a QDMEM driver such as QEMM you will need to

determine the RPCI identification number.  Trying different values for the RPCI

identification number and calling the 2FH vector using the QDMEM_CHECK_PRESENCE

subfunction your program can determine the identification code a QDMEM driver is

using.  If the call returns with a special response signature, then the

identification number being tested is the correct number for that system.

It is necessary that your program follow the convention of trying identification

numbers starting at D2H to FFH, then C0H to D1H.  Following this convention

exactly as specified is very important.

In order to determine whether an identification number is free (in addition to the

QDMEM_CHECK_PRESENCE call), the driver can utilize the following two facts:

If the segment of the previous INT 2FH vector is 0 (zero) then the vector has not

been initialized yet and hence nothing has hooked 2f.

If the AL register returns with a non-zero value (usually FFH) when it has

originally been set to 00H and the previous INT 2FH called, then the

identification number in AH is probably in use by a driver - this is a generalized

convention in use by many software manufacturers.  If however, the call returns

with AL equal to 00H, then that identification number is considered free.

The QDMEM_CHECK_PRESENCE call uses this convention, but also sets a response

signature in BX,CX and DX if a special calling signature resides in those

registers when the call was made.  This was done so that an RPCI driver can check

if another RPCI specific driver is loaded.

QDMEM_PREFERRED_ID  EQU  0D2H ;First function number (AH) to try to use

This is the first RPCI ID number to use when determining the QDMEM driver RPCI ID.

Once this ID number is determined, it should be passed in AH with each consecutive

call to INT 2f.

QDMEM_CHECK_PRESENCE          Check for Quarterdeck Memory Driver Presence

AL   =    00H

BX   =    'QD' ;(5144h)  QDMEM_BX_SIG

CX        'ME' ;(4D45h)  QDMEM_CX_SIG

DX   =    'M0' ;(4D30H)  QDMEM_DX_SIG

This subfunction should be called by your program to determine the QDMEM

identification number (RPCI identification).  If a QDMEM driver is installed using

this ID number, it will return the following:

AL   =    FFH  ;(-1)

BX   =    'ME' ;(4D45H)  QDMEM_BX_RESPONSE

CX   =    'MD' ;(4D44H)  QDMEM_CX_RESPONSE

DX   =    'VR' ;(5652H)  QDMEM_DX_RESPONSE

QDMEM_DVR_ID                  ;Determine Quarterdeck Memory Driver Type

AL   =    01H            ;Subfunction 1

This subfunction should be called with the following signatures to determine if

the QDMEM driver installed is QEMM, if there is High RAM present, and to determine

if VIDRAM and HIMEM are present:

Signature to check for QEMM (subfunc 1)

BX   =    'QE' ;(5145H)  QDMEM_QEMM_BX_SIG

CX   =    'MM' ;(4D4DH)  QDMEM_QEMM_CX_SIG

DX   =    '42' ;(3432H)  QDMEM_QEMM_DX_SIG

Response from QEMM (subfunc 1)

BX   =    'OK' ;(4F4B)   QDMEM_QEMM_BX_RESPONSE

Returns in ES:DI the function entry point.  The following calls can then be

made to that entry point:

NOTE: All routines return CY if not supported or if failed.

QEMM_GET_STATUS         EQU     0 ;AH

; Returns AL = 0 if on

;         AL = 1 if auto/on

;         AL = 2 if off

;         AL = 3 if auto/off

QEMM_SET_STATUS         EQU     1 ;AH

; Takes   AL as above

QEMM_GET_VERSION        EQU     3 ;AH

; Returns

BX = version number

Signature to check for high RAM driver (subfunc 1)

BX   =    'HI' ;(4849H)  QDMEM_HIGHRAM_BX_SIG

CX   =    'RA' ;(5241H)  QDMEM_HIGHRAM_CX_SIG

DX   =    'M0' ;(4D30H)  QDMEM_HIGHRAM_DX_SIG

Response from high RAM driver (subfunc 1)

BX   =    'OK' ;(4F4BH)  QDMEM_HIGHRAM_BX_RESPONSE

Returns in CX the first high segment, in DX the reserved owner.  QEMM uses

the same structure for the high memory chain as DOS does for its memory

chain.  You should follow the same procedures you would normally to traverse

the memory chain using the segment returned in CX as the start of the chain.  If

you are unfamiliar with the DOS memory chain, please see APPENDIX A for references

that will provide information on it.

Signature to check for HIMEM (subfunc 1)

BX   =    'HI' ;(4849H)  QDMEM_HIMEM_BX_SIG

CX   =    'ME' ;(4D45H)  QDMEM_HIMEM_CX_SIG

DX   =    'MQ' ;(4D51H)  QDMEM_HIMEM_DX_SIG

Response from QD HIMEM driver (subfunc 1)

BX   =    'OK' ;(4F4BH)  QDMEM_HIMEM_BX_RESPONSE

Returns in ES:DI the function entry point

Signature to check for VIDRAM (subfunc 1)

BX   =    'VI' ;(5649H)  QDMEM_VIDRAM_BX_SIG

CX   =    'DR' ;(4452H)  QDMEM_VIDRAM_CX_SIG

CX   =    'AM' ;(414DH)  QDMEM_VIDRAM_DX_SIG

Response from QD VIDRAM driver (subfunc 1)

BX   =    'OK' ;(4F4BH)  QDMEM_VIDRAM_BX_RESPONSE

Returns in ES:DI the function entry point

Following is an example procedure to check for the presence of a QDMEM driver and

determine the RPCI ID number that it is using:

include     qdmem.equ ; equates are described above

;�����������������������������������������������������������Ŀ

;� QDMEM_Presence (Checks for presence of QDMEM driver)      �

;�����������������������������������������������������������Ĵ

;� QDMEM_Presence bombs AX                                   �

;�����������������������������������������������������������Ĵ

;� Success:                                                  �

;�  ah=QDMEM QDI ID#                                         �

;� Failure:                                                  �

;�  ah=0                                                     �

;�������������������������������������������������������������

QDMEM_Presence   PROC

push    bx

push    cx

push    dx

mov     ax,352fh                ; DOS Get Vector returns es:bx

int     21h

mov     cx,es

test    cx,cx                   ; segment of 2F handler is 0?

jz      qdmem_noqdmem           ; no 2f handler found

mov     ah,QDMEM_PREFERRED_ID   ; RPCI ident. number

mov     al,QDMEM_CHECK_PRESENCE

qdmem_try_next:

mov     bx,QDMEM_BX_SIG

mov     cx,QDMEM_CX_SIG

mov     dx,QDMEM_DX_SIG

push    ax

push    ds

int     2fh

mov     di,ax

and     di,0ffh                 ;And off high byte

pop     ds

pop     ax

jz      qdmem_noqdmem           ;If no response, this func is free

cmp     di,0ffh                 ;Someone responded incorrectly?

jne     qdmem_bump_func         ;If so, cannot use this func

cmp     bx,QDMEM_BX_RESPONSE    ;Signature matches?

jne     qdmem_bump_func

cmp     cx,QDMEM_CX_RESPONSE

jne     qdmem_bump_func

cmp     dx,QDMEM_DX_RESPONSE

je      qdmem_done              ;AH = xdi id

qdmem_bump_func:

inc     ah                      ;Bump func byte

jnz     qdmem_check_done        ;Jump if not yet wrapped

mov     ah,0c0h                 ;Wrap down to 0C0h

qdmem_check_done:

cmp     ah,QDMEM_PREFERRED_ID   ;Wrapped back to start?

jne     qdmem_try_next          ;If not, try this func

qdmem_noqdmem:

xor     ah,ah                   ;no response so zero out ah for return

qdmem_done:

pop     dx

pop     cx

pop     bx

ret

QDMEM_Presence    ENDP

APPENDIX - A

References:

The Waite Group's MS-DOS Papers for MS-DOS Developers and Power Users.

Howard W. Sams & Company - ISBN# 0-672-22594-8

Chapter 6, Page 160.

The Waite Group's MS-DOS Bible - Second Edition by Steven Simrin

Howard W. Sams & Company - ISBN# 0-672-22617-0

Chapter 12, page 220.

APPENDIX - B

INT 2F Resident Program Communication Interface (RPCI)

Resident Program Communication Interface

Quarterdeck's specification for communicating with external drivers.  These

drivers may be implemented either as a DOS device driver (loaded by CONFIG.SYS),

or a Terminate and Stay Resident program (TSR).

For the rest of this documentation, the term 'driver' will normally refer to a

program that uses the RPCI irrespective of its method of implementation (device

driver or TSR).

Interrupt 2FH

All communication with the external driver occurs through interrupt 2FH. As this

interrupt vector is often multiplexed for several different functions, RPCI uses

an identification number (passed in the AH register) to identify the call as an

RPCI message.

For maximum flexibility, this identification number may vary from

configuration to configuration.  A later section describes the convention a driver

should use to determine the RPCI identification number.

Cascading Interrupt 2FH

"Hooking up" to INT 2FH is performed by pointing the INT 2FH vector to a program's

own INT 2FH handler code using the DOS Set Vector call.  It is this piece of code

which monitors the AH register whenever it is called to determine if the call is

in fact an RPCI call.

Note, however, that an RPCI driver must log the previous interrupt 2FH vector when

it is initializing since it is necessary for the RPCI subfunction routines to

"cascade" to any other driver that was loaded before itself. This requirement is

necessary for multiple RPCI drivers to coexist in the system.

"Cascading" is the process of calling the code that was being pointed to by an

interrupt vector before it was redirected to a program's new interrupt handling

code.

This is the basic principle behind the operation of the Resident Program

Communication Interface.

Driver Implementation Techniques

An RPCI driver can be implemented in one of two ways - as a device driver, or as

a Terminate and Stay Resident program (TSR).

RPCI Device Driver

If implementing an RPCI device driver (loaded by CONFIG.SYS), hooking up to the

RPCI will be performed in the driver's initialization procedure (driver command

code 0).  The 2FH interrupt and RPCI subfunction routines may be located anywhere

within the device driver.

RPCI TSR

A TSR program normally consists of two parts - a resident portion and a transient

portion.  The transient portion (which often performs initialization and installs

the resident portion) will usually contain the routines to hook up to the RPCI.

The resident portion will contain the 2FH interrupt and RPCI subfunction routines.

Hooking up to the RPCI

In order to hook up to the RPCI, a driver must perform several tasks.  It must

determine the RPCI identification number that it will use.  In addition, it must

both save the previous INT 2FH vector as well as setting the vector to point to

the driver's 2FH interrupt handler.

Note that it is the first RPCI driver loaded that will choose the RPCI

identification number (it does this by determining that it is the only RPCI driver

in the system) and all subsequent RPCI drivers inquire the RPCI driver loaded

previous to it what the identification number is. Because of this "chaining

effect", all RPCI drivers must implement an RPCI call (the RPCI_CHECK_PRESENCE

call, described later) correctly.

RPCI ID Number

Determining the RPCI number

One RPCI call that should be made is the RPCI_CHECK_PRESENCE call.  Trying

different values for the RPCI identification number and calling the previous 2FH

vector using the RPCI_CHECK_PRESENCE subfunction a driver can determine the

identification code when loaded after another RPCI driver.  If the call returns

with a special response signature, then the identification number previously used

is the correct number for that system.

If the RPCI driver is the first one loaded into the system, then this technique

used as is would fail and needs to be extended (since no driver would already

exist to respond to the RPCI_CHECK_PRESENCE call).  Consequently, in addition to

the above, if the RPCI driver finds a free identification number that is not being

used by any other driver (whether RPCI or otherwise) then it should assume that

it should mark it the RPCI identification number.

In order for this extended technique to perform correctly, it is necessary that

the driver follows the convention of trying identification numbers starting at DEH

up to FFH, then C0H to DDH.  Following this convention exactly as specified is

very important.

In order to determine whether an identification number is free (in addition to the

RPCI_CHECK_PRESENCE call), the driver can utilize the following two facts:

If the segment of the previous INT 2FH vector is 0 (zero) then the vector has not

been initialized yet and hence any identification number could (theoretically) be

chosen.

If the AL register returns with a non-zero value (usually FFH) when it has

originally been set to 00H and the previous INT 2FH called, then the

identification number in AH is probably already in use by some other driver - this

is a generalized convention in use by many software manufacturers. If however, the

call returns with AL equal to 00H, then that identification number is considered

free.

The RPCI_CHECK_PRESENCE call uses this convention, but also sets a response

signature in BX,CX and DX if a special calling signature resides in those

registers when the call was made.  This was done so that an RPCI driver can check

if another RPCI specific driver is loaded.

Setting the 2FH Vector

Once the identification number is found, the driver can point the INT 2FH vector

to its own 2FH interrupt handling routine.  This should be done by using the DOS

Set Vector call.

RPCI Installation

Pseudo Code example

The complete installation procedure for hooking up to the RPCI can be summed up

in the following Pseudo Code:

preferred_id = DEH

save INT 2FH vector as previous INT 2FH vector

IF segment of previous INT 2FH vector = 0 THEN

RPCI_id = preferred_id

ELSE

FOR id = preferred_id to FFH and then C0H to preferred_id - 1

CALL previous INT2FH vector with AH = id, AL = 0

and BX,CX,DX = RPCI signature

IF AL = 0 THEN

RPCI_id = id              /* This value is not in use */

JMP hook_vector:

ELSEIF AL = FFH and BX,CX,DX = RPCI response signature THEN

RPCI_id = id    /* Another RPCI driver uses this value */

JMP hook_vector:

ENDIF

/*Value returned is non-zero - this value must be in use    */

NEXT id

OUTPUT "No id numbers available"

END driver installation with error

ENDIF

LABEL hook_vector:

hook int 2FH to 2FH interrupt handler

END successful driver installation

Note that the RPCI calling and response signatures are defined in the

RPCI_CHECK_PRESENCE call - see that subfunction for details

2FH Interrupt Handler

The 2FH interrupt handler code is fairly simple.  It should check to see if AH

equals the RPCI identification number.  If it does not, then it should immediately

cascade to the previous 2FH vector.

If the identification numbers match, then the handler can dispatch the call to the

appropriate RPCI subfunction routine.

Here is the Pseudo Code for the 2FH interrupt handler:

IF AH = RPCI_id THEN

STI

JMP RPCI subfunction routine based on AL

ELSE

JMP previous 2FH handler

ENDIF

RPCI Subfunctions

The type of RPCI subfunction is determined by the value in the AL register when

an INT 2FH is made.

All calls must be passed on from one RPCI driver to the next.  For some calls, an

RPCI driver should do its work before passing the call along.  For others, it

should pass the call along first and then do its work.

The driver should not perform any DOS or BIOS calls during any work

performed in an RPCI subfunction.

************************************************************************

*This technical note may be copied and distributed freely as long as it*

*is distributed in its entirety and it is not distributed for profit.  *

*         Copyright (C) 1990-2 by Quarterdeck Office Systems           *

************************ E N D   O F   F I L E *************************