___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Performance Differences Between Stacker Software and Coprocessor

Versions (AT/16 and MC/16).

STACKER NOTE TEC010 - 2/20/92                          STAC FAX Index #2502

___________________________________________________________________________

Background.

Stacker's software is supplemented and enhanced by the coprocessor card.

The Stacker coprocessor card yields the highest compression of data, the

smallest device driver and the fastest I/O.

What performance increases will I see with the coprocessor?

1. Hardware compression offers as much as 10-20% more disk space;

2. Disk reads and writes are 5-15% faster.  This higher performance

is especially apparent on slower 8088/8086 and 80286 machines.

3. The device driver memory requirements with the Stacker coprocessor card

are as little as 14k vs. a minimum of 26k for the software version.

What if the coprocessor stops working?

If the coprocessor card does not respond to the Stacker device

driver when the system starts up, Stacker will run in the

software mode and produce the following message: <coprocessor not

functioning -- invoking software driver>.  This means that Stacker

is fully operational,  only it is running without the benefits of

the card.   For AT/16 trouble shooting guidelines see STACKER NOTE

Tec008 (STAC FAX index #2001), "Stacker Coprocessor Card Installation

Checklist".

Is is difficult to upgrade to the coprocessor from the software version?

Upgrading from the software to the coprocessor version does not

require that you re-stack your drives.  However, only those files

written to the drive with the coprocessor benefit from the higher

compression.  With Stacker 2.0, all the files can be recompressed

by the Stacker coprocessor by running Stacker's SDEFRAG disk optimizer

with the /R parameter.  For example, to recompress Stacker drive C:

type:  SDEFRAG /R C:   <ENTER>

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712