___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Stacker AT/16 Coprocessor Installation Checklist

STACKER NOTE TEC008 - 2/20/92                          STAC FAX Index #2001

___________________________________________________________________________

Background.

The Stacker AT/16 coprocessor card is installed by placing the card in

an available expansion slot in your computer, then running the Stacker

Install program to select an address for the card.  Normally, this is

all that needs to be done (no jumpers or special settings required).

This note outlines trouble shooting guidelines for those rare instances

when the card does not respond.

Symptoms.

A Stacker AT/16 coprocessor exhibiting the following symptoms will merit

further investigation:

1. During the I/O address test phase of the installation, you may

find that the card fails to respond at any of the addresses.

2. After successfully selecting an I/O address for the card, upon

reboot you may encounter the Stacker message: <Coprocessor card not

functioning. Invoking software driver>.

Checklist.

1. Check to ensure the card is properly seated in the expansion slot.

The connection should be snug, and the bracket anchored securely to the

case.

2. The expansion slot may be defective, try the coprocessor in a different

slot.

3. If you receive the message that the card is not functioning at

boot time, check the Stacker device driver statement in Config.sys

to ensure that a valid coprocessor address is listed.  The /B=

parameter designates the I/O address the coprocessor is to use.  The

following are possible address values: 200, 220, 240, 260, 280,

2A0, 2E0.  Run the Stacker Install to test all of these addresses.

4. If you are still unsuccessful after step 3, it is possible that your

machine is not properly resetting the addresses after Stacker

reboots the machine.  Try manually adding each address, starting with

200, to the Stacker device driver in Config.sys, then reboot.  For

example, to test address 200, edit the Stacker device driver in

Config.sys as follows:

DEVICE=C:\STACKER\STACKER.COM /B=200

Save the file and perform a "cold boot"  ( Use the Reset button or

power down then up).  A cold boot is necessary to reset addresses.

5. Try rearranging the order of the cards in the expansion slots.

6. Something (usually another add-in card) may be conflicting with the

address  of your AT/16 card. Check the hardware addresses for your

other cards or change the hardware address for the AT/16 card.

7. The system buss speed may be greater than the AT standard of 8 Mhz.

This value can sometimes be set in the system BIOS setup.  Examine your

BIOS to determine the speed of your buss.  It is usually set as the

clock speed (CLK) divided by some number.  Try setting the speed to

8 Mhz or less to test.  (See your computer documentation for more

information on editing the BIOS).

8. Try disabling Shadow RAM, if present. Shadow RAM is a set of system ROM

BIOS routines that have been copied into RAM for increased speed.

Shadow RAM can usually be enabled/disabled in your system BIOS setup or

with some memory managers.

9. If possible, try the card in a 2nd machine to see if the problem is

with the card itself.

10. If after completing the above steps you are still unable to get the

AT/16 coprocessor to function, call Stac Technical Support.

____________________________________________________________________________

Copyright 1992, Stac Electronics                     STAC FAX (619) 431-6712