___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Stacker 1.x Memory Requirements

STACKER NOTE TEC005 - 5/15/92                          STAC FAX Index #2501

___________________________________________________________________________

Background.

The Stacker device driver, STACKER.COM , requires a modest amount of

resident conventional memory.  The resident size of the Stacker device

driver depends on the Stacker product being used, and on the way

Stacker is configured.  During the initialization phase, as it is

loading and mounting the Stacker drives, it requires slightly more

memory than when resident.

Refer to the following tables to determine how much memory the Stacker

device driver requires.  These numbers should be used as guidelines.

Once Stacker is installed, observe the Stacker.COM display as the

system starts to see the exact amount of memory the driver is using.

Starting sizes in bytes, without Stacker cache (/M=0)

Coprocessor             Software

Version               Version

1st volume  (with 4KB clusters): 21,504                30,720

(with 8KB clusters): 25,600                38,912

add'l volumes:    3,072 each            3,072 each

When Stacker loads at boot time, the size of the driver (both initial

and resident) is displayed.

How does the Stacker cache affect the device driver size?

If the Stacker cache has been specified, the driver size will

increase by the amount specified (in Kbytes) for cache allocation.  The

size is specified with the /M=xx switch on the Stacker device line in

Config.sys.  Values range from 0-64.  For example:

DEVICE=C:\STACKER\STACKER.COM C:\STACVOL.000 /m=64

The /m=64 will create a 64 K cache and add 64K to the size of the driver.

If you find your programs complain about insufficient memory, or the

driver will not load into upper memory, reduce the size of the cache.

Note: the device driver can be reduced by an additional 9K if using the

coprocessor, and setting /m=0.

Can the Stacker driver be loaded into Upper Memory?

The Stacker device driver can be loaded "high" like any other driver.

Just follow your memory manager's instructions.  It is desirable to make

the driver as small as possible, so it may be necessary to reduce the size

of the cache. See STACKER NOTE Tec015 (STAC FAX index # 2102) "Loading

Stacker High with DOS 5.0" for more on using the DOS 5.0/Windows 3.1

memory manager to load Stacker high.

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712