LASERTST.COM causes any HP LaserJet printer to run a self-test and

print out the results.  Be sure the printer is turned on, is on line, and has

paper.  Just type LASERTST and press Enter to use the program.  The

self-test normally takes less than a minute, and there will be no output

to the monitor screen.  Depending on the model of LaserJet, the results

may include an example of the default font, ROM dates, installed memory,

and number of pages printed since it was manufactured.

The program works on some LaserJet clones as well.

For those who are interested, the assembly language debug script

for LASERTST.COM is given below:

DEBUG

A 0100

MOV     DL,1B ;put code for Esc in DL

MOV     AH,05 ;set up printer output function for INT 21

INT     21    ;execute printer output

MOV     DL,7A ;put code for z in DL

MOV     AH,05 ;set up printer output function for INT 21

INT     21    ;execute printer output

INT     20    ;terminate program

N LASERTST.COM

RCX

E

W

Q

This program was written by Michael Bruss, 569 Villanova Drive, Davis, CA

95616 and is donated to the public domain.  Comments on the program are

welcome.  No liability is assumed for any damage resulting from its use; no

claims are made for its fitness for a particular purpose; use it at your

own risk.