Sony CDU-31A Adapter Device Driver (ADD) for OS/2 2.x

Beta Release 05/05/93

Installation Notes

Trademarks and Copyrights

IBM(TM) and OS/2(TM) are registered trademarks of International Business

Machines Corporation.

Spectrum(TM) is a trademark of Media Vision, Inc.

Sony(R) and Desktop Library(TM) are registered trademarks of Sony

Corporation.

NOTICE:  The device driver (SONYB31A.ADD) accompanying this file is a BETA

version provided for testing.  Use at your own risk.  The device

driver and this file are owned by Sony and protected by United

States copyright laws, international treaty provisions, and all

other applicable national laws.  You may not reverse engineer,

decompile, or disassemble the device driver.

Instructions for installing SONYB31A.ADD device driver for OS/2 versions

2.0 and 2.1:

1)   Copy the file SONYB31A.ADD to the OS2 directory of the boot

drive:

ex.:  copy A:\SONYB31A.ADD C:\OS2\SONYB31A.ADD

2)   Add the following line to your OS/2 CONFIG.SYS file:

BASEDEV=SONYB31A.ADD

Note that it is not necessary to specify the full path to the

file - OS/2 expects the file to be in the root directory or

the OS2 directory.

3)  The valid parameters and the associated actions are shown below.

Parameter              Action

---------              ------

/A:d                   Identifies a specific adapter, d

/P:hhhh                Set base I/O port address for current adapter

(this should be a four-digit value - use

leading 0's if necessary)

(default = 0340)

/IRQ:dd                Set the interrupt level used to dd for current

adapter (default = none)

/AT:dd                 Set the adapter type.  Current legal values are:

00 = Sony CDB-334 (default)

08 = Media Vision PAS-16

For example, to access a CDB-334 host adapter at IO port 360h,

use the following line in your OS/2 2.x CONFIG.SYS file:

BASEDEV=SONYB31A.ADD /A:0 /P:0360

To access a Media Vision PAS-16 adapter, use the following

line in your OS/2 2.x CONFIG.SYS file:

BASEDEV=SONYB31A.ADD /A:0 /AT:08

(the default port address for the CDU-31A in this case is

1F88h.)

FOR USERS OF THE OS/2 2.1 BETA RELEASE(S):

4)  Open the OS/2 System Folder.  Open the System Setup sub-folder.

Open the Selective Install application.

5)  Click on the CD-ROM Device Support Checkbox.  Click on the OK

button.

6)  Select "OTHER" from the list of CD-ROM devices.  Click on the

OK button.

7)  Click on the Install button and follow the instructions displayed

on the screen.

FOR USERS OF OS/2 VERSION 2.0:

4)  Download the file OS2CDR.ZIP from Library 17 of the IBM OS/2

Support forum on CompuServe and follow the instructions given

in that file.  If you do not have an OS/2 account then you can

download the file OS2CDROM.ZIP from file area 17 on the IBM

PCC Bulletin Board.  The number for this BBS is (404) 835-6600.

FIXED IN THIS RELEASE:

Previous version could only access the first 10 tracks (songs) of an audio

CD.

Previous version could not initialize the Media Vision PAS-16 card after a

cold boot/power cycle/hard reset.

KNOWN PROBLEMS:

None.

Any other problems found with this device driver should be reported to Sony

by sending a message to Chuck Roman of Sony Corporation at CompuServe ID

74130,220.  Messages will be checked between the hours of 7:30 and 5:00

(Mountain Time Zone, USA) daily excluding (most) weekends and holidays.  In

order to prevent repetitive reports of the same problems, you may wish to post

your message in the OS2USER forum (section 3), in the PSPBETA forum or in

the CDROM forum (section 4).  These areas are checked several times a day, so

your message should be answered in a timely manner.

SONY DISCLAIMS ALL WARRANTIES, WHETHER EXPRESSED

OR IMPLIED, INCLUDING WITHOUT LIMITATION,

WARRANTIES OF FITNESS AND MERCHANTABILITY WITH

RESPECT TO THE INFORMATION IN THIS DOCUMENT.

BY FURNISHING THIS DOCUMENT, SONY GRANTS NO

LICENSES TO ANY RELATED PATENTS OR COPYRIGHTS.

Copyright Sony Corporation, 1993, all rights

reserved.