This is a modified version of the Mouse Systems/Logitec driver with added support for

the Mouse Systems bus cards and improved support when moving the mouse quickly.

I fixed the overflow routines in the driver so the mouse will respond properly

when the mouse is moved quickly.

The driver's interface is the same as the interface for the old driver but with

the following extension.

To install the new driver, first back up the old file pclogic.sys in X:\OS2, where X is the

drive letter where OS/2 boots up from.  Next, copy the new file pclogic.sys into

X:\OS2

Then add the following lines to config.sys or make sure they are present.

DEVICE=E:\OS2\MDOS\VMOUSE.SYS

DEVICE=E:\OS2\POINTDD.SYS

DEVICE=E:\OS2\PCLOGIC.SYS BUS=3,6

DEVICE=E:\OS2\MOUSE.SYS TYPE=PCLOGIC$

The above lines should be in the same order as listed above.  The above lines were

just copied out of my own config.sys.

Instead of just

SERIAL=x,

I have added

BUS=x,y

where x is the I/O port location and y is the IRQ.

The following table may be used for the I/O port location

1 = 3F8

2 = 2F8

3 = 338

4 = 238

The following numbers for y may be used for the IRQ location

1 = IRQ 2

2 = IRQ 3

3 = IRQ 4

4 = IRQ 5

5 = IRQ 7

6 = IRQ 10

7 = IRQ 11

8 = IRQ 12

9 = IRQ 15

So for example, if you had a BUS card installed set to port 338 and IRQ 10

(the settings I use on my own bus card), the following statement should be

placed in config.sys

DEVICE=X:\OS2\PCLOGIC.SYS BUS=3,6

If you find this useful or have problems with the driver, I can be reached at

aaronw@cats.ucsc.edu, or by snail mail,

Aaron Williams

35900 Turpin Way

Fremont, CA 94536-2666

By phone I can be reached at (408) 426-3053, which is my school number.  This number should

be valid until June, 1994.  After this time (or during vacation breaks), I can be reached

at (510) 792-0340.