~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~              README2.DOC OF SCANKIT UTILITY                      ~

~                                                                  ~

~              September 25, 1992                                  ~

~                                                                  ~

~              Copyright (c) 1992 Mustek, Incorporated             ~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This file contains the following topics:

*  About the	EMM386.EXE

*  Running Scankit Utility in either	Windows	version	3.0 or 3.1

*  Editing the scanner device line

Before proceeding, it is important that you have completed the

following steps:

Step 1:  connect the Prinscan Adapter with your computer.

Step 2:  Connected the scanner to the Prinscan Adapter.

Step 3:  Run the Install program.

If you have not yet completed the above 3 steps, please refer to the

scanner User's Guide for assistance in completing them.

ABOUT THE EMM386.EXE

~~~~~~~~~~~~~~~~~~~~

EMM386.EXE is an expansion memory (XMS) management driver supported

by Windows.

1. Do you need the EMM386.EXE?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

If, after installation, you are able to get good scanning results,

(i.e. good image quality), then you will not need to install the

EMM386.EXE. If, however, you are unable to get good scanning results

(i.e. poor image quality), with the Prinscan Adapter, AND your PC system

has more than two megabytes of extension memory, then it is recommended

that you should install EMM386.EXE to improve the scanning quality.

2. Installing the EMM386.EXE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Make sure that your	Windows	has EMM386.exe and add the following line

to the config.sys file.

device={Windows' Directory}\emm386.exe ram x=b000-c7ff 1664

for example: device=c:\windows\emm386.exe ram x=b000-c7ff 1664

NOTE: 1. Please be aware that the EMM386.EXE should	be used	with HIMEM.SYS.

Other XMS drivers like QEMM386.SYS	should not be used.

2. Recommended setting of expansion memeory is 1664K.

3. System Side Effects after installing the EMM386.EXE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

3.1	The extension memory installed will be used as expansion memory

and thus can not be used by Windows when running enhanced mode.

3.2	After installing the EMM386.EXE, Windows may be	run only under

standard mode if the system has	not enough extension memeory.

RUNNING SCANKIT UTILITY

~~~~~~~~~~~~~~~~~~~~~~~

1.  Start Windows by typing "win."

2.  Double-click the ScanKit Utility Icon (built after installation) to

run the Scankit Utility or,

2.1 Use	the Program Manager's File RUN command,	type:

"scanw"	(if you're using Windows 3.0 or	3.1)

Also include the DOS pathname.

(Examples: C:\SCANW\scanw  )

2.2 Click OK.

EDITING THE SCANNER DEVICE LINE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1.  INTRODUCTION

~~~~~~~~~~~~~~~~

For the computer to recognize that you have installed a scanner,

the CONFIG.SYS file should contain a device line that specifies

the scanner driver and the configuration (i.e.,  IRQ, and

I/O address settings) used by the prinscan adapter.

TAKE NOTE THAT INSTALLING SCANKIT UTILITY WILL AUTOMATICALLY

UPDATE THE CONFIG.SYS FILE FOR THE SCANNER DEVICE LINE.

However, there are instances when you may need to manually

change the parameters in the scanner device line. Like, for

example, you have to change the IRQ setting to avoid a

hardware conflict.

To edit the CONFIG.SYS file, you will need a text editor.

If you have DOS 5.0, you could use EDIT.EXE. If you have DOS

version below 5.0, you could use EDLIN.COM. Under Windows, you

could also use the Windows Notepad. Please refer to your DOS

(or Windows) manual for steps on how to use these text editors.

Read carefully the following sections to obtain more information

on how to set the parameters in the scanner device line.

2.  THE SCANNER DEVICE LINE

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The scanner device line should include four (4) items. These are:

*  Device driver

*  IRQ setting

*  Scanner type setting

*  I/O address setting

In the CONFIG.SYS file, the correct syntax for the scanner

device line is as follows:

DEVICE=[drive:\][path-directory\]driver-name	 In An Sn

where:

[drive:\][path-directory\]    is the drive and subdirectory

where the scanner driver is located

driver-name                   is the name of the scanner driver

(e.g., PRINSCAN.SYS)

In                            is the IRQ setting

(e.g., I7 for IRQ7)

An                            is the I/O address

(e.g., A378 for I/O address LPT1)

Sn                            is the scanner type

(e.g., S2 for HT-105 scanner)

3.  SETTING THE SCANNER DEVICE LINE PARAMETERS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When you edit the scanner device line, there are certain

precautions that you should take note of especially when

choosing the IRQ, and I/O address settings for the

prinscan adapter.

Avoid hardware and software conflicts that can adversely

affect the performance of the scanner by selecting only the

available and unused IRQ, and I/O address settings.

Use the following options for setting the scanner device line

parameters.

3.1  The Mustek scanner device driver  [driver-name]

The Mustek scanner device driver is found in the Scankit

Utility subdirectory. For the scanner to work, it is

important that the scanner driver is loaded properly when

you boot your computer. In the CONFIG.SYS file, make sure

the driver in the scanner device line has the correct path.

Driver-Name:    For Mustek hand-held scanners:

------------    ------------------------------

PRINSCAN.SYS          HT-105 / Matador 105

3.2  The IRQ level  [In]

When you assign the IRQ setting for the prinscan adapter,

make sure it is not used by any other device connected to

your computer. The available IRQ levels for the prinscan

adapter are:

I5        for  IRQ 5

I7        for  IRQ 7 (recommended setting)

I10       for  IRQ 10

I11       for  IRQ 11

I12       for  IRQ 12

I15       for  IRQ 15

3.4  The Scanner type  [Sn]

S2        for  HT-105 / Matador 105

3.5  The I/O Address  [An]

The PRINSCAN adapter has 3  available I/O addresses and is

equipped with a switch for selecting the address setting.

The 3 available I/O addresses normally are:

*  378 (LPT1)

*  278 (LPT2)

*  3BC (LPT3)

Note: The address which is actually used for LPT(1,2 or 3) depends

upon your system configuration. After installing the PrinScan

adapter, you can run the ScanTest program to find out what

address your system actually can be used in the Scanner Device

line in the CONFIG.SYS file.

3.6  Examples of Scanner Device Lines

DEVICE=C:\SCANW\PRINSCAN.SYS  I7 S2 A378

In the example above, the scanner driver PRINSCAN.SYS is

located in subdirectory SCANW of drive C. The parameters

used are  IRQ 7, and I/O address 378. The scanner

model is either the HT-105 or the Matador 105.

DEVICE=D:\SCANW\PRINSCAN.SYS  I5 S2 A278

In the example above, the scanner driver PRINSCAN.SYS is

located in subdirectory SCANW of drive D. The parameters

used are  IRQ 5, and I/O address 278. The scanner

model is either the HT-105 or the Matador 105.

AFTER YOU HAVE EDITED THE SCANNER DEVICE LINE IN THE

CONFIG.SYS FILE, REBOOT YOUR COMPUTER TO MAKE SURE THE

CONFIG.SYS FILE IS READ.