Databook

INCORPORATED

CardTalk™

PC Card Software for the TMI-140

Reference Manual

Document Version History

Document Version:	2.6

Release Date:	May 7, 1993

Proprietary Notice and Disclaimer

Version	Release Date
	2.2	January 25, 1993
	2.3	February 4, 1993
	2.4	February 17, 1993
	2.5	March 31, 1993
	2.6	May 7, 1993

Unless otherwise noted, this document and the information herein disclosed are proprietary to Databook Incorporated (“Databook”). Any person or entity to whom this document is furnished or who otherwise has possession thereof, by acceptance agrees that it will not be copied or reproduced in whole or in part, nor used in any manner except to meet the purposes for which it was delivered.

The information in this document is subject to change without notice, and should not be construed as a commitment by Databook. Although Databook will make every effort to inform users of substantive errors, Databook disclaims all liability for any loss or damage resulting from the use of this document or any hardware or software described herein, including without limitation contingent, special, or incidental liability.

PC and AT are trademarks of IBM Corporation.

Flash File System is a trademark of Microsoft, Inc.

CardTalk, ThinCard, and TCIC-2/N are trademarks of Databook Incorporated.

Copyright © 1993 by:	Databook Incorporated
	Babcock Hall
	112 Prospect St.
	Ithaca, New York 14850

Databook Software License Agreement

Read this agreement before opening the software. Once you have removed the software from its envelope, you have accepted the agreement.

I.	Usage Rights

II.	Your rights, with respect to the Software, are non-exclusive.

III.	The Software may only be used by one user, on one computer at a time.

IV.	The Software may be transferred to another computer, as long as the requirements to item III are satisfied.

V.	The Software and its documentation may not be copied or distributed to others.

VI.	You may not create, modify, alter, adapt, merge, decompile, or reverse-engineer the Software, and you may not remove or obscure Databook copyright or trademark notices.

VII.	Term of License

VIII.	This Agreement is effective until terminated.

IX.	Terminate the Agreement by destroying the Software, documentation and all backup copies.

X.	Backup Copies

XI.	You may make backup copies of the Software.

XII.	The copyright notice must be included on each backup copy.

XIII.	Copyrights

XIV.	The Software accompanying this manual is protected by United States copyright law.

XV.	The Software documentation is copyrighted.

XVI.	You may only copy the Software and the Software documentation for backup or to load the Software onto your computer as part of program execution.

XVII.	Title to Software and Confidentiality

XVIII.	The Software and all copies thereof are proprietary to Databook and title thereto remains in Databook. All applicable rights to patents, copyrights, trademarks and trade secrets in the Software are and shall remain in Databook.

XIX.	You may not sell, transfer, publish, disclose, display, or otherwise make available any Software or copies thereof to others. You acknowledge that the Software is a trade secret of Databook. You agree to secure and protect each program, software product and copies thereof in a manner consistent with the maintenance of Databook’s rights therein and to take appropriate action by instruction or agreement within your organization to satisfy your obligations hereunder. Violation of these provisions shall be the basis for immediate termination of this license. Termination of the license shall be in addition to and not in lieu of any other legal or equitable remedies available to Databook.

XX.	Limited Warranty

XXI.	SOFTWARE IS PROVIDED “AS IS” WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE LICENSED PROGRAM IS WITH YOU.

XXII.	Databook does not warrant that the functions contained in the Software will meet your requirements or that the operation of the Software will be uninterrupted or error-free.

XXIII.	However, Databook warrants the diskette(s) on which the Software is furnished to be free from defects under normal use for a period of ninety (90) days from the date of delivery to use as evidenced by a copy of your paid invoice or sales receipt.

XXIV.	If the diskette(s) fail due to neglect, accident, or abuse, Databook shall not be liable to replace the diskette(s) under this Limited Warranty.

XXV.	Databook’s entire liability and your exclusive remedy for a diskette not meeting Databook’s Limited Warranty will be the replacement of such diskette which is returned to Databook or to an authorized dealer with a copy of your paid invoice.

XXVI.	In no event will Databook be liable for any damages, including any lost profits, or other incidental or consequential damages arising out of the use or performance of the Software, even if Databook or an authorized dealer has been advised of the possibility of such damages.

XXVII.	You agree that Databook’s liability arising out of contract, negligence, strict liability in tort or in warranty shall not exceed any amounts paid by you for the Software.

XXVIII.	This Limited Warranty gives you specific legal rights. You may have additional rights, depending on the state in which you live. Some states do not allow the exclusion of incidental or consequential damages, or the limitation on how long an implied warranty lasts, so some provisions of this Limited Warranty may not apply to you.

XXIX.	General

XXX.	You acknowledge that you have read this Agreement and understand it, and agree to be bound by its terms, and further agree that it is the complete and exclusive statement of the Agreement, which supersedes and merges all prior proposals, understandings and other agreements, oral and written, between the parties to this Agreement. This Agreement may not be modified or altered except by a written instrument duly executed by both parties.

XXXI.	This agreement and performance hereunder shall be governed by and construed in accordance with the laws of the State of New York.

XXXII.	The waiver or failure of either party to exercise in any respect any right provided for herein shall not be deemed a waiver of any further right hereunder.

XXXIII.	No action, regardless of form, arising out of this Agreement may be brought by you more than two (2) years after the cause of action has arisen.

XXXIV.	Databook shall have the right to collect from you any reasonable expenses incurred in enforcing this Agreement, including attorney’s fees.

XXXV.	If any of the provisions, or portions thereof, of this Agreement are invalid under any applicable statute or rule of law, such invalidity shall not affect the validity of the balance of this Agreement, and such provisions or portions thereof are to the extent of their invalidity to be deemed omitted from this Agreement.

Table of Contents

1.	Introduction	1

1.1.	CardTalk Overview	1

1.2.	Manual Scope	1

1.3.	Manual Organization	1

1.4.	Typographic Conventions	3

1.5.	Unit Multiplier Prefix Convention	3

1.6.	Reference Documents	4

2.	Installing the Software	5

2.1.	Installation Program and Distribution Contents	5

2.2.	Flash File System Software	6

2.2.1.	FFS Updating	6

2.2.2.	Compatibility Problems	6

2.3.	Changing System Configuration to add PCMCIA Socket Support	6

2.3.1.	CardTalk Driver Installation	7

2.3.1.1.	Single Drive Socket Configuration	8

2.3.1.2.	Multiple Drive Socket Configuration	8

2.3.2.	Flash File System Installation	10

2.4.	Verifying Software Installation	10

2.5.	Installation Error Messages	12

3.	TC Utility Commands — Quick Reference	15

3.1.	File Systems	15

3.2.	Formatting a Single-Partition Card	16

3.2.1.	SRAM Card	16

3.2.2.	Flash Card	16

3.2.3.	Flash Series-2 Card	16

3.2.4.	Sundisk Card or ATA Interface Hard Disk	16

3.3.	Formatting a Multi-Partition Card	17

3.3.1.	Multi-Partition SRAM Card (Configuration File Only)	18

3.3.2.	Multi-partition Flash card  (Configuration File and Database)	19

3.4.	Formatting a Bootable Card	20

3.4.1.	Bootable SRAM Card	20

3.4.2.	Bootable Flash Card	20

4.	Using ATA Interface Devices	21

4.1.	Device Insertion	21

4.1.1.	SunDisk Cards	21

4.1.2.	Hard disk Drives	21

4.2.	Initialization	22

4.3.	Compatibility Issues	22

4.3.1.	SunDisk Cards	22

4.3.1.1.	SunDisk Technology Driver	22

4.3.1.2.	Write Limitations on Certain Computers	22

4.3.2.	Hard Disk Drives	22

4.4.	Start Delay	23

5.	Using Modem Cards	25

5.1.	Insertion Constraints	25

5.2.	Serial Port Assignment	25

5.3.	Troubleshooting Modems Under Windows	26

6.	Using Network Cards	27

6.1.	Network Cards Supported	27

6.2.	Installing Network Drivers	27

6.2.1.	ODI	27

6.2.2.	NDIS	28

6.3.	Installing a PCMCIA Ethernet Adapter Card	28

6.3.1.	XIRCOM CreditCard Sample Procedure	29

6.3.2.	IBM/National Semiconductor Card Sample Procedure	30

7.	CardTalk Driver Reference Information	33

7.1.	Specification Line for CardTalk Driver Installation	33

7.1.1.	Syntax	33

7.1.2.	Case and Space Sensitivity	34

7.1.3.	Loading Into High Memory	34

7.2.	Operation at Card Insertion	34

7.3.	CardTalk Support For Windows	35

8.	Specifying Memory Card Information	37

8.1.	Card Information Structure (CIS)	37

8.1.1.	Placing CIS On Cards	37

8.1.2.	Attribute Memory	38

8.1.3.	Common Memory	38

8.2.	Databook Card Information Database	39

8.2.1.	Specifying Part Numbers	39

8.2.2.	Specifying Generic Card Information	42

8.3.	Card Configuration Files	43

8.3.1.	General Information	43

8.3.1.1.	Specification Line Format and Notational Conventions	44

8.3.1.2.	Keyword Groups	44

8.3.1.3.	Delimiters	45

8.3.1.4.	Comment Lines	46

8.3.1.5.	Multiple-Word Strings	47

8.3.2.	Specification Lines (Keywords and Values)	47

8.3.2.1.	Top-Level Keywords	47

8.3.2.2.	Physical Layout Group	49

8.3.2.3.	Space Group	50

8.3.2.4.	Partition Group	51

8.3.2.5.	Geometry Group	53

8.3.2.6.	Battery Group	53

8.3.3.	Sample Card Configuration Files	54

9.	Erasing a Flash Memory Card	57

10.	Formatting a Memory Card	59

10.1.	Formatting a RAM Card	59

10.2.	Formatting a Flash Card	59

10.2.1.	Formatting with the Flash File Allocation Table	59

10.2.2.	Formatting with the Microsoft Flash File System	61

10.3.	Preparing a Single-Partition Memory Card for Booting	61

10.3.1.	Preparing a Single-Partition RAM Card for Booting	61

10.3.2.	Preparing a Single-Partition Flash Card for Booting	63

10.4.	Formatting a Multi-Partition Card	63

10.4.1.	Formatting a Two-Partition RAM Card	64

10.4.2.	Formatting a Two-Partition Flash Card	66

10.4.3.	FFS Partition Size Constraints	68

10.4.3.1.	FFS2 Partitions	68

10.4.3.2.	FFS1 Partitions	68

10.5.	Preparing a Multi-Partition Flash Card for Booting	68

11.	Reading and Writing a Memory Card	71

12.	TC Utility Commands — Detailed Description	73

12.1.	General Features	73

12.2.	Specifying Default Options	73

12.3.	Command Descriptions	74

12.3.1.	tcerase	75

12.3.1.1.	Command Line	75

12.3.1.2.	Options	75

12.3.1.3.	Environment Variables	77

12.3.2.	tcformat	78

12.3.2.1.	Command Line	78

12.3.2.2.	Options	78

12.3.2.3.	Environment Variables	79

12.3.3.	tcinit	80

12.3.3.1.	Command Line	80

12.3.3.2.	Options	80

12.3.4.	tcprog	82

12.3.4.1.	Command Line	82

12.3.4.2.	Options	82

12.3.4.3.	Environment Variables	84

12.3.4.4.	Programming Cards for Embedded Systems	84

12.3.5.	tcread	85

12.3.5.1.	Command Line	85

12.3.5.2.	Options	85

12.3.5.3.	Environment Variables	86

12.3.6.	tcxcopy	87

12.3.6.1.	Command Syntax	87

12.3.6.2.	Options	87

12.3.6.3.	File-Directives	89

12.3.6.4.	Global File-directive Options	90

12.3.6.5.	Copying a Single File	90

12.3.6.6.	Copying a Single Directory	91

12.3.6.7.	Copying Multiple Files	91

12.3.6.8.	Copying Files With Unusual Names	92

12.3.6.9.	Using UNIX-style Command Syntax	92

12.3.6.10.	Environment Variables	92

12.4.	Entering the Card Size	93

Appendix A.	Using MEMCARD.EXE	95

A.1	Overview	95

A.1.1	MEMCARD.EXE Capabilities	95

A.1.2	MEMCARD.EXE Limitations	95

A.2	Starting MEMCARD.EXE	96

A.3	Using MEMCARD.EXE with Menus	96

A.3.1	Displaying Partition Information	97

A.3.2	Creating and Formatting a New Partition	98

A.3.2.1	Flash Memory Card	98

A.3.2.2	RAM Card	100

A.3.3	Formatting an Existing Partition	101

A.3.3.1	MS-Flash Format	101

A.3.3.2	MS-DOS Format	102

A.3.4	Erasing a Memory Card	103

A.3.5	Deleting a Partition	104

A.3.6	Checking a Flash Memory Card for Errors	104

A.3.7	Selecting a Memory Card Slot	105

A.4.	Formatting an Existing Partition Directly	106

A.5.	Checking and Fixing a Flash Memory Card Directly	106

List of Figures

Figure 7-1	CardTalk Driver Installation Specification Line Space Sensitivity	34

Figure 8-1	Card Configuration Specification Keyword Group Organization	46

Table 2-1	Assignment of Logical Drives to Physical Card Sockets	9

List of Tables

Table 2-2	Effect of Relative Number of Card Partitions and Socket Drives	10

Table 2-3	CardTalk Driver Error Codes	13

Table 6-1	Network Cards Supported and Software Used for Testing	27

Table 8-1	Memory Cards Supported by TC Utilities	40

Table 10-1	Usual FFS2 Partition Size Constraints	68

Table 12-1	Decimal and Hexadecimal Equivalents of Card Sizes	93

# 1	Introduction

## 2	CardTalk Overview

Databook’s CardTalk™ software package consists of the CardTalk driver for the TCIC-2/N™ PC™ Card Controller IC (see Reference 5 in Section 1.6), and a set of six ThinCard™ (TC) utilities.

The TC utilities are DOS applications used to prepare PC memory cards for operation with standard DOS commands (e.g., copy, dir, del, etc.). They provide the capability to erase a card, place a card information structure (CIS) on a card, and format a card; these operations are beyond the capabilities of the standard DOS commands. After a memory card is prepared, normal operation to and from the card can be handled by DOS commands.

The TC utilities operate by interfacing with the CardTalk driver, which handles the actual PCMCIA standard card-specific operations of the TCIC-2/N for memory cards as well as I/O cards. (TC utilities are not needed for I/O card operation.)

## 3	Manual Scope

CardTalk is applicable to any computer system into which the TCIC-2/N is incorporated. This manual, however, is specific to CardTalk’s use with Databook’s TMI-140 Evaluation Board (see Reference 6). Certain portions of the manual do not apply to other systems. For example, when CardTalk is used with a system other than the TMI-140, …

…	some system messages (those that display the name of the system) will be different from those presented herein as examples;

…	discussions specific to the presence of two card sockets will not be appropriate for any system that has only one (and command line parameters related to socket selection will have different results from those described herein);

…	the procedure for installing the CardTalk software may be different from that discussed herein.

This manual also describes CardTalk’s support of Microsoft’s Flash File System™ with Flash cards; Databook provides the necessary Microsoft programs as part of the software package distributed with the TMI-140.

## 4	Manual Organization

The information presented in this manual is organized into 12 chapters and an appendix. This chapter presents a brief overview of the CardTalk package, describes the scope of this manual and the notational and typographical conventions used, and lists reference documentation.

Chapter 2 describes how to install CardTalk for the TMI-140 in your system. Chapter 3 provides a quick reference to the simple forms of the commands that invoke the most commonly-used TC utilities. Chapters 2 and 3 are intended to be sufficient to show you how to use the CardTalk software in most common straightforward memory card applications. Chapters 4 through 6 describe operation with various types of I/O cards: ATA and SunDisk bulk storage devices, modem cards, and network cards, respectively.

YOU DON’T NEED TO READ THIS WHOLE MANUAL TO GET STARTED USING CARDTALK. If you’re an experienced card user and are eager to get started, go to…  Chapter 2, Sections 2.1 and 2.4 … for information on installing and verifying installation of the CardTalk software. Then… To use memory cards, go to… • Chapter 3 … for the commands you’ll need to format a memory card that can be used with the most popular DOS commands in simple, straightforward applications. To use I/O cards, go to… • Chapter 4, 5, or 6 … depending on whether you’re working with ATA or SunDisk bulk storage devices (Ch. 4), modem cards (Ch. 5), or network cards (Ch. 6).

The remainder of this manual is designed to provide all the information any user will need to take full advantage of CardTalk’s flexibility and versatility. Which other parts of this manual are important to you will depend on your particular application — whether you need to put card information structure (CIS) on memory cards, or to prepare multi-partition cards, or to read/write binary data, or have some other unusual requirements.

Chapter 7 provides reference information for the CardTalk driver through which the TCIC-2/N interfaces with a PC card. Chapters 8 through 11 describe the use of the TC utilities to prepare and use memory cards in special or unusual applications. Chapter 8 is of interest only for any application in which you have to place a CIS on a memory card. Much of the detail in Chapters 9 through 11 is of interest only for special applications. Chapter 12 gives detailed descriptions of the syntax and options of all the commands used extensively in Chapters 8 through 11 to invoke the TC utilities.

Appendix A describes the use of Microsoft’s MEMCARD.EXE program to configure and maintain PC memory cards. MEMCARD.EXE is included with CardTalk in the distribution package for the TMI-140. The user may choose to use MEMCARD.EXE instead of one or more of the TC utilities, depending on individual requirements and preferences.

## 5	Typographic Conventions

This manual uses the following typographic conventions:

Plain typewriter font:

Represents what the system prints on the workstation screen, as well as system program, directory, and file names.

Boldface typewriter font:

Indicates literal input, by programmers writing software and by users giving commands and in other responses to prompts.

Italic typewriter font:

Used for class (or “place-holder”) words that represent names of commands, arguments, etc. , but that are not literally those names. When shown as user input, the user is to replace such a place-holder with an actual name or argument; when shown as system output, the system will print a case-specific word in place of such a place-holder.

Boldface text font:

Emphasizes information within the text.

Italic text font:

Used for document titles.

## 6	Unit Multiplier Prefix Convention

The use of “k” and “M” as memory size unit prefixes does not conform to the internationally-sanctioned metric standard (k º 103, M º 106); rather, this manual follows the ubiquitous but informal practice of the U.S. computer industry:

k	º	210 (= 1024 = 400h), and
	M	º	220 (= 1048576 = 100000h).

This convention applies only to memory size and relative locations (that is, only when prefixed to “bit” and “byte”). For all other measurements (e.g., frequency, including transfer rates measured in “bit/s” and “byte/s”), the internationally-sanctioned metric standard applies.

## 7	Reference Documents

This manual assumes the reader has a basic familiarity with the following documents:

1.	Personal Computer Memory Card International Association (PCMCIA), PC Card Standard, Release 2.01, PCMCIA, November 1992.

2.	Japan Electronic Industry Development Association (JEIDA), Card Standard, Version 4.1, JEIDA, September 1991.

3.	PCMCIA, Socket Services Interface Specification, Release 1.01, PCMCIA, September 1991.

4.	Institute of Electrical and Electronics Engineers (IEEE), Personal Computer Bus Standard P996, Draft D2.02 of proposed standard, IEEE, 13 July 1990.

5.	Databook Inc., TCIC-2/N PC Card Controller for Notebook PCs — Hardware Design Guide, Databook Inc., most current revision.

6.	Databook Inc., TMI-140 Evaluation Board — User’s Guide, Databook Inc., most current revision.

# 8	Installing the Software

## 9	Installation Program and Distribution Contents

First, back up the distribution diskettes provided with the TMI-140 Evaluation Board. The first distribution diskette contains an installation program, INSTALL.EXE, and a number of packed and unpacked files. To install the software, place the diskette in a drive on your system, then type:

drive:install

where drive is the identifying letter of the drive in which the diskette is inserted.

Do not copy the files off the diskette! Because many of the files are in a packed format, you need to use INSTALL.EXE to unpack them. INSTALL.EXE will ask you where the files should be installed, and will create any required directories for you.

When the installation is complete, you should have the following files on your target disk:

1.	CARDTALK.SYS, the device driver for the TCIC-2/N PC Card Controller.

2.	CARDTALK.386, the driver for Microsoft Windows 3.1 operating in '386 Enhanced Mode.

3.	TCINIT.EXE, a utility program for installing Card Information Structures on cards which don't have them already.

4.	TCFORMAT.EXE, a utility program for formatting memory cards.

5.	TCXCOPY.EXE, a utility program for copying DOS files to Flash memory cards.

6.	TCERASE.EXE, a utility program for erasing memory cards.

7.	TCPROG.EXE, a utility program for programming memory cards.

8.	TCREAD.EXE, a utility program for reading memory cards.

9.	README.1ST, a text file containing late-breaking information.

10.	MS-FLASH.SYS, the Microsoft Flash File System version 2.0 (FFS2) driver.

11.	DBLFLASH.EXE, Microsoft’s driver for optional automatic compression of FFS2 files.

12.	MEMCARD.EXE, Microsoft’s program for preparing memory cards.

INSTALL.EXE also creates a number of subdirectories that contains files with information about various cards supported. Do not remove any of these subdirectories or files.

## 10	Flash File System Software

Microsoft’s Flash File System version 2.0 (FFS2) is a special-purpose formatting scheme for a Flash memory card that allows such a card to be used much as if it were a RAM card. You need not use FFS2 to work with Flash cards. (Currently, very few PCMCIA/JEIDA-compatible systems support FFS2. In particular, you cannot read FFS2 cards on either a Poqet PC or an HP-95LX.) The CardTalk software includes a utility, TCXCOPY.EXE (see item 5 of the list of files in Section 2.1), for formatting Flash cards with regular DOS File Allocation Table (FAT). If you do not use FFS2 — that is, if you do not add instructions to load items 10 and 11 of the list of files in Section 2.1 (see Subsection 2.3.2), you may delete those items and item 12 of that list from your system.

### 11	FFS Updating

Earlier releases of Databook’s TC utilities included support software for Flash File System version 1.0 (FFS1). FFS1 is compatible with MS-DOS version 3.31 or later. Unlike FFS1, FFS2 is compatible only with MS-DOS version 5.0 or later. FFS1 and FFS2 are not mutually compatible, and should not be installed in the same system. If both are installed, FFS1 will override FFS2 so that FFS2 will never become active.

FFS1- and FFS2-formatted cards are also not mutually compatible. FFS2 will not operate with FFS1-formatted cards; FFS1 will generally accept FFS2-formatted cards, but this usually results in a system crash when the card is accessed. For these reasons, we recommend that, if your system includes FFS1, you remove it, and that you also convert your FFS1-formatted Flash cards to FFS2.

### 12	Compatibility Problems

The FFS2 driver MS-FLASH.SYS does not write correctly to a card partition formatted with the DOS File Allocation Table that uses any type of error detection (see Subsection 8.3.2.4, Note 2 of the ERROR-DETECT keyword description).

Fujitsu RAM cards with attribute space may not be erasable with MS-FLASH.SYS if the attribute space contains CIS information. If you want to put FFS2 format on a Fujitsu RAM card, its attribute space may be erased with the CardTalk driver (CARDTALK.SYS), using TCINIT.EXE (see Subsection 12.3.3).

## 13	Changing System Configuration to add PCMCIA Socket Support

Standard DOS does not provide support for PCMCIA standard socket operations. CARDTALK.SYS, the device driver for the TCIC-2/N PC Card Controller, is provided for this purpose. The DOS system configuration file, CONFIG.SYS, must be edited to add CARDTALK.SYS to the DOS configuration.

CONFIG.SYS always resides in the root directory of the system bootstrap device. CARDTALK.SYS may be placed in any subdirectory of the system bootstrap device. If you are operating in Windows 3.1 ’386 Enhanced Mode, CARDTALK.386 must also be added, and must be in the same subdirectory as CARDTALK.SYS. INSTALL.EXE (see Section 2.1) places the CardTalk software (items 1 through 9 of the file list in Section 2.1) into a subdirectory named \CARDTALK on the root directory of the system bootstrap device. We recommend that you place all additional CardTalk-related software (e.g., any configuration files used — see Section 8.3) in \CARDTALK.

When you execute INSTALL.EXE, it will ask whether you want it to add the necessary line(s) to your system configuration file (CONFIG.SYS), or whether you want to do it manually. If you want to change your CONFIG.SYS file manually, you must add a specification line to install the CARDTALK.SYS driver at power-up. If you are installing FFS2 (see Section 2.2), you must also add a line to install MS-FLASH.SYS, and you may add another line to use automatic file compression.

Subsections 2.3.1 and 2.3.2 discuss these various added specification lines. For visual clarity, and to conform to naming practice for the files provided, file names in all specification lines shown in this section are given in upper case letters; DOS is not case-sensitive, however, when it is processing CONFIG.SYS.

DOS is sensitive to the order in which certain commands appear within CONFIG.SYS. We recommend you add all the CardTalk-related commands toward the end of CONFIG.SYS.

Be careful when editing CONFIG.SYS. Errors in this file may prevent your system from booting correctly. We suggest you experiment by making a bootable floppy diskette using the DOS format command before editing your system’s CONFIG.SYS file.

Because DOS uses CONFIG.SYS when the system is booting, any changes you make to CONFIG.SYS have no effect until the system is rebooted.

### 14	CardTalk Driver Installation

Databook’s TCIC-2/N PC Card Controller supports use of two PCMCIA-compatible physical card sockets, called socket 0 and socket 1 (see Reference 5). A memory card inserted in either socket can be a “single-partition” card, formatted to use a single logical drive letter (like a floppy disk), or a “multi-partition” card, formatted to have two or more partitions each accessed by a separate logical drive letter (like a hard disk). CARDTALK.SYS must be configured to assign one or more logical drive letters to each socket, depending on whether the cards to be inserted are single- or multi-partition.

CARDTALK.SYS assigns letters starting with the first available letter after the last drive installed up to the point in the boot sequence that calls CARDTALK.SYS. If you heed our recommendation in Section 2.3 that you add all the CardTalk-related commands toward the end of CONFIG.SYS, the usual base system configuration drive assignments (for floppy and hard disks, and possibly other mass storage devices) will not be disturbed.

Installing and configuring CARDTALK.SYS is discussed in Subsections 2.3.1.1 and 2.3.1.2. Chapter 10 discusses in detail the full syntax of the specification line that installs it.

If your system is operating with MS-DOS v5.0 or later, you may specify that CARDTALK.SYS is to be loaded into high memory by substituting the word devicehigh for the word device in whichever specification line (in Subsection 2.3.1.1 or 2.3.1.2) is appropriate for your situation. (see Chapter 10 for more detail).

#### 15	Single Drive Socket Configuration

The default condition of the specification line that installs CARDTALK.SYS is that the driver assigns one logical drive letter to each socket. Assuming you are adding the line toward the end of CONFIG.SYS (after all other commands that assign logical drives) as suggested in Section 2.3, the drives assigned are the first two (alphabetically) not used in the system’s base configuration. For example, if drives A and B are floppy disk drives, drives C and D are hard disk drives, and there are no other mass storage devices in the system (examples throughout this document assume this system configuration), then drive E corresponds to socket 0 and drive F corresponds to socket 1.

The form of the specification line that must be added to CONFIG.SYS to configure CARDTALK.SYS to assign one logical drive letter to each socket is:

device = drive:path\CARDTALK.SYS

where drive is the drive letter and path the complete name of the directory you chose when installing the software. For example, with the driver files installed in c:\cardtalk, you would add the following line:

device = c:\cardtalk\CARDTALK.SYS

You must also set the LASTDRIVE specification in your CONFIG.SYS file to be a drive letter that is equal to or be;yond the total number of drives installed. For example, assume drives A through D are assigned in your base system configuration. If you install CARDTALK.SYS as in the above example, you will have drives E and F installed, and the LASTDRIVE specification should be G or beyond:

LASTDRIVE = <G|H|I|…>

#### 16	Multiple Drive Socket Configuration

As stated in Subsection 2.3.1.1, the default condition is that CARDTALK.SYS assigns one drive letter to each of the two sockets. If multi-partitioned cards are to be used in either socket, however, the default condition provides access to only the first partition of any such card. A parameter must be added to the CARDTALK.SYS installing specification line to allow access to multiple partitions; This parameter is a switch argument (/n:) that specifies the total number of drive letters to be assigned, as follows:

device = drive:path\CARDTALK.SYS /n:number_of_driveletters

where number_of_driveletters must be at least the number of sockets to be managed by CardTalk (one or two, depending on the system) but no larger than ten. For example, for four logical drives, specify /n:4, as follows (assuming the driver files are installed in c:\cardtalk):

device = c:\cardtalk\CARDTALK.SYS /n:4

The desired number of logical drives is determined by — but is not the same as — the maximum number of partitions you want to set up on any card to be inserted in each socket; it is determined as follows: The first half of logical drives (rounded up to the next integer if the total number of partitions is odd) correspond to socket 0, and the remainder to socket 1. This is shown in Table 2-1. Thus, for example, if socket 0 will be handling three-partition cards, the table shows that /n:5 or /n:6 is to be used.

Table 2-1	Assignment of Logical Drives to Physical Card Sockets

| /n: Switch |  | Drives Assigned † to |  |  |  |  | /n: Switch |  | Drives Assigned † to |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Entry | Socket 0 |  | Socket 1 |  |  | Entry |  | Socket 0 |  | Socket 1 |  |
| /n:1 | E |  | — |  |  | /n:6 |  | E,F,G |  | H,I,J |  |
| /n:2 ‡ | E |  | F |  |  | /n:7 |  | E,F,G,H |  | I,J,K |  |
| /n:3 | E,F |  | G |  |  | /n:8 |  | E,F,G,H |  | I,J,K,L |  |
| /n:4 | E,F |  | G,H |  |  | /n:9 |  | E,F,G,H,I |  | J,K,L,M |  |
| /n:5 | E,F,G |  | H,I |  |  | /n:10 |  | E,F,G,H,I |  | J,K,L,M,N |  |
| † assumes drives A — D are assigned in the base system configuration ‡ Default. |  |  |  |  |  |  |  |  |  |  |  |

In any socket to which multiple drives are assigned, drive letters correspond in alphabetical order to the order of partitions on a card. Table 2-2 shows the effects of the relationship between the number of partitions on a card and the number of drives assigned to the socket in which that card is inserted.

You must also set the LASTDRIVE specification in your CONFIG.SYS file to be a drive letter that is equal to or beyond the total number of drives installed. For example, assume drives A through D are assigned in your base system configuration. If you install CARDTALK.SYS to support four logical drives, as in the above example, you will have drives E, F, G, and H installed, and the LASTDRIVE specification should be I or beyond:

LASTDRIVE=<I|J|K|…>

17	Flash File System Installation

Table 2-2	Effect of Relative Number of Card Partitions and Socket Drives

| Relationship Between Card Partitions & Socket Drives | Effects |
| --- | --- |
| Equal Partitions and Drives | All card partitions are accessible, in corresponding alphabetical order |
| More Partitions than Drives | Only the first d card partitions (where d is the number of socket drives) are accessible, one per drive in alphabetical order; the remainder of the card partitions cannot be accessed. |
| Fewer Partitions than Drives | Only the first p socket drive letters (where p is the number of card partitions) are needed to access the card; attempts to access the card using higher drive letters will result in a “Drive not ready”, “Invalid media”, or “Invalid Drive” error message. |

If you want to use FFS2 and you are changing your CONFIG.SYS file manually, you must install MS-FLASH.SYS (the FFS2 driver), by adding the following line after the line that installs CARDTALK.SYS:

device = drive:path\MS-FLASH.SYS

If you are installing FFS2, and you want to use automatic file compression, you will have to install DBLFLASH.EXE (the FFS2 compression utility driver), by adding the following line after the CARDTALK.SYS line, but before the MS-FLASH.SYS line:

device = drive:path\DBLFLASH.EXE

To summarize, the lines added must be in the following order:

device = drive:path\CARDTALK.SYS	[required]

device = drive:path\DBLFLASH.EXE	[if using FFS2 with automatic file compression]

device = drive:path\MS-FLASH.SYS	[if using FFS2]

## 18	Verifying Software Installation

After you have installed the software, you must reboot your system to activate CARDTALK.SYS.

When you reboot the system, the screen quickly flashes the following message identifying the Databook BIOS Extension:

TMI-140 TCIC-2/N Evaluation Board

Databook Socket Services BIOS V2.20 [release date]

Copyright (C) Databook Incorporated 1990-93.

Performing Self Test...passed

Assuming there is no card in socket 0, the screen then displays the following message:

Databook CardTalk Bootstrap Manager V2.20 [release date]

Copyright (C) Databook Incorporated 1990-93.

No card present.

and proceeds with the boot process as normal. (For an explanation of how to boot from a memory card, see Section 10.3.)

If correctly installed, the software will print the following messages when booting is complete (in this illustration, the drive letter is “E”):

Databook CardTalk Device Driver V2.20

Copyright (C) Databook Incorporated 1990-93. All rights reserved.

Installed as DOS drive E:.

Installed as DOS drive F:.

Note the drive letter displayed (in this example, E); you will use this letter to access the memory card drive on the system.

To confirm that everything is working properly once your system has booted up, make sure there is no card in drive E (in this illustration), then enter the following command at the DOS prompt:

tcformat -v -now e:

The system should display one of the following two messages, depending on whether installation is successful or has failed:

tcformat V2.20

Copyright (C) Databook Incorporated 1990-93.

?tcformat-F-can't access card 'e:' -- no card in drive.

or

tcformat V2.20

Copyright (C) Databook Incorporated 1990-93.

?tcformat-F-can't access card 'e:' -- driver not responding.

The latter message indicates one of the following possible problems:

1.	an unsupported version of DOS. The software requires DOS version 3.3 or later;

2.	wrong drive letter — re-check the message displayed during system start-up;

3.	too few drives in DOS — add or modify the following specification line in CONFIG.SYS:

lastdrive = letter

letter must be beyond the letters displayed by the system during start up.

## 19	Installation Error Messages

The following message may be displayed immediately upon rebooting:

TMI-140 TCIC-2/N Evaluation Board

Databook Socket Services BIOS V2.20 [release date]

Copyright (C) Databook Incorporated 1990-93.

Performing Self Test...failed

This message indicates one of the following problems:

1.	an addressing conflict between the TCIC-2/N and another I/O-addressed component of the system;

2.	a malfunctioning TCIC-2/N controller chip;

3.	a malfunctioning Socket Services RAM.

The following error messages may be reported during system start-up, after the CARDTALK.SYS driver version and copyright notice are displayed:

Error in command tail: text from CONFIG.SYS

This message indicates an error in the CARDTALK.SYS installing specification line. The driver is not installed. Correct the specification line in your CONFIG.SYS file, and restart your system.

Databook CardTalk Device Driver V2.20

Copyright (C) Databook Incorporated 1990-93. All rights reserved.

No Socket Services BIOS Sockets found.

This message indicates one of the following problems:

1.	a self-test failure;

2.	an addressing conflict between Databook Socket Services and some other memory-addressed component of the system;

3.	presence of a third party’s Socket Services that conflicts with Databook’s; or

4.	the TMI-140 is not installed.

Drive installation failed: code failure-code. Consult manual.

This message indicates that an error occurred during the installation of the CARDTALK.SYS driver. Table 2-3 gives the possible error codes and their meanings.

Table 2-3	CardTalk Driver Error Codes

| Code | Meaning |
| --- | --- |
| 0301h | Auto-increment for byte or word access failed. |
| 0302h | Auto-increment mode for attribute address space failed. |
| 0402h | Auto increment for block transfer failed |
| 0501h | Increment hold mode testing failed |
| 0Cxxh | CRC tests failed (xx = ignored) |
| 13xxh | 16-bit data path test failed (xx = failed bit code) |
| 14xxh | 8-bit data path test failed (xx = failed bit code) |

# 20	TC Utility Commands — Quick Reference

The purpose of this chapter is to provide enough information to allow you to format a memory card that can be used with the most popular DOS commands in simple, straightforward applications. For most such applications, you should not have to consult succeeding chapters.

If you have a special or unusual application, you should see Chapters 8 through 12 for more comprehensive and detailed information; see especially Chapter 8 for specifying card information, Chapter 10 for formatting, and Chapter 12 for complete details on all the options that can be exercised with all the TC utility commands.

## 21	File Systems

A PC memory card is a storage device functionally identical to a floppy disk. Once a card is properly formatted, you can access it directly using standard DOS commands. A memory card may also be formatted to be a multi-partition card; each partition of a multi-partition card, when formatted, is treated as a separate DOS drive.

Formatting a PC memory card, or any partition of a multi-partition card, consists of applying a particular file system to it. There are three types of file systems that Databook’s TC utilities can apply; the one to use depends on the type of card and its expected use.

| File Allocation Table (FAT) | This is the standard DOS file system; it is placed on SRAM, ATA interface, and Sundisk cards. A FAT file system is required for booting hard drives. |
| --- | --- |
| Flash FAT | This is a special FAT used only for Flash cards. DOS commands can be used with a Flash FAT-formatted card, except as follows: • TCXCOPY.EXE must be used to copy files to the card. • The DOS del command does not work. • The DOS chkdsk does not work. • The DOS dir command always shows 0 bytes free instead of actual free space. DOS can be booted from a Flash FAT-formatted card. |
| Flash File System version 2.0 (FFS2), a Microsoft standard. | This provides the most flexible structure for Flash cards. DOS commands work except for chkdsk. DOS cannot be booted from a FFS2-formatted card. (Databook no longer supports FFS version 1.) |

## 22	Formatting a Single-Partition Card

The steps required to format a PC memory card depend on its type. In the following examples, the card is assumed to be in drive D.

### 23	SRAM Card

C:> tcformat d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FAT File System

Format complete.

C:>

### 24	Flash Card

C:> tcerase d:

Programming:   100%

Erasing:   100%

C:> tcformat -type flash d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FFS2 File System.

Format complete.

C:>

### 25	Flash Series-2 Card

C:> tcerase d:

Erasing:   100%

C:> tcformat -type flash d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FFS2 File System.

Format complete.

C:>

### 26	Sundisk Card or ATA Interface Hard Disk

Use the standard DOS format command.

## 27	Formatting a Multi-Partition Card

To format a multi-partition card, a Card Information Structure (CIS) must be placed on the card. The CIS is a set of data that describes a card’s physical attributes, and that may also (but need not) include a description of its logical structure. The tcinit command is used to place a CIS on the card.

tcinit requires the existence of a configuration file for the particular card. If the configuration file contains the physical as well as logical card information, it is sufficient. If the configuration file contains only the logical information, the physical information must be obtained from Databook’s database, included in the TMI-140’s software distribution package. The following two subsections illustrate these two cases respectively.

### 28	Multi-Partition SRAM Card (Configuration File Only)

In this case, the configuration file (displayed by the first command shown) contains the physical card information and the logical structure information; the tcinit command need only identify the configuration file.

C:> type 2psram.cfg

ECHO Two Partition SRAM card

# Physical Characteristics

CARDTYPE:   RAM

CARDSIZE    512K

PHYSLAYOUT:

REG-SPACE: NONE

COMMON-SPACE:

DEVICE-TYPE:  SRAM

# Must store CIS on the card

CIS: FULL

CIS-RESERVE: 1

CIS-CHECKSUM: YES

# Layout Information

PARTITIONS: 2

PARTITION:

SIZE: 200K

FILE-SYSTEM: DOS

ERROR-DETECT: CRC-INLINE

PARTITION:

FILE-SYSTEM: DOS

ERROR-DETECT: CRC-INLINE

C:> tcinit -config 2part d:

Two Partition SRAM card

C:> tcformat d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FAT File System

Format complete.

C:> tcformat e:

Insert new card in drive E:, and press ENTER when ready: <cr>

Formatting for FAT File System

Format complete.

C:>

### 29	Multi-partition Flash card  (Configuration File and Database)

In this case, the configuration file (displayed by the second command shown) contains only the card’s logical structure information; the tcinit command must include a parameter (-card) to identify the pertinent physical information in Databook’s database.

C:> tcerase d:

Programming:   100%

Erasing:   100%

C:> type 2pflash.cfg

ECHO Two Partition FLASH card

# Must store CIS on the card

CIS: FULL

CIS-RESERVE: 1

CIS-CHECKSUM: YES

# Layout Information

PARTITIONS: 2

PARTITION:

SIZE: 512K

FILE-SYSTEM: FFS2

ERROR-DETECT: NONE

PARTITION:

FILE-SYSTEM: FFS2

ERROR-DETECT: NONE

C:> tcinit -card intel/imc004 -config 2pflash d:

Two Partition FLASH card

C:> tcformat -type flash d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FFS2 File System.

Format complete.

C:> tcformat -type flash e:

Insert new card in drive E:, and press ENTER when ready: <cr>

Formatting for FFS2 File System.

Format complete.

C:>

## 30	Formatting a Bootable Card

### 31	Bootable SRAM Card

C:> tcformat -sys d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FAT File System.

Format Complete.

Copying system files

Complete

C:>

### 32	Bootable Flash Card

This operation must be performed on an erased partition or a card with only a CIS.

C:> tcformat -type flash -sys d:

Insert new card in drive D:, and press ENTER when ready: <cr>

Formatting for FAT File System.

Format Complete.

Copying system files

Complete

C:>

# 33	Using ATA Interface Devices

The two major classes of “ATA” (sometimes referred to as “IDE”) interface devices: solid-state cards and hard disk drives, are functionally identical from the viewpoint of the system. Each presents a logical disk drive interface to the system. CardTalk handles these devices somewhat differently from the way memory cards are handled; except for the initialization procedure, however (see Subsection 4.2), the differences are largely transparent to the user.

This chapter applies to the following types of ATA interface devices:

•	SunDisk solid-state ATA interface Flash memory cards, series SDP and SDPL (PCMCIA-compatible). (Certain older SunDisk cards are not PCMCIA-compatible and will not operate with CardTalk.)

•	“First-generation“ ATA interface 1.3-in and 1.8-in hard disk drives in PCMCIA form factor. These devices’ ATA interface signals are brought out to their PCMCIA connector. Although their interface is not PCMCIA-compatible, they are supported by the TCIC-2/N and CardTalk.

•	“PCMCIA/ATA“ interface 1.3-in and 1.8-in hard disk drives, whose I/O card interface is PCMCIA-compatible. (Except for possible markings, these are physically indistinguishable from "first-generation" ATA drives.)

## 34	Device Insertion

### 35	SunDisk Cards

SunDisk cards are PCMCIA Type 2 cards, which are 1.7 mm thicker than PCMCIA Type 1 cards (see Reference 1), and will not fit in socket 0 of the TMI-140 (the socket closest to the circuit board). SunDisk cards may therefore be inserted only in socket 1 (farthest from the circuit board).

WARNING:

Do not attempt to insert a SunDisk card into socket 0 of a TMI-140. Forcing will damage the connector.

### 36	Hard disk Drives

Most ATA drives are PCMCIA Type 3 devices, which are 5 mm thicker than PCMCIA Type 2 cards. Some ATA drives are even thicker. ATA drives can therefore not be directly installed into either socket of a TMI-140. Databook offers a socket extender card that allows ATA drives to be used in the TMI-140. The extender is a PCMCIA Type 2 card; it may therefore be inserted only into socket 1 of the TMI-140 (see Subsection 4.1.1).

## 37	Initialization

As explained in Section 4.1, Databook’s TC utilities are not used with I/O cards (including ATA interface devices). To initialize an ATA interface device (solid-state card or hard disk drive) after insertion, at the DOS prompt C>, type the standard DOS format command (not tcformat which is used as described in Chapter 10 for memory cards only), as follows:

C> FORMAT E:

The integrity of a formatted device can be verified with the DOS CHKDSK.EXE utility, as well as with such third party utilities as the "Norton Disk Doctor."

## 38	Compatibility Issues

### 39	SunDisk Cards

#### 40	SunDisk Technology Driver

SunDisk cards require special driver software in addition to standard PCMCIA memory card drivers. The CardTalk driver includes this special software. If you plan to use SunDisk cards to exchange data with another system, consult the reference manual for the other system or contact SunDisk to ensure that the other system supports SunDisk cards.

#### 41	Write Limitations on Certain Computers

Both SunDisk card series, SDP and SDPL, can be read on the HP-95LX or other 5 V-only palmtop computer, but only the SDPL series can be written on such a computer. For reads and writes, a special driver is requried for the HP-95LX; this driver can be obtained from SunDisk.

### 42	Hard Disk Drives

ATA drives require significantly more power than do most PCMCIA cards. An ATA drive must be used only with a PCMCIA socket whose vendor has approved it for use with ATA drives. The TMI-140’s socket 1 is approved for use with ATA drives.

WARNING:

Using an ATA drive with a PCMCIA socket whose vendor has not approved it for use with ATA drives may permanently damage the electronics of your system and render the socket inoperable.

## 43	Start Delay

When an ATA interface hard disk drive is inserted and recognized, a start delay of several seconds is required before accesses may occur.  The system is free to execute other programs, but an access to the ATA drive during this delay time will cause the system to pause until the delay has elapsed.

# 44	Using Modem Cards

This chapter discusses the basic principles and limitations of operating PCMCIA Modem and Fax/Modem cards with CardTalk in the DOS and Windows environments.

## 45	Insertion Constraints

All current modem and fax/modem cards are packaged in PCMCIA Type 2 configuration (see Reference 7). Type 2 cards are 1.7 mm thicker than Type 1 cards, and can easily be distinguished by a raised area in the middle of the card. A Type 2 modem card will not fit in socket 0 of the TMI-140 (the socket closest to the circuit board), and may therefore be inserted only in socket 1 (farthest from the circuit board).

Warning:

Do not force a Type 2 modem card into socket 0.
Forcing can physically damage the socket.

## 46	Serial Port Assignment

A modem card may be inserted into either PCMCIA socket at any time (except see Section 5.1). When a modem card is inserted, CardTalk scans the system state and automatically determines the serial port assignment for that modem card that is best for the system. The serial port number assignment scheme in CardTalk V2.20 has been significantly upgraded. The scheme now takes into account many more aspects of the system's operations, such as mouse drivers.

CardTalk does not reassign a modem card after its initial assignment. Therefore, for best results you should load your mouse driver before inserting the modem card. Because a card inserted at boot time is configured by CardTalk as soon as CARDTALK.SYS is loaded (see Subsection 2.3.1), we recommend that you load the mouse driver in CONFIG.SYS before loading CARDTALK.SYS.

If running under Windows, CardTalk conforms to the COM port assignments given in SYSTEM.INI's [386enh] section. Therefore, CardTalk may configure a modem card differently depending on whether the card is inserted before or after Windows is started.

Because of limitations in the BIOS port assignment scheme, CardTalk may not configure a modem card if both serial ports are active when the card is installed.

Because of limitations in the PCMCIA standard (Reference 8), a modem card's IRQ lines are not completely compatible with the ISA standard (P996 — Reference 9). If the modem is assigned to COM3 or COM4, CardTalk takes advantage of special hardware in the TCIC-2/N to better simulate the sharing of IRQs with standard serial ports. This special mode works well under Windows 3.1 in 386 Enhanced Mode, both with Windows communication applications and with DOS applications running in compatibility mode. However, it causes some older DOS applications  (e.g., KERMIT.EXE) to malfunction when they are running under DOS. Therefore, you may need to disable this COM port-sharing feature of the TCIC-2/N, to allow you to use certain DOS communications applications when not running under Windows. This can be done via a new switch, /noshare, on the line that installs CARDTALK.SYS, as follows:

device = \cardtalk\cardtalk.sys /noshare

If you must do this, you will not be able to use COM1 on your motherboard whenever a modem card is installed in your system.

## 47	Troubleshooting Modems Under Windows

If Windows does not recognize your modem card, you may find the following notes helpful:

•	Make sure your modem card is installed before starting your communication program. If your communication program (such as a fax program) is launched automatically when you start Windows, make sure your modem card is installed before you start Windows.

•	Before starting Windows, disable COM2 (if present), so that only COM1 is enabled. CardTalk will then assign your PCMCIA modem card to COM2. Select COM2 for your communication program, and check whether the problem is solved.

•	As a quick test, you can use the Windows Terminal program to check whether your modem is functioning.

—	Insert the card.

—	Double-click on the Windows Terminal icon.

—	Select the COM port to which the modem has been configured, using the Settings/Communications/Connector menu.

—	In the Windows Terminal pane, try typing the following:

ATZ

—	If the modem is working correctly, you should see:

OK

# 48	Using Network Cards

Most network operating systems use one of two kinds of driver: NDIS or ODI. (Of the two most popular systems, LAN Manager uses NDIS, and NetWare uses ODI.)

## 49	Network Cards Supported

Databook has tested CardTalk with several PCMCIA network cards using standard vendor-supplied software without modification in a 10net environment. 10net is a LAN Manager-compatible peer network for DOS-based systems.  In each case, the NDIS version only of the software that interfaces with Socket Services was used for testing. Table 6-1 shows the vendors whose cards have been tested and the vendor software packages used.

Table 6-1	Network Cards Supported and Software Used for Testing

| Vendor | Vendor S/W Version | Comments |
| --- | --- | --- |
| Xircom | v1.0 |  |
| IBM National Ethernet | v1.09 | Enabler & drivers from IBM support BBS; National Ethernet card is identical to the IBM card. |
| Socket Communications | v1.00 | Enabler from Socket Communications |

## 50	Installing Network Drivers

### 51	ODI

ODI drivers operate by loading several layers of protocols, usually from AUTOEXEC.BAT. The load sequence is typically:

C> lsl                    [link service layer]

C> hardware-driver

C> ipxodi                 [ipx-to-ODI converter]

C> netx                   [e.g., net3 :  the NetWare driver]

Note that these are all TSRs.

ODI configuration information comes from a text file named NET.CFG in the current directory. The first line of the file is:

Link Driver name

where name is the driver name, which you must insert.

### 52	NDIS

NDIS drivers operate by loading several layers of device drivers. The load sequence is in CONFIG.SYS:

rem load the NETBIOS registry

device=netprot.dos /i:path-to-setup-file

rem load the hardware driver

device=hardware-driver.dos

rem load the upper-level network control

device=netbeui.dos

Depending on the network operating system you may also need one or more additional commands in CONFIG.SYS. For 10net, the commands are:

C> 10bind                 [process the bindings]

C> 10net                  [start the network OS]

Configuration information about the network comes from a text file, PROTOCOL.INI. This file is in a different format from that of NET.CFG for ODI (see Subsection 6.3.1), and resides in the directory specified by the /i: switch on the first device driver loaded (see the load sequence listed above). It looks much like a Windows .INI file.

Sample  PROTOCOL.INI files that have been tested are shown in Subsections 6.3.1 and 6.3.2.

## 53	Installing a PCMCIA Ethernet Adapter Card

To install a PCMCIA Ethernet Adapter card in the protocol stack, you must do two things:

1.	Arrange for the PCMCIA interface and the card to be suitably configured. This is usually done with a piece of software called an "enabler", which is added as a device driver or executable command in the sequence of commands that loads the network.

2.	Install the driver for the PCMCIA interface.

The enabler must be told such things as which I/O ports to use, which memory addresses to use (if a memory window is to be used), etc. You are responsible for determining which addresses to use for this purpose. For Xircom cards, the information need only be entered once, into NET.CFG (for ODI) or PROTOCOL.INI (for NDIS). For IBM and National cards, the information must be entered twice: once into NET.CFG or PROTOCOL.INI, and a second time into the command line of the enabler itself.

The following two subsections show complete examples, using 10net as the example network.

### 54	XIRCOM CreditCard Sample Procedure

To start using the Xircom CreditCard Ethernet Adapter card, prepare the following PROTOCOL.INI file (with appropriate changes to accommodate your network, if it is not 10net):

;--- protocol.ini for Xircom Ethernet Credit Card, under 10net --

[protocol manager]

drivername=PROTMAN$

[10BEUI]

drivername="10BEUI$"

bindings=XIRCOMNET

; protocol.ini section for the Xircom CreditCard Ethernet Adapter.

[XIRCOMNET]

DRIVERNAME = XIRCOM$

PORT = 0x260

INT = 10

MEM = 0xD000

IOWORDSIZE = 16

;socket 1 is the first socket; socket 2 is the second....

SOCKET = 1

;---- end -----

Also, add the following lines in CONFIG.SYS (with appropriate changes to accommodate your network, if it is not 10net):

rem	.. the registry:

device=\10net\10prot.dos /i:\10net

rem	.. the enabler:

device=\xircom\cecfg.exe driver=ndis

rem	.. the driver:

device=\xircom\cendis.exe

rem	.. rest of the network software:

device=\10net\10beui.dos

The Xircom enabler will automatically search each socket for a Xircom card. It will then configure the card and the PCMCIA system, using the information given in the PROTOCOL.INI file.

### 55	IBM/National Semiconductor Card Sample Procedure

To start using the IBM or National Semiconductor Ethernet Adapter card, prepare the following PROTOCOL.INI file (with appropriate changes to accommodate your network, if it is not 10net).

;--- protocol.ini for IBM Ethernet Credit Card, under 10net --

[protocol manager]

drivername=PROTMAN$

[10BEUI]

drivername="10BEUI$"

bindings=PCMNIC_NIF

[PCMNIC_NIF]

; protocol.ini section for the IBM PCMCIA-NIC Card

IOBASE = 0x300

INTERRUPT = 5

RAM = 0xD400

DRIVERNAME = PC-NIC$

;--- end ---

Also, add the following lines in CONFIG.SYS (with appropriate changes to accommodate your network, if it is not 10net):

rem .. the registry:

device=\10net\10prot.dos /i:\10net

rem .. the enabler:

rem    Note that these parameters MUST match the parameters

rem    in PROTOCOL.INI:

device=\ibm\ethernet.sys sa irq=5 ds=16 sram=d400 io=300

rem .. the driver:

device=\ibm\pcmnic.dos

rem .. rest of the network software:

device=\10net\10beui.dos

It is essential that the parameters given on the ethernet.sys command line match the parameters given in PROTOCOL.INI. In addition, you must specify the socket in which to find the network card ("sa" or "sb" for socket 0 or socket 1 respectively).

Note:

The ECCTEST.EXE initialization program supplied by IBM and National Semiconductor is a platform-specific program; it is not compatible with and will not operate with CardTalk or the TCIC-2/N.

# 56	CardTalk Driver Reference Information

## 57	Specification Line for CardTalk Driver Installation

CARDTALK.SYS, the device driver for the TCIC-2/N PC Card Controller, must be added to the DOS system configuration to provide support for PCMCIA standard socket operations (see Section 2.3). This is accomplished by adding a specification line to the system configuration file, CONFIG.SYS, that installs CARDTALK.SYS at power-up.

### 58	Syntax

The full syntax of this CARDTALK.SYS specification line is as follows (square brackets ([]) enclosing an item indicates that entry of that item is optional — the brackets themselves are not to be entered):

device = [drive:][\path\]CARDTALK.SYS [/n:#] [/a] [/noshare]

where	drive	is the DOS drive letter for the storage device that contains CARDTALK.SYS;

path	is the directory (or sub-directory) that contains CARDTALK.SYS;

#	is the total number of drive letters to be assigned to the system’s card socket(s) by CARDTALK.SYS.

You should specify drive and path when editing the system’s file, as described in Subsection 2.3.1. We recommend, however, that you not specify drive or path when constructing a CONFIG.SYS file to be used for booting the system from a memory card, as described in Sections 10.3 and 10.5.

You may, but need not, specify /n:# when preparing CONFIG.SYS for use with only single-partition memory cards. You must, however, specify /n:# when preparing CONFIG.SYS for use with multi-partition cards. How to determine the value of # to specify in a particular application is discussed in Subsection 2.3.1.2 and illustrated in Table 2-1.

/a enables CardTalk’s annunciator feature, which provides card insertion feedback through the speaker. When /a is specified, card insertion will produce one, two, or three beeps, as follows:

| 1 beep: | CARDTALK.SYS recognizes the card as a properly initialized memory card. |
| --- | --- |
| 2 beeps: | CARDTALK.SYS does not recognize the card, and assumes that it is an uninitialized memory card (see list item 2b in Section 7.2). |
| 3 beeps: | CARDTALK.SYS recognizes the card, but initialization has failed; this is usually because some system resource necessary to support the card is not available. |

(The single, double, and triple beeps are at successively lower pitches.)

You should specify /noshare only if you are running any of several older DOS applications under DOS, which malfunction with the TCIC-2/N’s IRQ level-to-pulse conversion feature; this switch disables that feature (see Section 5.2).

### 59	Case and Space Sensitivity

DOS is not case-sensitive when it is processing CONFIG.SYS. You may therefore enter the specification line in any combination of upper and lower case letters.

DOS is not sensitive to the presence or absence of spaces on either side of the "=", and before the drive number switch. DOS is sensitive, however, to spaces within the file specifier string; there must be no such spaces. Figure 7-1 illustrates allowable and unallowed spaces in an example specification line.

Figure 7-1	CardTalk Driver Installation Specification Line Space Sensitivity

### 60	Loading Into High Memory

MS-DOS v5.0 or later allows device drivers to be loaded into high memory, if your system is suitably configured. CARDTALK.SYS is a fairly large driver (about 35 kbytes), so this may be of some advantage to you. If your system is operating with MS-DOS v5.0 or later, you may specify that CARDTALK.SYS is to be loaded into high memory by substituting the word devicehigh for the word device in the CARDTALK.SYS specification line. If CARDTALK.SYS cannot be loaded into high memory, it will be loaded into low memory.

## 61	Operation at Card Insertion

Using the DOS timer interrupt, CardTalk polls the TCIC-2/N to sense card insertion in either socket; when card insertion is sensed, CardTalk performs the following operations:

1.	Card power is switched on.

2.

a.	If the card contains a Card Information Structure (CIS — see Section 8.1), the CIS is interrogated to determine the card type. Every I/O card (modem; network card; ATA interface bulk memory storage device — SunDisk type solid-state and PCMCIA hard disk drive) has a CIS.

b.	If the card does not contain a CIS, CardTalk assumes that the card is a memory card of unknown type. To ensure that neither the card nor its data are damaged, the card will not be accessible when a utility program is invoked unless the user specifies its type; this may be done in either of two ways, depending on the arguments used with the utility-invoking command (see Chapter 8).

3.	When a card is recognized, its socket is configured appropriately, and the card is initialized if required. In general, memory cards do not require initialization, whereas I/O cards do.

## 62	CardTalk Support For Windows

CardTalk provides additional support for PCMCIA modem cards when Microsoft Windows 3.1 is operating in '386 Enhanced Mode (the default mode). This support allows Windows applications (as well as DOS applications running under Windows) to take full advantage of PCMCIA cards, and adds additional error checking and system management functions.

The support software is provided as a separate file, CARDTALK.386. This file is loaded by Windows at startup time. It must reside in the same directory as CARDTALK.SYS.

If Windows cannot find CARDTALK.386, the following error message will be displayed during Windows startup:

Cannot find a device file that may be needed to run Windows in 386 enhanced mode.

You need to run the Setup program again.

drive:path CARDTALK.386

Press a key to continue

Note the drive letter and directory (drive and path) shown in the above message, and then press a key. Windows will start up without CardTalk PCMCIA support. We recommend that you exit from Windows immediately and correct the problem before proceeding.

The above error message from Windows is misleading; the Windows "Setup program" has nothing to do with finding CARDTALK.386.  To correct this problem, you must use the DOS copy command to put CARDTALK.386 in the directory shown (drive:path) in the message.

# 63	Specifying Memory Card Information

## 64	Card Information Structure (CIS)

The TC utilities are designed to program various card types for different uses. Specifying the card type and size information (or the card's manufacturer and part number) gives the software the physical data to properly program the card. In addition, you can indicate how you want to logically store data on a memory card.

The logical information that you can specify includes the entire card configuration: the file system to use on the card, the error detection method, and the number of logical partitions into which to divide the card.

The card's physical attributes as well as its logical configuration can be permanently stored on the card. Collectively, these data are referred to as the Card Information Structure (CIS) or the Card Metaformat.

Any system that supports the PCMCIA CIS specifications (see Reference 1) can read the CIS stored on the card and can obtain sufficient information to read and write to the card.

If a single-partition memory card does not contain a valid CIS, the TC utilities will attempt to obtain card information from the command line specifications or from the pertinent card configuration file, depending on the arguments you use with the utility-invoking command (-type and -size, or -card and -config, respectively — see Chapter 12). You can therefore program a single-partition memory card without placing a CIS on the card first. If your card contains a valid CIS, however, it takes precedence over any command line specifications and card configuration file information.

A CIS is required on a card that is to be formatted as a multi-partition card (see Section 10.4).

### 65	Placing CIS On Cards

To place CIS on cards, you use the tcinit command to invoke the TCINIT.EXE utility. On some systems such as the HP-95LX, CIS is placed on the card during the formatting process. With TCINIT.EXE, you have the option of putting CIS on the card. This is designed as a separate and optional step because you may want to use your card with a system that does not support CIS, such as the Poqet PC.

When working with TCINIT.EXE, you need to distinguish between the card's physical data (such as the type and size), and its logical configuration (such as the file system to use).

If you are working with a card that appears in Table 8-1, you can provide the card's physical data simply by specifying the card's manufacturer and part number with the -card option (see Subsection 12.3.3.2). You provide the card’s logical layout in a card configuration file that you prepare separately, and which TCINIT.EXE reads when you specify the -config option.

For example, to put CIS on an Intel 1-Mbyte Flash card (part number IMC001 — see Table 8-1) in drive E using a card configuration file named flash.cfg, you would type:

tcinit -card intel/imc001 -config flash.cfg e:

If your card doesn't appear in Table 8-1 but is compatible with PCMCIA/JEIDA standards, you can provide the physical data about the card in the same card configuration file that contains the logical layout. At a minimum, you need to specify the card type and size (see the sample card configuration files in Subsection 8.3.3).

Assuming that you have all this information in a card configuration file called test.cfg and the card is in drive D, run TCINIT.EXE with just the -config option:

tcinit -config test.cfg d:

Subsequently, other TC commands will examine the CIS on the card to properly program and read from it.

The CIS information stays on the card until you erase the card using the tcerase command, if the card does not have a separate attribute memory (see Subsection 8.1.2). A card that does have attribute memory can be initialized with new CIS information via the tcinit command.

See Subsection 12.3.3 for complete information on the syntax of the tcinit command.

### 66	Attribute Memory

Some memory cards come with a separate memory area used to store Card Information Structure. This is referred to as attribute memory space.

When your card configuration file specifies that the card has attribute memory space (see description of the REG-SPACE keyword in Subsection 8.3.2.2), TCINIT.EXE attempts to save CIS in attribute memory. Consult the card's manufacturer specifications for information on whether the card has attribute memory space.

### 67	Common Memory

If a card does not have attribute memory space, TCINIT.EXE places the CIS starting at byte 0 of common memory. If attribute memory space is present, CardTalk examines it and assumes, unless specified otherwise, that the CIS continues in common memory. This mechanism allows cards with fixed attribute memory (e.g. Intel Series-2) to have additional CIS information placed in common memory.

## 68	Databook Card Information Database

The TC utilities support various card types, each of which requires a unique programming technique. To program the card correctly, the software needs to know the type of card you are working with. As you will see in the following sections, CardTalk provides different methods for specifying card information. If no card type is specified, the following TC utilities assume the indicated default card types:

| TCERASE.EXE: |  | Flash |  |
| --- | --- | --- | --- |
| TCFORMAT.EXE: |  | RAM |  |
| TCXCOPY.EXE: |  | Flash |  |

If you want to override the default card type, provide additional card information, or use one of the three utilities that has no default (TCINIT.EXE, TCPROG.EXE, or TCREAD.EXE), you can specify the card data by naming the card's manufacturer and part number (see Subsection 8.2.1), or by specifying the generic card type and size (see Subsection 8.2.2).

### 69	Specifying Part Numbers

Specifying the memory card's manufacturer and part number assures that the software takes into account all physical characteristics of the card when programming. These characteristics include the card size, type of chips, access speed, bus width, and whether or not the card has attribute memory (see Subsection 8.1.2).

All TC utilities let you specify the card's manufacturer and part number via the appropriate command line’s -card option (see all 12.3..X.2 under Subsection 12.3). The -card option has the format:

-card manufacturer/part-number

For example, to use TCFORMAT.EXE on a Fujitsu RAM card with part number MB98A90601-20, enter the command:

tcformat -card fujitsu/90601 drive:

To use TCERASE.EXE on a 4-Mbyte Intel Flash card, enter the command:

tcerase -card intel/imc004 drive:

Table 8-1 lists, by manufacturer and part number, all memory cards currently supported by the TC utilities, and the corresponding part-number code for the -card option in any TC utility command line. This table is current as of the publication date. The list of supported parts is regularly updated; for the most recent list, contact Databook.

Table 8-1	Memory Cards Supported by TC Utilities

| Manufacturer | Manufacturer’s Part Number | Technology | Size | Attribute Memory | -card option part-number |
| --- | --- | --- | --- | --- | --- |
| AMD | AmC001FLKA | Flash | 1 Mbyte | 512 byte | C001FLKA |
|  | AmC002FLKA |  | 2 Mbyte | 512 byte | C002FLKA |
|  | AmC004FLKA |  | 4 Mbyte | 512 byte | C004FLKA |
| ASCII | AF512-S | SRAM | 512 kbyte | 6 bytes | AF512-S |
| Dupont | 90100-504 | SRAM | 256 kbyte | yes | 90100504 |
|  | 90100-505 |  | 512 kbyte | yes | 90100505 |
|  | 90100-507 |  | 1 Mbyte | yes | 90100507 |
|  | 90100-509 |  | 2 Mbyte | yes | 90100509 |
|  | FLASH1M | Flash | 1 Mbyte | 2 kbyte | flash1m |
|  | FLASH2M |  | 2 Mbyte | 2 kbyte | flash2m |
| Epson | AWB 065SD20 | SRAM | 64 kbyte | 2 kbyte | A065SD20 |
|  | AWB 065SD30 |  | 64 kbyte | 16 bytes | A065SD30 |
|  | AWB 065SD40 |  | 64 kbyte | no | A065SD40 |
|  | AWB 101SD20 |  | 1 Mbyte | 2 kbyte | A101SD20 |
|  | AWB 101SD30 |  | 1 Mbyte | 16 bytes | A101SD30 |
|  | AWB 101SD40 |  | 1 Mbyte | no | A101SD40 |
|  | AWB 129SD20 |  | 128 kbyte | 2 kbyte | A129SD20 |
|  | AWB 129SD30 |  | 128 kbyte | 16 bytes | A129SD30 |
|  | AWB 129SD40 |  | 128 kbyte | no | A129SD40 |
|  | AWB 129SDX0 |  | 128 kbyte | 6 bytes | A129SDX0 |
|  | AWB 201SD20 |  | 2 Mbyte | 2 kbyte | A201SD20 |
|  | AWB 201SD30 |  | 2 Mbyte | 16 bytes | A201SD30 |
|  | AWB 201SD40 |  | 2 Mbyte | no | A201SD40 |
|  | AWB 257SD20 |  | 256 kbyte | 2 kbyte | A257SD20 |
|  | AWB 257SD30 |  | 256 kbyte | 16 bytes | A257SD30 |
|  | AWB 257SD40 |  | 256 kbyte | no | A257SD40 |
|  | AWB 513SD20 |  | 512 kbyte | 2 kbyte | A513SD20 |
|  | AWB 513SD30 |  | 512 kbyte | 16 bytes | A513SD30 |
|  | AWB 513SD40 |  | 512 kbyte | no | A513SD40 |
|  | HWB 101SDX0 | Flash | 1 Mbyte | 2 kbyte | H101SDX0 |
|  | HWB 101SDY0 |  | 1 Mbyte | no | H101SDY0 |
|  | HWB 201SDX0 |  | 2 Mbyte | 2 kbyte | H201SDX0 |
|  | HWB 201SDY0 |  | 2 Mbyte | no | H201SDY0 |
|  | HWB 257SDX0 |  | 256 kbyte | 2 kbyte | H257SDX0 |
|  | HWB 257SDY0 |  | 256 kbyte | no | H257SDY0 |
|  | HWB 513SDX0 |  | 512 kbyte | 2 kbyte | H513SDX0 |
|  | HWB 513SDY0 |  | 512 kbyte | no | H513SDY0 |
|  | LWB 065SD20 | SRAM | 64 kbyte | 2 kbyte | L065SD20 |
|  | LWB 065SD30 |  | 64 kbyte | 16 byte | L065SD30 |
|  | LWB 065SD40 |  | 64 kbyte | no | L065SD40 |
|  | LWB 101SD20 |  | 1 Mbyte | 2 kbyte | L101SD20 |
|  | LWB 101SD30 |  | 1 Mbyte | 16 byte | L101SD30 |
|  | LWB 101SD40 |  | 1 Mbyte | no | L101SD40 |
|  | LWB 129SD20 |  | 128 kbyte | 2 kbyte | L129SD20 |
|  | LWB 129SD30 |  | 128 kbyte | 16 byte | L129SD30 |
|  | LWB 129SD40 |  | 128 kbyte | no | L129SD40 |
|  | LWB 201SD20 |  | 2 Mbyte | 2 kbyte | L201SD20 |
|  | LWB 201SD30 |  | 2 Mbyte | 16 byte | L201SD30 |
|  | LWB 201SD40 |  | 2 Mbyte | no | L201SD40 |
|  | LWB 257SD20 |  | 256 kbyte | 2 kbyte | L257SD20 |
| Epson (cont) | LWB 257SD30 | SRAM (cont) | 256 kbyte | 16 byte | L257SD30 |
|  | LWB 257SD40 |  | 256 kbyte | no | L257SD40 |
|  | LWB 513SD20 |  | 512 kbyte | 2 kbyte | L513SD20 |
|  | LWB 513SD30 |  | 512 kbyte | 16 byte | L513SD30 |
|  | LWB 513SD40 |  | 512 kbyte | no | L513SD40 |
| FDK | SC9001-162H1 | SRAM | 512 kbyte | aliased | SC9001 |
|  | SC9008-82H1 | Flash | 1 Mbyte | yes | SC9008 |
| Fujisoku | JS1024G3-C-11 | SRAM | 1 Mbyte | yes | JS1024G3 |
|  | JS128G3-C-11 |  | 128 kbyte | yes | JS128G3 |
|  | JS256G3-C-11 |  | 256 kbyte | yes | JS256G3 |
|  | JS512G3-C-11 |  | 512 kbyte | yes | JS512G3 |
|  | JS64G3-C-11 |  | 64 kbyte | yes | JS64G3 |
| Fujitsu | MB98A9060 | SRAM | 64 kbyte | no | 9060 |
|  | MB98A90601-20 |  | 64 kbyte | aliased | 90601 |
|  | MB98A90602-25 |  | 64 kbyte | no | 90602 |
|  | MB98A90603-25 |  | 64 kbyte | 8 kbyte | 90603 |
|  | MB98A9070 |  | 128 kbyte | no | 9070 |
|  | MB98A90701-25 |  | 128 kbyte | aliased | 90701 |
|  | MB98A90702-25 |  | 128 kbyte | no | 90702 |
|  | MB98A90703-25 |  | 128 kbyte | 8 kbyte | 90703 |
|  | MB98A9080 |  | 256 kbyte | no | 9080 |
|  | MB98A90801-25 |  | 256 kbyte | aliased | 90801 |
|  | MB98A90802-25 |  | 256 kbyte | no | 90802 |
|  | MB98A90803-25 |  | 256 kbyte | 8 kbyte | 90803 |
|  | MB98A9090 |  | 512 kbyte | no | 9090 |
|  | MB98A90901-25 |  | 512 kbyte | aliased | 90901 |
|  | MB98A90902-25 |  | 512 kbyte | no | 90902 |
|  | MB98A90903-25 |  | 512 kbyte | 8 kbyte | 90903 |
|  | MB98A91021-20 |  | 1 Mbyte | aliased | 91021 |
|  | MB98A91022-20 |  | 1 Mbyte | no | 91022 |
|  | MB98A91023-20 |  | 1 Mbyte | 8 kbyte | 91023 |
|  | MB98A91121-20 |  | 2 Mbyte | aliased | 91121 |
|  | MB98A91122-20 |  | 2 Mbyte | no | 91122 |
|  | MB98A91123-20 |  | 2 Mbyte | 8 kbyte | 91123 |
| HP | F1002A | SRAM | 128 kbyte | yes | F1002A |
|  | F1003A |  | 512 kbyte | yes | F1003A |
| Intel | IMC001 | Flash | 1 Mbyte | aliased | IMC001 |
|  | IMC002 |  | 2 Mbyte | aliased | IMC002 |
|  | IMC004 |  | 4 Mbyte | aliased | IMC004 |
|  | IMC004s2 | Series 2 Flash | 4 Mbyte | read-only | IMC004S2 |
|  | IMC010s2 |  | 10 Mbyte | read-only | IMC010S2 |
|  | IMC020s2 |  | 20 Mbyte | read-only | IMC020S2 |
| ITT Cannon | CSCJ-0512F-A08A-04 | FLASH | 512 kbyte | 8 kbyte | 0512F |
|  | CSCJ-0512S-A08A-04 | SRAM | 512 kbyte | 8 kbyte | 0512S |
| Macro | 451F-3 | Flash | 1 Mbyte | aliased | 451F-3 |
|  | 451S-1 | SRAM | 64 kbyte | no | 451S-1 |
|  | 451S-2 |  | 128 kbyte | no | 451S-2 |
|  | 451S-3 |  | 256 kbyte | no | 451S-3 |
|  | 451S-4 |  | 512 kbyte | no | 451S-4 |
| Macro (cont) | 451S-5 | SRAM (cont) | 1 Mbyte | no | 451S-5 |
|  | 451S-6 |  | 2 Mbyte | no | 451S-6 |
| Mitsubishi | MF365A-L2DAT01 | SRAM | 64 kbyte | no | MF365A |
|  | MF3129-L2DAT01 |  | 128 kbyte | no | MF3129 |
|  | MF3257-L2DAT01 |  | 256 kbyte | no | MF3257 |
|  | MF3513-L2DAT01 |  | 512 kbyte | no | MF3513 |
|  | MF31M1-L2DAT01 |  | 1 Mbyte | no | MF31M1 |
|  | MF32M1-L2DAT01 |  | 2 Mbyte | no | MF32M1 |
| Panasonic | BN-064HMCE | SRAM | 64 kbyte | no | BN064HMC |
|  | BN-128HMCE |  | 128 kbyte | no | BN128HMC |
|  | BN-256HMCE |  | 256 kbyte | no | BN256HMC |
|  | BN-512HMCE |  | 512 kbyte | no | BN512HMC |
|  | BN-01MHMCE |  | 1 Mbyte | no | BN01MHMC |
|  | BN-02MHMCE |  | 2 Mbyte | no | BN02MHMC |
|  | BN-04MHMCE |  | 4 Mbyte | no | BN04MHMC |
| Poqet | SC9001-162H1 | SRAM | 512 kbyte | aliased | SC9001 |
|  | SC9008-82H1 | Flash | 1 Mbyte | yes | SC9008 |
| Seagate | ST72P | solid-state | 2.5 Mbyte | read-only |  |
|  | ST75P | ATA interface | 5 Mbyte | read-only |  |
|  | ST710P |  | 10 Mbyte | read-only |  |
|  | ST714P |  | 15 Mbyte | read-only |  |
|  | ST720P |  | 20 Mbyte | read-only |  |
|  | ST72P5 |  | 2.5 Mbyte | read-only |  |
|  | ST75P5 |  | 5 Mbyte | read-only |  |
|  | ST710P5 |  | 10 Mbyte | read-only |  |
| Sumitomo | SMC-01MFJ-NA | Flash | 1 Mbyte | no | 01MFJ-NA |
|  | SMC-02MFJ-NA |  | 2 Mbyte | no | 02MFJ-NA |
|  | SMC-04MFJ-NA |  | 4 Mbyte | no | 04MFJ-NA |
|  | SMC-256FJ-NA |  | 256 kbyte | no | 256FJ-NA |
|  | SMC-512FJ-NA |  | 512 kbyte | no | 512FJ-NA |
| SunDisk | SDP-2.5 | solid-state | 2.5 Mbyte | read-only |  |
|  | SDP-5 | ATA interface | 5 Mbyte | read-only |  |
|  | SDP-10 |  | 10 Mbyte | read-only |  |
|  | SDP-15 |  | 15 Mbyte | read-only |  |
|  | SDP-20 |  | 20 Mbyte | read-only |  |
|  | SDPL5-2.5 |  | 2.5 Mbyte | read-only |  |
|  | SDPL5-5 |  | 5 Mbyte | read-only |  |
|  | SDPL5-10 |  | 10 Mbyte | read-only |  |
| TI | CMP68F1MBN | Flash | 1 Mbyte | no | P68F1MBN |
| Toshiba | [unmarked] | Flash | 2 Mbyte | yes | TOS2MBFA |

### 70	Specifying Generic Card Information

If you don't know the part number of your card, or if the card is not specifically listed in Table 8-1, you can specify the card type and size using the -type and -size options on a utility’s command line. These two items provide the minimum information required to properly program a memory card. (Be sure that your card is a RAM or Flash card that complies with PCMCIA/JEIDA standards — see References 1 and 2.)

All TC utilities except TCINIT.EXE support the -type and -size options in the following command line format:

TCcommand -type card-type -size card-size

For example, to erase a 512-kbyte RAM card without specifying the card's manufacturer and part number, use the following command:

tcerase -type RAM -size 0x80000 drive:

See the description of the command of interest in Section 12.3 for supported card types and sizes.

## 71	Card Configuration Files

### 72	General Information

The physical description data for all cards listed in Table 8-1 are collected into a single large card configuration file named CARDINFO.DBK, located in the CARDTALK directory. If you want to create a physical description for a card that is not in the table, or if you should need to change any information for a card that is in the table, do not change CARDINFO.DBK; instead, create a new stand-alone card configuration file (using CARDINFO.DBK as a model), and give it the standard configuration file name extension .cfg. The TC utilities will first search for an individual card configuration file with the standard name extension and, only if such a file is not found, will then search in \CARDTALK\CARDINFO.DBK.

You can create or modify a card configuration file with any standard text editor. You need to create a different card configuration file for each unique card type to be initialized. Initializing a RAM card with CIS information intended for a programmable card could result in the RAM card being damaged by excessive voltages during the programming attempt.

The TCINIT.EXE utility uses the information in the card configuration file to place a CIS on the card. The command that invokes TCINIT.EXE, tcinit, has an option by which to specify the card configuration file to be used (see the -config option description in Subsection 12.3.3.2).

#### 73	Specification Line Format and Notational Conventions

A card configuration file has a relatively simple format; it is composed of card configuration specification lines, each of which consists of a KEYWORD field followed by one or more VALUE fields. A card configuration file need have only enough specification lines (keywords and associated values) to support the intended memory card application. The KEYWORD field consists of one or more words ending with a colon; multiple words are connected by hyphens:

KEYWORD:

MULTI-WORD-KEYWORD:

The VALUE field which may be a number or text string. Where there is not a pre-defined value or choice of values, the field is shown in Subsection 8.3.2 as a place-holder (see Section 1.3) for that number or string. If a value is required to be entered (the specification is incomplete without it), it is enclosed in angle brackets (<>). If entry of a value is optional, it is enclosed in square brackets ([ ]); in such a case, a default value exists which is used if no value is entered, or the card will be queried by the utility being run in an attempt to add any needed information to the CIS data. (The angle or square brackets themselves are not to be entered.)

In some cases, a value is to be selected from a limited defined group of mutually exclusive strings or numbers; the defined choices for such a value are shown as a list of literal strings or numbers (see Section 1.3) separated by vertical bars (|). (The entire list is enclosed in angle brackets or square brackets, depending on whether selection of a value is required or optional.)

For example, the specification line:

CARDSIZE: <number[K|M]>

(described in Subsection 8.3.2.1) indicates that the keyword CARDSIZE: must be followed by a numeric value, which may (but need not) be followed by one of the two letters K or M. (If neither letter is entered, a default value is assumed.)

To conform to sample files provided, keywords and literal values are all given in upper case letters in this chapter; the line syntax, however, is not case-sensitive.

#### 74	Keyword Groups

Many of the card configuration specification keywords are organized in groups. A group starts with a special group-start-keyword:

GROUP-START-KEYWORD:

GROUP-KEYWORD1: value

GROUP-KEYWORD2: value

There are two nested levels of keyword groups, as shown in Figure 8-1. Any second- or third-level keyword specification line (keyword and any associated values) must be preceded in a card configuration file by its group start keyword. All keywords encountered after any group start keyword are assumed to belong to the group defined by that start keyword, until the next group start keyword is encountered.

Top-level keywords may be in any order; also, the keywords in any group may be in any order.

#### 75	Delimiters

All white-space characters (space, tab, carriage return, line feed) are treated as keyword and value delimiters. The colon character that always ends a keyword also acts as a delimiter. This means you don't have to add a white space character to separate a keyword from its value.

Figure 8-1	Card Configuration Specification Keyword Group Organization

| Top Level ¯ | 2nd Level ¯ |  | 3rd Level ¯ |
| --- | --- | --- | --- |
| PHYSICAL LAYOUT GROUP START keyword |  |  |  |
|  | SPACE GROUP START keyword |  |  |
|  |  | space group keywords |  |
|  | other physical layout group keywords |  |  |
| PARTITION GROUP START keyword |  |  |  |
|  | GEOMETRY GROUP START keyword |  |  |
|  |  | geometry group keywords |  |
|  | other partition group keywords |  |  |
| BATTERY GROUP START keyword |  |  |  |
|  | battery group keywords |  |  |
| other top-level keywords |  |  |  |

You also can have multiple keywords and values on any line in the file. For clarity, all the sample card configuration files contain only one keyword per line.

#### 76	Comment Lines

To add a comment line, begin the line with a pound sign (#) in column 1. For example, each of the following five lines is treated as a comment:

#################################################################

#

# Sample Configuration for Intel 1-Mbyte flash cards:

#

#################################################################

Comments can be useful to add additional descriptive information to the card configuration file, such as the card type the file is intended for.

#### 77	Multiple-Word Strings

The ECHO keyword (see Subsection 8.3.2.1) is the only keyword that requires a string value that might consist of multiple words separated by spaces or tabs (the name of a card configuration file). If that name is a single word (no embedded spaces or tabs), that word alone suffices:

ECHO: card.config.file.name

If the name consists of multiple words separated by spaces or tabs, enclose the entire string in double quotes, to avoid having the spaces or tabs mistaken for delimiters (see Subsection 8.3.1.3):

ECHO: "card.config file name"

(In such a case, the double quotes act as the string delimiters.) If the name itself contains one or more double quote characters, precede each double quote character in the name with a backslash (\) character, to avoid having it mistaken for a string delimiter:

ECHO: "Special \"private\" file"

(In this last example, the name of the card configuration file is SPECIAL "PRIVATE" FILE.)

### 78	Specification Lines (Keywords and Values)

This subsection presents all allowed keywords, and corresponding values for each keyword. It is organized by keyword group; within each group, the keywords are presented alphabetically. See Subsection 8.3.3 for sample card configuration files for some typical applications.

#### 79	Top-Level Keywords

BATTERY:

This keyword indicates the start of a battery group. All keywords encountered after this keyword will be assumed to belong to the battery group until the next top-level keyword is encountered. See Subsection 8.3.2.6 for all keywords in the battery group.

CARD-DESCRIPTION: [string]

This keyword may be used to describe the intended end use of the card.

CARDSIZE: <number[K|M]>

This keyword specifies the size of the entire physical card. number must be entered in decimal notation. K and M indicate that the size is in kilobytes or megabytes respectively. If neither K nor M is entered, the size is assumed to be in bytes.

CARDTYPE: <FLASH|RAM>

This keyword specifies the type or name of the card.

CIS: <NONE|BASIC|FULL>

This keyword is used to specify the level of CIS data to be recorded on the card:

NONE specifies that no CIS information is written to the card. Use this option when preparing cards for systems that don't support CIS such as the Poqet PC.

BASIC specifies that only Level 1 CIS data is written to the card. These data describe the physical properties of the card but do not describe the data recording format.

FULL specifies that all known recording format information as well as the card's physical properties are included in the CIS information written to the card.

CIS-CHECKSUM: <YES|NO>

YES specifies that error detection is turned on, and any inadvertent changes to the CIS data will be detected. Use this option to insure that the CIS data interpreted by the TC commands are valid.

NO specifies no error detection.

We recommend that the YES value be used with this keyword, since accidental modification of the CIS data could result in damage to the card.

CIS-RESERVE: <number>

This keyword is used to specify the number of 512-byte blocks of card space that should be reserved for the CIS information. number must be entered in decimal notation. In most cases number should be 1. A value other than 1 allows space to be reserved for future expansion of the CIS data.

ECHO: [string]

Use this keyword if you need to know whether the contents of a particular card configuration file have been read in. The entire string will be echoed to STDERR. If no string is present, a blank line is echoed.

OEM-NAME: [string]

This keyword specifies the name of the manufacturer of the card. (Not recommended for most user applications.)

OEM-PART-NUMBER: [string]

This keyword specifies the manufacturer’s part number for the card. (Not recommended for most user applications.)

PARTITION:

This keyword indicates the start of a partition group. All keywords encountered after this keyword will be assumed to belong to the partition group until the next top-level keyword is encountered. See Subsection 8.3.2.4 for all keywords in the partition group.

PARTITIONS: <number>

This keyword specifies the number of separate partitions to be created on the card. number must be entered in decimal notation. The PCMCIA standard (see Reference 1) allows a card to be created that contains multiple partitions that support the same or differing file systems (see Sections 2.3.1 and 10.4), or one execute-in-place (XIP) partition.

PHYSLAYOUT:

This keyword indicates the start of a physical layout group. All keywords encountered after this keyword will be assumed to belong to the physical layout group until the next top-level keyword is encountered. See Subsection 8.3.2.2 for all keywords in the physical layout group.

TARGET-MACHINE: <HP95LX|POQET>

HP95LX specifies that the card being programmed is to be used with the HP-95LX, so that CIS data can be handled properly.

POQET specifies that the card being programmed is to be used with the Poqet PC, so that the special voltage requirements for the Poqet PC can be verified.

This keyword may be used only if the card is to be used with a HP-95LX or Poqet PC.

#### 80	Physical Layout Group

ATTRIBUTE-SPACE: or COMMON-SPACE:

These keywords specify the start of a space group. ATTRIBUTE-SPACE and COMMON-SPACE specify that the following space group applies to attribute space or common space respectively. All keywords encountered after this keyword will be assumed to belong to the space group until the next physical layout group or top-level keyword is encountered. See Subsection 8.3.2.3 for all keywords in the space group.

JEDEC-ID: <number1> <number2>

This keyword allows the specification of the JEDEC manufacturer and device IDs, respectively. number1 and number2 are two-digit values in hexadecimal notation.

REG-SPACE: <COMPLIES|ALIASED|NONE>

This keyword indicates how the card complies with the PCMCIA standard (see Reference 1) regarding the support of attribute space (see Subsection 8.1.2):

COMPLIES indicates that the card contains true attribute space that is separate and distinct from common space.

ALIASED indicates that the card maps attribute space addresses onto common space. (Intel Flash memory cards are aliased).

NONE indicates that the card has a read-only attribute space that is permanently empty. (This is usually indicated in the card’s data sheet by text similar to: “REG space = FFh”.)

WRITE-PROTECT: IGNORE

This keyword indicates that the software should ignore the fact that the card is write-protected, and should write to the card. This keyword is not usually useful with modern cards.

#### 81	Space Group

DEVICE-COUNT: <number>

This keyword is used to indicate the number of chips that comprise this physical section; number is in decimal notation.

DEVICE-SIZE: <number>

This keyword is used to indicate the size of the chips in this physical section of the card; number is in decimal notation, and may use a “k” or “M” suffix as a multiplier. (As used here, k º210  = 1024, and M º 220  = 1048576 ).

DEVICE-TYPE: <NULL|SRAM|DRAM|IO|EEPROM|FLASH>

This keyword indicates the chip type in this physical section of the card.

SPEED: <number>

This keyword is used to specify the access speed of the chips in this section of the card; number is entered in nanoseconds, in decimal notation.

BIT-WIDTH: <number>

This keyword indicates the bit width that should be used to access the card. This value is entered in decimal format and, in most cases, should be 16.

#### 82	Partition Group

BASE-ADDRESS: <number>

This keyword specifies the starting card address of the partition. If you supply the card size via the keyword CARDSIZE: (see Subsection 8.3.2.1), you don't need to specify the base address of the partition; the only partition group keyword that needs to be present is SIZE (see its description later in this subsection).

ERROR-DETECT: <NONE|CHECKSUM|CRC|PARTITION> <INLINE|TABLE> [SIZE:[0|1|2]]

NONE/CHECKSUM/CRC/PARTITION specify the type of error detection to be used with the partition. If a DOS file system is to be used, any one of these four values may be specified.

NONE specifies that no error detection should be used.

CHECKSUM specifies a one-byte checksum for each block in the partition.

CRC specifies a two-byte SDLC error detection for each block in the partition.

PARTITION specifies a one-byte checksum for the entire partition.

CHECKSUM executes faster than CRC, but CRC provides a much more reliable error detection algorithm. PARTITION should only be used when you don't expect to write (or have only limited writes) to the partition.

INLINE/TABLE specify where the error detection data is to be stored. No entry is required here if no error detection is used (if the value NONE is entered for the first parameter).

INLINE records the data immediately following a block.

TABLE causes a table of data to be created at the end of the partition. Do not specify this value if error detection is set to NONE.

If access speed is a concern, specify TABLE for the data location; this allows the partition data blocks to remain aligned with 16-byte address boundaries for optimal data access.

SIZE specifies the size of the error value to be calculated. If entered, SIZE should match the data byte implied by the first value: 0 for NONE, 1 for CHECKSUM or PARTITION, 2 for CRC. SIZE may be used with NONE to reserve room for error detection data without enabling the error detection code.

Note 1:

For Flash cards, set error detection to NONE and do not specify location (INLINE or TABLE) or SIZE.

Note 2:

If you intend to interchange a card between Databook’s CardTalk driver and a driver included with the Microsoft FFS2 OAK, set error detection to NONE for any partition that you plan to format with the DOS File Allocation Table (FAT — see Subsection 10.2.1). The FFS2 drivers do not write correctly to a DOS FAT partition that uses any type of error detection; data will be corrupted, rendering the partition unreadable by the CardTalk driver.

FILE-SYSTEM: <DOS|FFS2|FFS1>

This keyword indicates the type of file system that the card will contain. If this keyword is specified, the value entered for the ORGANIZATION keyword (see its description later in this subsection) should be DISK.

DOS should be entered if you intend to use TCFORMAT.EXE to format a RAM card, or TCXCOPY.EXE to format a Flash card with the DOS File Allocation Table (see Subsection 10.2.1).

FFS2 should be entered if you intend to use TCFORMAT.EXE to format a Flash card with the Microsoft Flash File Format version 2.0 (see Subsection 10.2.2).

FFS1 should be entered only if you intend to use TCFORMAT.EXE to format a Flash card in a system in which the Microsoft Flash File Format version 1.0 is installed, and you do not intend to replace it with version 2.0. This is not recommended (see Subsection 2.2.1).

GEOMETRY:

This keyword indicates the start of a geometry group. If this keyword is specified, the value entered for the ORGANIZATION keyword (see its description immediately following) should be DISK, and the value entered for the FILE-SYSTEM keyword (see its description immediately preceding) should be DOS. All keywords encountered after this keyword will be assumed to belong to the geometry group until the next partition group or top-level keyword is encountered. See Subsection 8.3.2.5 for all keywords in the geometry group.

ORGANIZATION: <DISK|MEMORY|APPLICATION>

This keyword indicates the basic type of recording format to be used. If the card is to be used with either the CARDTALK.SYS or MS-FLASH.SYS driver, the value entered should be DISK.

SIZE: <number[K|M]>

This keyword is used to specify the size of the partition. The number must be entered in decimal notation. K and M indicate that the size is in kilobytes or megabytes respectively. If neither K nor M is entered, the size is assumed to be in bytes.

#### 83	Geometry Group

CYLINDERS: <number>

This keyword specifies the number of simulated cylinders for this partition. number should be between 1 and 65,535, in decimal notation.

Note:

The value of number for the CYLINDERS: keyword is 1-based. Therefore, it should always exceed the corresponding boot parameter block (BPB) value (which is 0-based) by one.

SECTORS: <number>

This value specifies the number of simulated sectors per track for this partition. number should be a number between 1 and 255, in decimal notation.

TRACKS: <number>

This keyword specifies the number of simulated tracks per simulated cylinder. number should be a number between 1 and 255, in decimal notation.

#### 84	Battery Group

INSTALLED: <mm-dd-yy>

This keyword is used to indicate the installation date of the card battery. The format of the date is two digits specifying the month, followed by a hyphen and two digits specifying the date, followed by another hyphen and two digits specifying the year.

REPLACE-BY: <mm-dd-yy>

This keyword is used to indicate the date by which the card battery should be replaced. The format of the date is two digits specifying the month, followed by a hyphen and two digits specifying the date, followed by another hyphen and two digits specifying the year.

### 85	Sample Card Configuration Files

For clarity, the sample card configuration files given here are indented to show keyword groups and levels. Indentation is not necessary in the actual files.

The following card configuration file can be used to prepare an Intel 1-Mbyte Flash card to have the DOS File Allocation System (FAT). Since this is one of the supported cards (see Table 8-1), you can use the -card option to specify the card's physical attributes. You need to specify only the logical layout of the card in its configuration file:

# Format 1 partition for the card.

# Use DOS FAT for file system.

# No error detection since it's a Flash card.

PARTITIONS: 1

PARTITION:

FILE-SYSTEM: DOS

ERROR-DETECTION: NONE

# Record this info in the CIS.

CIS: FULL

CIS-CHECKSUM: YES

CIS-RESERVE: 1

To put a CIS based on this file (in this example the file name is test1.cfg) on the Intel 1-Mbyte Flash card in drive E, type:

tcinit -config test1.cfg -card intel/imc001 e:

The following card configuration file can be used to format a 512-kbyte RAM card that does not have attribute memory space and is not currently supported (see Table 8-1), so that it does not have the -card option available:

#-------------------------------------------------------

# This section specifies the card's physical properties.

CARDTYPE: RAM

CARDSIZE: 512K

# This card has no attribute memory.

PHYSLAYOUT:

REG-SPACE: NONE

COMMON-SPACE:

DEVICE-TYPE: SRAM

#-------------------------------------------------------

# Format 1 partition for the card.

# Use DOS FAT for file system.

# Use CRC error detection and store it in a table.

PARTITIONS: 1

PARTITION:

FILE-SYSTEM: DOS

ERROR-DETECTION: CRC TABLE

# Record this info in the CIS.

CIS: FULL

CIS-CHECKSUM: YES

CIS-RESERVE: 1

To put a CIS based on this file (in this example the file name is test2.cfg) onto the subject card, in drive E, type:

tcinit -config test2.cfg e:

If you are interested in preparing a multi-partition card, read Section 10.4. If you are interested in preparing a multi-partition card for booting, also read Section 10.5.

# 86	Erasing a Flash Memory Card

A Flash memory card that has a Flash FAT file system (formatted using TCXCOPY.EXE — see Subsection 10.2.1) must always be erased before being rewritten and before being formatted. (This is not true of RAM cards.) The TCERASE.EXE utility (see Subsection 12.3.1) is provided for that purpose. The command that invokes that utility, tcerase, has an optional argument, -all, that selects between erasing the whole card and erasing an individual partition. To erase the whole card, use

tcerase -all drive:

To erase an individual partition, use:

tcerase drive:

and (in either case) any other appropriate options (see Subsection 12.3.1.2).

Individual files cannot be deleted from a Flash memory card that has a Flash FAT system. To effectively delete individual files from such a card, the files to be retained should be copied, after which the card should be erased and formatted. The files to be retained may then be copied back onto the card.

Individual files can be deleted (using the DOS del command) from a card that has FFS2 (Flash File System version 2.0, formatted using TCFORMAT.EXE — see Subsection 10.2.2), without erasing or formatting the card first. The memory that the deleted files occupied will be reclaimed.

# 87	Formatting a Memory Card

A memory card is a storage device functionally identical to a floppy disk. Once a card is properly formatted, you can access it directly using standard DOS commands. Different memory card technologies require different formatting methods. The following sections describe how to format the specific card types supported by CardTalk.

## 88	Formatting a RAM Card

Like a diskette, a RAM card must be initialized before use. The TCFORMAT.EXE utility is provided for this purpose. Use TCFORMAT.EXE whenever you need to format a new RAM card or re-format one. To use TCFORMAT.EXE in its simplest form, type:

tcformat drive:

Substitute the letter of the memory card drive on your system for drive. Used in this way, tcformat is much like the ordinary DOS format command. It prompts you to insert a card and strike <enter>.

The format process itself is very brief. TCFORMAT.EXE simply determines the size of the card, then writes the appropriate information to the card.

Once a RAM card is formatted, you can use it as you would a floppy disk. You can use all the ordinary DOS commands and applications to read and write data to the card.

## 89	Formatting a Flash Card

The TC utilities are not needed for formatting solid-state memory cards that use SunDisk Flash technology; these cards are operable on directly by DOS commands. DOS commands cannot be used to format all other Flash cards, however, and the TC utilities must be used. The TC utilities software provides two ways to format a Flash card: with the Flash File Allocation Table (FAT) or with the Microsoft Flash File System (FFS2 — see Section 2.2). The choice depends on the system you intend to use with the Flash card.

If you intend to use your Flash card with a hand-held computer, you need to find out whether your machine supports FFS2. Many of the currently available systems (including the HP-95LX and the Poqet PC) do not support FFS2.

### 90	Formatting with the Flash File Allocation Table

To use Flash cards with a DOS system that does not support FFS2, you need to format the card with a Flash File Allocation Table (FAT) so that your system can read it. The TCXCOPY.EXE utility (see Subsection 12.3.6) is provided for this purpose.

TCXCOPY.EXE provides the following functions:

•	It automatically initializes blank cards in a DOS-compatible format.

•	It can copy files from any source onto a Flash card.

•	It can automatically create subdirectories while copying.

•	It can copy a large set of files into a subdirectory on the Flash card.

•	It can append additional files to a Flash card that has previously been prepared using TCXCOPY.EXE.

TCXCOPY.EXE is invoked with the tcxcopy command, which supports most of the options provided by the DOS XCOPY command, with a similar syntax.

Before using TCXCOPY.EXE, the card must be blank or freshly erased (see Chapter 9). To erase a Flash card, insert the card in the system’s drive and enter the following command:

tcerase -all drive:

It takes roughly one minute to erase a 2-Mbyte Flash card. When TCERASE.EXE is finished, you are ready for TCXCOPY.EXE.

To use TCXCOPY.EXE in its simplest configuration, do the following:

1.	Set up a directory or subdirectory on your hard drive that contains the files you wish to copy to the Flash card.

2.	Put a blank (or erased) Flash card in the system’s drive.

3.	Enter the following command:

tcxcopy path\*.* drive:

TCXCOPY.EXE will copy all files in the specified directory to the root directory of the specified drive. (drive indicates the target system memory card drive.)

4.	To check that the copy was successful, enter:

dir drive:

TCXCOPY.EXE uses an unusual method for putting files on the card. You will notice that the DOS dir command indicates the card has “0 bytes free” even when space remains. This is OK.

After a Flash card has been prepared with TCXCOPY.EXE, it is completely compatible with DOS for read-only operations. This means that most hand-held computers can read a Flash card prepared with TCXCOPY.EXE.

### 91	Formatting with the Microsoft Flash File System

The Microsoft Flash File System version 2.0 (FFS2) is a special-purpose formatting scheme that organizes the data on a Flash card in a novel way, allowing you to use a Flash card in much the same way as a RAM card. Any application can write data to a Flash card prepared with FFS2 just as easily as it can write to a diskette.

Unfortunately, very few PCMCIA/JEIDA-compatible systems support FFS2. In particular, you cannot read FFS2 cards on either a Poqet PC or an HP-95LX.

If your system supports FFS2, you can initialize a Flash card for use with FFS2. Be sure MS-FLASH.SYS is installed (see Section 2.2).

To format a Flash card with FFS2, use TCFORMAT.EXE. Enter the following command (see Subsection 12.3.2):

tcformat -type flash drive:

After the card has been formatted as above, you can use standard DOS commands on it if you've installed MS-FLASH.SYS.

## 92	Preparing a Single-Partition Memory Card for Booting

If your system contains a Databook TCIC-2/N BIOS (Basic Input/Output System) Extension, this BIOS Extension checks for the presence of a memory card in socket 0 (but not in socket 1) during the boot process. If socket 0 contains a memory card, and if that card is formatted to be bootable, the system boots from it. (Otherwise, the system boots from your hard disk or floppy disk as normal.) This section describes how to format a memory card to be bootable.

### 93	Preparing a Single-Partition RAM Card for Booting

To prepare a RAM card to be bootable, first make sure the card is blank or freshly erased. From your system’s normal boot drive, enter the tcformat command with the -sys option (see Subsection 12.3.2.2):

tcformat -sys card_drive_letter:

where card_drive_letter is the letter of the drive in which your card is installed. This command formats the card to use DOS File Allocation Table (FAT), and the -sys option copies COMMAND.COM and the two hidden system files from your system to the card in one step. This is very similar to the DOS format /s command.

Next, prepare a special directory  (in this example, c:\test) containing the files you want to put on the card. You will need at least the following two files:

1.	a special CONFIG.SYS, with the following specification line (in addition to any other required lines:

device = cardtalk.sys

Do not specify an explicit drive letter.

2.	CARDTALK.SYS

Next, copy these files to the card, using the following DOS command:

xcopy c:\test card_drive_letter:\ /s

The system will run these files from the RAM card during the boot process.

Leave the RAM card in the drive and reboot. You should see the following message:

Databook Card Talk Bootstrap Manager V2.20 [release date]

Copyright (C) Databook Incorporated 1990-93.

Card is present.

The system will boot from the RAM card at this point.

If you boot with a RAM card that does not contain a DOS file system (i.e., you have not used the tcformat command),  the system displays the following message:

Databook CardTalk Bootstrap Manager V2.20 [release date]

Copyright (C) Databook Incorporated 1990-93.

Card is present.

No valid BIOS Parameter Block

If you boot with a RAM card that contains a DOS file system but does not have the required boot block (i.e., you have used the tcformat command but without the -sys option), the system displays the following message:

Databook CardTalk Bootstrap Manager V2.20 [release date]

Copyright (C) Databook Incorporated 1990-93.

Card is present.

Non-System disk or disk error

Replace and press any key when ready.

### 94	Preparing a Single-Partition Flash Card for Booting

To prepare a Flash card for booting, first make sure the card is blank or freshly erased. To boot from a Flash card, you need to format the card to use Flash File Allocation Table (FAT) File System instead of Flash File System (FFS2). From your system’s normal boot drive, enter the tcformat command with the -sys and -type flash options (see Subsection 12.3.2.2):

tcformat -sys -type flash card_drive_letter:

where card_drive_letter is the letter of the drive in which your card is installed. The -sys option formats the Flash card to use Flash FAT, and copies COMMAND.COM and the two hidden DOS system files to the card in one step.

Next, prepare a special directory (in this example, c:\test) containing the files you want to put on the FAT partition. You will need at least the following two files:

1.	a special CONFIG.SYS, with the following specification line (in addition to any other required lines:

device = cardtalk.sys

Do not specify an explicit drive letter.

2.	CARDTALK.SYS

Next, copy these files to the card. Because the DOS FAT file system is not directly compatible with Flash technology, you must use TCXCOPY.EXE (see Subsection 10.2.1). The following commands will create the FAT partition on the card and copy the files in the special directory to it:

cd c:\test

tcxcopy -type flash *.* card_drive_letter:/s

The system will run these files from the RAM card during the boot process. When booting from this card, you will see the same messages shown in Subsection 10.3.1 for a RAM card.

You can also boot from a multi-partition Flash card that has at least one FAT partition (and may also have one or more FFS partitions). To prepare a multi-partition Flash card for booting, see Section 10.5.

## 95	Formatting a Multi-Partition Card

This section provides instructions for creating a memory card with more than one logical partition. This section assumes that you have read Subsection 2.3.1 and Sections 8.1 and 8.3. for background information on installing the CardTalk driver, CIS, and card configuration file format respectively.

A memory card can be formatted to have more than one logical partition, just as you format your hard disk to have more than one logical drive. Each partition on the memory card is treated as a separate DOS drive. For example, if you have a RAM card in drive D of your system, and you format the card to have three DOS partitions, the first partition will be accessed as drive D, the second as drive E, and the third as drive F.

If you have a Flash card, it may be useful to set up two partitions on the card: a DOS partition using Flash FAT for the file system (see Subsection 10.2.1), and a Flash partition using FFS2 (see Section 10.2.2). You can easily copy files between the two systems using the DOS copy command.

Note that two TC commands,  tcinit and tcread, work on the entire physical card as opposed to individual partitions (see Subsections 12.3.3 and 12.3.5 respectively). Also, the tcerase command, which by default operates on a single partition of a multi-partition card, has an option, -all, which causes it to operate on the entire physical card (see Subsection 12.3.1).

To format a multi-partition card, first make sure that the CardTalk driver has been installed to run in multi-partition mode (see Subsection 2.3.1.2). Next, prepare a card configuration file that contains information about the number of partitions on the card (see Subsection 8.3.2.1) and the size and format of each partition (see Subsection 8.3.2.4).

Use a new card or one freshly-erased with tcerase. Use tcinit to put the CIS data based on the above card configuration file on the card (see Subsection 8.1.1).

Next, use tcformat or tcxcopy to format each partition with the file system specified for it by the CIS on the card (see Sections 10.1 and 10.2). The following two subsections present examples to illustrate these procedures. When these procedures are completed, the card is ready for use with DOS.

### 96	Formatting a Two-Partition RAM Card

This subsection presents an example of how to prepare a multi-partition card, using the case of a two-partition RAM card to be inserted in socket 0.

First, install CARDTALK.SYS with the /N:4 switch in your CONFIG.SYS file; this assigns two drives to socket 0 (see Table 2-1). For this example, assume that, after installation, CARDTALK.SYS notifies you that drives E, F, G, and H are now installed on your system.

Assuming you are working with a 512-kbyte RAM card, prepare a card configuration file that has information similar to the following file:

# Physical characteristics

CARDTYPE: RAM

CARDSIZE: 512K

# This information should reflect the actual card

PHYSLAYOUT:

REG-SPACE: NONE

COMMON-SPACE:

DEVICE-TYPE: SRAM

# Must store CIS on card.

CIS: FULL

CIS-RESERVE: 1

CIS-CHECKSUM: YES

# 2 partitions on card

PARTITIONS:2

# 1st partition has 200K

PARTITION:

SIZE: 200K

FILE-SYSTEM: DOS

ERROR-DETECT: CRC INLINE

# 2nd partition has remainder of card.

PARTITION:

FILE-SYSTEM: DOS

ERROR-DETECT: CHECKSUM TABLE

The above file specifies two logical partitions for the RAM card. The first partition is 200 kbytes, has a DOS FAT system, and uses CRC error detection stored at the end of each sector. The second partition is 312 kbytes (since no size is specified, TCINIT.EXE assumes the rest of the card), has a DOS FAT file system, and uses the CHECKSUM error detection method stored in a table at the end of the partition.

Assuming that this card configuration file has the name ram.cfg, run TCINIT.EXE with the -config option to specify the above file. Insert the card in socket 0 (logical drives E and F), and enter:

tcinit -config ram.cfg e:

TCINIT.EXE records the above CIS in the first sector of the card, so that subsequent card commands can examine CIS for proper operations on the card. Note that the drive specified with tcinit must be the first drive identified by the CARDTALK.SYS driver. TCINIT.EXE places CIS data that apply to the entire card, so you only run tcinit once.

Finally, run TCFORMAT.EXE on each partition of the card to put the appropriate file system information in each partition. TCFORMAT.EXE operates on the individual partitions, so you need to run it once for each logical drive (the card is in socket 0):

tcformat E:

tcformat F:

TCFORMAT.EXE at this point can obtain complete card and partition information from the CIS on the card to format each partition properly.

You are now ready to use standard DOS commands and applications for reading and writing to the two-partition RAM card.

You can only use this card on another system if it supports CIS and multi-partition mode according to the PCMCIA/JEIDA standard (see Reference 10). Popular hand-held computers such as the Poqet PC and the HP-95LX currently do not.

Note:

In the preceding example, if you install CARDTALK.SYS to handle two partitions but prepare a card with only one partition, an attempt to access the second drive (drive F) will result in an “Abort, Retry, Fail” error.

### 97	Formatting a Two-Partition Flash Card

This subsection presents an example of how to prepare a multi-partition card, using the case of a two-partition Flash card of which the first partition has Flash FAT and the second has FFS2 the system, to be inserted in socket 0.

First, install CARDTALK.SYS with the /N:3 switch in your CONFIG.SYS file. For this example, assume that after installation, CARDTALK.SYS notifies you drives E, F, and G are now installed on your system.

Since you will be also reading and writing to the FFS2 partition on the Flash card, be sure MS-FLASH.SYS is installed (see Section 2.2).

Assuming you are working with an Intel 1-Mbyte Flash card, prepare a card configuration file that has information similar to the following file:

# Must store full CIS on card

CIS: FULL

CIS-RESERVE: 1

CIS-CHECKSUM: YES

# 2 partitions

PARTITIONS:2

# 1st partition is FAT

PARTITION:

SIZE: 261632

FILE-SYSTEM: DOS

ERROR-DETECT: NONE

# 2nd partition is FFS2

PARTITION:

FILE-SYSTEM: FFS2

ERROR-DETECT: NONE

This card configuration file specifies a CIS (which occupies the first 512 bytes), and two logical partitions for the Flash card.

The first partition is 261632 bytes, or 255.5 kbytes; this size is specified to make the remainder of the card (1 Mbyte - 255.5 kbytes - 0.5 kbyte for the CIS = 768 kbytes) an integral multiple of 256 kbytes (this satisfies the FFS2 partition size constraints for the card being used — see Table 10-1). The first partition has Flash FAT format, and has no error detection.

Because no size is specified for the second partition, TCINIT.EXE will use the rest of the card; the second partition is therefore 768 kbytes. It has FFS2 format, and also has no error detection.

The reason both partitions have no error detection is that all error detection schemes require the ability to update the EDC “on the fly”, and a Flash card is not capable of random writes (see Note 1 in the ERROR-DETECT keyword description in Subsection 8.3.2.4).

Note that you didn't need to specify any information regarding the physical properties of the card. Since the Intel 1-Mbyte Flash card is one of the supported cards listed in Table 8-1, you can simply specify the manufacturer and part number via the -card option (see Subsection 8.2.1) of the tcinit command as follows (for this example, the name of the above card configuration file is flash.cfg): Insert the Flash card in socket 0 (logical drives E and F), and enter:

tcinit -config flash.cfg -card intel/imc001 e:

TCINIT.EXE records the CIS based on flash.cfg in the first sector of the card, so that subsequent card commands can examine the CIS for proper operations on the card. Note that the drive specified with the tcinit command must be the first drive identified by the CARDTALK.SYS driver.

After placing CIS on the card, you must format the card to set up the appropriate file systems.

To put Flash FAT on the first partition, you must use TCXCOPY.EXE (see Subsection 12.3.6), which simultaneously formats the partition and copies files to it. To format and copy all .dat files to the first partition:

tcxcopy *.dat E:

You don't need to specify additional information about the card, since TCXCOPY.EXE can obtain information from the CIS on the card.

Run TCFORMAT.EXE on the second partition to put FFS2 on it:

tcformat F:

Again, you don't need to specify more information, since TCFORMAT.EXE can obtain complete card and partition information from the CIS on the card.

Assuming that you have installed MS-FLASH.SYS (see Subsection 2.2), you can now use standard DOS commands to read and write to the FFS2 partition (drive F). To copy files from the Flash FAT partition to the FFS2 partition:

copy E:*.dat F:

You can use standard DOS commands to read from drive E; to copy more files to this partition, you must use TCXCOPY.EXE.

### 98	FFS Partition Size Constraints

#### 99	FFS2 Partitions

FFS2 accesses a Flash card in terms of blocks. The block sizes of a particular card affect the allowable sizes of partitions. The usual partition size constraints are given in Table 10-1.

Table 10-1	Usual FFS2 Partition Size Constraints

| Flash Card Type | Minimum Size | Size Increment |
| --- | --- | --- |
| Series 2 | 256 kbytes | 128 kbytes |
| Series 1, < 2 Mbytes | 512 kbytes | 256 kbytes |
| Series 1, ³ 2 Mbytes | 1 Mbyte | 512 kbytes |

#### 100	FFS1 Partitions

If you are using FFS1 (not recommended — see Section 2.2), partitions should not be greater than 16 Mbytes, due to limitations in the FFS1 drivers. The reported size of a partition larger than 16 Mbytes will be 16 Mbytes less than the actual size. (This is not related to the Databook software.)

## 101	Preparing a Multi-Partition Flash Card for Booting

This section presents an example of how to format a bootable Flash card prepared with FFS2. Because DOS cannot boot directly from an FFS2 partition, the card must have two partitions; the first must be a FAT partition, which must contain the files required to boot the system.

Assuming you are working with an Intel 1-Mbyte Flash card, prepare the card configuration file described in Subsection 10.4.2. (Note that it divides the card into a small FAT partition followed by a FFS2 partition for the rest of the card.) Next, record the CIS based on that card configuration file on the card, using the tcinit command, just as shown in Subsection 10.4.2. With the same assumptions as in that example (the card is inserted in socket 0, the first corresponding drive letter is E), the command is:

tcinit -config flash.cfg -card intel/imc001 e:

Next, prepare a special directory (in this example, c:\test) containing the files you want to put on the FAT partition. You will need at least the following files:

1.	a special CONFIG.SYS, with the following specification line (in addition to any other required lines:

device = cardtalk.sys /n:4        [see Table 2-1]

device = dblflash.exe             [if desired — see item 3 of this list]

device = ms-flash.sys

Do not specify an explicit drive letter.

2.	CARDTALK.SYS

3.	DBLFLASH.EXE, the FFS2 compression utility (if desired)

4.	MS-FLASH.SYS, the FFS2 driver

Next, copy these files to the card. Because the DOS FAT file system is not directly compatible with Flash technology, you must use TCXCOPY.EXE (see Subsection 10.2.1). The following commands (still assuming drive E) will create the FAT partition on the card and copy the files in the special directory to it:

cd c:\test

tcxcopy -sys *.* e:

Finally, to put FFS2 on the second partition, run TCFORMAT.EXE (as described in Subsection 10.4.2):

tcformat F:

# 102	Reading and Writing a Memory Card

Two utilities, TCREAD.EXE and TCPROG.EXE, are provided to copy a binary image from a memory card to a file and from a file to a memory card. These functions are most useful when creating duplicate copies of cards.

The TCREAD.EXE utility copies from a memory card to a file; it is invoked with the tcread command (see Subsection 12.3.5). It allows the user to specify the start address and the amount of data to copy from the card. Attribute memory is selected by setting the most significant address bit; for example, to select attribute memory address 0, specify 80000000h.

The TCPROG.EXE utility copies the binary data from a file and places it on a memory card; it is invoked with the tcprog command (see Subsection 12.3.4). TCPROG.EXE always starts programming at address 0h and extends the length of the binary file. Attribute memory cannot be programmed using TCPROG.EXE.

# 103	TC Utility Commands — Detailed Description

Chapters 8 through 11 refer extensively to six commands used to invoke Databook’s utility files with which to prepare and use memory cards: tcinit, tcformat, tcxcopy, tcerase, tcprog, and tcread. [These commands are collectively referred to here as the TC (ThinCard) commands.] The name of each command is the name of the utility it invokes, without the trailing .exe (see items 3 – 8 of the list of files in Section 2.1). This chapter gives you complete information on the syntax of those six TC commands.

## 104	General Features

All the TC commands share a number of common features. Commands have the form:

command-name options parameters

All fields, unless specified to the contrary, are not case-sensitive.

•	command-name specifies the name of the TC program.

•	options allow you to modify the operation of the command from its default operation. Options always begin with a hyphen (-), and must be separated from surrounding information by one or more spaces. To enclose a special character within an option argument, enclose the entire argument in quotes.

•	parameters provide the substantive information to the command: what files to copy, what drive to operate upon, etc.

If the first parameter begins with a hyphen, a double hyphen (--) must mark the end of the options, to prevent the program from misidentifying the parameter as an option. For example, to copy the file -test-.dat to a Flash card, you'd enter the following command:

tcxcopy -- -test-.dat g:

## 105	Specifying Default Options

Each utility has an associated environment variable, whose name is formed by attaching the word FLAGS to the name of its invoking command. When run, each utility examines its environment variable for default options before checking the information from the command line. For example, TCXCOPY.EXE looks for an environment variable named TCXCOPYFLAGS. You may wish to change the default options to be assumed by the TC commands. Changing environment variables is discussed individually for each command (except tcinit) in the subsections of Section 12.3.

Options specified as defaults, whether in a command program itself or in an environment variable, can always be overridden on the command line.

NOTE:

It is essential that there be no blanks or tabs between the environment variable name and the equals sign (=). Under some versions of DOS, a trailing space will prevent a TC command from matching its flag.

## 106	Command Descriptions

The following subsections present the six TC commands in alphabetical order by command name. Each subsection gives the command line syntax, defines and describes all available command options, and presents other important information where pertinent.

Default values are identified for many options. These default values are operative only if all of the following three conditions exist: the option is not specified, there is no CIS on the card, and no value is specified in a card configuration file named in the command line.

The use of square brackets ([]) in the following subsections to enclose an item indicates that entry of that item is optional. (The brackets themselves are not to be entered.)

### 107	tcerase

Use tcerase to erase a RAM or Flash card’s common memory. If the card has attribute memory, the attribute memory is not erased. (Attribute memory may be erased, or overridden with a new CIS, using tcinit — see Subsection 12.3.3.)

In the case of a multi-partition card, tcerase erases only the single specified partition by default; an option is available (see the description of -all in Subsection 12.3.1.2) with which to erase all of common memory.

#### 108	Command Line

TCERASE.EXE is invoked by a command of the following form:

tcerase [options] drive:

Drive specifies the system’s memory card drive that contains the card to be erased. In the case of a multi-partition card, drive specifies the partition to be erased (except as explained in the description of -all in Subsection 12.3.1.2).

#### 109	Options

The options control the operation of TCERASE.EXE. They may be entered in any order, but must appear before the drive parameter. The valid options for tcerase are:

-?
TCERASE.EXE produces a brief command summary and stops.

-all
TCERASE.EXE erases the entire common memory of a multi-partitioned card. This option is not necessary for a card that has not been partitioned. If this option is used, drive must correspond to the card’s first (alphabetically) partition (see Subsection 2.3.1.2). If this option is not used, only the partition corresponding to the specified drive parameter will be erased.

-base address
Specifies the start address of the region of the card to be erased. You should ordinarily omit this option. The default is 0.

-buffer number-of-bytes
Specifies the size, in bytes, of the internal buffer to be used for programming operations. The default value is 1024. number-or-bytes may be in hexadecimal or decimal notation.

-card manufacturer/part-number
If the card is a supported card (see Table 8-1), use this option to specify the card's manufacturer and part number. TCERASE.EXE uses this information to look up the physical characteristics of the card.

If your card doesn't appear in Table 8-1, use the -type and -size options to specify card information.

-config filename
Specifies the card configuration file to be used by TCERASE.EXE. The filename field for this option may contain a file path. filename ends with the extension .cfg.

If you don't specify a card configuration file, TCERASE.EXE looks for a default file named tc.cfg provided as part of the distribution.

-debug
Specifies debug mode. Voluminous output is produced.

-nodebug
Suppresses debug output. This is the default.

-override
Specifies that the card type and size information entered in the command line override the information obtained from the card by TCERASE.EXE from any other source. You should not normally use this option.

-nooverride
TCERASE.EXE checks user input against information stored on the card, before erasing the card. If there are any conflicts, TCERASE.EXE stops with an error message. This is the default.

-progress
Requests TCERASE.EXE to produce progress reports as it's erasing the card. This is the default.

-noprogress
Suppresses progress reports.

-size number-of-bytes
Specifies the size of the card, in bytes. This option is not needed if you use the -card option, or if the card you are erasing has CIS on it or has been programmed with a DOS File Allocation Table (FAT). See Section 12.4 for the requirements for entering number-of-bytes.

-stats
Tells TCERASE.EXE to display programming statistics.

-nostats
Tells TCERASE.EXE to suppress display of programming statistics. This is the default.

-type card-type
Specifies the card type. This option is not needed if you use the -card option, or if the card you are erasing has CIS on it.

The card-type field must be either of the following two values:

•	flash. This is the default.

•	ram. Erasing a RAM card is a feature provided for test and checkout purposes; most of the logic of TCERASE.EXE is first used to set the RAM to zero, then fill with 0xFF.

-v
Specifies “verbose” mode. Much output is generated, and a different progress display is used. This option overrides -progress.

-nov
Suppresses “verbose” output. This is the default.

#### 110	Environment Variables

You can configure TCERASE.EXE to change its default command option values (see Section 12.2). Simply add the following command to your AUTOEXEC.BAT file:

set TCERASEFLAGS=options

For example, to make Intel 1-Mbyte Flash cards the default card type, and to display statistics by default, you'd use the following command:

set TCERASEFLAGS=-stats -card intel/imc001

### 111	tcformat

Use tcformat to format a RAM card or a Flash card. When formatting a RAM card, tcformat places the DOS File Allocation Table (FAT) on the card, behaving very much like the DOS format command does when formatting a floppy disk. When formatting a Flash card, tcformat places the Flash File System (FFS2) on the card, unless the -sys option is included in the command line, in which case the FAT format is placed on the card (see the description of -sys in Subsection 12.3.2.2).

#### 112	Command Line

TCFORMAT.EXE is invoked by a command of the following form:

tcformat [options] drive:

Drive specifies the system’s memory card drive that contains the card (or, in the case of a multi-partition card, the partition) to be formatted.

#### 113	Options

The options control the operation of TCFORMAT.EXE. They may be entered in any order, but must appear before the drive parameter. The valid options for tcformat are:

-?
TCFORMAT.EXE produces a brief command summary and stops.

-card manufacturer/part-number
If the card is a supported card (see Table 8-1), use this option to specify the card's manufacturer and part number. TCFORMAT.EXE uses this information to look up the physical characteristics of the card.

If your card doesn't appear in Table 8-1, use the -type option to specify card information.

-config filename
Specifies the card configuration file to be used by TCFORMAT.EXE. The filename field for this option may contain a file path. filename ends with the extension .cfg.

If you don't specify a card configuration file, TCFORMAT.EXE looks for a default file named tc.cfg provided as part of the distribution.

-now
Tells TCFORMAT.EXE to assume that the card is already in the drive and not to wait until the user presses Enter before formatting.

-sys
Requests TCFORMAT.EXE to format the card and place DOS system files on the card. For RAM cards, this option is equivalent to the /s switch of the DOS format command. For Flash cards, this option places the DOS FAT format (compatible with TCXCOPY.EXE) on the card rather than FFS2, and puts the system files on the card. Subsequently, you can use TCXCOPY.EXE to put files on the Flash card. You cannot use this option to create a card with Flash File System and DOS system files.

When using this option, you must be on the boot drive.

-type card-type
Specifies the card type. This option is not needed if you use the -card option, or if the card you are formatting has CIS on it.

The card-type field must be either of the following three values:

•	ram. Use with RAM cards. This is the default.

•	flash. Use with Flash cards.

-w
Tells TCFORMAT.EXE to prompt the user to insert the card in the drive and press Enter. This is the default.

-v
Specifies “verbose” mode. Much output is generated, and a different progress display is used.

-nov
Suppresses “verbose” output. This is the default.

#### 114	Environment Variables

You can configure TCFORMAT.EXE to change its default command option values (see Section 12.2). Simply add the following command to your AUTOEXEC.BAT file:

set TCFORMATFLAGS=options

For example, to display complete formatting information by default, and to search by default in c:\cardtalk for an additional card configuration file called sram.cfg, you'd use the following command:

set TCFORMATFLAGS=-v -config c:\cardtalk\sram.cfg

### 115	tcinit

Use tcinit when you want to place the Card Information Structure (CIS) on a memory card. If the card has attribute memory, tcinit stores as much of CIS data as possible in attribute memory. If the card doesn't have attribute memory, tcinit stores this information in the first sector on the card. All existing CIS information on the card, whether in attribute memory or common memory, is erased before the new CIS is placed on the card. See Chapter 8 for complete information on card configuration and CIS.

#### 116	Command Line

TCINIT.EXE is invoked by a command of the following form:

tcinit [options] drive:

Drive specifies the system’s memory card drive containing the card to be initialized.

#### 117	Options

The options control the operation of TCINIT.EXE. They may be entered in any order, but must appear before the drive parameter. The valid options for tcinit are:

-?
TCINIT.EXE produces a brief command summary and stops.

-card manufacturer/part-number
If the card is a supported card (see Table 8-1), use this option to specify the card's manufacturer and part number. TCINIT.EXE uses this information to look up the physical characteristics of the card.

If your card doesn't appear in Table 8-1 but is compatible with PCMCIA/JEIDA standards, you can add the card's physical data in the card configuration file used to describe the card's logical layout (see -config option).

-config filename
Specifies the card configuration file to be used by TCINIT.EXE. The filename field for this option may contain a file path. filename ends with the extension .cfg.

Typically, the card configuration file contains information about the logical layout of the card and how data are stored. If you don't specify a card configuration file, TCINIT.EXE looks for a default file named tc.cfg provided as part of the distribution.

The type of card may also be specified in the card configuration file (see Chapter 8).

-erase 
Use this option to simply erase the CIS from a card without placing a new CIS. This option is not necessary when any other option is used.

-p 
TCINIT.EXE checks whether the selected card configuration file’s syntax is correct, but does not initialize the card. This is useful when constructing new card configuration files.

### 118	tcprog

Use tcprog to program a binary image stored in a file to a memory card.

#### 119	Command Line

TCPROG.EXE is invoked by a command of the following form:

tcprog [options] drive: input-file(s)

Drive specifies the system’s memory card drive that contains the card (or, in  the case of a multi-partition card, the partition) to be programmed.

The input-file(s) field specifies one or more files that supply the data to be used in programming the card. This (each) file may be a binary file, an Intel OMF, or an Intel ASCII-hex file. See Subsection 12.3.4.4 for more information.

#### 120	Options

The options control the operation of TCPROG.EXE. They may be entered in any order, but must appear before the drive parameter. The valid options for tcprog are:

-?
TCPROG.EXE produces a brief command summary and stops.

-buffer number-of-bytes
Specifies the size, in bytes, of the memory buffer to be used in programming the card. The default value is 32768. number-or-bytes may be in hexadecimal or decimal notation.

-card manufacturer/part-number
If the card is a supported card (see Table 8-1), use this option to specify the card's manufacturer and part number. TCPROG.EXE uses this information to look up the physical characteristics of the card.

If your card doesn't appear in Table 8-1, use the -type and -size options to specify card information.

-config filename
Specifies the card configuration file to be used by TCPROG.EXE. The filename field for this option may contain a file path. filename ends with the extension .cfg.

If you don't specify a card configuration file, TCPROG.EXE looks for a default file named tc.cfg provided as part of the distribution.

-debug
Specifies debug mode. Voluminous output is produced.

-nodebug
Suppresses debug output. This is the default.

-filetype filetype
Requests that TCPROG.EXE use the specified file type when it programs the memory card. Normally, TCPROG.EXE determines the file type when it opens the file. Use this option to override that process. The valid file types are:

| OMF | Intel Absolute Object Module |
| --- | --- |
| HEX | Intel ASCII Hex |
| BIN | binary image (no format) |

-override
Specifies that the card type and size information entered in the command line override the information obtained from the card by TCPROG.EXE from any other source. You should not normally use this option.

-nooverride
TCPROG.EXE checks user input against information stored on the card, prior to programming the card. If there are any conflicts, TCPROG.EXE stops with an error message. This is the default.

-progress
Requests that TCPROG.EXE give the usual progress reports as it's working. This is the default.

-noprogress
Suppresses the progress reports.

-size number-of-bytes
Specifies the size of the card, in bytes. This option is not needed if you use the -card option, or if the card has the CIS on it. See Section 12.4 for the requirements for entering number-of-bytes.

-stats
Tells TCPROG.EXE to display statistics on programming efficiency at the end of a programming cycle.

-nostats
Tells TCPROG.EXE to suppress display of programming statistics. This is the default.

-type card-type
Specifies the card type. This option is not needed if you use the -card option, or if the card you are programming has CIS on it.

The card-type field must be one of the following two generic values:

•	flash

•	ram

This option is not needed if you use the -card option, or if the card has the CIS on it.

-v
Specifies “verbose” mode. Much output is generated, and a different progress display is used.

-nov
Suppresses “verbose” output. This is the default.

#### 121	Environment Variables

You can configure TCPROG.EXE to change its default command option values (see Section 12.2). Simply add the following command to your AUTOEXEC.BAT file:

set TCPROGFLAGS=options

For example, to select Flash cards by default and to select a 1-Mbyte default card size, you’d use the following command:

set TCPROGFLAGS=-type Flash -size 0x100000

#### 122	Programming Cards for Embedded Systems

In addition to its ability to program a card from a card image, TCPROG.EXE is able to program cards from Intel OMF or Intel ASCII-hex files. This allows you to use RAM and Flash cards as general-purpose memory components in embedded systems.

When used in this way, TCPROG.EXE has a significant characteristic: it simply programs bytes in the order in which it encounters them in the input OMF or ASCII-hex files. This works well for RAM cards, and in most cases it is entirely adequate for programming Flash cards.

### 123	tcread

Use tcread to read the image of a card and store it in a binary file. tcread always produces an exact byte-by-byte image of the entire card.

#### 124	Command Line

TCREAD.EXE is invoked by a command of the following form:

tcread [options] drive: output-file [start] [length]

Drive specifies the system’s memory card drive that contains the card to be read.

Output-file specifies where the data file is to be written.

Start and length, if entered, specify the amount of data to be copied from the card. These arguments have two purposes; they allow you to read a portion of the card and to specify the size of the card.

When copying file-structured RAM cards, TCREAD.EXE can usually determine the size fairly accurately; when copying arbitrary data, however, you may need to tell it the length of the card. Start and length are both numbers; they may be in decimal format (e.g., 512) or in hexadecimal format (e.g., 0x400).

#### 125	Options

The options control the operation of TCREAD.EXE. They may be entered in any order, but must appear before the drive parameter. The valid options for tcread are:

-?
TCREAD produces a brief command summary and stops.

-buffer number-of-bytes
Specifies the size, in bytes, of the internal buffer to be used in reading the card. The default value is 1024 bytes. For large copies, on systems with sufficient memory, increasing the buffer size may decrease execution time. number-or-bytes may be in hexadecimal or decimal notation.

-card manufacturer/part-number
If the card is a supported card (see Table 8-1), use this option to specify the card's manufacturer and part number. TCREAD.EXE uses this information to look up the physical characteristics of the card.

If your card doesn't appear in Table 8-1, use the -size option to specify card size.

-config filename
Specifies the card configuration file to be used by TCREAD.EXE. The filename field for this option may contain a file path. filename ends with the extension .cfg.

If you don't specify a card configuration file, TCREAD.EXE looks for a default file named tc.cfg provided as part of the distribution.

-size number-of-bytes
Specifies the size of the card, in bytes. This option is not needed if you use the -card option, or if the card you are erasing has CIS on it. See Section 12.4 for the requirements for entering number-of-bytes.

-v
Specifies “verbose” mode. Much output is generated, and a different progress display is used.

-nov
Suppresses “verbose” output. This is the default.

#### 126	Environment Variables

You can configure TCREAD.EXE to change its default command option values (see Section 12.2). Simply add the following command to your AUTOEXEC.BAT file:

set TCREADFLAGS=options

For example, to increase the default buffer size to 32768, you’d use the following command:

set TCREADFLAGS=-buffer 32768

### 127	tcxcopy

Use tcxcopy to create a DOS-compatible read-only File Allocation Table (FAT) file system on a Flash card (or, for test purposes, on a RAM card) and copy files to the card. tcxcopy’s function and syntax are somewhat similar to those of the DOS XCOPY command; hence the name.

Some of tcxcopy’s notable features are:

•	automatic formatting and initialization of card before copying;

•	file specification by name, by wild-card match, or by subdirectories search;

•	capability to read list of files to be copied from an input file;

•	capability to add files to a card.

#### 128	Command Syntax

TCXCOPY.EXE is invoked by a command of the following form.

tcxcopy [options] file-directives drive:[path]

Drive specifies the system’s memory card drive that contains the card (or, in the case of a multi-partition card, the partition) on which the FAT file system is to be placed and to which specified files are to be copied.

File-directives specifies the files to be copied (see Subsection 12.3.6.3).

Path is the output path on the drive.

#### 129	Options

The options control the operation of TCXCOPY.EXE. They may be entered in any order, but must appear before the drive parameter. The valid options for tcxcopy are:

-?
TCXCOPY.EXE produces a brief command summary and stops.

-buffer number-of-bytes
Specifies the size, in bytes, of the internal buffer to be used in copying the card. The default value is 32768. number-or-bytes may be in hexadecimal or decimal notation.

-card manufacturer/part-number
If the card is a supported card (see Table 8-1), use this option to specify the card's manufacturer and part number. TCXCOPY.EXE uses this information to look up the physical characteristics of the card.

If your card doesn't appear in Table 8-1, use the -type and -size options to specify card information.

-config filename
Specifies the card configuration file to be used by TCXCOPY.EXE. The filename field for this option may contain a file path. filename ends with the extension .cfg.

If you don't specify a card configuration file, TCXCOPY looks for a default file named tc.cfg provided as part of the distribution.

-debug
Specifies debug mode. Voluminous output is produced.

-nodebug
Suppresses debug output. This is the default.

-override
Specifies that the card type and size information entered in the command line override the information obtained from the card by TCXCOPY from any other source. You should not normally use this option.

-nooverride
TCXCOPY.EXE checks user input against information stored on the card, prior to erasing the card. If there are any conflicts, TCXCOPY.EXE stops with an error message. This is the default.

-precheck
Specifies execution of checks to assure that input data will fit on the output card before beginning work. TCXCOPY.EXE does this to protect you against erroneous operations that might otherwise cause you to misprogram a card. This check is somewhat conservative and might prevent you from doing an otherwise legal operation. This is the default.

-noprecheck
Suppresses the checks that determine whether the input data will fit on the output card.

-progress
Requests that TCXCOPY.EXE give progress reports as it's working. This is the default.

-noprogress
Suppresses progress reports.

-size number-of-bytes
Specifies the size of the card, in bytes. This option is not needed if you use the -card option, or if the card you are erasing has CIS on it, or if you are adding more files to a previously-created card. See Section 12.4 for the requirements for entering number-of-bytes.

-slash
Specifies that a preceding forward slash (/) instead of a hyphen (-) identifies a file-directive-option (see Subsection 12.3.6.3). This is the default.

-noslash
Disables identification of a file-directive-option (see Subsection 12.3.6.3) by a preceding forward slash (/).

-stats
Requests that TCXCOPY.EXE display statistics on programming efficiency at the end of a programming cycle.

-nostats
Suppresses statistical display. This is the default.

-sys
Requests TCXCOPY.EXE to initialize the card and place DOS system files on the card. For Flash cards, this option is equivalent to tcformat -sys -type flash (see -sys and -type descriptions in Subsection 12.3.2.2). You must be on the boot drive when using this option.

-type card-type
Specifies the card type. This option is not needed if you use the -card option, or if the card you are programming has CIS on it.

The card-type field must be one of the following two generic values:

•	flash. This is the default.

•	ram

-v
Specifies “verbose” mode. Much output is generated, and a different progress display is used. This option overrides -progress.

-nov
Suppresses “verbose” output. This is the default.

#### 130	File-Directives

The file-directives field specifies the files to be copied to the card; a file-directive specifies either:

•	the name of the file to be copied;

•	a wild-card pattern giving the names of files to be copied;

•	the name of a file containing the names of files to be copied; or

•	the name of a file containing more file-directives.

You may use as many file-directives as will fit on the tcxcopy command line.

File-directives all have a common form: they consist of a file specification optionally followed by one or more file-directive-options. Each file-directive-option is written as a forward slash (/) followed by a single letter. File-directive-options may, but need not, be separated from their preceding file specification or from each other. Valid file-directive-options are:

/s		
Search subdirectories. If the preceding file specification designates a directory, the directory and all subdirectories are copied to the card. If the specification does not designate a directory, the file specification is split into its directory and file-name portions. The directory and all subdirectories are searched for files that match the file-name; if any are found, they are copied to the card.

/nos	
Suppress subdirectory searches. This is the default.

/e		
Copy empty subdirectories. If used with /s, and if the preceding file specification designates a directory or all files in a directory, /s causes empty subdirectories to be copied. Otherwise it has no effect.

/noe	
Do not copy empty subdirectories. This is the default.

/h		
Copy hidden files.

/noh	
Do not copy hidden files. This is the default.

/f		
The preceding file specification specifies a file containing a list of file-directives. Each file-directive may be separated from the previous directive by spaces or tabs, or may be on a new line. If /s, /e, and/or /h are also specified, they are taken to apply to all files designated by the specified list of file-directives.

/i		
Read a list of file-directives from the standard-input device. The list is terminated by end-of-file (^Z when entering the list from the console). This file-directive-option is specified with no preceding file specification. Any preceding options are taken as applying to the previous file specification (if any); subsequent options (/s, /e, and/or /h), up to the next file specification, are taken as applying to all files designated by the input list of file-directives.

#### 131	Global File-directive Options

If file-directive-options are entered in the command line after the drive: parameter, they implicitly select default modes for TCXCOPY.EXE's operations. See appropriate examples in Subsection 12.3.6.7.

#### 132	Copying a Single File

A single file can be copied simply by specifying its name. For example:

tcxcopy tmm.dat e:

copies the file tmm.dat to the card in drive E.

#### 133	Copying a Single Directory

The contents of a single directory may be copied simply by specifying the name of the directory. The contents of the current directory may be copied using the special name “.”. For example:

tcxcopy . e:\mydir

creates the directory e:\mydir, if necessary, then copies all files in the current directory to e:\mydir.

By default, subdirectories are not copied. You can override this option using the /s option (see the description of /s in Subsection 12.3.6.3).

#### 134	Copying Multiple Files

You can copy a few files by simply listing them. For example, to copy the files file1.dat and app.exe to the card in drive E:

tcxcopy file1.dat app.exe e:

Multiple files may also be copied in a single specification using the standard DOS wild-card “*”. For example, to copy all .dat files to a Flash card in drive E:

tcxcopy *.dat e:

To copy a file called tmm.bat, all .wk2 files, and all .prn files to a subdirectory called \tmm on a card:

tcxcopy tmm.bat *.wk2 *.prn e:\tmm

To copy all .exe files from \bin and its subdirectories to a subdirectory called \tmm\bin on a card:

tcxcopy \bin\*.exe/s e:\tmm\bin

To copy all .exe files and all subdirectories from \bin, as well as all files named tmm.bat and all subdirectories in the current directory, to a subdirectory called /tmm on a card:

tcxcopy \bin\*.exe tmm.bat e:\tmm /s

In this example, subdirectories will be created below \tmm as needed.

To read a list of instructions from files.lis, implicitly searching all subdirectories; and to search all subdirectories of the current directory for tmm.bat, copying all matching files:

tcxcopy files.lis/f tmm.bat e: /s

#### 135	Copying Files With Unusual Names

If you need to copy a file whose name begins with “-”, specify either the explicit drive or the explicit directory containing the file to be copied. For example, the following command copies all files named -FILE-.* from the current directory and its subdirectories to the card in drive E.

tcxcopy .\-file-.*/s e:

If the “.\” were omitted, TCXCOPY.EXE would try to interpret “-file-.*” as a command option, and would print an error message.

#### 136	Using UNIX-style Command Syntax

For special applications, you can change the syntax of file-directive-options. The tcxcopy option -noslash (see its description in Subsection 12.3.6.2) has two effects:

•	The forward slash (/) no longer introduces options. It can be used as a directory separator interchangeably with the backslash (\) within file names.

•	File-directive-options are introduced with the character “-” just like command options. In this case, however, these options must be separated from file names, and from each other, by spaces or tabs.

For example, the following two commands are equivalent:

tcxcopy \bin\*.*/s/h \123\*.wks e:

tcxcopy -noslash /bin/*.* -s -h \123\*.wks e:

#### 137	Environment Variables

You can configure TCXCOPY.EXE to change its default command option values (see Section 12.2). Simply add the following command to your AUTOEXEC.BAT file:

set TCXCOPYFLAGS=options

For example, to make Flash cards the default card type, and to display statistics by default, you'd use the following command:

set TCXCOPYFLAGS=-stats -type flash

## 138	Entering the Card Size

Four of the commands described in Section 12.3 (tcerase, tcprog, tcread, and tcxcopy) accept the argument and value -size number-of-bytes. The value number-of-bytes may be entered in decimal or hexadecimal notation.

If entered in decimal notation, the number must be exact; given that “k” and “M” represent 210 and 220 (rather than 103 and 106) respectively (see Section 1.5), decimal entry is somewhat awkward. If entered in hexadecimal notation, the number must be preceded by “0x”.

For quick reference, the decimal and hexadecimal entries for the most common memory card sizes are given in Table 12-1.

Table 12-1	Decimal and Hexadecimal Equivalents of Card Sizes

| Card Size | Decimal | Hexadecimal |
| --- | --- | --- |
| 128 kbytes | 131072 | 0x20000 |
| 256 kbytes | 262144 | 0x40000 |
| 512 kbytes | 524288 | 0x80000 |
| 1 Mbyte | 1048576 | 0x100000 |
| 2 Mbytes | 2097152 | 0x200000 |
| 4 Mbytes | 4194304 | 0x400000 |
| 10 Mbytes | 10485760 | 0xA00000 |
| 20 Mbytes | 20971520 | 0x1400000 |

##### Appendix A.	Using MEMCARD.EXE

This appendix is based on Microsoft’s MEMCARD.EXE documentation. It has been edited, for clarity, organizational efficiency, applicability to MEMCARD.EXE’s use with the CardTalk driver, and format consistency with the rest of this manual. (Databook has not made any changes to the software itself.)

## A.1	Overview

### A.1.1	MEMCARD.EXE Capabilities

Microsoft’s MEMCARD.EXE is a configuration and maintenance program for Flash memory cards that are formatted with Flash File System version 2.0 (FFS2). You can use MEMCARD.EXE to do the following:

•	Create and format a new partition

•	Format an existing partition

•	Delete a partition

•	Display partition information

•	Erase an entire memory card

•	Check for and fix errors and display the status of a Flash memory card

•	Select a different memory card slot

### A.1.2	MEMCARD.EXE Limitations

MEMCARD.EXE may behave incorrectly with a Flash memory card prepared with DOS FAT partitions, and may incorrectly list partition information with such a card. The Databook TCXCOPY.EXE utility (see Subsection 12.3.6) should be used for formatting Flash memory cards with DOS FAT partitions.

MEMCARD.EXE will not use attribute space when preparing cards for multiple partitions. If you are preparing a card that has attribute space, we recommend that you use the Databook TC utilities (see Chapter 12) to prepare the card, to conserve common space memory.

New cards that do not contain a factory-installed CIS cannot initially be prepared with MEMCARD.EXE using the CardTalk software. New cards without a CIS should initially be prepared with Databook’s TCINIT.EXE utility (see Subsection 12.3.3).

## A.2	Starting MEMCARD.EXE

To start the MEMCARD.EXE program, type one of the following three commands at the command prompt:

memcard <cr>

…	to use a series of menus to perform any of the functions listed in Section A.1; or

memcard /fmt=drive: [/v=label] <cr>

…	to format an existing partition directly; or

memcard /chk=drive: [/f] <cr>

…	to check for and fix errors and display the status of a Flash memory card.

These three variations of the memcard command are described separately in the next three sections.

## A.3	Using MEMCARD.EXE with Menus

If you type memcard <cr>, the main menu appears:

PC Memory Card Setup Program Version 2.00

(C) 1991-1992 Microsoft Corporation

Memcard Options

Current memory card slot: [slot number]

Choose one of the following:

1. Create and format a new partition

2. Format an existing partition

3. Delete a partition

4. Display partition information

5. Erase entire memory card

6. Check memory card and fix errors

7. Select next memory card slot

Enter your choice.

To quit Memcard, press <esc>.

If your computer includes only one memory card slot, option 7 does not appear on the main menu.

To choose a menu option, type its number and press ENTER. When you choose any of menu options 1 through 6, another menu appears. To return to the main menu, press <esc>. To quit MEMCARD.EXE, return to the main menu, and press <esc> again.

Each menu displays a "Current memory card slot" message, followed by a slot number. If you have only one memory card slot, the number is always 1. If you have more than one slot, the number indicates which slot MEMCARD.EXE is currently using. You can select the next available slot by choosing option 7 from the main menu.

### A.3.1	Displaying Partition Information

To view information about the status, type, and size of the partitions on your memory card, perform the following procedure:

On the main menu, type 4 and press ENTER. A Display Partition Information screen such as the following appears:

Display Partition Information

Current memory card slot: 1

Device Information:

Memory Type        Size

———————————————————————

Flash             4096K

Partition Information:

Letter   Start Addr.   End Addr.    Type        Status        Size

——————————————————————————————————————————————————————————————————

D      0             4194303      MS-Flash    Formatted    4096K

.      .                .            .            .          .

.      .                .            .            .          .

.      .                .            .            .          .

The information on the Display Partition Information screen varies, depending on the number, size, and type of partitions on your memory card. The categories of information that appear on the Display Partition Information screen are:

(a) In the Device Information table:

| Memory Type: | Type of device or card in the slot. Possible types are: ROM: Read-only memory OTPROM: One-time programmable ROM EPROM: ROM erasable by ultraviolet radiation EEPROM or Flash: ROM erasable electrically SRAM: Static random-access memory DRAM: Dynamic random access memory I/O: input/output |
| --- | --- |
| Size: | Device size, in kbytes. |
| Vendor: | The card manufacturer. |

(b) In the Partition Information table:

| Letter: | Drive letter associated with each partition. |
| --- | --- |
| Start Addr: | Start address of that partition. |
| End Addr: | End address of that partition. |
| Type: | Type of partition. MEMCARD.EXE recognizes partitions created by: MS-DOS: the MS-DOS operating system; MS-Flash: the Microsoft Flash File System1; Old FFS: an older version of the Microsoft Flash File System. (Unpartitioned space is defined as "free.") |
| Status: | Whether the partition is formatted or unformatted. If the partition was created by the card manufacturer, "Unrecognized" appears in this column. |
| Size: | Partition size in kbytes. |

If there are more partitions than MEMCARD.EXE can display on one screen, press any key to view the next screen of information.

To return to the main menu, press <esc>.

### A.3.2	Creating and Formatting a New Partition[footnote mark artificially here to avoid page change; belongs at the MS-Flash def’n under (b) above]

You can use MEMCARD.EXE to create and format only MS-DOS and MS-Flash (not old FFS) partitions. If you want to change the size of a partition, you must delete the existing partition and then create and format a new one. For information about deleting a partition, see Subsection A.3.5.

#### A.3.2.1	Flash Memory Card

To create and format a new partition on a Flash memory card, perform the following procedure:

On the main menu, type 1 and press ENTER. A Create and Format a New Partition screen such as the following appears:

Create and Format a New Partition

Current memory card slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D         MS-Flash     Formatted     4096K

.            .             .           .

.            .             .           .

.            .             .           .

If the card already has one large MS-Flash partition, the following message appears at the bottom of the screen:

This card has already been formatted. Creating a partition will destroy all the data on the card.

Do you want to continue? (Y/N) [N]

Create a new partition only if you have backed up all the data that you want to save. To continue, type Y and press ENTER.

If the card is unformatted or has one large partition, the following message appears:

Do you want to use the entire card for MS-Flash? (Y/N) [Y]

Important:

If this message does not appear, press <esc> to return to the main menu. Before you can create and format the entire card as one large partition, you must delete all existing partitions. For more information about deleting a partition, see Subsection A.3.5.

Type N or Y, depending on whether you want to create and format a partition that occupies a portion of the card or the entire card, respectively.

If you have typed N, your next step is to type the size of the partition you want to create, and press ENTER.

The minimum size depends on the type of memory card you have (see Table 8-1). The maximum size depends on the amount of free space available on the memory card. The number that appears on your screen is the largest amount of available space. MEMCARD.EXE will adjust the partition size from your entry if necessary, based on the internal block size of the card.

If you have typed Y to create and format a single partition for the entire card, you of course do not have to specify the size of the partition.

In either event, the next step is to type the number of spare blocks that MS-FLASH.SYS should reserve for memory card cleanup , and press ENTER.

MS-FLASH must reserve one or more of the blocks on your memory card for cleaning up deleted files. Valid numbers range from 1 to one less than the total number of blocks on the memory card. For example, if the total number of blocks on the card is 4, the valid range of spare blocks is 1 to 3. The default is 1. Although reserving extra blocks may speed up the memory card slightly, you will not be able to use the reserved blocks for files. The default is recommended.

The last step is to type a volume label for the partition , and press ENTER.

If you don't want to specify a volume label, leave the field blank, and press ENTER.

#### A.3.2.2	RAM Card

To create and format a new partition on a RAM card, perform one of the following procedures.

On the main menu, type 1 and press ENTER. A Create and Format a New Partition screen such as the following appears:

Create and Format a New Partition

Current memory card slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D         MS-DOS       Formatted     1024K

.            .             .           .

.            .             .           .

.            .             .           .

If the card already has one large MS-DOS partition, the following message appears at the bottom of the screen:

This card has already been formatted. Creating a partition will destroy all the data on the card.

Do you want to continue? (Y/N) [N]

Create a new partition only if you have backed up all the data that you want to save. To continue, type Y and press ENTER.

If the card is unformatted or has one large partition, the following message appears:

Do you want to use the entire card for MS-DOS? (Y/N) [Y]

Important:

If this message does not appear, press <esc> to return to the main menu. Before you can create and format the entire card as one large partition, you must delete all existing partitions. For more information about deleting a partition, see Subsection A.3.5.

Type N or Y, depending on whether you want to create and format a partition that occupies a portion of the card or the entire card, respectively.

If you have typed N, your next step is to type the size of the partition you want to create, and press ENTER.

The minimum size is 16 kbytes. The maximum size depends on the amount of free space available on the memory card. The number that appears on your screen is the largest amount of available space.

If you have typed Y to create and format a single partition for the entire card, you of course do not have to specify the size of the partition.

In either event, the next step is to type the maximum number of files and subdirectories that you want in the root directory , and press ENTER.

Valid numbers range from 16 to 512. The default number that appears on your screen varies, depending on the size of the partition you are creating. The default is recommended.

The last step is to type a volume label for the partition, and press ENTER.

If you don't want to specify a volume label, leave the field blank, and press ENTER.

### A.3.3	Formatting an Existing Partition

You can format a partition that has already been created. However, when you format a partition, all the data on it are lost and cannot be recovered. Before formatting an existing partition, back up any data that you want to save.

#### A.3.3.1	MS-Flash Format

To format an existing MS-Flash partition, perform the following procedure:

On the main menu, type 2 and press ENTER. A Format an Existing Partition screen such as the following appears.

Format an Existing Partition

Current memory card slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D        MS-Flash     Formatted      4096K

.            .             .           .

.            .             .           .

.            .             .           .

Which partition do you want to format? [ ]

Type the letter of the partition you want to format , and press ENTER. If the partition is already formatted, a message such as the following appears:

The partition is already formatted. Reformatting the partition will destroy all the data on it.

Do you want to format partition D? (Y/N) [N]

To return to the main menu without formatting the partition, type N and press ENTER. To format the partition, make sure you have a backup copy of any data presently in the partition that you want to save, type Y and press ENTER.

Next, type the number of blocks that MS-FLASH.SYS should reserve for memory card cleanup, and press ENTER.

MS-FLASH.SYS must reserve one or more of the blocks on your memory card for cleaning up deleted files. Valid numbers range from 1 to one less than the total number of blocks on the memory card. For example, if the total number of blocks on the card is 4, the range of spare blocks is 1 to 3. The default is 1. Although reserving extra blocks may speed up the memory card slightly, you will not be able to use the reserved blocks for files. The default is recommended.

Finally, specify a volume label for the partition , and press ENTER.

If you don't want to specify a volume label, leave the field blank, and press ENTER.

#### A.3.3.2	MS-DOS Format

To format an existing MS-DOS partition, perform the following procedure:

On the main menu, type 2 and press ENTER. A Format an Existing Partition screen such as the following appears:

Format an Existing Partition

Current memory card slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D         MS-DOS       Formatted     1024K

.            .             .           .

.            .             .           .

.            .             .           .

Which partition do you want to format? [ ]

Type the letter of the partition you want to format , and press ENTER. If the partition is already formatted, a message such as the following appears:

The partition is already formatted. Reformatting the partition will destroy all the data on it

Do you want to format partition D? (Y/N) [N]

To return to the main menu without formatting the partition, type N and press ENTER. To format the partition, make sure you have a backup copy of any data presently in the partition that you want to save, type Y and press ENTER.

Next, type the total number of files and subdirectories that you want in the root directory, and press ENTER.

Valid numbers range from 16 to 512. The default number that appears on your screen varies, depending on the size of the partition you are creating. The default is recommended.

Finally, specify a volume label for the partition , and press ENTER.

If you don't want to specify a volume label, leave the field blank, and press ENTER.

### A.3.4	Erasing a Memory Card

When you buy a memory card, it may contain irrelevant data that you must erase before you can use the card. Erasing a card destroys all the data stored on it. If you want to erase a card that is not new, make sure you have backed up any data that you want to save.

To erase a memory card, perform the following procedure:

On the main menu, type 5 and press ENTER. An Erase Entire Memory Card screen such as the following appears:

Erase Entire Memory Card

Current memory card slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D         MS-Flash     Formatted      512K

E         MS-Flash     Formatted     3072K

.            .             .           .

.            .             .           .

.            .             .           .

Erasing a card destroys all the partition information and data on the card. Erase a card only if you do not want to save the information on it.

Do you want to continue? [Y/N] [N]

To return to the main menu without erasing the memory card, type N and press ENTER. To erase all the data on the memory card, type Y and press ENTER. All the data on the card will be erased, and MEMCARD.EXE displays the following message:

This operation may take some time. Do not remove the card from the slot while this message is displayed.

### A.3.5	Deleting a Partition

You may want to delete a partition. When you delete a partition, all the data on it are lost and cannot be recovered. Back up any data that you want to save before deleting the partition.

If there are other partitions on the card with drive letters alphabetically beyond the partition you want to delete, those drive letters will change to fill the alphabetical gap that would otherwise be left by the deleted partition. For example, if you delete partition D from a memory card with partitions D, E, and F, partitions E and F are renamed D and E respectively.

To delete a partition, perform the following procedure:

On the main menu, type 3 and press ENTER. A Delete a Partition screen such as the following appears.

Delete a Partition

Current Memory Card Slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D         MS-Flash     Formatted      512K

E         MS-Flash     Formatted     3072K

.            .             .           .

.            .             .           .

.            .             .           .

Which partition do you want to delete? [D]

To return to the main menu, press ESC. To delete a partition, specify the letter of the partition you want to delete , and press ENTER. A message such as the following appears:

Deleting the partition will destroy all the data on it. Delete a partition only if you do not want to save the information on it.

Do you want to delete partition D? (Y/N) [N]

To return to the main menu without deleting a partition, type N and press ENTER. To delete a partition, make sure you have a backup copy of any data you want to save, type Y and press ENTER.

### A.3.6	Checking a Flash Memory Card for Errors

You can use MEMCARD.EXE to check a Flash memory card for errors. MEMCARD.EXE displays a status report about your Flash memory card, and identifies and recovers any lost allocation units that may be taking up space on it. If your card is not a Flash memory card, use the MS-DOS chkdsk command to check it for errors.

Caution:

If you try to fix errors while programs are running, you might lose data. Quit all programs before you use MEMCARD.EXE to check for and fix errors. This includes programs such as MS-DOS Shell and Microsoft Windows. If you use memory-resident programs, disable the command lines in your CONFIG.SYS and AUTOEXEC.BAT files that load them, and restart your computer.

To check a Flash memory card for errors, perform the following procedure:

On the main menu, type 6 and press ENTER. A Check an MS-Flash Partition screen such as the following appears:

Check an MS-Flash Partition

Current memory card slot: 1

Letter      Type         Status         Size

—————————————————————————————————————————————

D         MS-Flash     Formatted      512K

E         MS-Flash     Formatted     3072K

.            .             .           .

.            .             .           .

.            .             .           .

Which partition do you want to check? [D]

To return to the main menu without checking any partition, press ESC. To check a partition, type the letter of the partition you want to check , and press ENTER. The following message appears:

Do you want to correct any errors that are found? (Y/N) [N]

To recover any lost allocation units and make the space available on the memory card, type Y and press ENTER. To check the memory card without fixing any errors, type N and press ENTER.

### A.3.7	Selecting a Memory Card Slot

If your system has more than one slot for memory cards, you can choose which one you want to work with.

To select a memory card slot, type 7 on the main menu, and press ENTER. The current memory card slot will change to the next available slot. If you have more than two slots, type 7 again to select succeeding slots sequentially. The current memory card slot is displayed at the top of the main menu.

## A.4.	Formatting an Existing Partition Directly

Parameter and switch definitions for the command syntax:

memcard /fmt=drive: [/v=label] <cr>

are as follows:

| /fmt= | Formats the memory card partition associated with the specified drive. |
| --- | --- |
| drive: | Letter specifying the drive you want to format. |
| /v= | Gives the newly formatted memory card the specified volume name . |
| label | volume label text. |

## A.5.	Checking and Fixing a Flash Memory Card Directly

Parameter and switch definitions for the command syntax:

memcard /chk=drive: [/f] <cr>

are as follows:

| /chk= | Checks the Flash memory card partition associated with the specified drive for errors, and displays a status report. If no drive is specified, the current drive is checked. |
| --- | --- |
| drive: | Letter specifying the drive you want to format. |
| /f | Fixes errors found on a Flash memory card. If this switch is not specified, the errors are displayed, but not fixed. |

MS-Flash is referred to as “FFS2” in the numbered chapters of this document.