JOYCAL  Version 1.00

(C) Copyright 1987 Stephen B. Hajducek

All Rights Reserved.

DISCLAIMER OF WARRANTY

----------------------

THIS SOFTWARE IS PROVIDED  "AS IS" AND  WITHOUT ANY WARRANTIES  EXPRESSED OR

IMPLIED WHATSOEVER.  DUE TO THE DIVERSITY  OF CONDITIONS AND  HARDWARE UNDER

WHICH THIS SOFTWARE MAY BE USED, NO WARRANTY  OF FITNESS FOR  ANY PARTICULAR

PURPOSE IS OFFERED. THE END USER IS ADVISED TO TEST THIS SOFTWARE THOROUGHLY

BEFORE RELYING ON IT.  THE USER  MUST ASSUME  THE  ENTIRE RISK OF USING THIS

SOFTWARE.

[DOCUMENTATION]

PROGRAM: JOYCAL.EXE  V1.00

PURPOSE: Professional Joystick calibration.

DETAIL: Joysticks are not created equal, there are various configurations

availible for many applications. The quality and capabilties of Joysticks

vary from simple to the most ornate.

This program was written for joysticks that take full advantage of

any software specifically developed to be enhanced by the use of a joystick,

by allowing precise adjustment at the center most X-Y coordinate.

Joysticks with positive-true-positioning and optional self-centering

modes of operation are of the type this package is aimed at. These offer the

best results when working with any package requiring accurate stepping and

smooth roll, also the various MOUSE/JOYSTICK drivers availble yield an even

more useful life for the tired old joysick.

USE:    This package handles two joysticks simultaneously, the screen displays

joysick "A" on the left and "B" on the right as indicated. Both displays will

be at zero (0) if no joysticks are attached else some appreciatable reading

will be displayed.

For proper operation a game adapter must be installed in the computer,

the joystick(s) must be plugged in, then "Joycal" can be started. Following

the instructions of the joysticks manufacturer for centering adjustments, the

screen will show variations in the X-Y coordinate, when centered, the readings

for both coordinates will be equal, and furthermore they will be half of the

maximum attainable reading when the stick is moved to its lateral extreme. For

example, if the maximum output was equal to 200 when the stick is moved to the

full bottom right, then when the unit is centered properly both X-Y readings

will be at 100.

P.S. - Happy wrists to all............