usr00.TXT

IT IS NOT ADVISED TO EDIT THE FILE. DO THIS WHITHIN THE EDITOR BY:

ESC  MENU  USER (templatate)

Significations in USRnn.CFG

If you do editing of USRnn.CFG anyway, please don't touch USR00.ORG (a copy of the  original). You might spoil the file to edit which will cause unpredictable behaviour of the LAYO1 program, even RUNTIME ERRORS. Perhaps you have to reboot. In that case, before loading with LAYO1E or TRACEURS type in DOS  in  \LAYO1EE\

COPY USR00.CFG USR00.ERR

COPY USR00.ORG USR00.CFG

It will be preferable to edit only in USR01.CFG and then run Layo1 with

LAYO1E /U01

_00010	no comment

_00020	Colors of layers

_00030	Color of pads on layers after a netlist is charged

_00040    "   cursor in arrow form, grid, and squares

_00050_1  Subdirectories for components

_00050_2  Same for CNF if you loaded LAYO1 with LAYO1E CNF\PROJECR5\TEST5.CNF

_00060    Same LIBs. If you created your own LIB place it on top of the list.

_00070  Sensibility mouse. If 0, mouse nou usable. Take your time to find the

comfortable sert_up in combination with  _0000  (AUTOMOVE DELAY)

_00080  Type cursor,  try SH_A in the program first.

Pins/pads numbered, toglle-key 'n' in the program.

Blocks\modules\components numbered Y/N

_00090  Default file for pad-shapes & tool nrs and track thicknesses.

This is the file wich will be loaded with any new CNF. You may load

another one. See \SNB\.  Existing CNF_PLYs contain their own

definitions. So, they don't load DEFAULT nor any other *.SMB.

_00110  Routing set_ups

_00120	Default_steps of cursor in millimeter mode. Can temporary be changed

in the program with Sh_F9, Sh_F10 (if you are in mm_mode KEY_'m'.

See F9 & F10 .

_00130  8 is the 8th value from the step_table as default:

1280 640 320 160 80 40 20 10 5 1

Same for millimeters (from list _00120)

_00140  Tool diameters

_00150  Don't edit here but use USER TEMPLATE in the popmenu.

_00160  _00170  No editing allowed

_00180  Thes values will be used or changed only if you use the PROJECT MANAGER