COPYRIGHT LAYO1 Peter Baas 1984,1985, 1986, 1987, 1988, 1989, 1989, 1990, 1991

������������������������������������������������������������������������Ŀ

�  THERE ARE SOME CHANGEMENTS COMPARED TO THIS MANUAL. READ: README.DOC  �

�  FIRST.  THE F1-HELP-KEY ON THE (Esc)POP-MENU-LINES BEFORE TYPING ENTER�

�  IS UPDATED   (READ OTHER DOC FILES ALSO LIKE QUICKREF and STRUCTURE  !�

��������������������������������������������������������������������������

THIS TEXT IS DIRECTLY TRANSFERRED FROM THE VENTURA MANUAL FILE

OF COURSE THE DESIGNS WERE NOT EXPORTED TO THIS FILE

SO THE TEXT MAY SOMETIMES BE STRANGE !

1.1 Introduction to Layo1

Layo1 is a vector-based PCB designer for the electronic engineer.

It makes it possible to simply and quickly design electronic PCBs

(printed circuit boards), to a maximum size of 650 x 650 mm (22.5

x 22.5 inch), with a resolution of 1/1280 inch ( mm). This accuracy

is made possible by the mouse (or cursor) operating on a grid that

is adjustable, even while designing, from 1 to 1/1280 inch. Thus,

tracks, pads, components and texts can be placed with great accuracy.

What is Layo1?

Actually Layo1 works in exactly the same way as PCBs have been made

for years ... all components, pads, tracks and texts had to be drawn

or pasted. This procedure required separate drawings for all layers

that were later to be joined together very carefully. A job that took

a lot of time and precision: each mistake could be disastrous. If

later any changes were necessary, the best idea usually was to start

all over again. Layo1 works in much the same way, but it is many times

faster, easier and more accurate. Changes are no problem for Layo1,

and in the databank there are hundreds of ready-to-use components

that you can simply add on to your design.

How does Layo1 work?

Think of a manually controlled plotter that has a memory and remembers

each move you make, each trace and each pad that you draw. It does

not only remember all you draw, but also the pen diameter, the coordinates,

and the layer. All this information is displayed on Layo1's screen

and is stored in so-called datalines. A dataline could

contain (encoded) information like:

Starting coordinate, pen diameter, layer number.

With Layo1 you need not draw pads and countless components yourself.

You make a selection of what you need and then just point at where

you want it. For pads you can specify the tool diameter that you require

straight away, for later, when everything is ready and done, Layo1

will also control the drilling machine for you. Each pad placed means

another dataline added to the file. This dataline contains:

Coordinate, pad number and tool number.

If at a later stage any changes are necessary, you just make them

on the screen and Layo1 will adjust the relevant datalines. Thus,

tracks can be adjusted and whole parts of your design can be moved

or rotated to make up a perfect design. When moving parts of your

design about, all tracks will simply be pulled along.

In Layo1 you can work on fifteen layers. This makes it possible for

you to design multi-layer prints. You can use these

layers to store all information of your electronic circuit in separate

layers, such as tracks, component profiles, electronic diagram, etc.

The following example will show how Layo1 works and how Layo1 stores

everything you draw on-screen with the mouse.

s

1. A pad is placed. Its coordinate, the pad number and the relevant

tool number are stored in the first dataline.

Ins

Ins arrow Ins

2. More pads are placed.

Esc

3. The pen is activated in the position of the first pad. This is

also stored in a dataline by Layo1.

t

4. A track is drawn between the two pads with the mouse. It is stored

in the dataline, together with the pen number and the layer number.

By making this drawing you have created a four line program which

will be the basis of later output to printer, (photo)plotter and drilling

machine.

-----------------------------------------------------------------------------

1.2 Drawing with Layo1

As you can see from the previous example, drawing with Layo1 is a

much simpler procedure than with pen and ruler. Using Layo1's drawing

function you can draw with 7 line widths (t p) that are carefully tuned

to current technical standards. Layo1 makes for pleasant work, quick

as lightning, extremely accurate.

For the display of a 160 x 100 mm euroboard as a color picture the

resolution of the screen (zoom factor 4) is exactly 1/80 inch per

pixel. Starting from 1/80 inch (0.3175 mm) with 2/80 inch intervals,

your thickest <169>pen<170> is 13/80 inch (4.1275 mm). To ensure the

most accurate display on screen the best procedure is using the 1/80

inch grid as much as possible (this stepsize can be adjusted at any

time in Layo1). This is because in electronics the inch is still the

predominant unit of measure. That is why there is always an auxiliary

1/10 inch grid available for you on Layo1's screen.

Millimeters

However, in Layo1 you can also work in millimeters. So, Layo1 is equipped

with two complete measuring systems. This makes it possible for you

to switch from inches to metric units at almost any time. There are

six different rulers available for you and all of them are kept up-to-date

for fast and easy pinpoint positioning.

>  This toglle key for squares will follow the mm stepsize if the m-key is used

but you have to toggle again after m.

.  This toglle key for Grid gives the Inch grid at all times

Status line

Layo1's status line continuously displays the current cursor coordinates

in 0.001 inches and 0.01 millimeters in the top right corner of the

screen.

Internally the Layo1 program uses steps of 1/1280 inch (0.01984370 mm)!

Stepsize

While working you can select ten different stepsizes which you can

define for yourself if you like and which are then stored for you

by Layo1. The measuring system of Layo1 (r key)is so powerful and versatile

that it is possible to position the cursor on exactly 12.5 mm, then

switch to <->inches to place pads, traces and components in steps

of 1/40 inch relative to the 12.5 mm. The reverse is also possible,

of course.

Simple control

All these possibilities and numerous others only take a keystroke

or a mouse command in Layo1. For Layo1's most important characteristic

is simplicity. Powerful functions that do the routine jobs for you,

quickly and accu<->rately, so that you can indeed use your creativity

in your PCB design.

-----------------------------------------------------------------------------

1.3 Layo1 in practice

The first time that you use Layo1 you may wish to supplement the library

with special components that are not yet available, and with subdrawings

that you frequently require, e.g. power supply sections.

First you use a scheme designer

a) OrCAD SDT) to make a scheme, of which you then make up a Calay compatible netlist and

component list.

b) with your text editor; see the exemples CMP and NET

c) in Layo graphic editor with F3 and Edit NET)

If the scheme contains any components

that are not yet available in Layo1's component databank

you must add them now. In this case the library (LIB) must

also be supplemented, so that Layo1 will know what components are

referred to in the component list.

Layo1 is started up and a new file is created. You give a name

to the New design, decide on its dimensions and offset, and you specify

the hole diameters that you require. In the configuration menu you

specify the name  of the component list, netlist and the library file in User    Menu)

When everything is ready and done, Layo1 will go to work for

you. It will read the component list, scan for the components in the

Layo1 component databank via the library and put them in place.

THERE ARE MANY EXEMPLES LIB IN \LIB\. YOU MAY GIVE THEM A TRY BUT IF

THE MESSAGE "NOT FOUND" IS DISPLAID TO OFTEN YOU HAVE TO CREATE A SECOND

LIB CONTAINING THE COMPONENTS WRITTEN BY LAYO IN NAME.LST

(ERRORS) and PLACE THIS LIB ON TOP IN THE LIST IN THE USER MENU

line: LIBRARIES

Next,

the netlist with all connections will be read and Layo1 will generate

the rats<->nest. Once all this is on your screen, you can still move

or rotate the components, if necessary, to bring the distribution

to perfection. Once everything is to your satisfaction, you start

to place the traces.

You can follow three procedures to do so:

- Manualy, using Layo1s powerful editor-functions

- Manual router

- Netlist router

Manual

Using the Layo1 Editor you can easily switch between the major functions

by just pressing one or more mouse buttons: placing pads and components

and drawing traces. This way you do all the drawing yourself, while

Layo1 checks for you if all connections are made in the right way.

Manual router

Using the Autorouter of Layo1, you simply point at the points that

have to be connected, and Layo1 will do so for you. If you do not

like the Autorouter's solution, you can interfere and change the trace

in any way you want to .

Netlist router

You can set up Layo1' netlist router exactly the way you want

it to work. Depending on the type of design you are working on you

can set the router to use regular patterns of traces (i.e. for RAM-boards

or RAM-sections). You can specify whether or not the router should

use vias to make any connection. Furthermore you can use the Lee-router

(using Lee's algoritm) that will find practically any possible connection.

You can also limit the space that the router can use and the maximum

deviation. Netlist router is one of Layo1's important powertools,

which lets you leave the tedious work to the computer.

Next, you activate Layo1's optimizer that will transform all square

angles in lines into neat 45� angles. If the autorouter has not

done so already. And now your design is ready for you: made in no

time with a few simple keystrokes, without any time-consuming manual

work. And finally, using the Layo1 Editor, you put the finishing touches

to your design.

Now that your design is ready you can send it to the plotter or photoplotter

and you can create a complete drill program on disk or papertape.

As a check, you can make a checkplot on a matrix printer at any stage.

Doing so, you can specify which of the 16 layers are to be printed

or not.

100 percent MANUAL WORK

Working with Layo1, the first thing you usually do is loading the

components from the Layo1 databank and placing them. It is also possible

to load subdrawings of frequently used elements. All these parts can

be moved and rotated until the distribution is entirely to your satisfaction.

Next, the traces can be drawn. This can be done manually using the

Layo1 Editor, as was described above for Layo1. It can also be

done with the help of the Layo1 router: you simply point

at the two points that have to be connected and Layo1 will find the

shortest route between them (without vias). Once all tracks have been

drawn the optimizer will transform all square angles in the lines

into neat 45� angles. If the autorouter has not done so already.

Once your design is ready you can make a checkplot on a matrix printer

or laser printer or you can plot it to the scale you require on a

penplotter straight away or you can make it into a file for a photoplotter.

You can also create the drill program on disk or papertape now.

-----------------------------------------------------------------------------

1.4 This manual

This manual has been written with the utmost care. It is intended

to quickly make the Layo1 user, beginning and advanced alike, familiar

with the many (new) possibilities that the program has to offer. The

main idea has been to structure the manual in the same way as the

program in order to make it as accessible as possible. The publisher

welcomes all comments from users on the contents and structure of

this manual.

Layo France will in no event be liable for any errors in both program

and manual nor for any incidental or consequential damages.

Read.me

In as much as this is possible, recent amendments will be supplied

as a READ.ME file on the program disk. Should such a file be present

on your system disk, please read it and, if you like, print it.

Layo1

This manual goes with Layo1 as well as Layo1. For Layo1 you can

also read Layo1 unless otherwise stated. The extra facilities

offered by Layo1 are clearly indicated.

Keystrokes

In the text you will find symbols or frequent keystrokes in bold print,

such as function-key F1 or Enter or c. In these

cases simply pressing key F1, Enter or CR (carriage

return) or small letter c will be enough. Where combinations

of two keys are referred to, the first of which usually is Shift

(capital letters), Alt or Ctrl (Control), the idea is

to hold the first key down until the second key has been pressed.

An example, Shift F1: hold down the Shift key (capitals)

and strike function-key F1.

-----------------------------------------------------------------------------

1.5 Mouse control

The best way to operate Layo1 is by way of a mouse with three keys.

The functions of the mouse-keys and their combinations may vary dependent

on the application and menu option in the Layo1 Editor. The best way

to familiarize yourself with these particular functions is pressing

F1 in different situations. This will display a complete overview

of all mouse functions at that particular moment.

Although the use of mouse-keys is variable, there is a consistency

to it:

Left mouse-key: stands for <169>do<170>. Sometimes

literally so, usually figuratively ... Thus, it may stand for <169>OK,

go ahead<170>. It will activate a selection or it will add something,

working as a kind of Enter or insert key.

Right mouse-key: stands for <169>stop<170>. Sometimes

literally so, usually figuratively ... Thus, it may mean <169>No,

don't<170> and <169>Go back to the previous option. It will then quit

from an option already activated and/or will take you back to the

main menu, then functioning as the Escape-key.

Middle mouse-key: in all respects it is in-between

the left and right ones. In menu selections it often works as a <169>pointer<170>.

It is used to point at something, to select or to center. While drawing

and so on it activates the zoom function, offering a choice from the

various zoom factors (enlarging or reducing the picture on display).

Zoom

When zooming, the picture on display is either enlarged or reduced,

with the current cursor position as the center point. This is very

convenient whenever you want to move the drawing on the screen a bit

... with the help of the mouse you place the cursor over the new center

point, press the middle mouse-key once to activate the zoom function

and once again to maintain the current zoom factor ... the picture

has now been moved with the cursor <->position as the center point

of the picture.

Double-clicking the zoom-button will center your design on screen

(if corners have beeen placed) and adapt the zoom factor in such a

way that your entire design will fit on screen.

Yes/no <->mouses

Combinations of two or three mouse-keys have different meanings dependent

on the active function, but they are always consistent with the mouse-logic

as described above. Thus, pressing the two left mouse-keys always

stands for yes or plus, pressing the two right mouse-keys

for no or minus whenever you are asked for a yes/no or

+/- decision. In this manual full information on the mouse-key functions

will be given for all functions.

F1 in a certain mode AFTER ENTER ON THE MENU LINE = Help

(REMEMBER TO USE F1 ALSO BEFORE TYPING ENTER ON THE MENULINES)

While working with the Layo1 Editor, pressing F1

is enough to see what exactly you can do with the mouse at that particular

moment. Especially in the beginning this is a convenient memory aid;

after a while you will be so familiar with Layo1's mouse control that

you may even forget what F1 stand for.

-----------------------------------------------------------------------------

1.6 Hardware

Layo1 requires PCDOS or MSDOS on an IBM or IBM-compatible XT/AT computer.

Layo1 is not very demanding as to the hardware it requires:

Minimal requirements

An XT 640 Kb personal computer or compatible, EGA video board, color

screen, one floppy disk drive, one fixed disk, and one parallel printer

port.

If you want top performance and fast and easy handling, you will at

least choose for:

Recom- mended:

An AT (80386) personal computer or compatible, with at least 640 Kb

RAM and 1 Mb extended memory (LIM standard/Lotus-Intel-Microsoft),

one floppy disk drive, a fixed disk, a VGA (or compatible) video board,

a VGA color screen and a mouse with three keys.

In addition, a matrix printer (9 or 24 pins), a laser printer, and

a penplotter are useful output devices.

-----------------------------------------------------------------------------

1.9 Layo1 software

Layo1

Completely menu/mouse driven design program for complex PCBs to a

maximum of 25,000 coordinates and 1,000 components per design, with

the following extra <->facilities:

Reading component and netlist from Calay compatible scheme designers

such as OrCAD SDT.

Autorouting from a netlist.

Check-up on the connections according to the netlist.

On installation this program is placed in the directory \LAYO1P. The

program can be started up by typing LAYO1P from this directory.

Layo1 Junior

All facilities of Layo1, but suitable for smaller and/or simpler

PCBs to a maximum of 5,000 coordinates and 100 components per design.

On installation this program is placed in the directory \LAYO1PJ.

The program can be started up by typing LAYO1PJ from this directory.

Layo1 Demo

Similar to Layo1, but only with a very limited component databank

and to a maximum of 1,000 coordinates . On installation this program will be placed in the directory

\LAYO1EE.

**************************************************************************

After installation you will find that the following subdirectories

have been created and filled with the relevant files:

\LAYO1EE

\LAYO1EE\CNF

\LAYO1EE\LIB

\LAYO1EE\CMP

\LAYO1EE\NET

\LAYO1\LMC\

LAYO1EE\FNT

\LAYO1EE\LML

Dependent on the version of the program, LMC has been subdivided

into several subdirectories.

Layo1 directory

The directory name \LAYO1 depends on the version that you have bought

and installed:

\LAYO1EE 	Layo1 Limited

\LAYO1H	Layo1 DOUBLE

\LAYO1Q	LAYO1 quadro

\LAYO1PJ		Layo1 Junior

\LAYO1P 		Layo1

From this point on in this manual we will always refer to the Layo1

directory.

-----------------------------------------------------------------------------

2.2 Starting up Layo1

===> Attention:

To start up the Layo1 Demo version, please refer to the next

section.

Mouse driver

If necessary, load the mouse driver, please refer to the <->documentation

of your mouse.

Then type subsequently:

C:

CD\LAYO1

LAYO1E

After LAYO1 you must type H, Q,  PJ, or P, dependent on the Layo1

version used. So, for Layo1 you type:

C:

CD\LAYO1P

LAYO1P

The C in the first command should be replaced by another letter

if you have not installed Layo1 on fixed disk C:.

Configuration menu   Type LAYO1E /c

Now, Layo1's configuration menu will be displayed where, among other

things, you must specify the screen and printer used.

Getting to work straight away...

If you should like to get to work with Layo1 straight away to get

the feel of it, just select the correct video board parameters in

the configuration menu and otherwise leave it as it is for the moment.

You can always return to the configuration menu later (cf. section

16). Layo1 starts from the assumption that you have an EGA screen.

Should you have a VGA screen, you can adjust the Graph Mode setting

by selecting the EGA option (H10) with your mouse and pressing the

+-key twice: now H12 (VGA) will appear. You can reverse your selection

by using the <196> key. The same goes for the setting of the resolution

and the character height.

Layo1 Editor

Once you have stored the specified configuration, Layo1's main menu

will appear. Select the graphic editor and press the right mouse-key

or the Enter key. Now Layo1's menu line will appear: select

pad and then place.

Zoom

Press the middle mouse-key which will call up the zoom menu line.

Select 6 and press the left mouse-key. Try using some of the

other zoom factors in the same way. During all of this procedure a

pad will be attached to the cursor. Just put it wherever you want

it and place it by pressing the left mouse-key.

Pad

Do you want another pad? Select zoom factor 6 by pressing the

middle mouse-key (or type z F6) and press the s-key or

the two left mouse-keys once the menu has disappeared from the screen.

Move the mouse and watch what happens at the cursor: the selected

pad will be displayed immediately. Make your selection and confirm

it by pressing the left mouse-key. Further on in this manual you will

find out how to customise the sizes of the pads.

When you are ready placing pads, press the right mouse-key or escape.

This will call up the menu line again. Now select draw. A submenu

will appear. Select draw again by pressing the left mouse-key.

The menus will now disappear again. You are ready to start drawing.

Help: F1

Now press the F1 key first ... you will find full information

on the functions of the mouse-keys and their combinations. While working

in Layo1's graphic editor you can always use the F1 key (unless

there is a selection menu on-screen) for a quick look at the mouse-key

functions. You can remove this quick-reference memory aid from the

screen by pressing Escape or the right mouse-key.

Drawing

Thus, the left mouse-key stands for drawing. Once you press it the

pen will be put on the paper and you can draw lines and traces. See

what happens when you move the mouse in a circular motion: the line

will remain attached to the starting point, but it will follow the

cursor and doing so will create horizontal, vertical and 45� diagonal

lines. Now press the p-key and then F4 in order to adjust

the pen diameter and see what happens. In this way Layo1 works on

layers 1 ... 7 which are particularly suitable for the design of PCBs.

Layers 8 ... 15 work in a different way: they are mainly meant for

more general purposes, such as auxiliary lines, texts, circles, housings,

etc. Just try a few things. Conclude your line by pressing the right

mouse-key. The pen will be lifted up from the paper and the cursor

is free.

Layers

Now press the L-key and then the F8 key. Now you are on

layer 8. Press the left mouse-key again in order to put the pen on

paper and make a circular motion with your mouse. The line left by

the pen is now thinner and, moreover, it will take any form without

being forced into 90� or 45� angles.

Pen <->diameter

Now switch back to layer 2 again and adjust the pen diameter by pressing

the p-key and then the F5 key to select pen 5. Now try

and do the same things as above.

Another exercise

Go to a <169>clean<170> part of the screen and draw the line from

the example hereunder on layer 1 or 2. Start at 1 by pressing the

left mouse-key, place every corner with the left mouse-key and conclude

your line at 6 by pressing the right mouse-key. The pen is now up.

Now move the cursor to point 4 and press all mouse-keys simultaneously.

Layo1 will make line-section 3 to 4 blink, asking whether that is

the one you want.

Confirm using the left mouse-key. The line-sections that connect at

point 4 are now attached to the cursor like rubber bands: you can

pull them in all directions.

If you now press the delete-key, point (dataline) 4 will disappear

and points 3 and 5 will be directly connected. Point 5 has now become

point 4 and can be moved about with the cursor:

Now make a new start in some other part of the screen. Again draw

the line from 1 to 6 as described above, lift the pen from the paper

at point 6 by pressing the right mouse-key. Place the cursor on point

4, press all mouse-keys simultaneously (or use the =-key) and,

when the line-section starts blinking, confirm by pressing the left

mouse-key:

Again the lines from and to point 4 are attached to the cursor. You

can move point 4 in all directions. Now press the p-key and

select pen 0 with the mouse by highlighting it and pressing

the left mouse-key. Now the line from 3 to 4 is drawn with an <169>empty<170>

pen

In the same way a line can be drawn using different pen diameters.

In the example above you would e.g. use pen 7 instead of pen 0:

Quitting

Press the right mouse-key as often as is necessary to <->return to

the menu line. Select FILE and go down the submenu to select QUIT.

Now you exit from the graphic editor and return to Layo1's main menu.

Meanwhile Layo1 will ask you whether you want to save your design.

For the moment, select no.

Easy-to-use

Trying out Layo1, you may now be getting the feel of how Layo1 works,

but this is only a first impression of Layo1's many extensive possibilities.

Layo1 is easy-to-use: in the same way in which you, as a beginner,

were placing pads and drawing traces a moment ago, you can also load

ready-to-use components, move, rotate and copy parts of your design,

and automatically connect two points. And these are just a few of

Layo1's many powerful functions.

Quick, easy and accurate

All functions are extensively discussed in this manual, but you will

soon find that working with Layo1 is quick, easy and accurate and

that Layo1 gives you every chance to exploit your creativity and craftmanship

to the fullest extent.

-----------------------------------------------------------------------------

Mouse

If you want work with Layo1 for yourself after the <->demonstration,

you must first load the mouse driver before starting up. Please refer

to the documentation of your mouse.

======================  Layo1 Editor

-----------------------------------------------------------------------------

3.1 Layo1 Editor introduction

The interactive editor is the powerfully beating heart of the Layo1

program. Everything that you draw on the screen with the help of the

mouse is made into a data file by Layo1 that will later be able to

control a (photo)plotter or a drilling machine. Moreover, everything

in this file can be deleted or adjusted in the same interactive way.

The Layo1 Editor enables you to create and store components and subdrawings

and later use them for your print design. You can place pads, draw

tracks or have them made by the Autorouter, add texts, etc.

You can work on 15 layers with 16 definable pads at your disposal

which can be placed on a sixteenth layer. You can choose from 7 definable

pen diameters for the drawing of the tracks and 8 tool numbers for

your drill program.

While working you can move, delete, copy and rotate all components,

pads and tracks.

Layo1

Layo1 offers the extra facility of reading the net and component

list that you have made with the help of a scheme designer beforehand.

In this case practically all connections can be made by the Layo1

Plus Netlist Autorouter.

Inches or millimeters

You can choose to have your measures in inches or millimeters on the

basis of an optional grid; you can adjust the cursor/mouse stepsize

from 1 to 1/1280 inch.

Zoom

You may at almost any time enlarge or reduce parts of your drawing

on the screen by means of the middle mouse-key (zooming); the current

cursor position is always taken for the new center point of the screen.

Autozoom: zz

Double clicking the zoom button will instantly put your entire design

(once corners have been placed) in the center of the screen.

-----------------------------------------------------------------------------

3.2 Components/subdrawings

Apart from complete layouts for PCBs, Layo1 can also be used for the

design of components and subdrawings (e.g. standard power supplies).

There are important differences between layouts on the one hand and

components and subdrawings on the other. This will be dealt with in

greater detail in the subsequent sections, but it is important to

make the distinction here and now.

Corners

As long as no undamaged corners have been placed in a drawing to define

the dimensions of your PCB, Layo1 will see your design as a component

or a subdrawing. When saved on disk, the file will be stored under

*.LMC The asterisk stands for the name specified by you

(max. 8 characters).

Layo1 PLY FILE

Layo1 will itself place the corners according to the dimensions

that you specified in the CNF configuration menu of your design. A

Layo1 design will be stored as *.PLY. Again, the asterisk

stands for a name specified by you (max. 8 characters).

*.CNF

Therefore, Layo1 has a fourth type of file: *.CNF. In this file

all sorts of data concerning a design are stored via the PCB configuration

menu. This menu will appear as soon as you want to create a Layo1

Plus design.

The *.CNF file will keep record of the sizes of the final PCB and

its offset, the hole diameters of the different tool numbers, and

in which (sub)directories the different relevant files are stored,

such as *.LMC files, *.PLY files, the netlist, the component list

and the different libraries. These libraries (*.LIB) contain

the translation of the components from the scheme designer to the

Layo1 component databank.

Each designs CNF-file can be edited through OPTIONS in the menu line

of Layo1 Editor and whenever some file can't be found or loaded when

starting up, you'll probably will have to edit the CNF file to point

to the right files an directories.

-----------------------------------------------------------------------------

3.3 The menu

This is what the menu line of the Layo1 Editor looks

like:

The mouse (or cursor) will take you to your option and

pressing the left mouse-key (or Enter key) once will activate

it. If you do not want to make a selection and want to return to Layo1's

main menu, just press the right mouse-key (or Esc).

When you are working with the Layo1 Editor and the Layo1

menu line is not on-screen, you can return to this menu by pressing

the right mouse-key (or Esc) once or twice. Pressing F1

will display the exact function of each mouse-key or combination of

mouse-keys within every operational mode.

All menu options and their submenus will be dealt with in detail in

subsequent sections. Here, we will just give an overview.

File

This option will call up a submenu for saving, loading, <->renaming,

and deleting particular files (layouts). Also, it features a directory

function such as you know from DOS (dir), enabling you to scan directories

and subdirectories on disks. Please refer to section 4: Layo1 Editor

File.

Draw

This option enables you to draw on the different layers. You may want

to draw components or subdrawings, but it is equally possible to draw

tracks, lines, rectangles, <->circles, and ellipses. Anything that

is drawn in this way can also be deleted or adjusted within this menu

option. Please refer to section 5: Layo1 Editor Draw.

Pad

This option enables you to directly place, change, copy or delete

the 16 definable Layo1 pads. For more <->details, please refer to

section 6: Layo1 Editor Pad.

Autorouter

The Autorouter is one of Layo1's powertools. It will connect almost

any two points in your design quickly and effectively. In Layo1

it is used to make all necessary connections automatically, based

on a Netlist that was generated beforehand by using Calay compatible

scheme design software such as OrCAD SDT. Layo1's Autorouter can be

configured to perfectly suit the job and your needs and preferences.

Section 7 tells you all about it.

Multi <->Function

This option offers you a choice of four ways to edit a <->design and

manipulate (parts of) it. This manipulation can take the form of copying,

deleting, moving, or rotating parts of the drawing and of one or more

lines of the data file that you have created while drawing. For detailed

<->information, please refer to section 8: Layo1 Editor Multi Function.

Options

This option hosts a number of important auxiliary tools of Layo1.

They will be discussed in detail in section 9 (Layo1 Editor Options).

It includes:

Layo1 Toolbox offers a number of convenient facilities, enabling

you to mirror your design, to move it away from the corner (offset),

or to reduce or enlarge it to a different scale. You can also delete

layers or move data from one layer to another.

Drill group makes it possible to assign a particular tool number

with corresponding tool diameter to a type of pad.

Set grid puts all coordinates of pads placed and components

and traces drawn on the set grid.

Optimizing to transform all corners of tracks into 45� and

to delete all datalines that have become superfluous.

Edit design CNF enables you to specify and adjust various information

on a design in Layo1: dimensions and offset, hole diameters and

the different files relevant to the design (*.CMP, *.NET, *.PLY, and

*.LIB).

Display cam offers a survey of the number of drawing commands

and pads on every layer of your design and the number of holes to

be drilled per tool number.

Configuration will call up the Layo1 configuration menu. Please

refer to section 16 (Layo1 Setup).

Printer checkplot makes it possible to make a quick print of

your design on a matrix or HP-laser printer.

-----------------------------------------------------------------------------

3.4 Basic functions of the Layo1 Editor

While working with the Layo1 Editor you should be familiar with a

number of rules and basic functions. All these will be dealt with

in detail in the following subsections.

Screen

The top line on the screen gives continuous information on the active

function, the coordinates of the mouse position, the current layer

and the current stepsize of the cursor.

16 layers

In Layo1 you can work on no less than 16 layers. Layer 0 is reserved

for the placing of symbols (standard pads). A number of these layers

has special characteristics or is automatically used for particular

purposes.

Grid

For your convenience a 1/10 inch grid of small dots (which can be

switched off) is displayed on the screen.

Cursor

The Layo1 cursor consists of two cross hairs running over the entire

height and width of the screen or of a small arrow indicating the

cursor position. You can specify and adjust the form and color of

the cursor in the Layo1 configuration menu. The cursor stepsize can

be adjusted at all times, from 1 to 1/1280 inch, so that you can work

with the greatest accuracy possible. When the program is started up,

the standard stepsize is set to 1/20 inch.

The cursor stepsize can also be set in millimeters.

Autopan

When the cursor exceeds the screen limits, the screen will pan along

in any direction the cursor moves. This Autopan feature is very usefull

when working on larger designs. Double clicking the zoom button (middle

mouse key) will put your entire design (once corners have been placed)

back in the center of the screen.

Zoom

The zoom function of Layo1 (middle mouse-key) enables you to quickly

switch from an overview of the design to a particular detail. After

zooming the current cursor position is the new screen center point.

Also when not <->enlarging or reducing, the zoom function is a quick

way to move your design about over the screen.

Key functions

Many settings can be adjusted by means of simple keystrokes, and this

same simple procedure will often even call up different functions.

They are listed at the end of this section.

-----------------------------------------------------------------------------

3.5 The screen...

Whenever the Layo1 Editor is operational, the top line will supply

all necessary information relevant to whatever you are doing at that

particular moment. This will be dealt with in more detail in the following

sections.

Usually the top line will tell you which function of Layo1 is active,

on which layer you are currently working, and what the current cursor

coordinates are (in inches and millimeters).

While drawing, it will e.g. tell you which pen is currently in use.

Pu here means pen up: the pen is not <169>on the paper<170>.

When placing pads, the pad used and the relevant tool number can also

be found in the menu line.

An example:

Draw 110  p1  layer 2

This tells you that the draw function is active. By pressing the left

mouse-key the pen has been put on the paper, for draw is followed

by the number of the dataline and not by pu (pen up). <->Apparently,

we are working with pen 1 on layer 2. In addition the top line on

the screen will also indicate the current cursor coordinates in inches

and millimeters.

-----------------------------------------------------------------------------

3.6 Layo1's 16 layers

Layo1 enables you to draw on 15 layers, which are all displayed in

different colors (which you choose yourself). In Layo1 some layers

have particular <->restrictions or extra features. Make yourself familiar

with Layo1's layers and their recommended use and make up your own

standard procedures, for you will get a great deal more practical

use out of Layo1 if you develop your own layer routines.

Circles and ellipses

Please note that drawing circles and ellipses on layers

8 and up will yield the best results, because on the other layers

Layo1 only uses 90� and 45� angles.

===> Attention:

Layo1 is a high speed and acurate PCB design programm, which implies

that true circles and ellipses take up a lot of datalines!

Pen selection

On layers 1 ... 7 you can use all 7 pen diameters. On layers 8 and

up (for auxiliary lines) pen 1 is used exclusively.

Layer 0

On this layer all standard pads are placed. This is done automatically.

It is not possible to draw on layer 0.

Layer 1

Usually, the first layer is used for drawing the tracks on the top

side of the PCB. Only straight lines and 90� and 45� angles.

While drawing, this layer can be directly activated by pressing F7.

Layer 2

Usually, the second layer is used for the tracks on the underside.

Only straight lines and 45� and 90� angles. While drawing,

this layer can be directly activated by pressing F8.

Layers 3 ... 7

These layers can be used in any way you wish e.g. for multi-layer

PCBs. Only straight lines and 45� and 90� angles.

Layer 8

On layer 8 the shapes of the components are placed. You can do circles

and ellipses on this layer, as well as lines at any angle. Once pressing

the comma-key will remove this layer from the screen or call

it up. Only pen 1 on this layer.

Layer 9

This layer usually contains the texts for the PCB. You can do circles

and ellipses on this layer, as well as lines at any angle. Once pressing

the semicolon-key (;) will remove this layer from the screen

or call it up. Only pen 1 on this layer.

Layers 10 ... 12

These layers can be used in any way you wish. You can do circles and

ellipses on this layer, as well as lines at any angle. Only pen 1

on this layer.

ATTENTION SMB   FUNCTION   b  enter   SH_F2  see quickref.DOC

Layer 13

In Layo1 this layer is reserved for the display of faulty

connections. You are recommended not to use this layer for other purposes.

Only pen 1 on this layer.

Layer 14

In Layo1 this layer is reserved for the ratsnest. You are

recommended not to use this layer for other purposes. Only pen 1 on

this layer.

Layer 15

This layer is reserved for the corners that define the dimensions

of the PCB. You are recommended not to use this layer for other purposes.

Only pen 1 on this layer.

Selecting layers

While designing double or multi-layer PCBs with Layo1, you will frequently

want to switch layers. Layers 1 and 2 which are most frequently used

can be called up quickly by a single keystroke.

F7 = layer 1

F8 = layer 2

If you should want to use different layers, you can assign them to

function-keys F7 and F8. By pressing Shift F7 or

Shift F8 you are free to specify which layers you want to control

with these respective keys.

A second procedure, which allows you to directly select from all Layo1

layers, is pressing l (small L, for layer). A menu line will

be displayed from which you can select 1 ... 15 with your mouse. Put

your mouse over the selected layer and press the left key. If you

do not want to make a selection and want to leave everything as it

was, just press the right mouse-key.

When selecting a layer from the menu line you can also use the function-keys.

Here, F1 stands for layer 1. Layers 11 ... 15 can be selected

by pressing Shift F1 etc.

Each layer its own color

In Layo1 each layer has its own particular color, but you are free

to change these colors as you wish. To do so, <->select options from

the menu line, then select Layo1 configuration. Another way to do

this is selecting Layo1 Setup in Layo1's main menu. In both cases

the Layo1 configuration menu will appear. This will be discussed in

greater detail in section 16 (Layo1 Setup).

Note that the color that is selected for layer 15 is also the color

of the grid.

Selecting layers for display

Normally, Layo1 will display all layers and the grid together. It

may sometimes be convenient to switch off particular layers for a

while. The grid can be switched on and off simply by pressing the

. (full stop). Pressing the ; (semi-colon) will switch

layer 9, with the texts (if any), on and off. Layer 8 can be switched

on and off by pressing the ,-key (comma): this layer contains

the profiles of the components.

d

All layers can be selectively switched off by pressing the d-key.

The menu line shown hereunder will appear, clearly indicating which

layers are in use for the design and which layers are actually being

displayed.

By selecting a layer with the mouse and then pressing the left mouse-key,

you can switch that particular layer on and off. In this way you can

either select various layers or switch them off.

It is also possible to select all layers or switch all layers off

by a single keystroke: to do so you can use the a-key (all on)

and the c-key (all off).

By pressing the right mouse-key the menu line will disappear and you

can see the results. For the selection of a layer from the menu line

you can also use the function-keys. Here F1 stands for layer

1. Layers 11 ... 15 can be selected by pressing Shift F1 etc.

F5

It will often be necessary to rewrite the screen after this by pressing

F5: this will remove the 'shadows' of the layers that have been

switched off and you will be able to see exactly what the selected

layer or layers look like.

Interconnecting layers

Vias

Different layers of a design can be interconnected by means of so-called

vias. In fact, these are minute pads in which a hole is drilled, through

which the top and bottom sides are interconnected by means of metallisation.

These vias are very useful whenever it is not possible to put a track

entirely on one layer because this would make tracks cross with each

other. This problem can be solved by switching layers a number of

times and interconnecting the change-over points by means of a via.

F2 = via

While designing, a via can be placed (and removed) by placing the

cursor in the required position and pressing F2.

-----------------------------------------------------------------------------

3.7 1/10 Inch grid

For greater accuracy Layo1 features an on-screen grid consisting of

small dots (in the color of layer 15). The distance between these

dots is 1/10 inch. This 1/10 inch grid setting is also valid while

working in millimeters.

Grid on/off

The grid can simply be removed or called up again by pressing the

.-key (full stop).

Cursor on grid**

The cursor can be put on the nearest grid point by simply pressing

the *-key (asterisk) twice.

Pressing the *-key once will put the cursor on the nearest point

within the distance of the current cursor stepsize. This can be necessary

to readjust the cursor after <->changing the stepsize or switching

between millimeters and inches.

-----------------------------------------------------------------------------

3.8 Cursor

The Layo1 cursor consists of two cross hairs running over the entire

height and width of the screen or of a small arrow. While drawing

or placing pads and components the exact X/Y coordinates of the cursor

(the center of the two crossing lines or the point of the arrow) is

on continuous display in the top line of the screen in millimeters

as well as inches. The form of the cursor can be <->selected and adjusted

in the Layo1 configuration menu (from the main menu or via the option

options in the menu line of the Layo1 Editor). This is discussed in

greater detail in section 16 (Layo1 Setup).

Cursor stepsize

Another device for greater accuracy in Layo1 is the <->adjustable

stepsize of the cursor/mouse. This stepsize, again in inches or millimeters,

can always be adjusted while working.

Where this is applicable, the current stepsize is indicated in the

top line of the screen. It may be enlarged or <->reduced by a simple

keystroke:

F9 = smaller stepsize

F10 = bigger stepsize

The following standard stepsizes in inches are at your disposal:

1 1/2 1/5 1/10 1/20 1/40 1/80 1/160 1/320 1/640 1/1280

Define s<->tepsize

The stepsizes can be defined by yourself using Shift F9 and

Shift F10. Once you press Shift F9 or Shift F10 you

are free to specify ten stepsizes from which you can hence make your

selection via F9 or F10. When you exit from the program,

these settings are saved on disk for you to use again in a later session.

Diagonal auxiliary lines q

In some cases it is convenient to have diagonal (45�) auxiliary

lines in addition to the two cross lines for pinpoint positioning.

While drawing, pressing the q-key is enough to put these auxiliary

lines in the place of the cursor or to remove them again.

Square auxiliary lines a

Please refer to <169>diagonal auxiliary lines<170> above. However,

pressing the a-key will now put square auxiliary lines in the

place of the cursor or remove them again.

Cursor on grid**

The cursor can be put on the nearest grid point by simply pressing

the *-key (asterisk) twice.

Pressing the *-key once will put the cursor on the nearest point

within the distance of the current cursor stepsize. This can be necessary

to readjust the cursor after <->changing the stepsize or switching

between millimeters and inches.

Lost your drawing?

After all kinds of manipulations it may happen that you cannot get

your drawing back on to the screen: it is in an off-screen position.

The following series of keystrokes will help you: zz d a. The

first key will activate the zoom function, the second z or middle

mouse click will put your design in the center of the screen, d

and a will then display all layers.

-----------------------------------------------------------------------------

3.9 Zoom

With most functions of the Layo1 Editor the middle mouse-key is used

to enlarge or reduce the on-screen picture (zooming). Layo1 offers

you a choice of 9 zoom factors 1 ... 9. The bigger the zoom factor

the more detail; the smaller the zoom factor the more complete the

overview. Zoom factor 1 will show 1/10 inch per pixel (screen dot).

For zoom factor 9 this is 1/880 inch per pixel.

If, where applicable, the middle mouse-key is pressed, a menu line

will appear at the top of the screen showing the various zoom factors

1 ... 9. You can make your selection from these with your mouse. Pressing

the middle mouse-key is enough to activate the new zoom factor.

When the menu line is on-screen you can also select from the zoom

factors by pressing one of the function-keys F1 ... F9.

Cursor position as new center point

The cursor position defines the new center point of your drawing on

the screen. So, whenever you want to take a closer look or want a

wider view, put the cursor in the relevant place and press the middle

mouse-key before <->selecting the new zoom factor by pressing the

middle mouse-key or by pressing one of the function-keys F1 ...

F9.

Moving the picture

This facility may also be of great use whenever you do not want to

enlarge or reduce but simply want to move your drawing over the screen

a bit. Place the cursor on the required new center point of the screen

and press the middle mouse-key twice.

Keyboard

The latter can also be done by simply pressing the c-key, moving

the picture over the screen taking the current cursor position for

the new center point.

Zoom/move to zero point

The zoom function can also be activated by pressing the <->z-key.

You can then specify the zoom factor that you <->require, followed

by pressing the Enter-key. However, if after z you press

the n-key, the picture will be moved in such a way that the

zero point of your drawing will be placed in the top left corner of

the screen. In the latter case you may also change the zoom factor

first in the way described above, before pressing the n-key.

The same can be done with the mouse by selecting a zoom factor and

pressing the two right mouse-keys. The drawing will be placed with

its zero point in the top left corner of the screen.

-----------------------------------------------------------------------------

3.10 Working with Layo1 Editor

Once the Layo1 Editor is activated there is no submenu, but the different

mouse-key functions make it possible to activate a great number of

Layo1's important standard functions directly, avoiding the roundabout

route via the menu line and the different submenus.

===> Attention:

Please note that when in the Layo1 Editor a particular function has

been selected (e.g. drawing), the mouse will from then on behave as

if that function had been selected from the menu line. You are referred

to the relevant sections for more information.

F1 = help

While working, pressing F1 will always call up an overview of

the current mouse-key functions. Pressing Esc removes the overview

again.

Drawing

By pressing the left mouse-key you can draw immediately as if you

had activated the option draw from the menu line option draw

(this can also be done by pressing t). From this option the

mouse-key functions are nearly identical to those in draw. Use the

F1 key to get an overview of the new mouse-key functions.

===> Attention:

Please refer to section 5 (drawing) for additional information.

Placing Pads

The left and middle mouse-key will activate the option place pads

(this can also be done by pressing the s-key). The mouse-keys

will then behave in the same way as described in the section Pads

before (use the F1 key to get an overview of all current mouse-key

functions).

Moving pads

Once move pad is activated existing pads can be moved. Using

the left and middle mouse-key a pad can be <->selected and moved to

a new position, where it can be placed by pressing the right mouse-key.

===> Attention:

Please refer to section 6 (pads) for additional information.

To the menu line

The right mouse-key will take you, as usual, to the menu line on the

screen. You can also use the Esc-key for this.

Zoom function

As always in the Editor, the middle mouse-key will activate the zoom

function, enlarging or reducing the picture with the current cursor

position as the new center point of the screen.

After you have pressed the middle mouse-key a menu line will appear

with the different zoom factors from which you can choose. To select

a zoom factor you can also use the function-keys when the menu line

is on-screen. F1 .. F9 stand for zoom factors 1 ... 9. Double

clicking the middle mouse-key will put your entire design in the center

of the screen (once corners have been placed).

Defining a window

Pressing the outer mouse-keys will enable you to define a window (this

can also be done by pressing the w-key). In this window you

can <169>capture<170> a part of your design and then move it. If there

any traces entering the window from outside, these will automatically

be pulled along to the new position.

===> Attention:

Please refer to section 8 for additional information and relevant

mouse-key commands.

Load component

By pressing the right and middle mouse-keys, you activate the option

Load Component avoiding the roundabout route of Layo1's menu

line and the file and load options. Thus, you can quickly import components

and subdrawings from the databank and add them to your design. This

can also be done by pressing F3. When loading a subdrawing in

this way, Layo1 will first ask you whether you want to load this as

a subdrawing (in which the components can still be manipulated separately)

or as a complete <169>component<170>, in which the separate parts

can no longer be manipulated directly.

===> Attention:

Please refer to section 4 File/Load/ Component for additional information.

Search

Pressing all three mouse-keys simultaneously will activate the search

function which enables you to edit your <->design. This can also be

done by pressing the =-key when the word Advance is in the top

left corner of the screen.

===> Attention:

Please refer to section 5.3 for additional information and the

relevant mouse commands.

Placing vias

While working in the Layo1 Editor, vias can also be placed in the

cursor position by pressing the F2 key.

A via is Layo1's smallest pad: it will create an interconnection between

two sides of a PCB. This makes it possible to draw complicated traces

alternating on both sides of the PCB.

Vias (in the cursor position) can also be deleted by again pressing

F2.

Key functions

In the Layo1 Editor many functions can be <->directly used by means

of simple keystrokes. Please see to it that Shift Lock and Caps

Lock are not active, for Layo1 will make a critical distinction

between small letters and capitals here.

For the key functions, please refer to the relevant sections (draw,

pad, etc.) of this manual or refer to the overview in this section.

-----------------------------------------------------------------------------

3.11 Making a subdrawing MANUALLY

We can make excellent use of a subdrawing that is already present

Layo1's component databank to demonstrate the operational procedure

of all Layo1 program versions, NOT even using the powerful extra facilities

offered by Layo1 "New Design". It is a subdrawing of a power supply, which

can be used in many designs: PWR03.LMC:

1

The various components for this subdrawing were first drawn

and then stored as components in the component databank. Next, they

were retrieved from the databank and positioned (moved/ rotated).

2

Some components were first copied and, if necessary, rotated in

Multi Function.

3

Using the Layo1 Editor the pads were added that do not belong

to the components.

4

After everything had been put in its right place with Multi Function,

the tracks were created in the drawing function. In this case

an extra thick pen was used on layer 1 and the traces were

drawn aiming at maximal insulation.

5

The profiles of the components were already on layer 8. On layer

9 the texts are now added, via drawing, and the contour line

is drawn around the subdrawing.

6

Finally this subdrawing can be stored in Layo1's component databank,

from where it can be added on to other designs. You are free to retrieve

this power supply as a component or as a subdrawing. In the latter

case you can still rearrange its separate components to accurately

fit into your design. If you retrieve the subdrawing as a component,

it can only be manipulated (copied, moved, rotated, or deleted) as

a whole.

Zero point of a component

When saving a component or subdrawing the cursor position is very

important. Once a component is stored in the component databank on

the fixed disk, the last cursor position will determine its zero point.

When this component is retrieved from the component databank, it will

appear with its zero point exactly at the cursor.

===> Attention:

Therefore, you are recommended to always place the cursor e.g. in

the top left corner of a component before storing it on disk.

-----------------------------------------------------------------------------

3.12 Key functions

Overview of all key functions in the Layo1 Editor:

ANY UPDATE OR CHANGEMENT YOU MAY FIND UNDER THE F1 KEY AMD IN THE FILE

QUICKREF.DOC

Ins

Adds a dataline when drawing, editing and copying. Usually the left

mouse-key.

Enter

This command does not add a dataline but concludes a trace. Usually

the right mouse-key.    GENERAL IDEA: CONFIRM

Del

Deletes dataline/pad/coordinate in cursor position.

Delete entire track under the cursor

F1

Mouse help. Displays current mouse functions.

F2

Places or removes a via (connection between top and underside of the

design) in the cursor position.

F3

Loads a component from the databank.

F4

Saves layout or component.

F5

This function-key rewrites the picture: it is especially useful to

rewrite the picture after placing and removing traces.

F6

This gives the current cursor position the coordinates 0.0 which makes

it simple to measure the distance between parts of your design with

the cursor. You can reset the original coordinates by pressing

r    1

F7

Selects layer 1 (or a layer defined by yourself, cf. Shift F7).

F8

Selects layer 2 (or a layer defined by yourself, cf. Shift F8).

F9

Reduces the stepsize in inches. Not within the Autorouter where the

stepsize is fixed at 1/20 or 1/40 inch.

In millimeters: selects the defined stepsize (10 steps) with Shift

F9.

F10

Enlarges the cursor stepsize in inches. As F9.

Shift F5

Switches between overlapping and mixed color display.

Shift F7

Selects a layer that can then be activated with F7.

Shift F8

Selects a layer that can then be activated with F8.

Shift F9

Defines the stepsizes in millimeters.

Shift F10

Idem.

Ctrl F1

Resets ratsnest (removes layer 14) in Layo1.

Ctrl F2

New ratsnest of the pad in the cursor position, with new calculated

shortest connections in Layo1.

Ctrl F3

New ratsnest of component at cursor, with new calculated shortest

connections in Layo1.

Ctrl F4

Updates the ratsnest in Layo1. All made connections are

removed from the ratsnest, so that you can easily pad which connections

have not yet been made.

Ctrl F6

Checks all connections according to the netlist in Layo1.

Faulty connections are marked with a small cross on layer 13.

CHANGEMENTS &UPDATES IN QUICKREF.DOC

a

Places or removes horizontal and vertical auxiliary lines on the screen.

b enter

Selects a block (component) in order to move or rotate it.

c

Centers the screen in the cursor position.

d

Displays the use of the layers. You can specify for each layer whether

or not it is to be displayed. You can do this with the mouse or with

function-keys F1 ... Shift F5. To display all layers first press

d, then a. To switch off all layers, first press d,

then c.

f

Centers cursor on the screen when the cursor is free (not while drawing,

editing and placing pads).

i

Switches from millimeters to inches.

l

Selects one of the layers. Selection can then be made with the mouse

or with function-keys F1 ... Shift F5.

m

Switches from inches to millimeters. (  > toglle will follow mm )

n  Toglles display of pad-numbers

Answers no to yes/no questions.

p

Selects pen number/diameter. Selection can then be made with the mouse

or function-keys F1 ... F7.

When moving: new zero point.

q

Places or removes diagonal auxiliary lines in the cursor position.

r

Ruler selection. You can select one from settings 1 ... 3:

1: sets zero point back to original 0,0 coordinate (after r 2, r 3)

2: sets zero point to coordinates of top left corner

3: sets zero point to current cursor position

In fact, Layo1 has 6 rulers because the three rulers mentioned above

are all kept up-to-date in inches as well as millimeters.

s     ss

While placing pads s: select a pad s again. Selection can then be made

with the mouse , arrows

The s-key is also used to activate the function for placing

pads. Once this function is active, this key works as described above.

t  (after s or =)

While placing pads or editing: select tool number. Selection

can then be made with the mouse or arrows keys

From the Layo1 Editor the t-key is used to activate the draw

function of Layo1.

w

Activates the window function in the Layo1 Editor, draw and

elsewhere, and simultaneously sets the starting point to the current

cursor position. Then, it is similar to pressing the two outer mouse-keys,

in which latter case it is still possible to select the starting point

with the cursor. When working in the Autorouter pressing the w-key

will make a special window appear.

x

Change the cursor position on the X-axis by typing the new position.

y

Idem, but now for the Y-axis.

Answers yes to yes/no questions.

z

Zoom setting (n = normal/Enter = center). Usually this

function is activated by pressing the middle mouse-key.

zz

Pressing the z-key twice or double clicking the middle mouse-key

will activate Autozoom, which puts your entire design on screen (only

when corners have been placed).

=

In the Layo1 Editor: searches a drawn item, rather near the cursor, in

order to edit this item by means of commands to be specified. Simultaneously

pressing the three mouse-keys has the same function. Cf. Edit.

+

Pages forward through datalines while editing a design. Usually, the

two left mouse-keys are used.

-

Pages backwards through datalines while editing a design. Usually,

the two right mouse-keys are used.

NOTE: iN THE = MODE:  If you want to add a dataline/vector with INS,

type one time - first. Mext  INS Arrow-keys. The vector is temporarely glued

to the cursor untill you drop it wit Enter or another INS.

(Observe the difference when you didn't type INS first. Do some exercising

on an empty screen with some tracks only.

The idea is that if you neglected the '-' before typing INS the vectorpoint

will be placed immediately but will not be 'glued' to the cursor to bring it

elsewhere.

*

(Asterisk) Places cursor on nearest point within the current stepsize.

**

(Two asterisks) Centers cursor on nearest grid point.

,

(Comma) Switches component imprint (layer 8) on/off on the screen.

;

(Semicolon) Switches layer 9 (texts) on/off on the screen.

(Full stop) Grid display on/off.

Esc

Concludes current activity. This will usually take you to the main

menu. This command is usually given with the right mouse-key.

-----------------------------------------------------------------------------

3.13 Save = safe

F4

A power failure can destroy hours of hard work in a split second.

Therefore, make it a habit while designing to save your design on

disk regularly. Pressing F4 every 10 minutes can limit the damage

to 10 minutes' work. A small effort to save you from big problems.

Auto save

In Layo1's configuration menu you can specify the frequency with which

you want the program to automatically save your design while you are

working on it. Then you can allow yourself to forget all about saving

for safety. Layo1 will take care of that for you and ask you at set

intervals whether you want to save yes or no. Answer yes and Layo1

will safely save your work on disk and you can go on with your work.

Answer no and you can go on straight away. The Layo1 configuration

menu can be called up from the menu line under Options or via

Layo1's main menu (Layo1 Setup). Please refer to section 16 for more

information on Auto save.

Backup

AS ALWAYS, IT IS HIGHLY RECOMMENDABLE TO MAKE BACKUP DISKS OF

ALL YOUR WORK REGULARLY.

**************************************************************************

======================  Layo1 Editor File

-----------------------------------------------------------------------------

4.1 File

If you select file from the menu line an additional submenu will appear

from which the following selection can be made by pressing the left

mouse-key.

F1 will not give information on the mouse-key functions within

the submenu of the file function. Please refer to section 4.3.

===> Attention:

When loading or saving files the file extension (e.g. .LMC and .PLY)

need not be typed: Layo1 will see to that for you.

Only LML is optional (in Menu � Options � Edit CNF)

-----------------------------------------------------------------------------

4.2 Types of files

Read STRUCTURE.DOC

Back up

OF COURSE, IT IS HIGHLY RECOMMENDED TO ALWAYS

BACK UP ALL YOUR WORK ON DISKS.

===> Attention:

Once you start to create files for a Gerber photoplotter,

Layo1 will create a separate subdirectory, GERBER\

NEW DESIGN

Once you start to make a design in Layo1, a CNF file will be placed

in the CNF\ subdirectory containing all information relevant

to that design. But first you must give a name to your design of max.

8 characters (keeping to DOS file name regulations), including a path

name. In the beginning you are recommended to use the subdirectory CNF\ of the Layo1

directory. The PCB configuration menu will be displayed for you to

specify the dimensions of your design and the diameters of the different

tools that will be used in production. You will also specify in this

menu which other files are used for this design.

4.2.0.1 Dimensions

You must specify the X/Y dimensions of your design so that the corners

can be placed. Maximum sizes are 650 x 650 mm (22.5 x 22.5 inch).

In addition, you may specify an X/Y offset.

These dimensions may be changed at a later stage by calling up the

Edit Design CNF menu again via the selection options from the

menu line.

New Offset can also be specified in TOOLBOX.

You can change ofset also in the plotmenu of TRACEURS.EXE  � F9.

Inch/mm

If you want to specify dimensions, Layo1 will ask whether you want

to do so in metric units (millimeters) yes or no. You are free to

specify the dimensions either in millimeters or in inches.

4.2.0.2 Hole diameters

Layo1 can handle 8 hole diameters for you which you can define to

your own liking. While placing pads ( s mode) you can select these hole    diameters

(tool numbers) by pressing the t-key or the two right mouse-keys.

4.2.0.3 File

In this field you specify where the relevant files are to be found

or where they are to be stored later. Extensions (.PLY etc.) are not

necessary.

LML optional

PLY

Here you specify the name of your design.

CMP

Here the  name of the component list must be

given.

NET

Here the name of the netlist must be specified.

IN USER MENU:

LIB1 etc.

Here you can specify THE  name of the different library files.

These files contain the information Layo1 needs to find the components

that are in the component list.

*.CNF

Next, a *.CNF file is stored in the CNF\ subdirectory. The asterisk

stands for any name you may have given to your design. You can change

the directories indicated to your own requirements.

The .CNF file contains the information that you specified in the     configuration

menu. The .PLY file contains the actual design and will only be

stored at the moment that you give the command to do so from the Layo1

Editor. The .PLY file contains the coordinates, the electrical

data and the component numbers.

Layouts

The layout files (the designs) can be stored in Layo1's subdirectory

LML\ with the extension .LML or\librar

.PLY

after the name given by yourself.

LML

From this file your design can

be sent to the photo/penplotter or punch tape.

Layo1

Layo1 will save both a .PLY and an optional LML file

Library

The *.LIB files are stored in the LIB\ subdirectory. They contain

a translation table that will refer to the right components in Layo1's

component databank when reading a component list from a scheme designer.

These library files can be created and edited using an ordinary word

processor. They take the following form:

SN7400

SN7401

SN7402

=LMC\ACTIVE\DIL-14

Netlist

The netlist and the component list that can be made with the help

of a scheme designer (e.g. OrCAD SDT) for a new design must be placed

in Layo1's subdirectory Netlist\.

Or use  your texteditor

Or make the list witn INS on the pads in the EDIT NET mode

Netlist

The netlist contains all connections. The file has the extension .NET.

Component list

The component list contains a list of all components

that are to be used in the design. This file has the extension .CMP

after its name.

LMC

The subdirectory LMC of Layo1 contains an extensive databank

with standard components and subdrawings to which you can add as many

as you like. All files (components) in this databank have the extension

.LMC after their names.

Not counting the capacity of your fixed disk, Layo1 may in principle

contain an unlimited number of components. There are hundreds of components

that are part of the standard Layo1 package. Therefore, LMC has

again been subdivided into several subdirectories.

Demo

The component databank of the demo version of Layo1 is limited

to the components that are used in the <->demonstrations.

Gerber  TRACEURS.EXE

The Gerber  subdirectories are only created by Layo1 at

the moment they are needed. As soon as a file is created for a Gerber

or Glaser photoplotter the relevant subdirectory is created (if not

already existing) and aN APERTURE file will be stored in it AND A GERBER FILE.

This file containing the plotter specifications

will usually be put on a floppy disk so that it can be sent to a PCB

manufacturer or a plotter service agency. If you want to, you can

put files for both a Gerber and a Glaser plotter on the disk, because

for each type a subdirectory is created in order to keep file storage

well- <->organized and efficient.

the file GERBER.DAT in \LAYO1EE\ may be modified and the data will be used

in TRACEURS.EXE

-----------------------------------------------------------------------------

4.3 Introduction

All options will be discussed extensively at a later stage in this

section. For now, just an overview:

Reset Layo1

The current design is deleted and you can make a fresh start.

Load

This option is used to load any type of Layo1 file from disk through

a special submenu.

Save

The save option is used to save your current work to disk.

New Design

Reset Layo1 and start a new CNF design. WITH CMP-NET

Don't use for manual work

Directory

Displays the directory (list of contents) of the files on the fixed

disk or floppy disk.

Rename

An existing file or design can be given a new name.

Delete

You can select a file or design and remove it from fixed or floppy

disk.

Quit

Exits from the editor and takes you back to Layo1's main menu. Of

course, for safety's sake there is a check question if you really

want to do this and if you would not like to save your work.

' /quit ' exit wihout saving !!!!!

Specifying a file name

Frequently you will be requested to specify a file name, e.g. when

loading or saving a design. Layo1 lends a hand by keeping the ten

files that were last used in mind for you.

4.3.0.1 Selection list

Once Layo1 asks you for a file name pressing the middle mouse-key

is enough to see an on-screen selection list of files from which you

can make your selection with the mouse. The left mouse-key will select

the file of your choice. The right mouse-key will help you leave this

<->selection menu so that you can type a different name yourself.

F7

Instead of the left mouse-key you may also use F7 here.

The selection list will show the ten files that you used last of the

type that you require (*.CNF / *.PLY / *.LML / *.FNT / *.LMC) If you

have not yet used files of the type that you require, there will be

no selection list and you will have to enter the name of the file

yourself. You need not enter the extension after the name.

4.3.0.2 Directory

If you need an overview of all files and various directories when

you are asked for a name, you must press the left mouse-key. A complete

overview will be displayed of all files (in red) in the current directory

and all subdirectories (in black). From here you can also go back

one <->directory (..) or all the way back to the root directory (.)

of your disk. In this way you can page through the whole disk.

Please refer to the previous section on types of files and Layo1's

subdirectory structure.

-----------------------------------------------------------------------------

4.4 Reset Layo1

This menu option will clear screen and memory so that you can make

a fresh start on a new design.

If there is an existing design on-screen, Layo1 will for safety's

sake ask you if you really want to start with a clean slate. You can

select either yes or no. Only if you point at yes

with the mouse and press the left mouse-key the screen will be cleared

and you can start anew. Yes can also be selected by pressing

the two left mouse-keys and no by pressing the two right mouse-keys.

In all other cases Layo1 will just return to the program.

No mouse

The selection of either yes or no can also be made using

the keyboard by pressing y or n; or by using the arrow

keys to go to your choice and pressing Enter.

-----------------------------------------------------------------------------

4.5 Load

This option enables you to load a lAYO1 file from disk

Layo1

The submenu as shown above is what you will see using Layo1.

Component

Will load a component from Layo1's database into the design on screen.

Drawing

A LML layout can be loaded after renaming it to LMC.

Design

This option loads a Layo1 design (PLY) from disk with all other

involved files (LIB / NET / PLY / CMP), through the CNF file.

Component list

To load a componentlist to be specified (in .CNF). This componentlist

should have been created with F3, or with a text editor not producing control characters or generated by a Calay compatible scheme designer (e.g.

OrCAD SDT III IV).

Netlist

To load a netlist to be specified (in .CNF). This netlist should have

been created with OPTIONS � Edit � NET, or with a text editor not producing control characters or  generated by a Calay compatible scheme designer (e.g. OrCAD SDT).

Textfont

This option enables you to load another (user designed) textfont for

use within the Layo1 Editor.

Selecting a file

In all cases Layo1 will ask you in the top left corner for the name

of the file that you require ....

C:\LAYO1EE\...

C:\LAYO1EE\... in this example will in reality be the disk and

directory where the program has been installed.

Having chosen for a certain type of file in the submenu a filename

extension is not wanted as Layo1 will only look for files with the

right extension ( CNF / LMC / LML IS NOT ACCEPTED RENAME TO LMC).

Layo1 remembers the names of the 10 files last used of each type.

When you are asked to specify a file name, you can call up the names

of the 10 files last used by pressing the middle mouse-key. Then you

can make your selection.

Pressing the left mouse-key will call up a complete directory overview.

Please refer to section 4.2 and 4.3 for information on types of files

and Layo1's directory structure.

Load component BNK

The menu option Load component enables you to retrieve components

or subdrawings from the databank and add them to your design. Afterwards,

components can be ma<->nipulated as a whole (moving/rotating/copying/deleting);

subdrawings are loaded as separate parts that can also be manipulated

separately on the different layers.

Thus, it is possible to import the subdrawing of e.g. a frequently

used power supply section as a whole, while its separate parts can

still be moved.

4.5.0.1 Finding a component/subdrawing

Layo1 will sometimes ask whether the .LMC you select has to be loaded as

a component or a subdrawing.

Component

A component will be loaded as a single block that can only be manipulated

entirely.

Subdrawing

A subdrawing consists of seperate elements and components. When Layo1

encounters a subdrawing you will be asked whether you want to load

it as such or as one single block.

When loaded as a subdrawing (single blocks) you will be able to rearrange

the elements that make up the subdrawing, whereas when loaded as a

single block (component) you can only manipulate it as a whole. Layo1

leaves the choice up to you.

4.5.0.2 Placing a component/subdrawing

Once a component or subdrawing is selected it will appear on screen

at the cursor position by the original zero point. Now set the cursor

at the required position and place it by pressing the left mouse-key

(or the Ins-key). Once you have finished placing, press the

right mouse-key or Esc. If you want to place more copies of

a component, place the first component by double clicking the left

mouse-key. You can now place as many copies as you like (watch the

counter in the upper left corner of the screen) untill you abort the

operation by pressing the right mouse-key.

4.5.0.3 Rotating a component

By pressing the two outer mouse-keys (or function-key F2) you

can rotate the component in steps of 90� clockwise (as long as

the word <169>move<170> is in the upper left corner of the screen).

The original zero point of the component is the point of rotation.

4.5.0.4 Original zero point

When a component is created and added to the component databank, the

cursor position of the moment that it was saved will be the original

zero point of the component

When the component is loaded, this will be the point with which the

component will be placed at the cursor position.

Load textfont

FNT

For the placing of texts on a PCB Layo1 features a stan<->dard font

which can be enlarged, reduced, rotated and mirrored in the design

(cf. Draw/Text).

However, you can use Layo1's textfont editor (cf. section 14) to design

several different fonts for yourself.

The menu option as shown above offers you a choice from different

fonts. However, if you have not yet created and stored fonts yourself,

there is little choice of course. The only file that Layo1 will find

in that case is FONT1.FNT in the subdirectory FONTS.

*.FNT

When loading fonts Layo1 always searches for files with the extension

.FNT. However, these need not be stored in the FONTS subdirectory.

It is up to you where you actually want to store them. You must not

type the extension .FNT after the name.

-----------------------------------------------------------------------------

4.6 Save

The menu option Save will safely transfer the current design

from the computer's RAM memory to fixed disk or floppy disk. You are

recommended to use this <->option regularly while working on a design.

It will prevent you losing many hours of work because of a power failure

or other mishaps. Even if you make a bad mistake while working on

a design and delete parts of it, there will always be the latest back-up

version to go back to. The more recent it is, the less work you have

to do all over again.

When this menu option is activated, Layo1 will show the most recently

used name and will ask for your permission to overwrite this version.

===> Attention:

However, when working on a .PLY design in Layo1, Layo1

will not ask for your permission and will overwrite the existing version

straight away. This is due to the fact that the right file names are

in the .CNF file! If you want to preserve the existing version, you

must first rename it with the rename option.

If you want the design saved under the current name and just go on

working, there are two faster ways in Layo1 to regularly save your

work.

Auto save

In Layo1's configuration menu you can specify the frequency with which

you want the program to automatically save your design while you are

working on it. Then this will be looked after for you by Layo1, although

Layo1 will ask for your permission before saving the current design.

You can call up the configuration menu from the menu line via options

or via Layo1's main menu (Layo1 Setup). Please refer to section 16

for more details on Auto save.

F4 = save

If you want to decide for yourself when to save your <->design on

disk, F4 is the easiest way to do so. Pressing it once will

safely save your design and you can go on working straight away.

In order to avoid the unintentional loss of your design, Layo1 will

always ask you for a confirmation when you quit the program in whatever

way without having saved your latest changes first.

Back up

AS ALWAYS, IT IS HIGHLY RECOMMENDABLE TO MAKE BACKUP DISKS OF

ALL YOUR WORK REGULARLY.

-----------------------------------------------------------------------------

4.7 New design

This option offers a fast way to start a new design. Of course Layo1

will ask you if you are sure and want to reset Layo1. If you are sure

you will prompted to give a name for your new design.

Layo1

When working in Layo1 the PCB configuration menu will appear

on screen. Please refer to section 4.2.1.

-----------------------------------------------------------------------------

4.8  Directory

This menu option is similar to the DOS DIR command that you know,

but it is easier to use and it is mouse-driven. Moreover, it presents

the files in alphabetical order. Thus, all directories and subdirectories

of all disks are directly accessible.

Please refer to the introduction, section 4.2.

-----------------------------------------------------------------------------

4.9 Rename

Any file on any disk can be given a new name with this menu option.

For the selection of the file to be renamed you can use Layo1's powerful

mouse-driven directory function.

Please refer to the introduction, section 4.2.

-----------------------------------------------------------------------------

4.10 Delete

Removing any file from any disk is equally well possible as renaming

it. For the selection of the file to be deleted you can use Layo1's

powerful mouse-driven directory function. This is an easy and quick

procedure: you must be careful not to delete the wrong file by accident.

Please refer to the introduction, section 4.2.

-----------------------------------------------------------------------------

4.11 Quit

When finished working on a design, this is the way to quit designing.

For safety's sake Layo1 will ask you if you really want to exit. If

you do not explicitly answer yes by clicking yes with

the left mouse-key, you will stay wherever you are with no harm done.

Yes can also be selected by pressing the two left mouse-keys.

Pressing the two right mouse-keys stands for no in this case.

Whenever you are asked a yes/no question, you can always answer in

this same way.

Keyboard

The selection of either yes or no can also be made using the keyboard

by pressing y or n; or by using the arrow keys to

go to your choice and pressing Enter.

**************************************************************************

======================  Layo1 Editor Draw

-----------------------------------------------------------------------------

5.1 Draw introduction

When you select draw from the menu line, there will <->appear a submenu

under the menu line from which a next selection can be made by pressing

the left mouse-key.

F1 = Help

Whenever you need information on the functions of the mouse-keys while

drawing, just press F1 and you will be presented an overview

of all key combinations and their functions.

In a nutshell ...

At a later stage in this section all options will be discussed in

detail. For now, just an overview of the draw functions.

Draw

This option enables you to draw components, lines and tracks. You

can select the pen diameter and the layer that you require.

Edit

This option makes it possible to edit existing lines and tracks.

Copy

The same as Edit, but the lines and tracks selected here can be copied

to other places.

Delete

Lines and tracks are removed from the design.

Text

With the help of this option you can add your own text to the design.

In doing so, you can accurately define size and position.

Box

This option draws a box on the screen. Very convenient for drawing

components, or for keeping free a part of the PCB on one or more layers

when using the Autorouter.

Circle/ Ellipse

Draws a circle or ellipse within a square/rectangle to be defined.

This is best done on layers 8 and up and only with pen 1. You

can select the layer you want by pressing small letter L on

the keyboard, after which you can <->select the layer you require

with the mouse or the cursor. If you attempt drawing a circle or ellipse

on layers 1 ... 7, Layo1 will automatically display the layer selection

menu line.

Corners

This option defines the final corners of your PCB. Only after this

has been done Layo1 will recognize your design as a PCB design. As

long as no corners have been placed your design is viewed as a subdrawing

or component.

From any menu option under draw it is possible to activate two powerful

Layo1 functions with your mouse: edit and window.

Edit function

This is activated by pressing all mouse-keys simultaneously or by

pressing the =-key. In this way you can edit parts (datalines)

of your design. This function is similar to the menu option edit.

It is discussed in section 5.3.

Window

The window function is called up by pressing the outer mouse-keys

or the w-key. It enables you to capture part of your drawing

in a window in order to move it. It is similar to the function Window-move

in Multi Function. Rotating is not possible in this case.

This function is discussed in detail in section 8.5.2.

Rewrite screen

F5

While drawing, but especially after making changes in a drawing or

deleting and copying, there may be all sorts of remnants and shadows

on the screen. Pressing F5 will <->rewrite the screen completely,

so that you know what you are doing again.

-----------------------------------------------------------------------------

5.2 Draw

This will activate one of Layo1's most powerful functions, for drawing

and designing with Layo1 is quick, easy and accurate. You can use

it to create components and subdrawings and to draw tracks, but you

can also edit them again straight away.

Drawing may well be the most basic function of Layo1: therefore it

can also be called up directly from the Layo1 Editor by pressing the

left mouse-key.

Different layers

While drawing you can switch layers all the time (by pressing small

letter L on the keyboard).

Layers 1 and 2

As most tracks are drawn on layers 1 and 2, these can be quickly selected

by pressing function-keys F7 and F8, <->unless you have

assigned different layers to these keys using Shift F7 and Shift

F8.

Pen diameter

Even while drawing it is possible to adjust the pen diameter to your

needs (small letter P on the keyboard). This will call up the

pen selection menu line from which you can make your choice with the

mouse or the corresponding function-keys F1 ... F7. Your selection

is then indicated in the top line of the screen.

Drawing

Drawing tracks is a fast and easy procedure in Layo1 <->because it

is mouse-controlled. Place the cursor (cross lines) at the point where

the track is to begin and press the left mouse-key. Move the cursor

to the next point and again press the left mouse-key. Go on like this

till you reach the end of the line: here you press the right mouse-key.

Should you have forgotten a connection or should you want to adjust

the track, you can always go back into the track with your <169>drawing

pen<170> and still adjust it or even remove it.

Once the pen is on the paper, the mouse-key functions are somewhat

different:

Example

For drawing a box, first place the cursor in the initial position.

Press the left mouse-key, move the cursor to the next corner, press

the left mouse-key again and go on like this until you come to the

starting point again. There press the right mouse-key. Now the box

is ready and you can start drawing again in some other place. This

example is just to demonstrate the easy procedure, for Layo1 has an

even quicker way to draw a box by means of only two mouse clicks.

Shortest route

Layo1 will always take the shortest route from A to B, but will on

layers 1 ... 7 only use straight line-sections with angles of 45�

and 90� relative to the axes. The example will show you how this

works.

The line is begun at A by pressing the left mouse-key. The pen is

put on the paper and is drawn to C with the cursor. The angle at B

is made by Layo1 itself. At point C the line can be concluded by pressing

the right mouse-key which will lift the pen up from the paper. If

you want to go on drawing from C straight away, you must press the

left mouse-key. The line is then fixed to point C and you can go on

to D etc.

If you want B in another place, follow this procedure: start drawing

again in some other part of the screen. Place A by pressing the left

mouse-key and move the cursor to C. Now press the two outer mouse-keys

Drawing backwards

The pen is still on the paper and you can now draw backwards in the

previously drawn line. Move the cursor to the left, back into the

line and you will notice that the points A and C will remain fixed,

but that you can <->manoeuver the line in between into all sorts of

twists. By pressing the left mouse-key or the two left or right mouse-keys

you can fix the required pattern in the line. If you press two mouse-keys,

the cursor will jump to the next or previous point in the line.

Inserting

Once you have defined all new angles Layo1 has created (inserted)

as many datalines. You can now go forward and backwards through the

drawn line via the data points, with the + and <196> key

respectively or by pressing the two left (forward) or right (backwards)

mouse-keys.

Forward/ backwards

The cursor jumps from point to point and you can see the dataline

counter in the top left corner of the screen go up or down. Any point

on which the cursor is placed in this way can be moved. As soon as

you jump to the next or previous point the adjustment is fixed.

Once you are ready with your adjustments you go back with the +-key

to point C and you can go on drawing.

F2 = via

A via is the smallest pad in Layo1. Its main purpose is to interconnect

tracks on two sides of the PCB by means of a soldering joint. In complicated

designs a via may often be an easy way-out. A via is placed (or deleted)

in the cursor position by pressing F2.

Layo1

Free lines

When working on a .PLY design in Layo1, you can only draw

tracks on the protected layers 1 and 2 that have their starting point

on a pad or on an existing trace that is present in the netlist that

has been read. Free (unconnected) tracks cannot be drawn on these

layers.

If the design has been created by means of a netlist and component

list, layers 1 and 2 are protected. If you try to begin a track on

pads that belong to pins of components which have been read in, this

will result in a message. An example of such a message in the top

of the screen:

NET 5  6 PINS  3 SUBNETS

This line tells you to which connection number (cf. netlist in Appendix)

that point belongs, the number of pins <->belonging to it and the

number of subnets.

===> Attention:

Before you can start drawing a track you must double-click the left

mouse-key on the starting point again.

5.2.0.1 Faulty connections

Layo1

If you use the Layo1 Editor on the basis of a rats<->nest, Layo1

Plus will check whether the connections made by you are all right.

If a faulty connection is made, it is marked with a small cross on

layer 13 in the place of the relevant pin.

Ctrl F6 will display the faulty connections.

-----------------------------------------------------------------------------

5.3 Edit

This function can be activated via the menu option draw in the

menu line, as indicated above. However, it can usually also be activated

directly from the other drawing functions and from the Layo1 Editor

(by pressing all three mouse-keys or the =-key).

If edit is activated when the cursor is on a line-section or a pad,

this will start blinking. Layo1 will ask you if this is the one that

you wanted...

Found? yes/no

If this is not the line or pad you wanted, press the right mouse-key

and start anew. You can also make Layo1 continue searching in the

same place by pressing the middle mouse-key.

If it is the right element, you can confirm this by pressing the left

mouse-key. The mouse-key functions will now differ from the above

to some extent.

In the top left corner of the screen there is the word edit

followed by the number of the dataline in which the <->selected element

of your design is stored. By using the + and <196> keys

or the two left or right mouse-keys you can go forward and backwards

through the datalines. Thus, you can page through the datalines of

the program that you have created while drawing. All lines, tracks

and pads form datalines and are stored in the same order as you have

created them. Thus, by paging through the datalines, you can check

everything that you have done in the same order in which you did it.

Once you arrive at line-section that you wanted to edit, you can manipulate

it with the mouse until it is to your liking. Pressing the right mouse-key

will conclude the procedure.

Insert-

Please refer to section 5.2.

F1 = help

Dependent on the way that you activated the edit function the mouse

functions may vary somewhat. Pressing F1 will always call up

an overview of the current mouse-key functions. Pressing Esc

removes the overview again.

F5

If you have made any changes in your design, you are recommended to

press F5. This will rewrite and update your on-screen picture

completely according the new situation.

Shift F5

Shift F5 switches between overlapping and mixed color display:

this may make it easier to perceive the starting points and finishing

points of tracks (datalines).

Layo1

If you start editing in a design made on the basis of a netlist, you

will notice that, as soon as you select a pad or a trace, the pads

and tracks connected with it will also start blinking: now you can

clearly see what consequences your editing may have.

In addition, the top line of the screen will give you information

on the relevant point, e.g.:

NET 5  6 PINS  3 SUBNETS

This line tells you to which connection number (cf. netlist in Appendix)

that point belongs, the number of pins belonging to it and the number

of subnets.

-----------------------------------------------------------------------------

5.4 Copy

This menu option is also activated by pressing the left mouse-key.

Next you place the cursor on the line-section that you want to copy

and press the left mouse-key. The selected line-section will start

blinking and Layo1 will ask you:

Found?     yes / no

If this is not the line you want, press the right mouse-key or press

the middle mouse-key to make Layo1 go on searching in the same place.

If it is the right line section, press the left mouse-key. The mouse-key

functions have now been adjusted to the new situation as will appear

from the following illustration. The selected line-section will now

move along with the cursor: you can now move it to the required position

with the mouse. Pressing the left mouse-key once will place the line

in the new position. This can be repeated until you press the right

mouse-key to conclude this procedure.

You can now select a different line-section for copying in the same

way or you can exit from this function by pressing the right mouse-key,

which will call up the menu line again.

In addition to lines and tracks you can also copy other elements in

the same way, such as circles, ellipses, boxes and texts.

-----------------------------------------------------------------------------

5.5 Delete

This menu option is also activated by pressing the left mouse-key.

Next, you place the cursor on the line-section that you want to delete

and press the left mouse-key. The selected line-section will start

blinking and Layo1 will ask you:

Found?     yes / no

If this is not the line you want, press the right mouse-key or press

the middle mouse-key to make Layo1 go on searching in the same place.

If it is the right line section, press the left mouse-key. The selected

line-section is deleted.

You can now select a different line-section for deleting in the same

way or you can exit from this function by pressing the right mouse-key,

which will call up the menu line again.

In addition to lines and tracks you can also delete other elements

in the same way, such as circles, ellipses, boxes and text.

-----------------------------------------------------------------------------

5.6 Text

This menu option enables you to add texts to your design. Again, this

option is activated by pressing the left mouse-key.

In the top left corner of the screen there will appear an <->indication

that you are about to place text. The mouse-keys are as follows:

Draw

Press the left mouse-key in order to place text. If you have not placed

any text since Layo1 was last started or since the last reset, Layo1

will ask you first which font you want to use.

This is because it is possible in Layo1 to design and store your own

fonts (and/or logos) in addition to Layo1's standard font. Cf. section

14.

In the top left corner you will be asked for the required .FNT file

and the full DOS path name (i.e. including <->directory and any subdirectories).

However, pressing the middle mouse-key will call up a selection menu

with the .FNT file names that are present in Layo1's FONTS subdirectory.

From this selection menu you can make a choice with the left mouse-key.

Once your font has been selected, Layo1 will ask you to type the text

you require. Conclude your text by pressing Enter or CR.

The text that you type will now be attached to the cursor. Before

positioning the text and placing it with the left mouse-key, you are

still free to position, enlarge, reduce, rotate and/or mirror the

text. Cf. hereafter.

Zoom

Here too, you can use the zoom function (middle mouse-key) to move

the picture of your design over the screen, or to enlarge or reduce

it.

The zoom function cannot be used once you have typed text and this

text is still <169>attached<170> to the cursor.

Select pen diameter

Before placing the text, when it is still attached to the cursor,

you can still adjust the pen diameter if you want to by pressing the

p-key. This will call up a menu line with the different pen

diameters.

Select layer

Preferably, text is placed on layers 8 and up (layer 9 is recommended).

You can select the layer you want by pressing the l-key and

then selecting the required layer with the mouse or function-keys.

Enlarge text

You can enlarge your text by pressing the two left mouse-keys: you

can repeat this until the size is to your liking. Instead you can

use the +-key.

Reduce text

You can reduce your text by pressing the two right mouse-keys: you

can repeat this until the size is to your liking. Instead you can

use the <196> key (minus).

Rotate text

You can rotate the text attached to your cursor in steps of 90�

by pressing the two outer mouse-keys or the r-key.

Mirror text

By pressing all mouse-keys simultaneously the text will be mirrored.

You can also use F2.

Place text

Once you are satisfied with the text you can position it with the

mouse and place it by pressing the left mouse-key or the Ins key.

Delete/move text

Once a text has been placed, it can be deleted, moved, <->copied or

rotated with the help of the options under the Multi Function option

in the menu line. Please refer to section 8.2.

Settings not lost

The settings made here (pen diameter, text size, rotation, mirror)

are saved by Layo1. As soon as you add another text, it will appear

on the screen in the same way as the one before. However, this text

in its turn can again be manipulated as was described above.

-----------------------------------------------------------------------------

5.7 Box

This option will draw a rectangle on the screen. This is useful for

the design of components, or for keeping a part of the PCB free on

one or more layers when using the Autorouter, or for limiting the

Autorouter's traces to a particular area. This can easily be done

by drawing a box round the area within which the Autorouter can draw

its traces.

Place box

Placing a box is done in the following way: first select menu option

Box by pressing the left mouse-key. The menu will disappear

and your back in your design again. Next, place the cursor on one

of the corners of the rectangle to be drawn and press the left mouse-key

(or Ins-key). Then move the cursor to the opposite corner and

you will see the box show up on the screen. Once it is as you want

it to be, press the left mouse-key (or Enter) again. However,

if you want to abort the procedure or e.g. want to select a new starting

point for the rectangle, press the right mouse-key and you can start

the procedure over again.

If you want to exit from the Box option and return to the menu

line, press the right mouse-key (or Esc) again.

As when drawing lines and tracks, you can use various layers (l-key)

and pen diameters (p-key) while making boxes. Here too, the

middle mouse-key controls zooming in order to enlarge, reduce and/or

move the picture.

-----------------------------------------------------------------------------

5.8 Circle

===> Attention:

Layers 8 and up! You can draw circles on all layers of Layo1,

but layers 8 and up will yield the best results. Layo1 will remind

you of this by showing the layer selection menu for you to select

the required layer, as soon as you are about to draw a circle on layers

1 ... 7.

Drawing a circle is done in the same way as drawing a rectangle (cf.

box).

-----------------------------------------------------------------------------

5.9 Ellipse

===> Attention:

Layers 8 and up! You can draw ellipses on all layers of Layo1,

but layers 8 and up will yield the best results. Layo1 will remind

you of this by showing the layer selection menu for you to select

the required layer, as soon as you are about to draw an ellipse on

layers 1 ... 7.

Drawing an ellipse can be done in the same as drawing a rectangle

or a circle (cf. box and circle).

-----------------------------------------------------------------------------

5.10 Corners

With this menu option you can define dimensions for your design. This

is only necessary for complete layouts. Components and subdrawings

need no corners for storage on disk and will automatically receive

the extension .LMC after their file name.

Complete layouts have the extension .PLY or LML (optional).

Place corners

Placing the corners is a simple mouse-controlled procedure. Place

the cursor on one of the corners, press the left mouse-key (or

Ins-key). Next, move the cursor to the opposite corner of your

PCB and press the left mouse-key again (or Enter) for confirmation.

While doing so, you will see the rectangle show up that will outline

your design. If there were any existing corners these will be replaced

by the new ones.

Layo1

In Layo1 the PCB dimensions and offset will usually be

defined in Advance in a .CNF file via the PCB configuration menu.

When you are editing an existing Layo1 layout and you select

the corner menu option, the new corners will be placed and the new

PCB dimensions will be automatically adjusted to the new values in

the relevant .CNF file (PCB configuration).

===> Attention:

The most practical procedure is to start placing the corners from

the offset: thus you will be able to read the exact PCB sizes from

the coordinates in the top right corner. You can also use function

key F6 to do this. This will give the current cursor position

the coordinates 0,0. You can reset the original coordinates later

by pressing r 1.

Incidentally, you can always readjust the corners later by placing

them all over again following the above procedure.

Layer 15

Corners are automatically placed on layer 15 by Layo1. Therefore,

you are recommended not to use this layer for other purposes.

Zoom

As always, you can zoom and move the picture over the screen by using

the middle mouse-key.

-----------------------------------------------------------------------------

5.11 Define

This option enables you to define Layo1's preset pen diameters. Once

you have activated this option you are prompted to select pen1 ...

pen7, after which you can specifcy the pen diameter required. In this

manner you can alter all pen diameters. After resetting Layo1 all

pen diameters will return to the default settings of Layo1.

Layo1

In Layo1 the pendiameters will be saved in the .CNF file that

is attached to your .PLY design, and become active again when you

load that design.

**************************************************************************

======================  Layo1 Editor Pad

-----------------------------------------------------------------------------

6.1 Pads

When you select the pad option from the menu line, a submenu

will appear under the menu line from which you can make your next

selection by pressing the left mouse-key:

F1 = help

While working, pressing F1 will always call up an overview of

the current mouse-key functions. Pressing Esc removes the overview

again.

In a nutshell ...

The menu options are self-evident: you can place stan<->dard pads,

copy, replace and delete them.

===> Attention:

Please note that in Layo1 pads that belong to components

that have been placed via a component list had better not be

deleted: they contain netlist information. Deletion could render the

ratsnest and the Autorouter unserviceable. If should you try to do

so anyway, Layo1 will warn you and ask for your confirmation.

Window

This function, which can be activated by pressing the outer mouse-keys,

makes it possible to manipulate parts of the design. For more details,

please refer to section 8.5.2.

Tool numbers

To each pad you can assign one from 8 tool numbers. The hole diameters

to go with these tool numbers can be specified via the menu option

drill program in Layo1's main menu, which is discussed at greater

length in section 12.

Tools can also be specified by means of the TOOL option of PADS. Cf.

section 6.6.

Layo1

In Layo1 the hole diameters are specified in the PCB configuration

menu that is created for each new design.

-----------------------------------------------------------------------------

6.2 Place pads

The menu option place pad can also be activated directly (without

the help of the menu line), by pressing the left and middle mouse-key,

when working in the Layo1 Editor.

Once the place pad option is activated, one of Layo1's standard pads

will be visible in the position of the cursor. With asymmetric pads

the cursor position is identical with the connecting point (drilling

hole). A pad can be placed by simply pressing the left mouse-key.

Next, you can place the same pad in the same way in another position

if you like.

Layer 0

Standard pads are automatically placed on layer 0 by Layo1.

Select pads

Pressing the left and middle mouse-key (or a single keystroke on s)

will call up the menu line from which you can select the various standard

pads 0 ... 15. As soon as you select one of them with the mouse, you

can see its shape and size appear in the cursor position. Once pressing

the left mouse-key is enough to select the pad that you require. From

the menu line you can also do this using the function-keys: F1

for pad 1 and Shift F1 for pad 11.

Sizes

The sizes of the pads are the following:

Pad nr 	in inches	in millimeters

0 	3/80	0.95	via

1 ... 4	8/80 x 5/80	2.54 x 1.59

5 and 6 	11/80 x 5/80 	3.49 x 1.59

7 	5/80	1.59

8 	7/80	2.22

9 	9/80 	1.27

10 	11/80	3.49

11 	13/80	4.13

12  	15/80	4.75

13 	17/80	5.40

14	19/80	6.03

15 	21/80	6.67

Here you can see the pads in actual size

Via

The smallest pad, the via, is used to interconnect tracks on different

layers in multi-layer PCBS.

Custom pads

Size and shape of the pads can be adjusted by the user. Cf. section

6.7.

===> Attention:

KST = If you place pads in components that you later want

to connect with the Autorouter, please remember that they are in the

right <169>grid<170> for the Autorouter. Therefore, the best procedure

for placing pads in this case is setting the cursor stepsize to 1/20

inch or bigger.

Tool selection

By pressing the middle and right mouse-key (or pressing the t-key)

you can select the required tool (tool number) for the relevant pad.

This can be done using the mouse, but from the menu line you can also

do this with the function-keys: F1 for tool number 1 etc.

The diameters going with the various tool numbers are specified in

the PCB configuration menu (Layo1 only) and can be adjusted

there. This PCB configuration menu can be called up via the options

option in the menu line. This procedure stores the tool data with

the other <->information on the relevant design.

Besides, the hole diameters are always specified when <->creating a drill   program.

===> Attention:

For the exact positioning of several pads at fixed intervals it may

be very convenient to adjust the cursor stepsize. Reducing or enlarging

the cursor stepsize is activated by means of F9 and F10

respectively. At the top of the screen the current stepsize will be

displayed. Once the cursor stepsize is identical with the interval

between the pads that you require, you will be able to place these

pads very quickly by using the arrow keys and the Ins-key. You

can also use the mouse to do so, of course.

Move pads

If a pad is not positioned to your liking you can easily move it by

placing a pad over it. In the top left corner of the screen the word

move will appear. Now you can move the pad to a new position and place

it there by pressing the right mouse-key.

If the pad to be moved is of the same size as the pad that was already

attached to the cursor, they will coincide completely and hence be

invisible if the color selection is unfortunate. Once you have placed

the pad that was to be moved by pressing the right mouse-key

both pads will be visible again.

You are now back in the place pad function again.

-----------------------------------------------------------------------------

6.3 Replace pads

Placed pads can simply be replaced by other pads.

This function is identical with the place pads function on the understanding

that when a new pad is placed in the position of an existing pad,

the latter will be removed and replaced by the new pad.

Here too, it is possible to select another tool number (t-key)

or pad (s-key). Please refer to the previous section place

pads.

===> Attention:

Please note that in Layo1 pads that belong to components

that have been placed via a component list cannot be replaced,

copied and deleted since this would render the ratsnest and the Autorouter

unserviceable.

-----------------------------------------------------------------------------

6.4 Copy pads

Instead of selecting a new type of pad time and again, this option

makes it possible to select a pad from elsewhere in the design and

copy it to other positions in the design. Pads copied in this way

are identical (including the tool number) with the original.

The pad to be copied is selected by pressing the left mouse-key. After

that, the function is the same as place pads. Use the left mouse-key

to place the pad in the <->required position(s). Please refer to the

previous section place pads.

===> Attention:

Please note that in Layo1 pads that belong to components

that have been placed via a component list cannot be replaced,

copied and deleted since this would render the ratsnest and the Autorouter

unserviceable.

Copy groups of pads

In multi function, which can be activated via the menu line,

it is possible to copy groups of pads as a whole. Please refer to

section 8.

-----------------------------------------------------------------------------

6.5 Delete pads

As all others, this menu option can be activated by pressing the left

mouse-key. This will make the menu disappear and your are back with

your design again.

Deleting a pad does not take more than pressing the left mouse-key

when the cursor is exactly over the center of the pad to be

deleted.

For pinpoint positioning it may be convenient to enlarge or reduce

the cursor stepsize with F9 and F10 respectively, or to

enlarge the picture by pressing the zoom-key: press the middle mouse-key

and select a bigger zoom factor. Remember that in the latter case

the current cursor <->position will be the new center of the screen.

===> Attention:

Please note that in Layo1 pads that belong to components

that have been placed via a component list cannot be deleted

since this would render the ratsnest and the Autorouter unserviceable.

-----------------------------------------------------------------------------

6.6 Tool

This option enables you to define the toolnumber to be used for each

pad separately.

-----------------------------------------------------------------------------

6.7 Define

Size and shape of each of the 16 pads in Layo1 can be user defined.

These user defined setting will be reset to Layo1 default after resetting

Layo1. They will however be saved in the .CNF file that is attached

to every Layo1 design (.PLY).

**************************************************************************

======================  Layo1 Editor Autorouter

-----------------------------------------------------------------------------

7.1 Autorouter

Layo1

When selecting Autorouter from the menu line a submenu will

appear. Above is the Autorouter submenu for Layo1 when a design

has been loaded.

An overview:

Set up

Enables you to configure the Autorouter just the way you want it work.

Grid size, vias yes or no, layerselection, etcetera you can set up

the Autorouter to perfectly fit the job and your needs and preferences.

Define window

Set up a window in which the Autorouter can place its tracks. A useful

option when you want to restrict the area that is available to router.

Manual

In this operational mode you point at the two pads or points to be

connected and the Layo1 Autorouter will try to find the shortest way

without crossing existing tracks on the same layer.

Layo1

Once a netlist and componentlist have been loaded from a scheme designer

and all components have been put in their proper place in your design,

the following powerful tools are at your disposal in Layo1, within

the limitations that you specified in Autorouter Set up.

Route track

You can select any track in the netlist and have Layo1 find the

shortest possible connection.

Route net

Same as Rout track, but now Layo1 will work out all connections

within a net.

Route all nets

Again the same as above but this time the Layo1 Autorouter will

try to make all the connections in the design.

Reroute

After Autorouting some nets or tracks seem to need a different router

configuration to obtain a better result. With the reroute options

you can delete the track or net(s) and have it routed again with a

different configuration. The Reroute options enable you keep control

over the Autorouting process. After some manual adjustments you can

have the Layo1 Autorouter to reroute a track, a net or all nets in

the design.

===> Attention:

The Autorouter functions can only be used after the corners have

been placed, or after defining a router window.

Read.me

The Autorouter is one of Layo1's latest powerful features: its possibilities

are still being added to all the time. The <->latest developments

are discussed in a read.me file on the original program disk.

If there is such a file on your disk, please refer to it first.

-----------------------------------------------------------------------------

7.2 Autorouter Set up

Selecting this option will display a second submenu:

It enables you to configure the Autorouter to your preferences and

to the specific design you're working on. Changing the options can

be done by selecting them and using the + and <196> keys

or the both left or right mouse buttons respectively.

Grid size

You can use a 1/40 or a 1/20 inch grid.

Bus-router

The Bus-router option is extremely useful when routing RAM-boards

or RAM-banks in your design. It uses regular patterns for the tracks

so that repeating track patterns are perfectly fitted to your design.

Lee-router

The Lee-router option use the Lee-algoritm. Not the fastest way to

make connections but sure to find the path if it's there. It can be

switched on or off.

Via-router

This option enables or disables the use of vias if anything else fails.

You might use it to get the last couple of connections allright.

45� routing<$I45�

routing>

Switched off all tracks will use 90� bends, switched on you'll

get 45� bends.

Pad to pad

If switched on the router will only work from pad to pad and not from

track to pad or track to track.

Horizontal/vertical

It's good PCB designers practice to put all mainly horizontal tracks

on one side of the PCB and all mainly vertical tracks on the other.

With these options you can preset the layers to be used for horizontal

and vertical tracks. By default they are set to layers 1 and 2 respectively

and it is best not to change that. When designing single sided PCB's

horizontal and vertical must be set to the same layer (1 or 2).

Max xy error / Max len error <$Imax

error>

This parameters restrict the margin of movement of the Autorouter.

To the Autorouter the ideal connection between two points is a straight

line, straight or diagonal along the grid.

XY

With the Max xy error set to 0 the Autorouter will only rout these

straight line connections. So you will have to give the Autorouter

some margin to work within. The higher the value, the more connections

will be made (try 8 or 16 or higher). The value is set grid-steps

(1/20 or 1/40 inch).

len error

To the Autorouter it is also obvious that the shortest track between

two points equals the distance between these points. If you set the

Max len error to 0 the Autorouter will only try to make this short

connections. Again you will have to give the Autorouter some margin

for a roundabout road to avoid other pads and sometimes loop around

other tracks. Again the value is set in grid steps. If the length

of a possible connection exceeds the distance between points plus

the max len error the Autorouter will not make that connection.

Lee-router

The setting of the Max len error can be important when using the Lee-router.

When a connection is possible the Lee-router will find the way, but

sometimes it takes a long way to get there.

-----------------------------------------------------------------------------

7.3 Define window    (autorouter)>

With this option you can define a window on screen to restrict the

Autorouter. The router will only make connections within the window

defined.

===> Attention:

For more information on using a window with the Autorouter please

refer to section 7.7.

-----------------------------------------------------------------------------

7.4 Manual autorouting

Once you have set up your favourite Autorouter configuration. You

can get to work. In order for the Autorouter to work properly, corners

should be set or a window should be placed (w-key) around the

area that is available for the Autorouter. If not, the Autorouter

will try to self-determine an invisible window around the elements

already placed in the design.

Creating tracks with the Autorouter is a very simple procedure. Once

you have <->activated the option you move the cursor to the point

where the track is to begin. Press the left mouse-key. Now move the

cursor to the next point that is to be connected via the track and

press the left mouse-key again. At the point where the track ends

and you want to go to a new connection, press the right mouse-key

to conclude the track. Now you can start on a new track, if you like.

Pressing F1 will always call up an overview of the current mouse-key

functions.

Window

When a window is placed (press both the left and right mouse-keys,

drag the window to the desired position and click the left mouse-key)

the Autorouter will only try to make connections within that window.

In this way it is possible to rout different areas of your

design using different Autorouter settings. To do so, first place

a window over a selected area set the Autorouter configuration for

that particular area and start the Autorouter. When finished remove

the window by first pressing the outer two mouse-keys and then the

right mouse-key. Reconfigure the Autorouter to the desired settings

and start the Autorouter again. It will now route the remaining nets

and tracks according to the new setting.

-----------------------------------------------------------------------------

7.5 Route track / net / all nets

Layo1 ONLY!

Once netlist and componentlist have been loaded and all components

are placed properly you can start the Layo1 Autorouter to make all

connections. First you might want to alter the settings in the Autorouter

Set up menu.

Route track

When this option is activated Layo1 expects you to select a track

by clicking the left mouse button when it's positioned on that track.

The Layo1 Autorouter will now find the best connection within the

given restrictions.

Route net

Same as above. Now the Layo1 Autorouter will try to solve all connections

within a selected net.

Route all nets

This time you don't select anything. You've selected everything. The

Layo1 Autorouter will start to make all connections according to the

netlist. Again within the given limitations.

Your choice

Routing track by track or net by net or routing all nets at once is

your choice. You might find that some parts of your design could best

be routed using only the Bus-router without any vias and that some

other connections should be made by the Lee-router with vias enabled.

For the odd connection that won't work either way you might want to

accept a larger XY or LEN error. It's all up to you. Layo1 offers

you all the flexibility you need.

===> Attention:

For general information on the use of the Layo1 Autorouter please

also refer to section 7.7.

-----------------------------------------------------------------------------

7.6 Reroute track / net / all nets

Rerouting of tracks, nets or all nets enables you to evaluate the

result of an Autorouting process, to make some adjustments to your

design or to alter the Autorouter's Set up and then to Reroute some

of the tracks or nets or all nets.

Rerouting is nothing more and nothing less than to have the Autorouter

rout a track or net(s) anew. The selected item is first deleted after

which the Autorouter is started again to rout that item. Cf. section

7.3

===> Attention:

For general information on the use of the Layo1 Autorouter please

also refer to section 7.7.

-----------------------------------------------------------------------------

7.7 Autorouting general notes

Quiting

You can always interrupt the Autorouter by pressing Esc.

Pad colors

Layo1

When using the Autorouter on the basis of an imported netlist or component

list, the colors of pads and traces will be different. This makes

it easy to see which pads are part of the netlist, and which connections

have been made and which have not. Once a pad from the netlist is

selected with the mouse it is highlighted, as are all other pads that

are to be connected with this point. If the connection has already

been made, this will also light up so that it is clearly visible on

the screen which pads are connected by which traces.

This can also be seen in the small window in the top left corner of

the screen where all pads of the entire design are displayed. Thus,

you can also see the pads to be connected which are off-screen at

that moment.

Not solved

The Autorouter will not automatically create vias (unless permitted

in the Autorouter Set up menu) and will not create any tracks that

cross other tracks on the same layer. Apparently, you have set the

Autorouter an impossible task. In that case press the right mouse-key

and if possible try again on another layer or make the connection

with the help of vias over the different layers.

You might also wish to review your settings in the Autorouter Set

up menu for this particular connection. Try switching on the Lee-router

if it is off or enable vias. If that does not work increase the XY

and LEN errors for this connection and try to route the track or net

again.

Select layer

You can always select the layer that you want to work on with the

Autorouter. Pressing the l-key will offer you an on-screen choice

from all layers. With function-keys F7 and F8 you can

quickly switch between the layers most used, 1 and 2, or the layers

that you have assigned to these keys yourself. You can also use the

two left or right mouse-keys to do this.

Only pen 1

You can select a pen diameter when working with the Autorouter. Be

carefull however: the Autorouter does not take pen diameters in consideration

when making connections. Using a large pen diameter could lead to

shorts in your PCB. Of course you can adjust the width of particular

tracks later.

Faulty connections

Layo1

If you use the Layo1 Editor on the basis of a rats<->nest, Layo1

Plus will check whether the connections made by you are all right.

If a faulty connection is made, it is marked with a small cross on

layer 13 in the place of the relevant pin.

Ctrl F6 will display the faulty connections.

Keeping an area free

If you do not want the Autorouter to draw its tracks in a particular

area of your design, you can keep it free by drawing a box around

it. The Autorouter will then draw its tracks around this area. In

the same way you can avoid the Autorouter drawing its tracks outside

a particular area. You can define the Autorouter's operational area

by drawing a window around the area within which the Autorouter should

stay. Combinations are also possible, e.g. limiting the operational

area of the Autorouter while within that operational area particular

areas are inaccessible to the Autorouter.

You can start drawing a box by pressing the two left mouse-keys. To

draw a window use the right and left mouse-keys.

Windows and boxes are also useful if you want to apply different ways

of Autorouting to some parts of your design (e.g. RAM-banks). In that

way you can rout some restricted areas (within a window) with the

Bus-router, while routing the remainder using the Lee-router with

vias enabled (after deleting the window of course).

Deleting a track

A track can be deleted quickly by pressing the two right mouse-keys

while the cursor is on the track to be deleted. The question Found?

yes/no will be displayed on the screen and the track will start

blinking. Select the required answer with your mouse and confirm by

pressing the left mouse-key. By pressing the right mouse-key you can

quit from this question and everything is left as it was. If you press

the middle mouse-key Layo1 will start searching for another track

in the cursor position.

Edit track

The tracks created by the Autorouter can be edited in a simple way.

To do so you press all three mouse-keys simultaneously. Please refer

to section 5.

Ratsnest function-keys

Layo1

For the maintenance of an imported netlist during work Layo1 has a

number of powerful functions that can be <->activated in the following

way:

Ctrl F1

Reset ratsnest (remove layer 14) in Layo1.

Ctrl F2

New ratsnest of the pad in the cursor position, with new calculated

shortest connections in Layo1.

Ctrl F3

New ratsnest of the component at the cursor, with new calculated shortest

connections in Layo1.

Ctrl F4

Update the ratsnest in Layo1. All existing connections

are removed from the ratsnest so that you can see which connections

are yet to be made.

Ctrl F6

Check all connections according to the netlist in Layo1.

Faulty connections are marked with a small cross on layer 13.

**************************************************************************

======================  Layo1 Editor Multi Function

-----------------------------------------------------------------------------

8.1 Multi Function

When you select Multi Function from the menu line, a submenu

will appear under the menu line from which you make your next selection

by pressing the left mouse-key:

===> Attention:

As long as your design is still blank, it is not possible to select

Multi Function: there is nothing to manipulate.

F1 = help

While working, pressing F1 will always call up an overview of

the current mouse-key functions. Pressing Esc removes the overview

again.

Introduction

In Multi Function you can manipulate existing data as will appear

from the new submenu that appears once you select one of the four

options (Block/P1..P2/Line/ Window) with the help of the left mouse-key.

Multi Function enables you to copy, move, rotate or delete parts of

your drawing or parts of the data file with which your drawing was

created.

Block

A block is a component or a subdrawing imported from Layo1's component

databank or a text that has been typed. Such a block can be manipulated

as a whole.

P1 .. P2

This function makes it possible to delete, move, rotate or copy datalines

that are in between two points (P1..P2).

Line

Every track can be selected for further editing. The <->selected trace

can be deleted, rotated, moved or copied.

Window

In several Layo1 functions a window (box) can be defined (usually

with the help of the two outer mouse-keys). All information (components,

pads, tracks, etc.) within that window can be moved, deleted, rotated

or <->copied. When moving or rotating, all traces (connections) are

pulled along to the new position.

===> Attention:

You are recommended to press F5 after every manipulation in

Multi Function. This will rewrite and update your on-screen picture

and will remove any shadows of the previous version. Thus, you will

be able to see what you are doing again.

Autorouter

If you manipulate parts of a drawing that you want to connect with

the help of the Autorouter later, it is important to set the cursor

stepsize to 1/20 or 1/10 inch and to position the cursor on the grid

correctly by twice pressing the *-key (asterisk). Else,

there could be the risk that pins are placed just missing the Autorouter

grid which will result in faulty connections.

A practical example

Using the subdrawing RAM-4.LMC from the Layo1 component databank may

clarify the various possibilities of Multi Function to save much time:

1.

The profile of the left component is drawn on layer 8 using the drawing

function of the Layo1 Editor.

2.

The right array of pads is placed in this component with the pad function

of the Layo1 Editor.

3.

Using the options window, copy block or P1..P2 of Multi Function the

array of pads is selected and copied to the left half of the component.

(The component thus created could now be stored in the component databank.)

4.

Using the option copy window of Multi Function the <->entire

component is now selected and copied three times.

5.

With the interactive Autorouter (Bus-router) the topmost track

is quickly created by pointing at the points to be connected. The

Autorouter will automatically steer clear of the pads that are not

to be connected.

6.

If necessary, the tracks can still be adjusted with the edit function.

7.

With the option copy line and rotate line of Multi Function

the created line is quickly copied in order to create the other connections

which all have the same shape, <->albeit that half of the tracks is

rotated by 180�.

8.

The subdrawing is now ready and can be stored in Layo1's component

databank. It can be used later for <->designs or as a basis for other

subdrawings, such as example RAM-8.LMC:

-----------------------------------------------------------------------------

8.2 Manipulation

In all Multi Function options you can copy, move, rotate or delete.

The procedure of manipulation is always the same: it is described

hereunder. However, there are differences in the ways the elements

to be manipulated (block, P1..P2, line and window) are selected. This

will be dealt with hereafter in separate subsections.

Once a part of the drawing is selected, it can be manipulated according

to the submenu options (copy, move/ <->rotate, or delete).

Copying

The selected part of the drawing can be copied to any place by pressing

the left mouse-key. Thus, the elements can be copied to more places

than one until the procedure is concluded by pressing the right mouse-key.

Repeated copying

If the element is to be copied several times at fixed intervals (e.g.

a track or a component), Layo1 can do this for you quickly and accurately.

Copy the element to the first position by pressing the left mouse-key.

Do not move the mouse and press the two left mouse-keys or the p-key

in order to set the zero point to the current cursor position. Now

accurately move the cursor to the next position, type a number between

0 and 100 on the horizontal numeric keys that you can find over the

letter keys on your keyboard (this number, with a maximum of 99, will

now be displayed in the top of the screen) and press the insert

key or the left mouse-key. The element will now be <->copied the required

number of times at the interval and with the orientation that you

specified with your cursor. Please refer to the practical example

above.

Thus, it is possible to quickly place a series of identical elements

next to, under, over, below and above each other at exactly identical

distances.

Zero point

Pressing the two left mouse-keys or the p-key you can define

a new zero point as indicated in the top of the screen. Pressing the

space bar will remove this text again.

Autorouter

If you manipulate parts of a drawing that you want to connect with

the Autorouter later, it is important to set the cursor stepsize to

1/20 or bigger (1/10 inch etc.) and to position the cursor on the

grid accurately by twice pressing the *-key (asterisk).

Moving/Rotating

When you have selected the element that is to be moved or rotated,

the block will be attached to the cursor. You can move it by moving

the cursor to the correct new position and there pressing the left

mouse-key.

Rotating is done (clockwise) in steps of 90� by pressing the two

outer mouse-keys, the cursor position being the turning point. Once

the element is correctly oriented and placed in the right position,

you can place it there by pressing the left mouse-key. All traces

will be pulled along to the new position, but they may need some revision.

Layo1

Once you have read the netlist and component list of a new .PLY design,

this function (block) can be used to correctly position the

components. You will notice that the lines of the ratsnest will be

pulled along. Of course, they remain intact because they only connect

the pins that are later to be connected with a trace.

Ctrl F4 will update the ratsnest using the shortest possible

routes.

Deleting

Once an element is selected and highlighted on the screen, Layo1 will

ask you whether this is to be deleted yes or no. Select

yes or no with the cursor and press the left mouse-key or enter

key, or press the y or n key.

Layo1

Whenever you want to manipulate an element that has been imported

via a component list the program will warn you if there is the risk

that your action would render the netlist, ratsnest, component list

or Autorouter unserviceable. So, be extremely careful here.

-----------------------------------------------------------------------------

8.3 Block

A block is an imported component or subdrawing or a text element.

With the block option this element can be copied to another place

within the design, moved, rotated or deleted.

The block will always have to be selected first by pressing the left

mouse-key. The entire selected block will <->appear highlighted on

the screen. If this is not the block you want you can either quit

from this function by pressing the right mouse-key or you can press

the two right mouse-keys to make Layo1 continue searching for another

block in the same cursor position.

If it is the block that you want, confirm by pressing the left mouse-key.

Please refer to section 8.2.

-----------------------------------------------------------------------------

8.4 P1..P2

The P stands for Position here, meaning dataline number. With this

function you can manipulate one or more <->datalines, i.e. deleting,

copying, rotating or moving.

Whichever you want to do (copying/moving/rotating/ <->deleting), you

will first have to select the datalines that you want to manipulate.

To do so, place the cursor on a pad or a line-section that belongs

to the group that you want to select and press the left mouse-key.

If Layo1 can find something in that position, it will start blinking

and Layo1 will ask you if that is the one you want. If so, press the

left mouse-key for confirmation. The mouse-key functions will now

be different.

Using the two left mouse-keys you can now page forward through the

datalines of your design; using the two right mouse-keys you can page

backwards (you can also use the + and <196>-key). The

cursor will jump from point to point. In this way, go to the starting

point of the part that you want to select and mark it by pressing

function-key F7: the relevant line number will be displayed

in the top left corner of the screen. Now, using the two left mouse-keys

or the +-key, go to the end of the part that you want to select

and mark the end by pressing function-key F8.

In the top left corner of the screen the relevant datalines are indicated

and the relevant elements are clearly visible in your design.

Please also refer to section 8.2.

-----------------------------------------------------------------------------

8.5 Line

This menu option enables you to manipulate a track as a whole. Once

you have selected the required manipulation from the submenu with

the left mouse-key, you can select the track by placing the cursor

on it and pressing the left mouse-key. The relevant track will start

blinking and Layo1 will ask you if this is the right one. By pressing

the left mouse-key you confirm Layo1's suggestion; pressing the middle

mouse-key will make Layo1 continue searching in the same cursor position.

Pressing the right mouse-key will take you back to the menu line.

Please refer to section 8.2.

-----------------------------------------------------------------------------

8.6 Window

This function enables you to place a box around a particular part

of your design and to manipulate anything within that box, i.e. copying,

moving, rotating or deleting. You will first have to select the required

function and confirm your selection by pressing the left mouse-key.

You can now define the window to be manipulated. Place the cursor

on one of the corners of the window to be defined and press the left

mouse-key. Moving the cursor now, you will notice a box appearing

on the screen. Move the cursor so that it frames all information that

is to be manipulated and finally press the left mouse-key. If anything

goes wrong, quit from the function by pressing the right mouse-key

and make a fresh start.

Visible layers only

Only the layers that are visible in the window are affected by the

manipulation. So first make sure that all active layers are displayed

on the screen unless you deliberately want to manipulate the information

of particular layers while leaving the information on other layers

unaffected.

Pressing the d-key will call up the layer selection menu line,

showing which layers are in use and which layers are being displayed.

Once a window is selected, the function of the left mouse-key changes:

===> Attention:

The option move window can also be called up directly from other Layo1

Editor functions (e.g.: draw, Autorouter), usually by pressing the

two outer mouse-keys or the w-key (in the latter case the current

cursor <->position will be the starting point of the window). If the

window function has been selected from a function other than Multi

Function, a window can be selected in the way described above, but

the resulting window can only be moved.

Please refer to section 8.2.

**************************************************************************

======================  Layo1 Editor Options

-----------------------------------------------------------------------------

9.1 Options

When you select Options from the menu line, a submenu will appear

from which you can make your next selection by pressing the left mouse-key:

F1 = help

While working, F1 will always call up an overview of

the current mouse-key functions. Pressing Esc removes the overview

again. If you STAY on the menu-line (don't type Enter) F1 will give some

telex-message advise on the subject. If there is nothing to explain, some

general advise will be given. (Layo1-versions in F1 Files � Delete file)

Options submenu

Only a short description of the submenu options can be given

here. Please refer to subsequent subsections for a more detailed discussion

of the various possibilities.

Edit design CNF

Editing of the configuration (.CNF) of the PCB design you are working

on.

Edit NETLISTE

If a CMP-file exist or you are preparing it this function enables you

to make connections. INS / mousebutton on pads. Esc to leave. SAVE IT

EXPLICITELY in Files � Save � Save Net.

Layo1 Toolbox

This is a magic box whose possibilities are being added to all the

time (cf. any read.me file on the Layo1 program disk). The current

version features the following possibilities: moving your design with

a fixed offset, mirroring it around the X or Y axis, adjusting the

scaling, adjusting the grid, deleting or editing layers.

Optimizing

This enables you to replace all square angles in tracks by 45�

angles. In addition there is a suboption that offers the possibility

to delete all superfluous datalines which will save memory space and

improve speed and capacity.

Display CAM

Overview of the generated program lines of all layers of the design.

Configu- ration

Edit the current configuration of Layo1.

Printer checkplot

Makes it possible to make a checkplot on a dot matrix or laser printer.

-----------------------------------------------------------------------------

9.2 Layo1 Toolbox

If you select the option Layo1 Toolbox, a new submenu will appear

(cf. above).

You can select the following options which will be discussed in greater

detail in the subsequent subsections:

X Mirror

Mirror the entire design around the X axis. Not with .PLY designs

in Layo1.

Y Mirror

Idem, but now around the Y axis. Not with .PLY designs in Layo1

Plus.

XY offset

Change the XY offset (displacement relative to the zero point) of

the design. In Layo1 this change is updated in the PCB configuration

menu straight away and consequently stored in the relevant .CNF file.

Delete layer

Is self-evident.

Change layer

With this option all parts of the drawing on a particular layer can

be transferred to another layer. Layer 0 cannot be transferred.

Drill group

Will call up the drill group menu in which it is specified how manu

drill positions there are per layer and per tool number.

Scaling

This option makes it possible to rescale the measuring of the entire

design.

Set grid

This option can place a drawing exactly on the grid which is particularly

important when a digitizer is used.

X Mirror

The mirror function to mirror your entire design can be used

for .PLY and .LMC and .LML designs.

Using this option  may damage the

data file of your design. Therefore, you are recommended to save your

design as a .LML file first and then mirror that version.

If you want to mirror a complete design around the X axis, you must

select this option from the Toolbox submenu. In the top left corner

you can read X mirror. Pressing the left mouse-key is now enough

to mirror the design relative to the zero point (corner).

Design without corners

When mirroring a design without corners, e.g. a component or a subdrawing,

you must first place the cursor on the required zero point before

pressing the left mouse-key.

Y mirror

Same as X mirror, but now around the Y axis.

XY offset

The XY offset is the displacement of the top left corner of the design

relative to the zero point of the screen.

Layo1

In Layo1 this offset is usually stored in the PCB configuration

of the relevant design and can be adjusted there as well.

Once the option XY offset is activated in the submenu of Layo1's

Toolbox, the word XY mirror will appear in the top left corner of

the screen. You can now use the mouse to move the cursor to the required

new zero point and <->select it by pressing the left mouse-key.

As the 1/10 inch grid on the screen relates to the zero point of the

design, moving the zero point will entail <->moving the grid (relative

to the design).

Delete layer

This option makes it possible to delete one or more layers from your

design permanently. The following table will be displayed:

This table shows all 16 layers of Layo1 and their use in the current

design. Going through this list with the mouse (cursor), you will

notice that the cursor skips the layers that are not in use. After

each layer the corresponding number of coordinates is given. Select

the required layer with the mouse and confirm by pressing the left

mouse-key. Layo1 will ask you if the layer is indeed to be deleted.

Answer yes or no.

===> Attention:

Layer 0 cannot be deleted.

Change <$Itoolbox;change

layer>layer

This option will display the following table:

Select the required layer in the left column by pressing the left

mouse-key or the Enter-key. Then point at the new layer with

the mouse in the right column and again press the left mouse-key.

Then select start by pressing the left mouse-key.

===> Attention:

Please remember that combining layers at this stage is <->irrevocable

and that e.g. combining layers 1 and 2 (top and underside of a double-layer

PCB will merge the traces of both layers into one from then on.

Save = safe

Therefore, you are recommended to save your design on the hard disk

first by pressing the F4 key. Then, you can always go back to

the old situation.

===> Attention:

Layer 0 cannot be deleted.

Drill group

This option will call up the following list:

This table shows the use (number of coordinates) of the layers of

your design. It also indicates the number of holes per standard pad

per tool number.

When designing a PCB, tool numbers are not usually taken into consideration.

Components are very frequently copied and moved. Consequently, the

tool numbers will usually vary. Usually the pad sizes are selected

while <->designing in relation with the hole later to be drilled.

Consequently, there is a relation between the pad number and the tool

number. The drill group option allows this relation to be ordered

and stored.

Exceptions can always be adjusted separately later in the Edit

function.

The right table shows the number of standard pads per type and the

corresponding tool numbers. Now it is possible to decide that the

4 pads number 7 that were to be drilled with tool number 1 must be

changed to tool number 2. To do so place the cursor on the 4 pads

number 7 under tool number 1 and press the left mouse-key. Layo1 will

show what the changes are going to be and will ask for your permission

(yes/no).

Scaling

This function is only meant for general drawing and not for PCB design.

This option allows you to set the plotting scale for your design.

To do so, specify the new scale and the design will appear on the

screen according to the new scale (after your confirmation). Layo1

will use the scale specified by you (0.1000 ... 10) and will multiply

all coordinates by that factor. However, the width of traces and the

size of pads remain unaffected. Thus, scaling factor 2 will enlarge

your design by two.

This option is particularly useful when designing PCBs of high accuracy

and density (e.g. SMT).

Fronts

It can also be very useful whenever you use Layo1 to <->design fronts

or schemes and you want to load symbols that have a different scale.

In that case you adjust the <->scaling of your design to that of the

symbol to be loaded, load the symbol, and rescale your design to its

original dimensions.

Set grid

This option makes it possible to place e.g. a PCB design imported

with a digitizer exactly on the grid.

In all other cases this option should be used with caution, preferably

immediately after placing the components.

Digitizer

As it is not possible to position on 1/100 inch with a digitizer,

Layo1 lends a helping hand. This option will place all specified coordinates

exactly on the nearest point of the specified inch grid.

-----------------------------------------------------------------------------

9.3  Optimizing

This option has a submenu with three more options. The first two are

involved with changing all square angles in tracks into 45� angles.

The Optimize 45 option will free space in your program by deleting

all superfluous datalines.

Sq 45 opt 1/40<$Isq 45 opt

1/40>

This option replaces all square angles in tracks by 45� angles,

rounded off by 1/40 inch.

Sq 45 opt 1/80<$Ioptimizing;sq 45 opt

1/80>

Similar to the option above, but rounded off by 1/80 inch.

Optimize 45

This option deletes all superfluous datalines. As Layo1 will often

place corners automatically when drawing tracks, it is not necessary

to store every change of direction of a track in a dataline with an

insert or by pressing the left mouse-key. The datalines created this

way only take up precious space: they can be deleted with the <->Optimize

45 option. A regular use of this option will <->improve working speed

and save memory space and <->capacity.

-----------------------------------------------------------------------------

9.4 Edit design PCB <$Itoolbox;edit design

PCB>

Layo1 only

If you are not working on a .PLY design on the screen when you select

this option, Layo1 will ask you if you want to create a new file.

If you select no, you will be asked for a name for the new file.

If you select yes, you will be asked if you want to reset Layo1

(and make a fresh start) yes or no. Next, you can enter your specifications

for a new configuration menu.

The PCB configuration menu will appear on the screen. Please refer

to section 4.2.1.

-----------------------------------------------------------------------------

9.5 Display Cam <$Itoolbox;display

cam>

This option will call up the CAM list of the design. The left table

shows which layers are being used and the number of coordinates per

layer. The total is given at the bottom, where you can also see how

many coordinates can still be added. This example shows the CAM list

of a <->design in the Layo1 Demo version which can contain a maximum

of 1000 coordinates.

-----------------------------------------------------------------------------

9.6 Layo1

configuration

This option will call up the configuration menu of Layo1 with all

standard specifications for:

Video board

Mouse

Printer

Screen settings:

Cursor type

Pin numbers

Tool numbers

Block numbers

Display X/Y metric/inches

Auto save time

Color Table Selector

Please refer to section 16 (Layo1 Setup) where this menu is discussed

in great detail.

-----------------------------------------------------------------------------

9.7 Printer <$Itoolbox;printer

checkplot>checkplot

With this menu option you can make a checkplot of your design on the

dot matrix printer or laserjet.

The procedure depends on the printersetting specified in the configuration

menu (cf. section 16).

Laserjet

When a (HP) laserjet (75 / 100 / 150 / 300 dpi) is specified in the

configuration menu and the printer checkplot option is activated,

Layo1 will ask for the scale on which to print. Then Layo1 will ask

you whether you want pads to be printed with open centers or not.

After that Layo1 starts transmitting the data to your laserjet.

Matrix printer

If you specified a dot matrix printer in the configuration menu (60x72

/ 72x72 /180x180 dpi) the procedure is the same as with a laserjet

(scale and open/closed centers). Cf. above.

In this case however Layo1 will also prompt you for a repeat value.

If you select 1 the printer head will move over the paper just once:

your print is soon ready. If you select 4 the printing process will

take more time but will result in a blacker and better print.

**************************************************************************

======================  Plotter checkplot

-----------------------------------------------------------------------------

10.1 Checkplot

When you select checkplot from Layo1's main menu, a plotter

driver table will appear on your screen. Some of the specifications

found here are extracted from the plotter and port configuration (please

refer to section 15).

Entering the specifi- cations

With the mouse or using the arrow keys you can move to the various

fields to be filled in. Except for the file name all specifications

are entered by means of the + and <196> key, or the two

left or right mouse-keys respectively. If you hold down the Shift-key

while doing this, the procedure is speeded up (bigger steps).

Input

You must first specify the design that is to be plotted. If

there is a resident design, Layo1 will provide its path and name.

However, you are free to select another file for plotting. The selection

of a file is done as was described in section 4.3.

Once a design is selected, it is displayed on the screen at a reduced

scale. Pressing Esc will take you back to the table.

Display design

The latter can also be done with function-key F5. Pressing it

will display the selected design, pressing it again will take you

back to the plotter driver table.

Plotter/port

Next, Layo1 will want to know what plotter you are working with and

to which parallel (LPTx) or serial (COMx) port it is connected. If

a serial connection is used, baudrate (speed of transmission) etc.

must be specified in the plotter configuration menu (please refer

to section 15). These values can be adjusted by means of the +

or <196>-key, or the two left or right mouse-keys respectively.

If several plotters have been installed in the plotter configuration

menu, you can adjust the selection of a plotter by pressing the +

or <196>-key, or the two left or right mouse-keys.

Output

Instead of a port, you can also specify a path and a file name. In

that case, a plot file will be stored a fixed disk or floppy. If necessary,

you can later send this file to a plotter using another computer.

You do not need Layo1 to do that: you can use ordinary DOS commands

such as COPY or PRINT.

Plot area

Min/max plot area: here the results are displayed of what was specified

for XY settings.

Plot scale

Setting the plot scale is also done by means of the + and <196>key,

or the two left or right mouse-keys respectively. If necessary, you

can simultaneously press Shift for bigger steps. The factor

that you specify here, will be the multiplication factor for all coordinates

of the design. This value can be specified to within a thousandth.

This is useful if you want to correct the scaling of your plotter.

Offset

Here you can specify the distance that you want

your <->design displaced relative to the top left corner of the plotter.

A positive X value will result in the design being plotted further

to the right, a negative further to the right. The same is true for

the Y value, but then vertically.

Mirror/Rotate

These options offer the possibility of plotting your design in reverse

or rotating it (clockwise in steps of 90). You can specify a value

0 ... 3. For 0 there is no rotation, for 1 a rotation of 90, etc.

Velocity

Under output you can specify the velocity (pen speed), within the

limits of the possibilities of the plotter used, and only if the plotter

allows you to do so from your computer. The velocity that you need

is dependent on the type of plotter pen that you use and the material

that you are plotting on. Please refer to specifications of the pen

and plotter manufacturer.

Auto pen change

must be always on YES

Here you can choose for automatic or manual pen change. If you select

no, Layo1 will warn you for every pen change necessary and will wait

for you to give your Ok before going on. This is only possible when

you are working with a plotter directly. You cannot do this when creating

a plot file.

Plot open pads

You can also decide whether or not you want pads to be plotted with

a hole in the center. For double-layer PCBs and PCBs that are to be

drilled via a drilling file it is better to have closed pads. For

single-layer PCBs that are to be drilled manually, <169>open<170>

pads can be helpful for the drilling.

All of these selections are made with the +/<196>-keys or the

mouse, after you have selected the relevant field.

10 config pages

In this way you can make up 10 different configuration pages for your

plotter(s). Once you have made more than one, you can page through

these pages using the +/<196>-keys or the PgUp/PgDn-keys.

You can reserve a particular page for a particular type of plot, e.g.:

Page 1: Plot of the soldering side with corners.

Page 2: Plot of the component side with corners.

Page 3: Plot of the component layout with frame.

Etc.

Thus, you do not have to specify all the settings all over again,

but you can store your favorite settings on separate pages. These

pages are stored for you, but they are not tied to a particular design.

Pen selection

Finally, in the left table, you will have to specify (for each configuration

page) which pens are to be used for the plotting of the different

layers. By not specifying a pen number for all layers you decide which

layers are going to be plotted or not. For each layer there is an

indication of the number of datalines (drawing commands), which makes

it simple to check which layers have been used in the design. You

can also select a pen for plotting along the data/file name, the 1/10

inch grid and the tool symbols.

Tool symbols

You can use the tool symbols to check if the tool numbers assigned

to the pads are correct. A plot with tool symbols can also serve as

a reference for the manual drilling of PCBs.

1/10 inch grid

Plotting the 1/10 inch grid along can be very useful for checking

the positioning of the components.

Start/stop

Once everything is to your liking, you can start plotting by pressing

the left mouse-key when the cursor is on the STOP field. You

can discontinue the plotting with Esc. If you do so, it is necessary

to initialize your plotter before making a new plot.

Pause

You can interrupt and resume the plotting by pressing the Enter-key.

Plotters that have an internal buffer memory will only interrupt once

the buffer is empty.

Quit

Once the plotting is done, you can return to Layo1's main menu by

pressing Esc or the right mouse-key.

**************************************************************************

======================  Photoplotter

-----------------------------------------------------------------------------

11.1 Photoplotter introduction

A photoplotter is a high-tech output device that can directly plot

on to photographic material with extreme accuracy.

Layo1 can control the most important photoplotters;

1. Gerber photoplotter

Gerber

By using different apertures, the Gerber photoplotter is able to plot

different line widths. This results in more succinct files and a much

quicker operation. Moreover, Gerber photoplotters that feature a laser

can plot negatively. These plotters work with a resolution

of 1/4000 inch.

Once one of these options is activated Layo1 will ask you for the

name of the file to be plotted. You can specify the file name

as was described in section 4.3. The procedures for both photoplotters

from that point on will be discussed separately hereafter.

-----------------------------------------------------------------------------

11.2 Gerber photoplotter      TRACEURS.EXE

After selecting the Gerber photoplotter and specifying

the relevant file name, the circuit will appear after F5:

Type Esc and the CAM list of the design is displayed, with

further information on the layers, the pads and tool numbers, cf.

above.

After pressing a key, the following table is displayed:

Here you can specify the data for 20 different photoplot files

(*.GE1 ... *.GE8) of the same design. You can specify per design which

layers are to be plotted. The layers are selected by placing the cursor

on them and pressing the left mouse-key.

The above table has already been specified for the file DEMO_SMD

and will show the following:

Plot file

c: or a:\gerber\DEMO1.GE1,

you can switch the letter of the disk on to which the different files

are to be stored. You can only adjust the letter of the disk (A, B,

C etc.) using the + and - keys or the two left or

right mouse-keys.

The files will always be placed in the directory \GERBER (which will

be created by Layo1 if necessary) and will always have the name

of the relevant  file.

Output

If a Gerber photoplotter is directly connected to your computer the

plotting information can also be sent there directly. You can specify

the right port (LPTx or COMx) in the field where in the example a

file is specified, by using the + and <196>-keys. If a

serial port COMx) is used, you can also adjust the baudrate, parity,

number of bits/stopbits. Please refer to section 16 on port configuration.

Save data; Exit

Making this selection with the left mouse-key will create the files

on the selected floppy or fixed disk.

\gerber\*.GER

In addition to the plot files *.G01 ... *.G04, the file *.GER

is placed in the specified directory. This file contains information

on the Gerber photoplotter settings: it is meant for the photoplot

supplier.   Data in *.GER will be taken from the file \LAYO1EE\GERBER.DAT

This is a source file used by TRACEURS.EXE   You may modify the

mills column in GERBER.DAT if you need different apertures.

Exit

By selecting Exit with the left mouse-key or Escape you

can exit without creating any files.

======================  Textfont editor

-----------------------------------------------------------------------------

14.1 Textfont Editor

Layo1's textfont editor is an auxiliary program that was included

in the Layo1 program package on request of <->experienced users.

It can be used for:

1. designing your own character sets (alphabets, symbols or logos)

2. editing an existing character set.

The textfont editor works in much the same way as the Layo Editor,

but it requires some experience with the Layo Editor and knowledge

of the commands used.

===> Attention:

Beginning users are therefore recommended not to use the textfont

editor without any previous experience. For usual applications Layo1's

standard character set has everything you might need.

Character set

A character set (font) can contain a maximum of 4000 coordinates (drawing

commands), divided over the characters 32 ... 255. Each of these characters

is on a separate <169>page<170> of the .FNT file. The page

numbers correspond with the ASCII codes for the different letters

and signs and are also referred to with these letters or signs.

-----------------------------------------------------------------------------

14.2 Differences from the Layo1 Editor

Although the procedures of the textfont editor are very similar to

those of the Layo1 Editor, there is a number of differences.

1. In the textfont editor only one layer is used to draw on.

2. The textfont editor does not feature pen diameters.

3. The displayed grid is set to 2 or 80 steps, dependent on the selected

zoom factor (1, 2, 4, 8, or 16).

4. Stepsizes in the textfont editor are: 1, 2, 5, 10, 15, 20, 25,

and 50 steps.

5. The maximum sizes per character are 255 x 255 steps.

6. The textfont editor has two operational modes: draw and edit, which

can be activated with the t and e-key <->respectively.

In the top left corner of the screen there is an E when the

edit mode is active, and a T when the drawing mode is selected

and the pen is on the paper.

7. A font file (*.FNT) consists of pages 32 ... 255: each ASCII sign

has its own page.

One layer

The layer on which the character is drawn cannot be <->selected. However,

once a character set is ready, its characters can be placed on any

layer in the Layo1 Editor, <->although on layers 0 ... 7 there is

the restriction of the 45� and 90� angles. If you create a

character set with other angles than 45� and 90�, it can only

be used on layers 8 ... 15, with pen 1, in order to achieve the best

results.

Pen diameter

While designing a character set the shape and the definition of the

coordinates per character is the most important thing. That is why

the textfont editor does not have diffe<->rent pen diameters. Once

the letters and symbols are used in a PCB design they will receive

the selected pen diameter which you can then select for them.

Zoom

As in the Layo1 Editor, the middle mouse-key and the z-key control

zooming. You can choose from the following zoom factors: 1, 2, 4,

8, and 16. Zoom factor 8 is the standard (default) setting.

The size of the characters is decided when the text is placed in the

Layo1 Editor. Then, text can also be rotated and/or mirrored. Please

refer to section 5.

Stepsize

The cursor stepsize can be reduced and enlarged with the function

keys F9 and F10. The following settings are available:

1, 2, 5, 10, 15, 20, 25, and 50 steps.

Draw

Drawing can be activated by pressing the left mouse-key or the

t-key. The pen is put on the paper in the cursor position. Once

drawing is concluded by pressing the right mouse-key, the T

in the top left corner of the screen will disappear again.

You can go to the previous or next character (page) using the two

left and right mouse-keys respectively.

Finishing point

For the correct positioning of the characters later (the space between

them) a finishing point must be placed for each character. The top

left corner of each character (absolute coordinate 0,0) defines the

starting point of the character. This is the point with which the

character will be <->attached to the cursor once it is loaded into

the Layo1 Editor, or with which it follows the previous character.

For each character you must place the finishing point, i.e. the point

where the next character in the text is to begin.

You can place a finishing point by placing the cursor in the required

position top right of the character and pressing the F2 key.

When placing the cursor you must remember to leave enough space relative

to the next character. The finishing point thus placed will determine

the position of the zero point of the next character in a text.

Edit

By pressing the e or =-key or all three mouse-keys the

edit mode is activated: as in the Layo1 Editor existing lines can

be edited and deleted.

If the edit function is activated by pressing the e-key the

cursor will jump to the first coordinate point of the relevant character.

However, if you use the =-key or all three mouse-keys, the line-section

under the cursor will start to blink and Layo1 will ask you if this

is the line-section that you require, as in the Layo1 Editor.

All mouse-key functions in the textfont editor's edit function relate

to coordinates, as in the Layo1 Editor's edit function.

Pages

Each character in the character set has its own page from 32 ... 255.

These numbers correspond with the ASCII character codes. When placing

text in the Layo1 Editor, the characters of a character set can be

called up by pressing the corresponding key. Pressing the Shift

A key will produce the character of page 65 of the selected character

set. Small e is on page 101 of this character set.

While designing the different characters you can select the pages

by pressing the c-key, followed by the required letter or number

key: pressing the small letter e will take you to page 101 etc.

You can also go to the relevant page directly by first pressing the

c-key and then typing the required ASCII (page) number on the

numerical keypad while holding down the Alt key.

By pressing Shift + or Shift <196> you can page quickly

through the character set. You can do the same with the function keys

F7 and F8 and with the two left (forwards) or right (backward)

mouse-keys.

===> Attention:

Selecting pages in the textfont editor is only possible when the cursor

is free: there is no T or E in the top left corner of

the screen for draw or edit respectively.

-----------------------------------------------------------------------------

14.3 Key functions<$Itextfont

editor;key functions>

a

horizontal/vertical auxiliary lines on the screen

c

character selection, followed by the required character or Alt

xxx, where xxx stands for the ASCII code of the required character

(32 ... 255), to be typed via the numeric keypad.

e

activate edit on first coordinate of character

p

pen up or down in edit function

q

diagonal auxiliary lines on screen

r

select ruler 1 ... 3

t

activate draw (pen on paper)

x

enter X coordinate

y

enter Y coordinate

z

zoom 1 ... 16 (8=default)

=

activate edit and search for line section under cursor

+/-

page through datalines (coordinates)

Shift +/-

page through character sets/pages

(full stop) grid on/off

F1

help text on screen

F2

place finishing point of character

F3

load a character set from disk

F4

save a character set on disk

F5

rewrite screen

F6

select ruler 3 and set position counters to 0

F7/F8

to previous/next character (you can also use Shift -/+ or the

two right/left mouse-keys

F9/F10

reduce/enlarge cursor stepsize

Ins

place coordinate point (same as left mouse-key)

Del

delete coordinate point under cursor in edit function

**************************************************************************

======================  Plotter/Port configuration

-----------------------------------------------------------------------------

-----------------------------------------------------------------------------

15.2 Plotter configuration  TRACEURS.EXE   F9

It is possible to store the data of 10 different plotters in Layo1.

The plotter configuration menu F9  has 10 different pages (plot number center bottom).

You can page through these pages using the + and <196>-key

or the two left or right mouse-keys.

All data can be adjusted by selecting them with the mouse and pressing

the left mouse-key. or Enter key. Once a selection is made,

the setting can be adjusted using the + and <196>-key

or the two left or right mouse-keys.

Layo1 is compatible with plotters that use the HPGL or DMPL standard

and knows the specifications of a great number of well-known plotters

that use those standards.

You can select these in the field plotter name. If your plotter is

not in the list, you can specify the data here yourself according

to the documentation of your plotter.

The port to be used and any communication parameters will usually

have been specified in the port table already.

X/Y moving presets

Usually you will want to use the full range of your plotter. If however

you want to limit that range, you can do so by adjusting the minimum

or maximum limit. The former relates to the top left corner of the

screen, the latter to the bottom right corner.

Output example

This is an example of the commands that will be sent to the plotter.

===> Attention:

If your plotter is different from Layo1's standard plotters, please

refer to the documentation of your plotter.

**************************************************************************

======================  Layo1 Setup

-----------------------------------------------------------------------------

16.1 Layo1 Setup

This is Layo1's configuration menu. It will be presented to you when

you use Layo1 for the first time. It is always possible to make adjustments

later, if you like: LAYO1E /C

From Layo1's USER MENU menu

In the configuration menu you can specify the video board that your

computer uses and you can specify a number of personal preferences

(and change them again later).

Making the adjustments is a simple procedure, as indicated in the

Help window at the bottom. With the mouse or the arrow keys you select

one of the items and make your adjustment of the setting with the

+ or <196>-keys. <->Instead of the + and <196>-keys

you can also use the two left or right mouse-keys respectively.

For some settings you can only choose between on and off.

For others there are more possibilities. They will be discussed in

detail hereafter.

Video board parameters

Here you select the setting of the video board you are using. The

first time it will say H12 (EGA) here. Pressing the +

key twice will set this to H14 (VGA). Please refer to

the documentation of your video board and screen and any read.me file

on your Layo1 program disks.

Cursor movement

Mouse sense can be adjusted separately for text and graphics. The

following settings are possible: 0 ... 9.

Mouse off

Please note that setting 0 switches off the mouse completely. If this

should happen accidentally, use the arrow keys to move to the relevant

field and adjust the setting using the +-key.

Printer

This concerns the setting for the dot matrix printer (9 or 24 pins)

that is used for the checkplots.

Paper

You can choose from two paper widths: 8 or 13.8 inch.

Bit map mode

Please refer to the documentation of your printer for this setting.

There are seven settings:

60 x 72 dpi (Esc"K")

72 x 72 dpi (Esc"*5")

180 x 180 dpi (Esc"*39") (24 pins)

Laserjet 75 dpi

Laserjet 100 dpi

Laserjet 150 dpi

Laserjet 300 dpi

Screen setting

The options here are not tied to hardware: you can select and adjust

to your own liking.

16.1.0.1 Cursor type

Layo1 features two types of cursors:

SH_a

long-small

The long cursor in Layo1 consists of two square crossing lines running

over the entire height and width of the screen. These lines make it

easier to line up pads, traces and components accurately.

arrow

The second type of cursor in Layo1 is the arrow. Very useful for selecting

elements of your design in the editing stage.

16.1.0.2 Pin numbers

Layo1

Display of pin numbers in pads can be switched on and off.

16.1.0.3 Tool numbers

Display of tool numbers in pads can be switched on and off.

16.1.0.4 Block numbers

Layo1

Display of block numbers of components can be switched on and off.

16.1.0.5 Display XY metric

Display of the cursor coordinates in millimeters in the top right

corner of the screen can be switched on and off.

16.1.0.6 Display XY inch

Display of the cursor coordinates in inches in the top right corner

of the screen can be switched on and off.

16.1.0.7 Display XY mode

r

Layo1 keeps 3 rulers for you, and these in millimeters and inches

separately:

absolute

1. Here the zero point is on the actual corner of your <->design.

screen

2. Here the zero point is in the top left corner of the screen.

relative

3. Here the zero point is in the cursor position.

The selection that you make here defines the ruler that Layo1 starts

up with. While working you can switch <->between these different rulers

by pressing the r key: now you can activate rulers 1, 2 and

3.

16.1.0.8 Auto save time

Much time and work can be lost because of power failures or other

mishaps. Therefore, you are recommended to safeguard your work by

saving it on disk regularly. It is an easy procedure in Layo1. Simply

pressing F4 will do the trick. However, it easily slips your

mind when you are absorbed in your work.

Layo1 will remind you, if you like. Dependent on the set Auto save

time, Layo1 will ask you if you want to save your work once every

5 to 60 minutes. The answer yes will allow Layo1 to save your work

on disk and you can go on with your work. If you answer no, you can

go on straight away until the set time has run out again and Layo1

will remind you once again. In the mean time you can always press

F4 whenever you like.

The Auto save time can be set from 0 to 60 minutes in steps of 5.

Color table selector

Each of Layo1 's 16 layers has its own color. You can <->adjust these

colors as you wish with the help of this option.

When you select the Color Table Selector you will see a screen with

the 16 layers and colors displayed in horizontal as well as vertical

bars. Thus, you can easily see the contrasts between them. Moreover,

if you press Shift F5 you can see the effect of two overlapping

colors/layers. You can switch back with F5. With the mouse you

can <->select a layer and using the two left and right mouse-keys

(or the + and <196>-keys) you can adjust the color per

layer.

Please refer to section 3 for information on the use of the different

layers.

Save Config Exit

Pressing the left mouse-key or the Enter-key on this option

will save the specified configuration menu on disk. Next time, Layo1

will start up using these settings. If you do not want to save the

new settings and quit from the configuration menu, just press the

right mouse-key or the Esc-key.

**************************************************************************

======================  Options

The mouse-key functions of a  digitizer are:

Key 1:  same as left mouse-key of ordinary mouse

Key 2:  center in cursor position

Key 3:  move cursor

Key 4:  enter

**************************************************************************

======================  Appendices

-----------------------------------------------------------------------------

18.1 Netlist

An example of a netlist, as generated by Calay compatible scheme designers

such as e.g. OrCAD SDT:

TUTOR.NET

/N00001 U4(8) U2(6) U5(1) U4(2);

/N00002 U5(12) U4(5);

/N00003 U5(8) U4(4);

/N00004 U4(6) U1(5);

/N00005 U3(1) U2(11);

/N00006 U3(2) U3(5) U3(10) U3(13) U4(3);

/N00007 U3(4) U2(10);

/N00008 U2(9) U3(9);

/N00009 U4(9);

/N00010 U4(10);

/N00011 U5(10);

/N00012 U2(1) U1(1);

/N00013 U2(2) U1(2);

/N00014 U1(6);

/N00015 U1(4) U2(5);

/N00016 U3(3);

/N00017 U3(6);

/N00018 U3(8);

/N00019 U2(4) U4(1);

/N00020 U3(11);

/VCC U4(14) U3(14) U2(16) U1(16) U5(14);

/GND U4(7) U3(7) U2(8) U1(8) U1(3) U5(7);

/N00023 U1(15);

/N00024 U1(14);

/N00025 U1(13);

/N00026 U1(12);

-----------------------------------------------------------------------------

18.2 Component list

An example of a component list, as generated by Calay compatible scheme

designers such as e.g. OrCAD SDT:

TUTOR.CMP

74LS00 		U4  shape -X- -Y- 0

74LS08		U3  shape -X- -Y- 0

74LS138		U1  shape -X- -Y- 0

74LS138		U2  shape -X- -Y- 0

TIME DELAY 		U5  shape -X- -Y- 0

-----------------------------------------------------------------------------

18.3 Library files

Example of a library file:

TTL.LIB

74LS00

74LS08

=\LAYOPLUS\LMC\ACTIVE\DIL-14

74LS138

TIME DELAY

=\LAYOPLUS\LMC\ACTIVE\DIL-16

Such a library file can be created with an ordinary word processor,

on condition that it is saved as an ASCII file.

On every line there is the name of a component as used by the scheme.

After this name or group of names the next line contains the name

of the corresponding Layo1 component, complete with full path name

and preceded by the =-sign. Components names are not allowed

to begin with the =-sign.

Once you import a component list of a new design in Layo1, the

program will look for the right Layo1 components in the libraries

specified in the PCB configuration and place them.

-----------------------------------------------------------------------------

18.4 Hints and tricks

There may be some hints in this section that might be of practical

use to you.

Making a copper face

Draw the first line of the copper face. Select the line in Multi Function

and copy it next to each other as often as necessary. A rectangle

appears. Using a window you can adjust the <169>ends<170> of the face

if necessary.

Two layers next to each other

Suppose you want the two layers of a double-layer PCB next to each

other on the screen. This is what you do. Switch off layer 1 by typing

d F1. Now place a window around the print in Multi Function,

within the corner lines, and make a copy of layers 2 and 15 next to

the <->original drawing. Now type d a.

Making a text plate

It is a simple procedure in Layo1 to make a text plate for a test

casing or a test series. For a carrier take clear LEXAN plate, or

single-sided matted plot film. Make the plot in reverse. (I.e. on

the <169>back<170> of the carrier). Spray a covering paint on the

plot in the color required for the text plate. Use synthetic paint,

e.g. a spray-can. Apply a film of glue on the front plate which has

already been drilled. Use spray glue or adhesive film. The latter

is available on peel-off carriers. Now carefully apply the plot to

the front plate: the paint on the glue. Now you have a neat smear-proof

text plate. If you use colors the results will be even more beautiful.

PCBs with irregular shapes

It will often happen that a PCB does not have a rectangular shape.

In that case you can define the irregular shape with pads that you

place so close together that their (big) hole diameters overlap.

Of course, it would be much better if your PCB could be cut in the

right shape by the PCB manufacturer. In that case draw the required

shape of the PCB e.g. on layer 12 (higher than 7 in any case) and

make a separate plot file of this layer to send to the PCB manufacturer

together with the other data.

Increasing working speed

The working speed of Layo1 (not the junior version) can be increased

significantly by placing the following files on a RAM-disk (if possible)

before starting Layo1:

LAYO1EDI.EXE

and all files with a .LPU and .DAT extension.

Consult your DOS manual on how to set up a RAM-disk.

-----------------------------------------------------------------------------

18.5 Problems

Autorouter draws tracks missing

the pads

The Autorouter works on a basis of a 1/20 or 1/40 inch grid. If any

pads were placed (e.g. while drawing the components) while a smaller

stepsize was active, there is a great risk that they will be placed

just missing the Autorouter grid which results in faulty connections.

The Autorouter does not work

First draw a window around the design: this will define the area in

which the Autorouter is allowed to draw its tracks.

The Autorouter <169>hangs<170> the computer

Difficult connections may take some time. Especially when the Lee-router

is activated and large error values are set. If it takes to long just

press Esc.

Move/copy/delete pad does not work

Pads that belong to components that have been imported via a netlist

and a component list in Layo1 cannot be manipulated.

F1 does not work

Help-key F1 will only work if you are working in a function

within the Layo1 Editor. It will not work whenever there is a menu

or submenu on the screen.

Multi Function does not

work

In Multi Function you can manipulate parts of your <->design. As long

as your screen is blank there is little to be done for Multi Function:

it will not work.

Plot open pads does not work

If you have specified that the pads are to remain open (drilling hole)

for the plotting and this is not the case everywhere, you may have

drawn a track across these pads without placing a coordinate point

in that track (with Ins or the left mouse-key).

Plotting from a plot file does not work

If you do not send your design to a plotter directly but to a plot

file and if you want to send it from DOS via a <->serial port to a

plotter later, please see to it that the serial port is defined as

the printer port and that the settings of baudrate, bits, stopbits

and parity are correct. You can <->adjust these parameters with the

DOS command MODE.

Lost drawing

After all kinds of manipulations it may happen that you cannot get

your drawing back on to the screen: it is in an off-screen position.

The following series of keystrokes will help you: z z d a. The first

key will activate the zoom function, the second z (or middle

mouse click) puts your entire design (if it has corners) in the centre

of the screen, d and a will then display all layers.

If your design doesn't have corners yet try this: z F1 d a.

-----------------------------------------------------------------------------

18.6 Layo1 help guide

Pads

Pad nr	in inches	in millimeters

0 	<+>3/80	0.95

1 ... 4 	<+>8/80 x 5/80 	2.54 x 1.59

5 and 6	11/80 x 5/80 	3.49 x 1.59

7	<+>5/80	1.59

8	<+>7/80	2.22

9	<+>9/80	1.27

10	11/80	3.49

11	13/80	4.13

12	15/80	4.75

13	17/80	5.40

14	19/80	6.03

15	21/80 	6.67

Here you can see the pads in actual size

Tool symbols

**************************************************************************

-----------------------------------------------------------------------------

.1 Editor key functions

Ins

Adds a dataline when drawing, editing and copying. Usually the left

mouse-key.

Enter

This command does not add a dataline but concludes a trace. Usually

the right mouse-key.

Del

Deletes dataline/spot/coordinate in cursor position.

F1 ... F10

In addition to the functions listed hereunder, the function-keys can

also be used to select a zoom factor, a spot, a pen or a tool number.

However, this is only possible when the relevant menu line is on-screen.

F1 will then select zoom factor, pen, spot, tool number or layer

1. For selections over 10, use Shift F1 etc.

F1

Displays current mouse functions.

F2

Places or removes a via (connection between top and underside of the

design) in the cursor position.

F3

Loads a component from the databank in the Advance function.

F4

Saves layout or component.

F5

This function-key rewrites the picture: it is especially useful to

rewrite the picture after placing and removing traces.

F6

This gives the current cursor position the coordinates 0.0 which makes

it simple to measure the distance between parts of your design with

the cursor. You can reset the original coordinates by pressing r

1.

F7

Selects layer 1 (or a layer defined by yourself, cf. Shift F7).

F8

Selects layer 2 (or a layer defined by yourself, cf. Shift F8).

F9

Reduces the stepsize in inches. Not within the Autorouter where the

stepsize is fixed at 1/20 or 1/40 inch.

In millimeters: selects the defined stepsize (10 steps) with Shift

F9.

F10

Enlarges the cursor stepsize in inches. As F9.

Shift F5

Switches between overlapping and mixed color display.

Shift F7 / F8

Selects a layer that can then be activated with F7/F8.

Shift F9 / F10

Defines the stepsizes in millimeters for F9/F10.

Ctrl F1

Resets ratsnest (removes layer 14) in Layo1.

Ctrl F2

New ratsnest of the spot in the cursor position, with new calculated

shortest connections in Layo1.

Ctrl F3

New ratsnest of component at cursor, with new calculated shortest

connections in Layo1.

Ctrl F4

Updates the ratsnest in Layo1. All made connections are

removed from the ratsnest, so that you can easily spot which connections

have not yet been made.

Ctrl F6

Checks all connections according to the netlist in Layo1.

Faulty connections are marked with a small cross on layer 13.

a

Places or removes horizontal and vertical auxiliary lines on the screen.

b

Selects a block (component) in order to move or rotate it.

c

Centers the screen in the cursor position.

d

Displays the use of the layers. You can specify for each layer whether

or not it is to be displayed. You can do this with the mouse or with

function-keys F1 ... Shift F5. To display all layers first press

d, then a. To switch off all layers, first press d,

then c.

f

Centers cursor on the screen when the cursor is free (not while drawing,

editing and placing spots).

i

Switches from millimeters to inches.

l

Selects one of the layers. Selection can then be made with the mouse

or with function-keys F1 ... Shift F5.

m

Switches from inches to millimeters.

n

Answers no to yes/no questions.

p

Selects pen number/diameter. Selection can then be made with the mouse

or function-keys F1 ... F7.

When moving: new zero point.

q

Places or removes diagonal auxiliary lines in the cursor position.

r

Ruler selection. You can select one from settings 1 ... 3:

1: sets zero point back to original 0,0 coordinate

2: sets zero point to coordinates of top left corner

3: sets zero point to current cursor position

In fact, Layo1 has 6 rulers because the three rulers mentioned above

are all kept up-to-date in inches as well as millimeters.

s

While placing spots: select a spot. Selection can then be made

with the mouse or the function-keys F1 ... Shift F5.

In Advance the s-key is used to activate the function

for placing spots. Once this function is active, this key works as

described above.

t

While placing spots or editing: select tool number. Selection

can then be made with the mouse or the function-keys F1 ... F7.

In Advance the t-key is used to activate the draw function

of Layo1. While <->placing spots in Advance this keys works as described

above.

w

Activates the window function in Advance, draw and elsewhere,

and simultaneously sets the starting point to the current cursor position.

Then, it is similar to pressing the two outer mouse-keys, in which

latter case it is still possible to select the starting point with

the cursor.

x

Change the cursor position on the X-axis by typing the new position.

y

Idem, but now for the Y-axis.

Answers yes to yes/no questions.

z

Zoom setting (n = normal/Enter = center). Usually this

function is activated by pressing the middle mouse-key.

=

In Advance: searches a drawn symbol, near the cursor, in order

to edit by means of commands to be specified. Simultaneously pressing

the three mouse-keys has the same function. Cf. Edit.

+

Pages forward through datalines while editing a design. Usually, the

two left mouse-keys are used.

-

Pages backwards through datalines while editing a design. Usually,

the two right mouse-keys are used.

*

(Asterisk) Places cursor on nearest point within the current stepsize.

**

(Two asterisks) Centers cursor on nearest grid point.

,

(Comma) Switches component imprint (layer 8) on/off on the screen.

;

(Semicolon) Switches layer 9 (texts) on/off on the screen.

.

(Full stop) Grid display on/off.

Esc

Concludes current activity. This will usually take you to the main

menu. This command is usually given with the right mouse-key.

Spots

Spot nr	in inches	in millimeters

0 	<+>3/80	0.95 (via)

1 ... 4 	<+>8/80 x 5/80 	2.54 x 1.59

5 and 6	11/80 x 5/80 	3.49 x 1.59

7	<+>5/80	1.59

8	<+>7/80	2.22

9	<+>9/80	1.27

10	11/80	3.49

11	13/80	4.13

12	15/80	4.75

13	17/80	5.40

14	19/80	6.03

15	21/80 	6.67

Here you can see the spots in actual size

Tool- symbols

To plot out via TRACEURS

hgpl or other plotter

DRILLMENU

Each tool is represented by a cross with closed side clockwise different for each tool