SLODOWN

Version 1.0

Copywrite (c) 1987 by Mark Webster

Program Description:

SLODOWN is a resident addition to DOS that reduces the

processing speed of your computer by approximately 500%.  It was

designed to allow games written for a standard PC or XT to run on

faster systems such as AT's which use an 80286 chip. The program

functions by intercepting the system clock tick interrupt.  Once

installed (by entering the command SLODOWN), at every tick (18.2

times a second) the system is put in a processing loop of 4500

cycles.  (This is enough to slow an 8mhz 80286 system down to the

equivalent of a standard 5 mhz 8088 pc.)

The program can be turned ON (slow speed) or OFF (normal

speed) through the commands SLODOWN ON and SLODOWN OFF.

Communication with the installed program (to turn it ON and OFF)

is accomplished through use of the unformatted data area of the

program segment prefix of the installed program.  This

communication can be disrupted by other programs which intercept

the system clock tick (such as a printer spooler).  If

communication is not possible the user is notified.

SLODOWN should be compatible with most programs.  If

interference is caused by another resident program, try switching

the order in which the programs are installed.

Commands:

SLODOWN - Entering SLODOWN without parameters installs the

resident portion of the program and slows down the

system.  As SLODOWN is a resident addition to DOS,

available memory will be decreased by the size of the

program (300 bytes).  SLODOWN checks during installation

to see if the resident portion has already been

installed.  If it has, or if another resident program

exists which may interfere with the operation of

SLODOWN, the user is given the option to abort or

continue  with the installation.

Installing more than one copy of SLODOWN serves no

useful purpose.  Every copy installed consumes 300 bytes

of memory, increases clock tick interrupt service

overhead, and causes the setting of all but the last

copy installed to be fixed (it can no longer be changed

from ON to OFF or OFF to ON).

SLODOWN ON - Turns SLODOWN ON.  System will be slowed down by

approximately 500%.  Error messages are returned if

SLODOWN has not been installed or if changing the

setting is not possible.

SLODOWN OFF - Turns SLODOWN OFF.  System will be returned to

normal speed.  Error messages are returned if SLODOWN

has not been installed or if changing the setting is not

possible.

SLODOWN ? - Program information (you're reading it).