CoreView(tm)

A Window Into Your PC Memory

Version 1.01

Copyright (c) 1989-1991 RolySoft, Denmark

This product is donated to the public domain

What is CoreView?

-----------------

CoreView is a small but fast memory  viewer.  With help of CoreView you

may  look through  the  memory of  your  computer.   CoreView also  has

facilities  for   searching  memory  for   specific  strings   or  BIOS

extensions.

CoreView is  particular a great  tool if you  are going to  install new

hardware into  your computer.   Often you  have to  set jumbers  on the

add-on card to install its BIOS into the right address space.  But WHAT

is the right address space? With CoreView you may browse through ROM at

C000h through F000h and look for empty slots.

CoreView  is also  capable of  viewing memory  in real  time.  You  may

enable a function  called "timed display".  While this  function is on,

CoreView continously updates the screen with  data from memory.  If any

bits change, you will see it immediately on the screen.

What's new in version 1.01?

---------------------------

Version 1.01 is only a minor update.  No new functions have been added.

The  only  new  facility  is the  better  co-existence  with  DESQview.

CoreView now recognizes DESQview  and enables you to work with  it in a

window.  CoreView has  also been recompiled using the  new Borland C++,

which produced a slightly faster code.

A quick tour through the CoreView functions

-------------------------------------------

CoreView has many functions.  As all functions  are easy to use, I will

only use little  space to explain them.   You may call the  help screen

inside CoreView  by hitting  [Alt+H] from the  main screen.   This help

screen summarises the functions.

First of all you  may scroll through memory.  To do  this, you use [#],

[#], [Home], [End] etc.   All PCs are capable of addressing  at least 1

Mb of memory.  With  CoreView you may inspect this 1  Mb address space.

By using the  [#], [#], [PgUp], [PgDn], [Home], [End]  and many control

keys, you may browse through this 1 Mb address space.  At the bottom of

the PC  memory there is  always RAM installed.   When you go  to higher

addresses,  the PC  has ROMs  installed.  RAM  is memory  which may  be

changed by the CPU.  ROM is Read Only memory, which means it may NOT be

changed.  If CoreView encounters  a ROM area, you will see  a stars (*)

right after the addresses.  This star tells you, that you are currently

looking at ROM locations.

If you are using a colour monitor, you may have noticed that some bytes

are high-lighted while others aren't.  All standard figures and letters

are shown in a  bright colour by CoreView to make it  easier for you to

recognize human-readable strings.

If you  want to  search for a  specific string, you  may do  that.  Hit

[Alt+S] to enter the search input screen.  Enter the string you want to

search for.   In the upper  input window  you may enter  all characters

between 20h  and FFh.  If  you also need to  search any of  the control

characters, you  should press [Tab] once,  and enter the  characters as

two-digit hex numbers.  Hit [Enter] to begin the search or hit [Esc] to

abort. [F10] will clear the input fields.

You may choose  to search for an exact  match, or you may  search for a

match, which  is case-insensitive, eg. uppercase and  lowercase letters

are equal.  Use [Alt+C] in the main screen to change the search case.

If CoreView finds your search string, and you want to repeat the search

for the next copy, then press [Alt+R] to repeat.

CoreView displays  a search message  in the  lower right corner  of the

screen, while  it is searching.  The  search is NOT  interruptable.  On

386s a  1 Mb search  takes approximately 2  seconds.  On XTs  and other

relatively slow  computers, the search may  take 15 seconds or  more to

finish.

If you want to jump to a total different address, you may enter it into

the address  window.  When you press  [Enter], CoreView is set  to this

new address.  You may also pick up  the current address you are looking

at,  and put  it into  the input  box.   Hit [Ctrl+Enter]  to read  the

address from  the current  window position.  If  you want  to normalize

this address,  just hit [Alt+N].  A  normalized address, is  an address

with an offset part always in the range 0000h through 000Fh.

CoreView provides you with two functions to specify how data are shown.

If you press  [Alt+W] memory is displayed in wide  format.  This allows

you  to see  more data  on  a single  screen.   To switch  back to  the

standard hex display, hit [Alt+W] once more.  [Alt+A] acts as a filter.

If you press [Alt+A], only ASCII characters are displayed.  This filter

is useful  when you are  looking into text  strings and don't  want the

code displayed.  Most code contains opcodes outside the ASCII range.

A final word...

---------------

The CoreView program and its documentation have been donated the public

domain.

Please report  any problems to ROLAND  LYNGVIG at the  Josti-BBS, phone

+45 47 38 05 24 (DANE TECH 2:230/31 or SDNet/Work: #551.13).

Thank you for using CoreView...

RolySoft, Denmark