Windows BarCode Generator

Version 1.6

(C)opyright 1990-1991

All Rights Reserved

Stellar Technologies

2510 Metro Blvd

St. Louis, MO 63043

Wbar is user-supported software provided to you at no

charge on a trial basis. If you use this software and

find it of value please send $15 for a registered copy

and support the shareware concept. Registered users

will be provided with information about program

updates. Please make checks payable to Stu Gisburne at

the above address. You are encouraged to share this

software with others provided that it is distributed

complete with documentation and in unmodified form and

that no fee or other consideration is charged or

accepted. All commercial, business and government users

must register each copy in use. I hope you enjoy using

Wbar. Your comments and suggestions to improve this

product are always welcome.

PROGRAM DESCRIPTION

Wbar is a program which generates bar codes (ie: 3 of 9, UPC,

Postnet, etc) that can be copied to the Windows Clipboard. Once

in the Clipboard, you can then paste the barcode into other

programs such as Windows Write, MicroSoft Word for Windows,

Paintbrush, as well as other Windows programs that can accept

bitmaps or metafiles from the Windows Clipboard.

HARDWARE REQUIREMENTS

IBM Personal Computer or Compatible

MicroSoft Windows 3.0

Mouse

PROGRAM FILES

The Wbar system comes in two files:

WBAR.EXE	executable program module

WBAR.DOC	document that you are reading now

GENERAL PROGRAM OVERVIEW

Upon execution, Wbar produces a typical Windows program

window. The menu consists of File, Edit, Action, Type

and Help. When first loaded, the About Wbar Dialog Box

appears if you have an unregistered copy of Wbar,

otherwise you are presented by blank window. The cursor

takes on the shape of a barcode while in the workspace

of the Wbar window. To make a barcode, you simply

position the cursor in the window where you want to

place the barcode and click the left button on your

mouse. A dialog box will appear allowing you to toggle

various options, select what type of barcode to produce

and to input the barcode data. After all this is done,

clicking on OK will produce the desired barcode on the

display. If you have the AutoMeta mode ON then a

metafile of the barcode is copied to the clipboard. If

wish to copy a bitmap representation of the barcode to

the clipboard, you then	choose ACTION from the main menu.

Clicking on SELECT from the ACTION submenu will toggle

the cursor to become a cross. You then press the left

mouse button and drag a box around the barcode. After

releasing the left mouse button, clicking on COPY from

the EDIT menu copies the selected box area to be pasted

in Bitmap form to the Windows Clipboard.

MENU FUNCTIONS

FILE

NEW     basically clears the Wbar window of any barcodes

EXIT	exits the Wbar program and closes the window

EDIT

COPY	copies a selected area to the Clipboard

PASTE	pastes from the Clipboard to the upper left area

of the Wbar window

CLEAR	another way to clear the Wbar window

ACTION

BARCODE	toggles the cursor to barcode mode

SELECT	toggles the cursor to the area select mode

TYPE

COLOR	when checked, any copies to the clipboard will

a color bitmap

MONOCHROME  when checked, any copies to the clipboard will

be a monochrome bitmap

AUTOMETA	when checked, any barcodes produced on the display

will be automatically copied to the clipboard in the Windows

MetaFile format. Very useful to produce barcodes

of exact size. Also the best way to paste barcodes

into Corel Draw 2.0 and procduce POSTNET barcodes suitable

for printing on envelopes.

Note: Metafiles pasted into other applications may not look

correct on the display, but should print correctly.

HELP

HELP	currently not used at this time

ABOUT	displays a dialog box containing information about

the Wbar Program

Barcode Types currently supported in Wbar:

UPC-A

Universal Product Code. Commonly used in the supermarket industry

since 1973. UPC is a coding system as well as a symbology, used to identify

a product and its manufacturer. UPC-A is a fixed length numeric barcode

which encodes 12 digits.

0 04560 17100 8

The first digit is the industry code. For example 0 is grocery and 3

is pharmaceutical. Digits 2 thru 6 specify the manufacturers product

number. Digits 7 thru 11 contain the manufacturer ID number. The remaining

12th digit is a check digit calculated from the preceding digits.

Interleaved 2 of 5

A variable length, high density code containing the digits 0 thru 9.

3 of 9

An alphanumeric code consisting of the upper case letters "A" thru

"Z", the digits "0" thru "9", and the characters "-", "+", ".", "$", "/",

"%" and the space character. It is of a variable length and can contain up

to 100 charcters in this version of Wbar.

POSTNET

The POSTNET bar code was developed by the Postal Service to provide a

system of encoding Zip Code information on letter mail, which can be read

reliably by relatively inexpensive bar code sorters. The code is made up of

binary elements printed in the lower right corner of the mail peice as tall

and short bars representing the Zip Code or Zip+4 code.

The location of the complete Zip+4 is very presice. The baseline of

the code should be 1/4" (+/- 1/16") from the bottom edge of the mail piece.

The start (left-most) bar should lie between 3 1/4" and 4" from the right

edge of the mail piece.

STELLAR TECHNOLOGIES MAKES NO WARRANTIES, EXPRESS OR

IMPLIED, AND SPECIFICALLY DISCLAIMS ANY IMPLIED WARRANTIES

OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE.

STELLAR TECHNOLOGIES FURTHER RESERVES THE RIGHT TO MAKE

CHANGES TO THE SPECIFICATIONS OF THE PRODUCT WITHOUT OBLIGATION

TO NOTIFY ANY PERSON OR ORGANIZATION OF SUCH CHANGES.

LIMITATION OF LIABILITY:  The Customer's sole remedies are

set forth in the warranty clause above.  Stellar Technologies

liability for damages shall not exceed the current registration

fee for the Product. In no event will Stellar Technologies be

liable for any special, incidental or consequential damages even

if Stellar Technologies has been advised of the possibility of

the same.

Comments and inquiries may be

sent to me through Compuserve ID number

74516,2776

or to:

Stellar Technologies

c/o Stu Gisburne

2510 Metro Blvd.

St. Louis, MO 63043

Revision History

================

Version 1.0  -	07-14-90	- Pastes UPC-A bar code to the clipboard

Version 1.1  -	08-05-90	- Added 3 of 9 barcode

Version 1.2  -	08-22-90	- Added Postnet barcode

Version 1.3  -	09-13-90	- Added monochrome bitmap capabilites to allow

pastes into Corel Draw 1.21

Version 1.4  -	11-30-90	- Added Interleaved 2 of 5 barcode

Version 1.5  -	01-21-91	- Included documentation with file

Version 1.6  -	02-17-91	- Fixed Postnet, incorrectly calculated check

digit when the sum of digits in the zipcode was

zero.

- Added Windows Metafile capabilites. This enables

Wbar to produce exact size barcodes and sets up

Wbar for Dynamic Data Exchange with other

programs.