+----------------------------------------------------------------------+

|                           JOYSTICK                                   |

| Version 4.1               4/23/1988           Steven Rich            |

|                                               24162 Gleneyrie        |

|                                               Southfield, Mi. 48034  |

|                                                                      |

|     Released to the Public Domain for Non-Profit, Non-Commercial     |

|     Use ONLY.                                                        |

|----------------------------------------------------------------------+

REVISION HISTORY

Version 4.1 - 04/23/1988 -Couldn't leave well enough alone.

1) Recompiled using QuickBasic 4.0 (and I thought 2.0 was

good!)

2) Added MAX, MIN, and AVERAGE for X & Y Axis.

3) Cleaned up problem when buttons were tested (messed up

screen).

4) Mild graphics added to display.

Version 3.1 - 01/11/1987 -First Version released to Public Domain

-----------------------------------------------------------------------------

Being very picky about all my computer equipment, I saw a need for a

program that would quickly test out the operation of a joystick, and make the

calibration of the trim pots easier. I came up with JOYSTICK.

Operation of the program is straightforward.  An opening screen is shown

and you are asked which joystick port (1 or 2) you are hooked up to.

Answering that, the program begins.

NOTE:  The only way to change the port is to exit (with <CTRL>-<BREAK>)

and start again.  If you have two joysticks, run the program for one

port, then exit, then select the second port and run it again.

Seven areas of information are presented:

JOYSTICK UNDER TEST - At the top center.  Shows 1 or 2 depending on which

port you have chosen to test.

JOYSTICK TREND - Shows with arrows the apparent direction the Joystick

is being moved in.  If no motion is detected, a "0"

appears in the center of the arrows.  Look for relative

smoothness of operation.  This test revealed a bad

potentiometer in my first joystick.

BUTTON 1 - Flashes whenever the first (usually RED) trigger is

pressed.  This is usually the button found at the top

of more expensive joysticks.  The final color of the

display has NO significance at all.

BUTTON 2 - Flashes whenever the second (usually BLACK) trigger is

pressed.  The final color of the display has NO

significance at all.

Note that some slightly erratic behavior may be noticed

in the flashing of the button displays.  This is

because I was forced to STOP the execution of the

button "Interrupts" while doing screen writes.  The

behavior is the result of wanting a clean display and

does NOT mean the buttons are bad.  Try not touching

the joystick when you test the buttons for a fairly

accurate display.

ABSOLUTE VALUE - Probably the most useful display.  Shows the numeric

value that Basic is receiving for the current joystick

position.

MAX, MIN, AVG  - Shows the Maximum, Minimum, and Average (Mean) values

for the X & Y potentiometers.  this information is

necessary to accurately adjust the joystick.

Most joysticks require initial calibration of trim.  When the values

displayed obtained when joystick is CENTERED are ** BOTH ** EXACTLY one HALF

those shown when the joystick is moved to the LOWER RIGHT position, the

joystick is properly trimmed.

EXAMPLE:

1)  Rotate the Joystick to every possible location you can (i.e make a

big clockwise circle covering all points).  The program will note

the maximum and minimum positions for both the X & Y axis.  The

Average of the max and min along each axis is also computed.

2)  Let go of the joystick.  The ABSOLUTE VALUE display should show

values close to the respective X & Y AVERAGE display.

ABSOLUTE VALUE

X-Axis         120          125        Y-Axis

MIN   MAX   AVG     ^            ^     AVG   MIN   MAX

0    246   123  <--|            |-->  130    10   250

The absolute value is unimportant. The fact that it is equal to the

Average is what counts.

Adjust the potentiometers, if accessable, per the maunfacturers

recommendations, so that the ABSOLUTE VALUE and X & Y AVERAGES Match.

----------------

PROGRAMMING NOTE

This program is written in QuickBasic V4.0.  The .ARC file should

contain this documentation, a standalone .EXE file compiled with BCOM40.LIB,

and the source code.

I have included the source code for several reasons.  First, this is

for the Public Domain, so I feel including source code is ESSENTIAL.  Second,

if you have QB V4.0, you can create a 'Standard' version which uses BRUN to

conserve space in your system.  Third, if this can form the basis of someone

elses program, so be it. Fourth, if you want to make changes, have fun.

Forth, there is no better insurance against "Viruses" than to recompile

source code.

This program was no big deal, but it has shown me some of the power

(and speed) of QuickBasic.  I think it is an excellent tool.

Steven Rich