HGCIBM.COM Version 2.01

Copyright (C) Athena Digital, Gary Batson 1986, 1987

145 Green Hills Rd. - Athens, Ga. 30605 - (404) 354-4522

This manual dated April 20,1987

1

CONTENTS

HGCIBM.COM

Acknowledgements .............................................  3

Trademarks ...................................................  3

Using HGCIBM.COM .............................................  3

Precautions ..................................................  3

Installation .................................................  4

Using the Command Line .......................................  5

HGCTEST program ..............................................  5

Sales and Distribution Policies ..............................  6

User Comments ................................................  6

History ......................................................  6

Future .......................................................  7

Changes and Corrections ......................................  7

Technicalities ...............................................  8

Extended Functions ...........................................  8

ATHENA.SYS

Purpose ...................................................... 11

Features ..................................................... 11

Control Program .............................................. 12

Sales and Distribution Policies .............................. 12

Upgrades ..................................................... 13

Software Author Note ......................................... 13

Extended Functions ........................................... 14

Library Versions ............................................. 18

QBDEMOC program .............................................. 18

This program is distributed as Shareware. All rights are

reserved except for distribution of the program and its

documentation IN ITS ENTIRETY and un-modified. You may test and

use this program for 30 days, after which you must register it,

destroy it, or pass in along to a friend. If you would like to

keep HGCIBM, see the pricing below. This DOES NOT APPLY to those

who contributed to HGCIBM Version 1.11. Those individuals are

welcome to this upgrade without further remuneration. Please

accept it with my THANKS!!

Athena Digital

145 Green Hills Rd.  -   Athens, Ga.   -   (404) - 354 - 4522

2

ACKNOWLEDGEMENTS

Athena Digital would like to thank IBM for creating the

Personal Computer and Hercules Computer Technology for making PC

graphics as pleasurable as PC TTL text.

I would also like to thank my wife for putting up with my

pre-occupation all these years, and one Scott Schultz for his

proding and enthusiasm without which the first simple emulator

would probably never have been uploaded.

TRADEMARKS MENTIONED

Hercules Graphics Card is a trademark of Hercules Computer

Technology

IBM is a trademark of International Business Machines

Microsoft Mouse and Quickbasic are trademarks of Microsoft

Corporation

ATHENA.SYS is a trademark of Athena Digital

USING HGCIBM.COM

The purpose of this program is to provide owners of Hercules

Graphics Cards* a means to run software written for the IBM*

Color Graphics Adapter, and to introduce a commercial version of

the Athena Digital emulator called ATHENA.SYS (T) which offers

added functionality.

PRECAUTIONS

MONOCHROME DISPLAY DAMAGE !!!

This program will not damage your display, but certain

application software certainly can. Programs which attempt to by-

pass the BIOS (Basic Input/Output System) to set the CRT

operating parameters will cause severe problems. Fortunately

since most software operates in standard modes, most software

vendors permit BIOS to set up the controller for them even if

they later write to screen ram directly. HGCIBM will intercept

the BIOS calls for all standard modes and program the controller

correctly. Writing directly to screen ram is permitted and the

emulator will still work properly. When trying an un-tested (with

HGCIBM) program for the first time, watch your display carefully

for signs of gross distortion or shrinking when the program sets

graphics mode. IF THIS OCCURS, RE-BOOT OR TURN OFF YOUR COMPUTER

IMMEDIATELY. The display should return to normal quickly when you

re-boot.

3

INSTALLATION

To install the emulator, you must tell it what mode to start

out in, and a little information about your hardware. There are

three "modes" of operation for HGCIBM as follows:

The /E[mulate] mode enables the emulator to respond to bios

calls to set standard CGA modes and provides maximum

compatibility with programs expecting to use the Color Graphics

Adapter, but displays the information on your Hercules adapter.

When the emulator is in this mode, the text buffer is at segment

B800 and the equipment flag is set to indicate that a CGA card is

present. Alas, you can't fool all the programs all the time. Some

of them read the equipment flag and then try to manipulate the

crt controller directly in order to provide faster screen output.

Naturally they try to do this at the CGA port addresses (which

don't exist) and will hang the system (ATHENA.SYS provides

algorithms to circumvent these hang-ups). A re-boot will cure the

problem and you should then remember to either un-install the

emulator before running these programs or simply place it in

/M(ono) mode or /C[GA] mode (if you have a physical CGA) as

described below.

The /M[ono] mode effectively disables HGCIBM without

uninstalling it. The monochrome card is selected and Video BIOS

control returns to the computer's ROM (Read only Memory).

The /C[ga] mode also disables HGCIBM and returns control to

ROM except the physical Color Graphics Adapter is selected if it

exists.

There are also 3 installation switches available to warn

HGCIBM about special hardware. During initialization, the program

tries to determine if a Color Graphics Adapter is present. If it

is, the program will not install unless you assure it that the

Color Graphics Adapter in your system is compatible with Hercules

Graphics cards. If yours is or you have a Leading Edge computer,

then you should use these switches:

The /H[ercules compatible] switch instructs HGCIBM to

install and allow the emulation mode even though a Color Graphics

Adapter is present. You may use this switch on installation if

you have a Hercules compatible CGA card in your system. If a CGA

card is present, HGCIBM will abort installation unless the /H

switch is used. Also note that some software will output to the

CGA registers (3D4, 3D5, 3D8, etc.) to adjust video parameters.

This can wreak havoc on the real CGA adapter while in Emulation

mode. If the CGA is not physically present, this output causes no

problems and is ignored by Hercules adapters.

The /L[eading Edge] switch instructs HGCIBM to install if

the Leading Edge Model D is connected to a TTL display.

4

The /U[n-install] switch instructs the emulator to reset all

vectors and remove itself from memory, freeing the space for

other programs. You may un-install the emulator provided that

other programs have not altered vectors 8h and 10h, but memory

may become segmented if you have loaded another resident program

after HGCIBM. The system will be left in the same condition it

was in before the installation. (i.e. monochrome display).

USING THE COMMAND LINE

Simply type the command HGCIBM with a mode switch to install

or change to that mode. If you have special hardware, ADD the

hardware switch to the mode switch on the same command line as

shown in the following examples:

HGCIBM /M      - installs in (or changes to) MONO mode (emulation

disabled)

HGCIBM /C      - installs in (or changes to) CGA mode (emulation

disabled)

HGCIBM /E      - installs in (or changes to) Emulation mode.

HGCIBM /U      - un-installs the emulator

HGCIBM /M/H    - informs HGCIBM that a Hercules compatible CGA is

physically present.

HGCIBM /M/L    - informs HGCIBM that you have a Leading Edge

computer.

These commands may be issued from a batch file if desired.

Hardware switches are needed only on installation.

To change modes after installation, just enter the command as

above. HGCIBM always checks to see if it is already installed and

will process the command correctly.

HGCTEST PROGRAM

This program is provided for you to test the operation of

HGCIBM with your computer configuration. Before running HGCTEST,

Load the emulator with the /E[mulate] switch. HGCTEST.BAS runs

under BASICA. Just enter BASICA HGCTEST to run it (assuming

BASICA is on the default drive with HGCTEST). The program is a

simple graphics demo and is self explanatory. If you have a basic

compiler, the demo program can be compiled to test the compiler.

5

SALES AND DISTRIBUTION POLICIES - HGCIBM.COM

There is no limitation on distribution of HGCIBM.COM as long

as no remuneration is accepted in connection with it's

distribution, and as long as it is distributed un-modified and in

it's entirety. You are permitted a 30 day trial period to decide

whether or not you wish to keep HGCIBM or purchase ATHENA.SYS

after which you must either register it with Athena Digital,

destroy it, or pass it along to someone else. The registration

fee for HGCIBM.COM is $9.95 in check or money order.

OEMS and Dealers are encouraged to support ATHENA.SYS with

their products, but may be permitted to distribute HGCIBM with

their materials provided that Athena Digital receives the

registration fee. Those interested should contact Athena Digital

for details.

USER COMMENTS

Let us know what you think. Any criticism is better than

none and will help to steer us along a path which is in the

majority's best interest. Your suggestions need not be limited to

graphics. Any comment regarding Athena Digital programs or

documentation is welcome.

Thank you for your support.

HISTORY

In June of 1986, Athena Digital uploaded a simple "Freeware"

emulator for Hercules cards which supported only the 200 by 640

Color Graphics Card mode (HGCIBM.COM Version 1.11). To our

knowledge, this was the first such program publicly announced. At

first glance there would appear to be no physical way to

effectively display CGA graphics on a ttl card. It not only could

be done, but done in such a way that resulting display quality is

actually better than the real CGA though it is in shades of black

and white. Support came in from users as far as Israel and

eventually from CGA software developers. This support inspired

Athena Digital to create HGCIBM.COM Version 2.00 which offers

emulation support of all of the CGA modes and to start the

development of Athena's own Video BIOS to support not only

emulation, but to provide an upgradeable and expandable Video

BIOS which enhances any display system.

Version 2.01 is no longer offered as freeware. It is not

copy restricted, but it is required that that it be registered if

and only if the copy is kept. If the user determines that he does

not wish to keep HGCIBM, he must pass his copy on to someone

else, or destroy (erase) it. This does not apply to those who

contributed monetarily to HGCIBM Version 1.11. Those individuals

are more than welcome to version 2.01 with no further

contribution.

6

FUTURE

Version 2.01 does not provide all of the Athena Digital

support functions. It is a fully functional emulator only, and

does not offer support for Microsoft Mouse* or extended video

BIOS calls except for a subset of calls 0B0h (SetContext) and

0BFh (AthenaState) as described below. Due to restrictions

imposed by the "resident" utility implementation, the next

release of Athena's BIOS (ATHENA.SYS Version 3.00) will be in the

form of an MS-DOS device driver. Installed in this way, the

program becomes as much a part of the operating system as is the

ROM video BIOS, except that it can be easily upgraded and

enhanced. Future enhancements will be limited to ATHENA.SYS, thus

obseleting HGCIBM.COM except as an emulator only. If bugs are

found in HGCIBM, they will be corrected. For more information

about ATHENA.SYS, read the section called ATHENA.SYS Video BIOS

in this document.

HGCIBM CHANGE AND CORRECTION NOTES

BUGS FIXED

o    Version 1.11 would sometimes crash with programs that

intercepted the timer interrupt while in Graphics, and then

changed to text.

o    If interrupts were redirected, Version 1 could not find

itself for a mode change and would install a second copy. Now it

can find and control itself unless it is bypassed altogether.

o    Version 2.00 (beta test version) would not install on a

Leading Edge due to differences in the way the Leading Edge video

buffers are implemented. This was corrected with the /L[eading

Edge] switch.

ENHANCEMENTS

o    Version 2.01 contains a SUBSET of Athena Digital's own

video bios and will support a wider range of compatibles as well

as the AT. This also provided a means for improved screen

behavior in color text mode, and faster screen operations if bios

is used.

o    All standard video modes are now supported. The 320 x

200 mode supports two intensities(colors). Color 1 and color 2

will appear identical, where color 3 will be high intensity.

Forty column text is supported but will  be displayed on the left

half of the screen only at the same resolution as eighty column

text. All four pages of 80 column text (or 8 pages of 40 column

text) are available on true Hercules cards. Some of the

compatibles will support only one text page at 80 columns, and 2

at 40 columns.

o    Version 2.01 provides extended bios calls which

assembly language programs can use to determine if HGCIBM is

present, and what version it is.

7

TECHNICALITIES

HGCIBM installs itself between two BIOS (Basic Input/Output

System) interrupts. The two interrupts used in this way are hex

10 (BIOS video) and hex 8 (timer). The later is used only while

in graphics mode. HGCIBM will fail to function properly if it is

not given control during the above interrupts. Other software

using interrupt 8 be called on a priority basis, preempting

HGCIBM if necessary. HGCIBM version 2.01 no longer passes

interrupt 10h (while in emulation mode) since it was learned that

compatible ROM bios was not always entirely compatible. If you

have other software using interrupt 10h, try loading it after

HGCIBM. You can still control HGCIBM after the interrupt is

changed but you will not be allowed to un-install it. HGCIBM need

be loaded only once each time the computer is started since it

attaches itself to BIOS and stays in memory until re-boot or it

is commanded to un-install. The memory usage has increased to

about 5k now due to bypassing of ROM video bios in emulation

mode, but does provide much improved color text emulation and

should be compatible with a wider variety of machines.

HGCIBM.COM EXTENDED VIDEO FUNCTIONS

This section is provided for those familiar with IBM PC

assembly language. The following calls are made through the

standard video interrupt 10H.

;

(AH)=B0h     SetContext - (AL) contains context value

In HGCIBM, these functions simply enable or disable the

emulation modes. As can be seen, most mode numbers mimic those

provided by the standard PC bios. The difference between these

modes and the standard AH=0 call is that the driver behavior

switches are modified before the mode change. A call here for any

of the standard modes will disable the emulator bios and pass

calls to the computer's rom. A call here higher than 7 will

enable the emulation modes. These changes affect the way HGCIBM

interprets calls to the standard bios function 0 as shown below.

These calls cannot be used to overide the hardware setup

determined by HGCIBM during initialization. Calls for invalid

context switches are ignored and the call returns with the carry

flag set:

8

HGCIBM function calls:

TARGET MODE         EMULATION      TARGET ADAPTER

Standard Modes:  (HGCIBM will re-enable ROM bios)

(AL)=0              disabled       CGA 40x25 b/w text

(AL)=1              disabled       CGA 40x25 Color text

(AL)=2              disabled       CGA 80x25 BW text

(AL)=3              disabled       CGA 80x25 Color text

(AL)=4              disabled       CGA 200x320 Color grph

(AL)=5              disabled       CGA 200x320 BW grph

(AL)=6              disabled       CGA 200x640 BW grph

(AL)=7              disabled       MONO 80x25 BW text

CGA Emulation Modes: (correspond to Standard modes 0 - 6)

(AL)=20 - (AL)=26   enabled        HERC page at B800h

Hercules only modes are not supported by HGCIBM.

;

(AH)=BFh      AthenaState - Return the current driver state.

(AL) = Current mode (as set in call B0h)

(AH) = Character Columns on screen

(BH) = Current Active Display Page

(DX) = Installed options, switch states

D0 = 1 if mono or hercules card is

installed.

D1 = 1 if cga card is present

D3 = 1 if hercules modes/cga emulation are

allowed.

D4 = 1 if emulation modes are enabled

*HGCIBM supports D0,D1,D3,and D5

only. Others will always be 0.

(CX) = ATHENA/HGCIBM version number

(CH) has major version

(CL) has minor version

* set cx to 0 before calling function BFh,

if 0 is returned, ATHENA.SYS or

HGCIBM.COM (version 2 or higher) are not

installed. If the major version is

greater than or equal to 3, the response

is from ATHENA.SYS. HGCIBM.COM will not

be upgraded past major version 2.00. All

future enhancements will be limited to

ATHENA.SYS or language libraries.

9

HGCIBM function calls:

;

(AH) = 0      SetMode - (AL) contains new mode value

The new mode is set according to the current

switches and the screen is cleared.

STANDARD MODES:

NEW MODE         EMULATION FLAG         TARGET SCREEN

(AL)=0              disabled            CGA 40x25 b/w text

enabled             HERC 40x25 emulation

(AL)=1              disabled            CGA 40x25 Color text

enabled             HERC 40x25 emulation

(AL)=2              disabled            CGA 80x25 BW text

enabled             HERC 80x25 emulation

(AL)=3              disabled            CGA 80x25 Color text

enabled             HERC 80x25 emulation

(AL)=4              disabled            CGA 200x320 Color grph

enabled             HERC 200x320   emulation

(AL)=5              disabled            CGA 200x320 BW grph

enabled             HERC 200x320   emulation

(AL)=6              disabled            CGA 200x640 BW grph

enabled             HERC 200x640 emulation

(AL)=7              don't care          MONO 80x25 BW text one page

(resets emulation flag and returns to ROM)

10

ATHENA.SYS (T) Version 3.00

Athena Digital Video BIOS for MS-DOS (T) Computers

scheduled release - May 15, 1987

PURPOSE

For some unknown reason, hardware and software vendors have

not seen fit to establish adequate specifications and graphics

support for the widely used Hercules Graphics standard. The most

peculiar aspect of this is the fact that the Hercules standard

was the third available for the IBM personal computer and the

first which offered truly high graphics resolution as well as the

first to support graphics on the ttl monochrome display. Today,

this standard is almost universally accepted as the display

adapter of choice for ttl monochrome PC systems and its display

quality is yet challenged only by the expensive EGA standard

which also requires an expensive color monitor to exploit it's

capabilities. Even then, the EGA provides a lower horizontal

resolution (640 horizontal, rather than 720). Athena Digital will

attempt to remedy this by making it as simple as possible for

vendors to exploit the capabilities of the Hercules card by

providing a BIOS which fully supports the Hercules monochrome

card and/or Color Graphics Card (and eventually others) with fast

support for standard modes, CGA emulation, and several new modes.

One of the primary enhancements provided by ATHENA BIOS is

emulation of the Color Graphics Adapter on the Hercules Graphics

Card. This allows Hercules owners to run the majority of graphics

programs which do not already support the hercules adapter. This

includes programs like BASICA and programs running under the

BASIC compiler, as well as increasingly popular programs like

QUICKBASIC. This eliminates the need for a Color monitor and

graphics card in many cases with obvious price advantages.

ATHENA also provides bios support for the higher resolution

Hercules 350 vertical by 720 horizontal modes, and a new 350

vertical by 360 mode with three intensities and 4 logical pixel

colors. ATHENA provides ALL of the Standard BIOS call support for

these new modes including character generation and scrolling,

which greatly simplifies direct support of hercules high

resolution modes by software vendors and makes it universally

possible for applications developers to determine the computer's

video hardware capabilities and behave accordingly.

ATHENA FEATURES

o    Supports 5 new Hercules only modes, which are as

readily available to applications developers as are the

standard modes.

o    Provides extended equipment, system capability information

for applications.

11

o    Supports Color Graphics Adapter Programs on the Hercules

Graphics Card, and includes algorithms to support Microsoft

Mouse sofware and prevent many of the "hangups" experienced

with earlier software emulators.

o    Supports high speed ANSI console escape sequences.

o    Supports new "context switching" between adapters and

certain modes.

o    Supports faster video operation in ALL modes than Standard

ROM BIOS.

o    Simple replacement for "buggy compatible" BIOS.

o    Installs as a device driver for MS-DOS version 2.00 or later

o    Adds PC-AT BIOS print string functions to the PC

Once installed, ATHENA replaces the standard MS-DOS "CON"

device driver and ROM video bios unless an EGA card is present.

By integrating BIOS with DOS, faster DOS screen output is

possible. Installation is a matter of copying ATHENA.SYS to your

boot disk, and inserting a one line command in your MS-DOS

config.sys file. This MUST be the FIRST "DEVICE =" command in the

file and has the following basic form:

DEVICE = ATHENA.SYS

ATHENA.SYS is compatible with a variety of MS-DOS computers,

including the PC, PC-AT, and Leading Edge. Our aim is to work

with as many different computers and display adapters as

possible, so enhancements can be expected periodically.

Version 3.00 video bios does not support EGA so it aborts

installation if the EGA is detected. Future products will likely

support the EGA for uniformity but EGA graphics are supported in

EGA bios, and there is no apparent need to extend it at this

time. The EGA hardware standard also conflicts directly with the

Hercules standard. Our present purpose is to provide support for

the multitude of Hercules and/or CGA systems.

12

CONTROLLING ATHENA BIOS AFTER INSTALLATION

Along with ATHENA.SYS, you are provided ATHENA.EXE which

allows you keyboard control over ATHENA's variety of modes.

ATHENA.EXE operates as any other external DOS command. Simply

place it in your program directory and use it with switches as

below. If you forget the switches, just type ATHENA and it will

give you a help screen.

ATHENA /M[ono]      - Places BIOS in MONO mode (emulation disabled)

ATHENA /C[ga]       - Places BIOS in CGA mode (emulation disabled)

ATHENA /E[mulate]   - Places BIOS in CGA EMULATION mode.

ATHENA /?           - Gives a report of ATHENA's status.

ATHENA /Q[uiet]     - ADD this switch to others to suppress messages

These commands may be issued from a batch file if desired.

SALES AND DISTRIBUTION POLICIES - ATHENA.SYS

ATHENA.SYS is a fully restricted program, and will soon be

offered for sale by mail order, through OEMS, and dealers.

Illegal distribution of this program is punishable in the same

manner as any other copyrighted work (if someone has provided you

a copy of ATHENA.SYS you must destroy it immediately). For the

purchase price of $34.95, you will receive the program on a 5.25"

IBM format diskette with documentation sheets, and be notified of

upgrades at low cost as they become available.

Dealers, OEMS, and those desiring Site Licenses are welcome and

encouraged to contact Athena Digital about liberal discounts.

UPGRADING FROM HGCIBM

If you have already registered HGCIBM and wish to upgrade to

ATHENA, you may do so at a reduced rate for a limited time.

To order the upgrade, send your registration name and

address along with 29.95 in a check or money order (normally

34.95).

SOFTWARE AUTHORS

You do not have to distribute ATHENA to support it, Athena

Digital for one will make the program infinitely available to

users. If you do decide to enhance your product specifications by

supporting the emulation modes (your product probably already

does) or the extended modes, simply send us your company name,

your product(s) name, your name, your address, and your telephone

number. From time to time, Athena Digital will upload a

"supporting product listing", and will keep you informed of

updates and enhancements. If you would like to mention your

support in your product specifications, you are encouraged to do

so provided you include the trademark notice as in the example

below.

13

<program name> supports the Hercules emulation modes of

ATHENA.SYS (T) Video Bios.

<bottom or top of page>

ATHENA.SYS is a registered trademark of Athena Digital

ATHENA.SYS FUNCTION CALLS

This information is provided for technical users, and

presumes familiarity with Standard BIOS function calls.

; *****

(AH)=B0h     SetContext - (AL) contains context value

These functions first change the current state switches to

that indicated below and then switch to the requested context. As

can be seen, most mode numbers mimic those provided by the

Standard PC BIOS.

The difference between these modes and the standard AH=0

call is that the driver behavior switches are modified before the

mode change and the concept of contexts is introduced for each

mode. For instance, if the driver were currently set to

acknowledge an active CGA card, but it is desired to switch the

active display to emulation CGA or one of the Hercules only

modes, a call here for the desired mode would freeze the CGA

screen and switch contexts to the requested screen.

If the target of the switch is the same as it was when it

was last active the target screen is not cleared. If a target

mode change is required then the target screen is cleared. In

either case, the context switches to the new active screen.

Context switches also affect the way ATHENA interprets calls to

the standard bios function 0 as shown below. These calls cannot

be used to overide the hardware setup determined by ATHENA during

initialization. Calls for invalid context switches are ignored

and the call returns with the carry flag set:

14

ATHENA.SYS function calls:

To force Athena to clear the target screen, set D7 in

register AL (or the mode value with 80H).

;

TARGET MODE         EMULATION           TARGET SCREEN

Standard Modes

(AL)=0              deactivated         CGA 40x25 b/w text

(AL)=1              deactivated         CGA 40x25 Color text

(AL)=2              deactivated         CGA 80x25 BW text

(AL)=3              deactivated         CGA 80x25 Color text

(AL)=4              deactivated         CGA 200x320 Color grph

(AL)=5              deactivated         CGA 200x320 BW grph

(AL)=6              deactivated         CGA 200x640 BW grph

(AL)=7              deactivated         MONO 80x25 BW text

Hercules only modes

CGA Emulation Modes: (correspond to Standard modes 0 - 6)

(AL)=20

.                activated           HERC page at B800h

.

(AL)=26

Hercules Text Mode: (supports 8 pages)

(AL)=27             activated           HERC 80x25 BW text

Herc Modes:

(AL)=28             no change           HERC 350x720 at B000

(AL)=29             no change           HERC 350x720 at B800

(AL)=30             no change           HERC 350x360 at B000*

(AL)=31             no change           HERC 350x360 at B800*

*modes 18 and 19 support low and high intensity

Certain mode changes are not possible without erasure of a

previous context and screen. The following HERCULES context

changes can be accomplished without erasure of a previous context.

(Emulation modes 24 - 26) and (modes 7,27,28 or 30):

These two groups use separate Hercules pages and do not

destroy each other. Changes made within each group are

destructive to other modes in the same group.

(modes 28 and 30) and (modes 29 and 31):

Context is retained when switching between groups, but lost

when switching between modes within the same group.

Emulation modes 20 - 23 destroy the context of all other

Hercules modes because they use both adapter pages.

15

ATHENA.SYS function calls:

(modes 0 - 6) and all others:

Context is retained when switching adapters.

; *****

(AH)=BFh      AthenaState - Return the current driver state.

OUTPUT:

(AL) = Current mode (as in call B0h)

(AH) = Character Columns on screen

(BH) = Current Active Display Page

(DX) = Installed options, switch states

D0 = 1 if mono or hercules card is installed

D1 = 1 if cga card is installed

D2 =   reserved

D3 = 1 if hercules modes/cga emulation are allowed

D4 = 1 if emulation modes are activated

D5 = 1 if ansi console codes are enabled

D6 = 1 if it is necessary to wait for

horizontal retrace before CGA text

screen updates. This bit is reset by

default when ATHENA initializes. If the

/W(ait) option switch is used on the

installation command line, this bit

will be set to cause ATHENA to wait for

retrace when updating CGA text. This

switch has no effect on Hercules or

Mono modes.

Application software should never

try to wait in Hercules modes since the

CGA registers may not even exist and

the Hercules standard does not require

it.

(CX) = ATHENA/HGCIBM version number

(CH) has major version

(CL) has minor version

* set cx to 0 before calling function BFh, if 0 is returned,

ATHENA.SYS or HGCIBM.COM is not installed. If the major version is

greater than or equal to 3, the response is from ATHENA.SYS.

HGCIBM.COM will not be upgraded past major version 2. All

future enhancements will be limited to ATHENA.SYS or language

libraries.

16

; *****

(AH) = 0      SetMode - (AL) contains new mode value

The new mode is set according to the current

switches and the screen is cleared.

STANDARD MODES:

NEW MODE         EMULATION FLAG         TARGET SCREEN

(AL)=0              deactivated         CGA 40x25 b/w text

activated           HERC 40x25 emulation

(AL)=1              deactivated         CGA 40x25 Color text

activated           HERC 40x25 emulation

(AL)=2              deactivated         CGA 80x25 BW text

activated           HERC 80x25 emulation

(AL)=3              deactivated         CGA 80x25 Color text

activated           HERC 80x25 emulation

(AL)=4              deactivated         CGA 200x320 Color grph

activated           HERC 200x320   emulation

(AL)=5              deactivated         CGA 200x320 BW grph

activated           HERC 200x320   emulation

(AL)=6              deactivated         CGA 200x640 BW grph

activated           HERC 200x640 emulation

(AL)=7              don't care          MONO 80x25 BW text one page

HERC MODES:     If Hercules modes are not allowed, these calls

are ignored.

(AL)=27                                 HERC page at B000h

Herc mono mode (supports 8 text pages)

Herc Modes:

(AL)=28                                 HERC 350x720 at B000

(AL)=29                                 HERC 350x720 at B800

(AL)=30                                 HERC 350x360 at B000*

(AL)=31                                 HERC 350x360 at B800*

*modes 30 and 31 support low and high intensity

ATHENA.SYS Version 3.00 provides the standard bios support

functions for all of the listed modes.

17

ATHENA LIBRARY VERSIONS

These are introduced in the accompanying QBDEMO program.

The demo is a short first-hand introduction to ATHENA.SYS and

Athena Language Libraries for programmers. There are 2 files

required to run the demo:

QBDEMOC.EXE    ; QuickBasic library DEMO - BCOM20 program module

FEATURES.PRX   ; DEMO text

o         To run the demo, place these two files in your default

directory and type "QBDEMOC".

o         If you have an older style Color Graphics Adapter which

"snows" when text is printed, then start the demo as

"QBDEMOC/W".

o         If you have a Hercules Card AND a Hercules CGA card AND

both are attached to monitors, then start the demo as

"QBDEMOC/H".

o         If you have a Leading Edge, no switches are required.

o         You may use both "/H" AND "/W" on the same command line.

o         It makes no difference whether HGCIBM is installed or

not to run the demo, since ATHENA BIOS is linked with the

demo and will be used in either case.

18