(>-------------------------------------

--------------------------------------<

)

The LOD/H Technical Journal: File #2

of 13

Custom Local Area

Signalling Services

Written by:

The Videosmith

Version

- 1.1

----------------------------(c)

Copyright 1994-------------------------

--

This article will explain the newly

developed LASS system (AT&T Bell Labs),

and how it may affect us in the near

future. Note that the service as it

appears for customers is called

"CLASS", the C standing for Custom. I

assume this is just for looks.

LASS

----

The telephone was destined to

become a well used and powerful tool

for

otherwise tedious tasks. Gas meters

and other metered services would be

surveyed through the use of automatic

data retrieval employing telephone

communications. All in all, some have

big plans for the uses one could put

the telephone system up to, and CLASS

is one plan that is going to drop

an innovative bombshell on the

telecommunicating world.

At this moment, a local CCIS

network feature is being developed by

Bell Laboratories. This feature will

change the way people use fones, and

will also change the attitude in

which they use them. It will give far

more control of the telephone to the

user than ever before. This feature

is called CLASS (Custom Local Area

Signalling Services).

Everyone will find something

useful in this newly developed

telephone

feature. Pizza parlours will no

longer have to worry about fraudulent

italian

food mongers, and little old ladies

won't have to worry about prank calls

by certain dubious characters.

What are all these fantastic

features?  These features will

include call back of the last caller,

regardless of whether you have their

telephone number or not. Another will

be distinct call waiting tones, and

preselected call forwarding (only

those people whom you wish to speak to

will be forwarded). This is a

rudimentary list of CLASS features to

come.

It is a very powerful system, and it

all relys on LCCIS (Local Common

Channel Interoffice Signalling), an

intra-LATA version of the ever-popular

CCIS.

CCIS Background

---------------

CCIS was originally introduced in

1976 as, basically, the signalling

system to end all signalling systems.

Instead of using the voice grade

trunks to carry signalling

information on, a data network would

be used. This

network is comprised of data links

from each TO [involved with CCIS] to

the appropriate STP (signal transfer

point). Signalling information is sent

through these links at 4800 bps to

the STPs (Note that baud rates may

increase

due to the economic availability of

faster data communications hardware),

where stored program control routes

the signalling information to the

needed

offices in order to open and complete

the call path. SPC checks automatically

for on-hook/off-hook status before

opening the path, and if the status is

off-hook (in this case the customer

does not have the call waiting custom

calling feature), returns information

to the originating CO to apply a busy

signal to the customer. This is but

one of many features toll CCIS provides

the network with.

Since this text is not centered on

the topic of toll CCIS, technical

aspects aren't as important (except

for the comparison between the local

and toll networks for observational

purposes): yet it is important to

notice how automated and flexible

this type of signalling method is, as

well

as its speed and efficiency. All the

software control involved with local

and toll networks is called,

fittingly, the "stored program control

network."

or ISDN (Integrated Services Digital

Network). LCCIS will be addressed in a

future article.

CLASS/LCCIS Features

--------------------

LCCIS would look like this:

/--\

CO-2

ESS#

/----I-T-G-----1A-----I-T-G----\

\--/

LCCIS

----------

/--\--LCCIS--CCIS/S

PC--LCCIS--/--\

CO-1

----------         CO-3

ESS#

ESS#

-1A----interoffice

trunk group---1A-

\--/

\--/

SPC = Stored Program Control (Network

control and Signal Transfer Point)

ITG = Interoffice Trunk Group

Using a high-speed data link

between local offices creates a much

more

flexible and more effecient way for

intra-LATA central offices to communi-

cate. Instead of using per-trunk

signalling (using the same trunk used

for

voice transmission to send routing

and billing information), such data

would

be sent thru a 2400 bps dedicated

data link, which interacts with a local

signal processing and transfer point.

From that point, signalling information

is distributed to appropriate central

offices or tandem switches.

At the during which this article

was being initially researched, CLASS

was

only being developed for the #1A

ESS switch due to the flexibility of

it's

memory handling, it's speed and what

Bell Labs called 'cost efficiency'. At

the end of the research involved with

this article, CLASS was already

implemented in data stage on ESS#5.

LCCIS will work with the local

switches using stored program con-

trol, keeping track of call data. The

1A switches will use what

is called "scratch pad" memory (also

known as call store), in conjuction

with LCCIS's database, to accomplish

all the features that LASS provides.

This memory will hold such data as

"line history", and a "screening list".

That information will make it

possible for autoredial, selective call

forwarding, nuisance call rejection,

and distinctive call waiting tones.

Selective CF

------------

Selective call forwarding is

defined by the subscriber (the sub-

scriber must have conventional call

forwarding to request this service).

Using call store, or more

specifically the screening list, one

will

be able to selectively forward a call

to another directory number by

executing a few simple commands on

the friendly home-bound telephone

(unlike migrating telephones most

frequently found in hotel rooms). An

access code (a list will appear at

the end of the file) will be entered,

and a special tone will be issued

from the subscriber's CO. The cus-

tomer will then dial in the numbers

he wants forwarded to the particular

number. After each number, a tone

will sound indicating the acceptance

of the number. Individual BOC's (Bell

Operating Companies) will be

able to define the amount of numbers

which may be screened. Once this is

done, the cusomter hangs up and the

ESS takes over. Now, whenever some

one calls this particular customer,

the customer's switch will compare

the calling line's directory number

with those stored in scratch pad

memory. If the CLID matches one of

the numbers in 1A memory associated

with

the called directory number, the

number is forwarded. If not, the phone

will

ring at the original destination.

This in particular could make it very

difficult on system hackers, as you

could probably imagine. A company can

subscribe to this CLASS feature, and

enter only the numbers of authorized

users to be forwarded to a computer.

Bureaus inside the various telephone

companies and other sensitive

operations can screen calls to

particular

numbers by using this service.

This is a security that's hard to

beat, but of course there is a way

(simple law of nature: nothing is

fail-safe). There will always be the

obvious way of finding numbers which

are being forwarded to, like auto-

dialing entire exchanges (one after

the other). Unfortunetly, CLASS will

be providing other services which

might make "scanning" seem less

attractive.

Distinctive Ringing

-------------------

Distinctive ringing is handled in

the same fashion as selective call

forwarding is: the screen list in

scratch pad memory. The customer may

enter numbers which the ESS should

give special precedence to, and when-

ever a call is placed to this

particular customer's number, ESS

checks

to see whether the CLID matches a

directory number listed in the

switch's memory. If a match is made,

the subscriber's CO gives the off-hook

line a special call waiting tone, or

the on-hook phone a distinctive ring

(possibly using abnormally timed

ringing voltage... some readers may

picture

a British Telecom ring as an example,

although many foreign audible rings

tend to be different).

Call Rejection

--------------

Nuisance call rejection, a feature

making it possible to block certain

idiots from ringing your fone (a

feature we can all benefit from at

one time or another... or all the

time), uses the information retrieved

from LCCIS (CLID).  Let's say

customer A calls customer B:

----LCCIS----

A ---> CO<

>CO ---> B

----trunk----

Customer B happens to despise

customer A, and keys in a special *##

code. ESS again takes over and looks

at the CLID information, and stores

the calling line directory number in

a special screen list associated with

with customer B. The next time

customer A tries calling customer B,

the

terminating office will reroute the

call to a local (the originating CO)

digitized recording telling customer

A that the call he made cannot be

completed due to customer B's request

("I'm sorry, but the customer you

have tried to reach wishes you were

eaten by a rabid canibal on drugs").

Dial Back

---------

To create such a feature as "dial

back" (for called or calling party),

the ESS scratch pad memory is used

again. The same principles are

used as are employed in the already

established custom calling feature,

auto-redial. CLID will be used in

this way:

(received from CLID)

last-called-mem

last-caller-mem

----------

----------

###-####

###-####

----------

----------

Your ESS switch will keep track of

who you called last, and who called

you last, thru the retrieval of

calling line information provided by

LCCIS in conjunction with your switch

(Your switch will know what number

you called last by directly storing

the digits you dialed previously. Local

signalling will provide calling line

information via LCCIS call

information forwarding using the data

link mentioned). This way, with your

access code (*##), you will have

total re-dial service.

Customer Trace

--------------

This type of memory handling and

signalling method will also allow the

feature that everyone was afraid

would abolish "phreaking". Subscriber

initiated tracing, using the last

caller directory number stored at your

CO,

will be available as far as Bell

Laboratories is concerned. There seems

to be

two types of "customer originated

trace". One will forward the number to

local

authorities, at  which it will be

handled through the police. The other

feature AT&T/Bell Labs is working on

will be a display module that will sit

by

your fone, and will display calling

directory numbers. All other CLASS

features that use the calling line

information are used at the descretion

of

the caller. The customer originated

trace, however, using the individual or

bulk calling line identification

features ("trace") allow the customer

to view

the calling number. The world is not

ending... yet, in any case. Individual

customers will be able to employ a

special "privacy code", which when

dialed,

tells the far-end switch not to

forward the calling number to a desk

display.

Whether there will be a way to

override this or not is obvious: of

course.

The police, the military and

government agencies are all likely to

have a

higher priority level than your

privacy. It seems that long distance

carriers could benefit greatly from

CLASS. Why Bell/AT&T should give any

type

of special services to OCCs not given

to other non-telephone companies,

especially after equal access is

fully implemented, I don't know (but

then

again, it is EQUAL access). It's

always possible. It is also possible

that

there will be no desk display. There

are those phone phreaks who feel that

BOC's will never give the end party

the priviledge of retrieving the

calling

party's number directly, if not due

to plain old Bell policy on the issue

of

privacy. We'll have to wait and see

about that point: the desk display is,

in

fact, operational and is being used

in test stage. Whether Bell Labs feels

that this feature can and will be

used in a full scale non-beta stage BOC

situation is a different story. The

economic feasability is questionable.

End Notes

---------

CLASS, using local CCIS, will not

function on inter-LATA calls. The

local CCIS network is exactly that:

local, and does not extend into the

realm of "toll network". This will

eventually be corrected (allowing toll

CCIS to interact with LCCIS as far as

CLID information is concerned). How

the various long distance networks

will exchange information with the

local

BOC network has not been determined

[by the writer of this article]. It

would seem like a monumental task to

try to integrate the emerging long

distance companies into the AT&T/BOC

ISDN, be it because of equipment

inconsistancies or lack of

cooperation on the part of the OCC,

etc. This

will be discussed in an upcoming

article dealing with toll CCIS.

Although CLASS has been built around

the ESS #1A switch, it has, as has been

mentioned, been co-developed for use

with the ESS #5 switching machine.

CLASS is going to cause problems,

as well as create a new environment

for telephone users. Of course, those

problems are only problems to people

who will generally be reading this

article, but the more you know about

CLASS

the more comfortable you'll feel

about the service. It can be used to

one's advantage, even as a

telecommunications hobbyist. Just as a

corporation will be able to set up a

complete history of who is calling

their

system, and eventually keep people

off the system using the screen list in

memory, the same features can be

applied to bulletin board systems and

the

like. Imagine being able to keep all

the local bozos off your board, or

being able to screen all but your

private local users (making your system

completely inaccessible through the

PSTN network from any telephone but

that of one of your users). It would

seem to be a useful feature, if nothing

else but an easy feature, to

implement.

It is a little difficult, if not

plain awkward, to write an article

about

a topic which is subject to change at

the researcher's ignorance. I think

that CLASS is enough of a momentous

issue that at least some text by a

hobbyist should be released for

public knowledge purposes.  Yet my

awareness

of the fact that some of this text

may be outdated, or inaccurate, by the

time CLASS is released as a BOC

service, is in itself the explanation

of why

there is a version number at the head

of this article. Most likely, when

CLASS

becomes public, the second version

will be released with update notes

(if need be...most probably so). I

hope you enjoyed it,

The Videosmith.

LOD/LOH!

--------------------------------------

-

Test stage defaults for some features:

DTMF ! Pulse ! Description of Service

--------------------------------------

-

*66 !  1166 ! Reconnect last caller

--------------------------------------

-

*63 !  1163 ! Selective Call Forward

--------------------------------------

-

*60 !  1160 ! Nuisance Call Blocking

--------------------------------------

-

*57 !  1157 ! Customer "Trace"

--------------------------------------

-

Note: These command codes may vary

from BOC to BOC. The codes listed above

were found in a general description

of CLASS and did not specify a

particular

implementation of these services.

Acknowledgements:

Mark Tabas for his views on various

included topics... for example,

subscriber

tracing ("FUCK NO").

Doctor <413> Who

Mr. DNA

(>-------------------------------------

--------------------------------------<#############################################################################