185

CHAPTER 18 - PORTS

In order to communicate with the outside world, the outside world

being your printer, your disk drives, your modem etc., the 8086

uses ports. A port is an address distinct from a memory address

where the i/o device can be reached.{1}  When IBM set up the

first PC, they decided what these port addresses would be and

what the function of each bit at each address would be. Any VGA,

EGA, CGA or monochrome card has to have it's ports at the same

addresses as those set by IBM, and these ports have to do the

same thing.

Normally, an i/o device has more than one port address. The

device not only has to transfer the data, it has to tell the

computer whether it is ready, find out if the computer wants to

send information, confirm that the transfer was successful etc.

COM1 is port addresses 3F8 - 3FF. COM2 is 2F8 - 2FF. The CGA

adapter is ports 3D0 - 3DF.

In the CGA, port 3D9 sets the different colors. The 8086 writes

to it, but can't read it. Port 3DA, it's neighbor, tells certain

status information and is read only.

All control information for the video cards is passed back and

forth through the ports. However, the video card comes into

memory 50 times a second to see what it should write to the

screen. The characters on the screen don't pass through the

ports. The mountain comes to Mohammed.

The data does pass through a port on the way to your serial

printer. The 8086 sends your printer control information through

one port address and sends the data through a different port

address.

The port instructions can be either with AL or AX. AL and AX are

the only registers you can use, AL for a byte transfer and AX for

a word transfer. There are two forms to the input instruction:

in   al, port_address

or

in   al, dx

port_address is a constant and is limited to 0 - 255. It is a one

byte constant. DX is the only register than can be used with the

second form. DX can contain any number from 0 - 65535. The

____________________

1. There is memory address 0000 and there is port address

0000; they are two entirely different things. You can reach

memory address 0000 with the DS:SI pair 0000:0000, but DS:SI

can't get to port address 0000. The instruction "out 0, al"

writes the contents of al to port address 0000,  but the OUT

instruction can't get anywhere near memory address 0000.

______________________

The PC Assembler Tutor - Copyright (C) 1989 Chuck Nelson

The PC Assembler Tutor                                        186

______________________

following two pieces of code do the same thing:

in   al, 155

mov  dx, 155

in   al, dx

This code moves one byte of data from port 155 to register AL.

The output instruction is similar. We have:

out  port_address, al

or

out  dx, al

where port_address is a constant 0-255 and DX can contain a

number 0 - 65535.

out  97, ax

moves a word from AX to port addresses 97-98. Notice that both

the IN and the OUT instructions follow the  DESTINATION, SOURCE

ordering found in all the other 8086 instructions.

The  "in  ax, port_address" form is not all that useful. If you

look at the addresses for the video ports, for COM1 and COM2, you

will see that they are all larger than 255. Also, most equipment

can be at several different addresses. A modem can be at COM1,

COM2, COM3, or COM4. If the port address is hard coded into the

instructions, you need 4 subprograms to handle the 4 different

addresses. It is easier to find out where the modem is, then use:

modem_data_address dw   ?

mov  dx, modem_data_address

in   al, dx

and

mov  dx, modem_data_address

out  dx, al

If you aren't planning on writing device drivers this is for

background information only, since all standard i/o is done

through DOS or BIOS interrupts.

One last thing is the parity flag. It has been sitting on the

screen with all the other flags. What is it for?  When you send

information over a modem, there is a large possibility of line

interference. If it is text, and an occasional screw up is not

too bad, using a parity check may be enough.{2}  Parity can be

even or odd. Even means that an even number of bits are set to 1;

odd means that an odd number of bits are set to 1. This is not

whether the number is even or odd, it is whether the number of 1

____________________

2. It is NEVER enough if you are transferring binary data or

executable files.

Chapter 18 - Ports                                            187

__________________

BITS is even or odd. Here's a short list.

NUMBER         BINARY         PARITY

6           0110         even

7           0111         odd

8           1000         odd

9           1001         even

10           1010         even

8 is an even number but has odd parity, 9 is an odd number but

has even parity. 6 has two 1 bits (even), 7 has three 1 bits

(odd), 8 has one 1 bit (odd), 9 has two 1 bits (even) and 10 has

two 1 bits (even).

How does this help us? If there is a chance of 1/100 of screwing

up a single bit in a byte, there is only a chance of 1/10,000 of

screwing up two bits in a byte. What this means is that of every

100 errors, 99 of them will will be detectable because of a

change in parity (by changing a bit from 0 to 1 or from 1 to 0)

and only 1 of them will go undetected because two changes will

keep the same parity. This is a little obscure, so make sure you

understand why before continuing.

This doesn't help us much yet, because we don't know what the

parity was originally. 9 has even parity, 7 has odd parity. What

communications programs do is FORCE the parity. They can either

force it even or force it odd.

The standard ASCII characters end at 127 - that is, 0111 1111

(7F) is the highest legal number you can transmit. The left bit

is unused, so we can use it to force the parity. We are going to

force it even, but forcing it odd uses the same technique.

The steps are:

(1) Find the parity of the byte.

(2) If it is even, leave it alone, if it is odd, put a 1 in

the left hand bit. The parity is now even.

If both the sending and receiving program have agreed on even

parity, then on the receiving end, the program:

(1) Checks for even parity. If the parity is odd, it is an

error.

(2) Puts a 0 back in the left hand bit. The original number

is restored.

We are going to make a loop with both parts and use show_regs to

watch the parity flag. Here's the program:

; - - - - - - - - - - ENTER DATA BELOW THIS LINE

error_banner  db  "Whoa! We have a screw up.", 13, 10, 0

The PC Assembler Tutor                                        188

______________________

; - - - - - - - - - - ENTER DATA ABOVE THIS LINE

; - - - - - - - - - - ENTER CODE BELOW THIS LINE

mov  ax_byte, 0A3h            ; al binary

mov  bx_byte, 0A2h            ; unsigned half registers

mov  cx_byte, 0A2h            ; unsigned half registers

mov  dx_byte, 0A3h            ; dl binary

lea  ax, ax_byte

call set_reg_style

comm_loop:

mov  ax, 0

call set_count                ; reset count to 0

mov  bx, 0                    ; clear registers

mov  cx, 0

mov  dx, 0

call show_regs                ; (1)

; the sending program

call get_unsigned_byte

and  al, 7Fh             ; 0111 1111 - make sure al < 128

mov  bl, al                   ; copy to bl

call show_regs_and_wait       ; (2)

and  al, al                   ; check parity

call show_regs_and_wait       ; (3)

jpe  do_nothing

or   al, 80h                  ; if parity odd, set left bit

do_nothing:

and  al, al                   ; check parity for show_regs

call show_regs_and_wait       ; (4)

; the receiving program

mov  dl, al                   ; transmit from al to dl

mov  cl, dl                   ; copy to cl

call show_regs_and_wait       ; (5)

and  dl, dl

jpe  zero_left_bit            ; is parity even?

mov  ax, error_banner

call print_string

zero_left_bit:

and  dl, 7Fh             ; 0111 1111  binary, left bit 0

mov  cl, dl                   ; copy to cl

call show_regs_and_wait       ; (6)

jmp  comm_loop

; - - - - - - - - - - ENTER CODE ABOVE THIS LINE

First, we set the register style so AL and DL are binary, BL and

CL are unsigned. We'll use AL and BL for sending, CL and DL for

receiving. Next, we reset the counter to 0 so we can see where we

Chapter 18 - Ports                                            189

__________________

are, and zero AX, BX, CX, and DX. We get a byte and make sure

that it is 127 or less,{3} then send a copy to BL. BL stays

unchanged for the rest of the loop. We test the parity. and if it

is odd, change it. Finally, we send it off to DL.

On the receiving end, we test the parity. If it is odd, we print

an error message. Then we zero the left hand bit. It is faster to

zero the left hand bit than to check to see if it needs to be

zeroed, so we do it for all data. The result is put in CL for

comparison. AL and DL are shown in binary form so you can count

the number of 1 bits in the byte.

____________________

3. The following could happen if we didn't take this step. We

get a number > 128 with odd parity. 131 (1000 0011  83h) is an

example. The sending program sees that it is odd parity, so it

puts a 1 in the left hand bit. But there is already a 1 in the

left hand bit, so the parity doesn't change, it is odd. The

receiving program gets the byte, tests it, notices that it is

odd, and sends back a transmission error. Since the sending

program didn't see anything wrong, it just sends the same byte

again. It will never get through correctly. In real life, the

sending program would probably test the byte to see if it was

greater than 127 and print an error if it was.

The PC Assembler Tutor                                        190

______________________

SUMMARY

POSSIBLE I/O INSTRUCTIONS

in  ax, constant (= port address 0 - 255)

in  ax, DX

in  al, constant (= port address 0 - 255)

in  al, DX

out constant, ax  (constant = port address 0 - 255)

out DX, ax

out constant, al  (constant = port address 0 - 255)

out DX, al

In these instructions DX holds a port address and

can be from  0 - 65535.

PARITY

Parity is whether the number of 1 bits in a byte (or word) is

even or odd. The 8086 sets the parity flag after most arithmetic

and all logical operations. If parity is even, the flag is 1, if

parity is odd, the flag is 0.