Chapter  7

Fixed Disk Service (INT 13H)

----------------------------

1.      Introduction

1.1     Disk Controller Basic Function

1.2     Standard Disk Interface

A.      ST-506/412 Winchester

B.      Enhanced Small Device Interface (ESDI)

C.      Storage Module Device (SMD)

D.      Enhanced SMD

E.      Small Computer Systems Interface (SCSI)

F.      Intelligent Peripheral Interface (IPI)

G.      AT-Bus

1.3     Coding

A.      Frequency Modulation (FM)

B.      Modified Frequency Modulation (MFM)

C.      Run Length Limited (RLL)

D.      Group Code Recording (GCR)

E.      Zone Bit Recording (ZBR)*

1.4     Data Format

1.5     Sector Interleaving

1.6     Disk Formatting

A.      Physical Format

B.      Partitioning

C.      Logical Format

1.7     Translative Versus Non-Translative Mode

2.      Winchester Disk Controller

2.1     System Configuration for Fixed Disk System

2.2     I/O Port Definition

2.3     Timing Consideration

3.      Programming Sequence in Fixed Disk Service Program

4.	     SPEC

5.      Q & A

1.1     Basic Function of a Disk Controller

1.      Write data to the disk

2.      Read data from the disk

3.      Serialize and de-serialize data

4.      Control the disk drive (option)

5.      Ensure data integrity

6.      Provide reasonable system interface (option)

7.      Buffer data to/from disk

1.2     Disk Interface Standard

�

High  �

�

�

�                                                  . IPI

�

�                   . SMD           . ESMD

�

�                                           . SCSI

COST     �                                . ESDI

�

�

�

�                     . ST-412HP

�

�          . ST-506/412        . AT Bus

�

Low   �   . 3.5 Floppy

�

��������������������������������������������������������>

Low                                            High

PERFORMANCE

Fig 1 :    Fixed Disk Interface Versus Market

A.      ST-506 412 Winchester Interface

1.      Industry standard since 1981

2.      Low capacity Winchester (5 - 150 Mbytes)

3.      small size

4.      Serial data transfer rate of 5 Mbits/second

5.      Cost (not performance) oriented

6.      Simple disk interface (most of the effort is in the

disk controller)

B.      Enhanced Small Device Interface (ESDI)

1.      Substantial upgrade to ST-506/412 interface

2.      Oriented towards high-capacity, high speed small

winchester

3.      Capable of handling magnetic tapes and optical disks

4.      Serial transfer rate 10 Mbits/second

5.      Simpler controller

C.      Storage Module Device (SMD)

D.      Enhanced SMD

E.      Small Computer Systems Interface(SCSI)

1.      ANSI defined intelligent, low-cost peripheral interface

2.      Derived from SASI (Shgart Assoc. Systems Interface)

3.      8-bit parallel data transfer, 1.5 to 4 Mbytes/second

4.      Provides device independent software

5.      Function more as a computer interface than a disk

interface

6.      Requires lots of intelligence in the peripheral

F.      Intelligent Peripheral Interface (IPI)

G.      AT-Bus

1.      Cost dowm

2.      Data transfer rate

3.      One I/O slot for another add-on card

4.      The fixed disk controller is no longer required

1.3     Coding

The following are four different ways of improving the storage

capacity of disk drives:

1)  Add more platters

2)  Add more tracks per platter

3)  Increase BPI (bit per inch)

4)  Use better encoding methods

For method 1, it will increase manufacture cost for extra

platter and extra read/write head set.

For method 2, it will increase control logic complexity for

write current precompensation and interfere.

For method 3, it will use better read/write to shorten

the length between read/write head and platter.

For method 4, it will increase the encoding/decoding complexity.

A.      Frequency Modulation (FM)

1 bit cell

�<----->�

�������Ŀ       �������Ŀ                       �������Ŀ

�   1   �   0   �   1   �    0       0       0  �   1   �  0

NRZ Data    ���       ���������       �������������������������       ����

c   d   c       c   d   c       c       c       c   d   c

�Ŀ �Ŀ �Ŀ     �Ŀ �Ŀ �Ŀ     �Ŀ     �Ŀ     �Ŀ �Ŀ �Ŀ

� � � � � �     � � � � � �     � �     � �     � � � � � �

FM Data     ��� ��� ��� ������� ��� ��� ������� ������� ������� ��� ��� ��

���Ŀ   �������Ŀ   ���Ŀ       �������Ŀ       ���Ŀ   ����

Disk Flux     �   �   �       �   �   �       �       �       �   �   �

Transitions ���   �����       �����   ���������       ���������   �����

FM Encoding Rules:

1.      Insert clock pulse for each bit cell

2.      Insert data pulse when NRZ data is "1"

B.      Modified Frequency Modulation(MFM)

1 bit cell

�<----->�

�������Ŀ       �������Ŀ                       �������Ŀ

�   1   �   0   �   1   �    0       0       0  �   1   �  0

NRZ Data    ���       ���������       �������������������������       �����

�Ŀ     �Ŀ

� �     � �

Clock       ����������������������������������� ������� ���������������

�Ŀ             �Ŀ                             �Ŀ

� �             � �                             � �

Data        ������� ��������������� ������������������������������� �����

d               d           c       c           d

�Ŀ             �Ŀ         �Ŀ     �Ŀ         �Ŀ

� �             � �         � �     � �         � �

MFM Data    ������� ��������������� ����������� ������� ����������� ���

���������������Ŀ           �������Ŀ           �����

Disk Flux         �               �           �       �           �

Transitions �������               �������������       �������������

MFM Encoding Rules :

1. Insert clock in pulse at the beginning of a bit cell only

if NRZ data = 0 for two consecutive bits.

2.  If data bit = 1, insert data pulse in the middle of the

bit cell.

3. If data bit = 0, no insertion of data pulses in the bit

cell.

C.      Run Length Limited (RLL)

RLL 2,7 Encoding Rules:

The following table shows encoding/decoding:

������������������������������������������������������������Ŀ

� NRZ Data�10  �11  �000   �010   �011     �0010    �0011    �

������������������������������������������������������������Ĵ

� RLL 2,7 �1000�0100�100100�001000�000100  �00001000�00100100�

��������������������������������������������������������������

D.      Group Code Recording( GCR)

4,5 GCR Encoding Rules :

The following table shows encoding/decoding:

���������������������������������������������������������Ŀ

� NRZ Data�0000 �000 1�0010 �0011 �0100 �0101 �0110 �0111 �

���������������������������������������������������������Ĵ

� 4,5 GCR �11001�11011�10010�10011�11101�10101�10110�10111�

�����������������������������������������������������������

���������������������������������������������������������Ŀ

� NRZ Data�1000 �1001 �1010 �1011 �1100 �1101 �1110 �1111 �

���������������������������������������������������������Ĵ

� 4,5 GCR �11010�01001�01010�01011�11110�01101�01110�01111�

�����������������������������������������������������������

E.      Zone Bit Recording(ZBR)

According to the track number, the fixed disk drive is divided

into several zones. The tracks in the same zone have the same

format, but the track in different zone may have different

format.

1.4     Data Format

�Ŀ                                                          �Ŀ

� �                                                          � �

Index �� ������������������������������������������������������������ ���

��������������������������������������������������������������Ŀ

�Index delay�Sector�Sector�    . . . .     �Sector�Sector�Final�

�  Field    �  0   �  1   �    . . . .     �  N-1 �  N   � Gap �

����������������������������������������������������������������

�      �

�������������      ���������������������������������������������Ŀ

����������������������������������������������������������������Ŀ

�Preamble 1 �Address� I.D.   �EDC�Postamble 1�Sector�Inter sector�

�(Synch 1)  � Mark  �(Header)�ECC�           �Data  �     Gap    �

������������������������������������������������������������������

�        �               �      �

���������������������        �����Ŀ         �      �

����������������������������������Ŀ         �      �

�ID AM�Track�Track�Head�Sector� ID �         �      �

�     �(MSD)�(LSD)� #  �  #   �Flag�         �      �

������������������������������������         �      �

�      �

�      �

�����������������������������      ������������Ŀ

�����������������������������������������������Ŀ

�Preamble 2 �Address�Data� Data �EDC�Postamble 2�

�(Synch 2)  � Mark  �Mark�Record�ECC�           �

�������������������������������������������������

1.5     Sector Interleaving

The initial formatting of a disk determines where sectors are

located within a track. It is not necessary to allocate sectors

sequentially around the track. The technique known as sector

interleaving optimizes access times when, although sectors are

accessed sequentially, a small amount of processing must be

performed between sector read/write.

interleave factor = 3

interleave factor = 2

1.6     Disk Formatting

A.  Physical Format (initialization)

Initializes the fixed disk with sectors and addresses to

access data.

B.  Partitioning

Assigns a partical amount of disk space to DOS.

C.  Logical Format

Verifies that the initialization of the fixed disk drive

was successfully completed.

1.7     Translative Versus Non-Translative Mode

Owing to the use of advanced technique, the new WDD contains

more cylinders, heads, and sectors per track. But due to DOS

limitation, some DOS  software can't access the extra disk

space. So some WDD manufacturer support "Translative mode" to

translate the extra disk space into DOS disk space.

Example 1 :

WDD have 1033 cylinders, 2 heads, 33  sectors/track in

non-translative mode, but it can have 1002 cylinders, 4 heads,

and 17 sectors/track in translative mode.

1033 * 2 * 33 = 68178 sectors/drive in non-translative mode

1002 * 4 * 17 = 68136 sectors/drive in translative mode

About 42 sectors in translative mode will be lost.

2.1     System Configuration for Fixed Disk Subsystem

��������Ŀ

������������Ĵ        �<����������Ŀ

�            � 8259A  �            �

�       ����>�        �            �

�       �    ����������            �                 ��������Ŀ

� ��������Ŀ                   ��������Ŀ      �---->� WDD 0  �

� �  CPU   �������������������>�        �<�����|����>�        �

� � INT 13H�<----�-------------�  WDC   �      |  �  ����������

� � -------�     |             �        �<-----|  �  ��������Ŀ

�>� INT 0EH�     |             ����������      |  ��>� WDD 1  �

����������     |                 ^           �---->�        �

|                 |                 ����������

|                 |

��������Ŀ            |

� DRAM   �<-----------�

����������

Command/Control  ��������

Data/Status      --------

Fig :    System Configuration for Fixed Disk System

2.2     I/O Port Definition

I/O Port Addr	  Read		       Write

1F0H/170H		  Data register	       Data register

1F1H/171H		  Error register       Write Precomp

1F2H/172H		  Sector Count         Sector Count

1F3H/173H		  Sector number	       Sector number

1F4H/174H		  Cylinder Low	       Cylinder Low

1F5H/175H		  Cylinder High	       Cylinder High

1F6H/176H		  Drive/Head(SDH)      Drive/Head(SDH)

1F7H/177H		  Status register      Command register

3F6H		  Alternate Status     Fixed Disk Control

3F7H		  Digital Input

A.	 Data Register

Transfers through this register are 16 bits wide for all

data transfer except ECC byte transfers during read/write

long command, which are 8 bits wide.

B.	 Error Register

There are two mode in all WDC operation: operational mode and

diagnostic mode. The controller is set into diagnostic mode

upon power on or receiving a diagnostic command. In diagnostic

mode, the status register contents are invalid and the error

register contents reflects a self test completion code. If bit

7 of completion code is set, the slave drive will fail.

Completion Code		Description

01H/81H			No error

02H/82H			Winchester Controller

03H/83H			Sector Buffer Error

04H/84H			ECC Device Error

05H/85H			Control Processor Error

The controller is set into operational mode by issuing any

command other than the diagnostic command. While in operational

mode, the error register is valid only when the error bit in the

status register is set. The bit definitions for operational mode

are shown:

bit 7		Bad Block

bit 6		Data ECC Error

bit 5		Reserved

bit 4		ID Not Found

bit 3		Reserved

bit 2		Aborted Command

bit 1		Track 0 Error

bit 0		Address Mark Not Found

C.  Write Precompensation Register

The Write precompensation register holds the value of the

starting  write precompensation cylinder divided by four. The

controller may use this value to determine when to turn on the

write precompensation logic.

D.  Sector Count Register

This register holds the number of sector to be transferred

during a read or write command, and the number of sectors per

per track during a format command. This register is decremented

for every sector during a multi-sector operation. Therefore, if

an error occurs during a multi-sector operation, this register

contains the number of sectors left to be transferred.

E.	 Sector Number Register

This register holds the starting sector number for any disk

operation. This register is updated as each sector is processed

in a multi-sector operation.

F.  Cylinder Number Register

The two eight bit cylinder number registers (Low and High)

specify the starting cylinder argument for disk operation.

G.  Drive/Head Register(SDH)

This register is used to specify the drive and head number

to be operated on during any following disk operation. During

the set parameters commands it is used to specify the maximun

number of heads on the drive. The bits are defined as follow:

bit 7		Reserved

bit 6		0

bit 5		1

bit 4		Drive select

bit 3		Reserved

bit 2-0		Head select

H.  Status Register

This register contains the results of the last command executed

and the status of the drive. Reading the status register also

clears any interrupt request to the host. The bits are defined

as follows:

bit 7		WDC Busy

bit 6		WDC Ready to accept command

bit 5		Write Fault

bit 4		Seek Completed

bit 3		Data request

bit 2		Corrected Data

bit 1		Index

bit 0		Non-coverable error

I.  Command Register

This register contains the code for the command to be performed.

All other command information should be written to the task

file before the command register is loaded. The BUSY bit in

the status register will be set when this register is written.

Interrupt request to the host is also cleared when this register

is written.

Valid command formats are as follows:

Recalibrate 		0 0 0 1 0 0 0 0

Seek			0 1 1 1 0 0 0 0

Read Sector		0 0 1 0 0 0 L R

Write Sector		0 0 1 1 0 0 L R

Format Track		0 1 0 1 0 0 0 0

Read Verify		0 1 0 0 0 0 0 R

Write Verify		0 0 1 1 1 1 0 0

Diagnostic		1 0 0 1 0 0 0 0

Read Buffer		1 1 1 0 0 1 0 0

Write Buffer		1 1 1 0 1 0 0 0

Set Parameters		1 0 0 1 0 0 0 1

L : ECC		R : Retry

J.  Alternate Status Register

This register contains the same information of the status

register. However, this register may be read anytime without

clearing a pending interrupt.

K.	 Fixed Disk Control Register

This register contains the software reset bit and the enable

bit to enable interrupt requests to the host. The bits are

defined as follows:

bit 7-3		0

bit 2		Reset(1 for 5 us minimun)

bit 1		Interrupt enable

bit 0		0

L.	 Digital Input Register

This register contains information about the state of the

drive. The bits are defined as follows:

bit 7		Reserved

bit 6		Write Gate status(0 - on, 1 - off)

bit 5-2		Head Select

bit 1-0		Drive Select

2.3     Timing Consideration

A.	  WDD Start Time & Stop Time

The WDD will speed up spindle to desired speed when user

powers on.

The time from power on to spindle settling down is called

start time. Do not attempt to issue any command during WDD

start time, for it will interfere with WDD self test. Generally

, the WDD start time is approximately 15 seconds.

The WDD will speed down spindle until stopped when powered off.

The time from power off to spindle halt is called stop time.

Generally, the stop time will be approximately 15 seconds.

B.	  WDD Access time & Seek Time

The performance of WDD is relative to the access time and seek

Time. Generally, the access time and seek time does not affect

the  WDD  programming sequence. In some WDD,  it supports

different seek speed that is set using seek command.

C.	  WDD  Restart time

In some WDD, it will turn off spindle automatically when it

detects unstable condition of the power supply, delay some time

, and turn on the spindle automatically. The time between turn

off and on of the spindle is called restart time.  The WDD

restart time must be  considered in WDD programming sequence

when using CONNER WDD.

3.      Programming Sequence in Fixed Disk Service Program

;++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

;Module_Name: read_sector_hd

;Calling Sequence:

;

;        Input :   AH = 02h

;                  AH (in stack) = no_error

;                  DL = drive no.(not value check)

;                  DH = head no.

;                  CH = cylinder low

;                  CL =  bit 7-6  -- cylinder high

;                        bit 5-0  -- sector no.(1 base)

;                  AL = no. of sectors to be read

;        output :  AH (in stack) = status of operation

;

;PDL:    IF WDC valid THEN

;           prepare task file

;           IF WDC ready THEN

;              write READ command to WDC

;           ENDIF

;           DO

;		   IF interrupt occurred THEN

;                read one sector from data register

;                get result back

;		   ELSE

;		      return error code = 80H (timeout)

;		   ENDIF

;           UNTIL no data to be transferred

;        ELSE

;           return error code = 01H (invalid function call)

;        ENDIF

;+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

page

f02_00:  ; check valid_WDC

call      valid_wdc            ; check valid_wdc

jc        f02_60               ; goto f02_40 if error occurred

;     send read command block to WDC

call      write_data           ; Send command

mov       ah, 021h             ; BX = 021h for read command

call      out_com              ; out command byte to WDC

jc        f02_60               ; goto f02_60 if WDC is not ready

mov       di, [bp-4]           ; DI = offset of transfer address

call      wait_drq             ;

jc        f02_60               ;

f02_20:  ; wait DRQ and test 64K boundary

cmp       di, 0fe00h           ; in 64K boundary

jb        f02_30               ; goto f02_30 if there is no boundary error

mov       ax, es               ;

add       ax, 20h              ;

mov       es, ax               ; adjust ES

sub       di, 200h             ; adjust DI

f02_30:

call      wait_int             ; wait_interrupt

jc        f02_60               ; goto f02_60 if time out

mov       dx, hd_port0         ; data register

mov       cx, 100h             ; count = 256 words

cld                            ;

cli                            ;

rep       insw                 ;

sti                            ;

;     get result back

call      get_result           ; get result byte into AL

jc        f02_60               ;

dec       byte ptr [bp-2]      ;

jnz       f02_20               ;

f02_60:

jmp       disk_end

5.      Q & A