Interrupts Table of H/W, BASIC, & DOS

Interrupt       Address   Function      Name            BIOS entry

-----------------------------------------------------------------------

0             0-3     Divide by Zero        D11

1             4-7     Single Step           D11

2             8-B     Nonmaskable           NMI INT

3             C-F     Breakpoint            D11

4            10-13    Overflow              D11

5            14-17    Print Screen          PRINT_SCREEN

6            18-1B    Reserved              D11

7            1C-1F    Reserved              D11

8            20-23    Time of Day           TIMER_INT (IRQ 0)

9            24-27    Keyboard              KB_INT    (IRQ 1)

A            28-2B    Reserved              D11       (IRQ 2)

B            2C-2F    Communications        D11       (IRQ 3)

C            30-33    Communications        D11       (IRQ 4)

D            34-37    Alternate Printer     D11       (IRQ 5)

E            38-3B    Diskette              DISK_INT  (IRQ 6)

F            3C-3F    Printer               D11       (IRQ 7)

10            40-43    Video                 VIDEO_IO

11            44-47    Equipment Check       EQUIPMENT

12            48-4B    Memory                MEMORY_SIZE_DETERMINE

13            4C-4F    Diskette/Disk         DISKETTE_IO

14            50-53    Communication         RS232_IO

15            54-57    Cassette              CASETTE_IO/System Extensions

16            58-5B    Keyboard Service      KEYBOARD_IO

17            5C-5F    Printer               PRINTER_IO

18            60-63    Resident BASIC        F600:0000

19            64-67    Bootstrap             BOOTSTRAP

1A            68-6B    Time of Day           TIME_OF_DAY

1B            6C-6F    Keyboard Break        DUMMY_RETURN

1C            70-73    Timer Tick            DUMMY_RETURN

1D            74-77    Video Parameters      VIDEO_PARMS

1E            78-7B    Diskette Parameters   DISK_BASE

1F            7C-7F    Video Graphics Chars  O

20            80-83    DOS program terminate

21            84-87    DOS function call

22            88-8B    DOS terminate address

23            8C-8F    DOS Ctrl Break exit address

24            90-93    DOS fatal error vector

25            94-97    DOS absolute disk read

26            98-9B    DOS absolute disk write

27            9C-9F    DOS terminate, fix in storage

28-3F          A0-FF    Reserved for DOS

40-5F         100-17F   Reserved for BIOS

60-67         180-19F   Reserved for user program interrupts

68-6F         1A0-1BF   Not used

70           1C0-1C3   Realtime Clock INT    RTC INT   (IRQ 8)

71           1C4-1C7   Redirect to INT 0Ah   RE_DIRECT (IRQ 9)

72           1C8-1CB   Reserved              D11       (IRQ 10)

73           1CC-1CF   Reserved              D11       (IRQ 11)

74           1D0-1D3   Reserved              D11       (IRQ 12)

75           1D4-1D7   Redirect to NMI int.  INT_287   (IRQ 13)

76           1D8-1DB   Disk hardware INT     HD_INT    (IRQ 14)

77           1DC-1DF   Reserved              D11       (IRQ 15)

78-7F         1E0-1FF   Not used

80-85         200-217   Reserved for BASIC

86-F0         218-3C3   Used by BASIC interpreter while BASIC is running

F1-FF         3C4-3FF   Not used