INT 05H - PRINT_SCREEN

This logic eill be invoked by interrupt 05H to print the screen.

The cursor position at the time this routine is invoked will be saved

and restored upon completion. The routine is intended to run with

interrupts enabled. If a subsequent Print Screen Key is depressed

while this routine is printing it will be ignored. The base printers

status is checked for not busy and not out of paper. An initial

status error will abend the print request.

Address 0050:0000 contains the status of the print screen:

50:00   = 0     Print Screen has not been called or upon return

from a call. this indicates a successful operation.

= 1     Print Screen is in progress - Ignore the request.

= 255   Error encountered during printing.

PDL:

Save all registers

IF STATUS_BYTE (50:00) is in progress (= 01)

THEN

jump to EXIT: (ignore this key progress)

ELSE

get currnt page, number of rows, and number of columns on screen

get printer status from INT 17h

IF printer stataus is Busy or Out_of_Paper

THEN

set STATUS_BYTE to error status (= 0FFh),

jump to EXIT:

ELSE

print the screen

IF printer stataus is error during process

THEN

set STATUS_BYTE to error status (= 0FFh),

jump to EXIT:

ELSE

set STATUS_BYTE to no error status (= 00h)

ENDIF

ENDIF

ENDIF

EXIT:

restore all registers

return