INT 08H (IRQ 0) - TIMER_INT hardwre interrupt

This routine handles the Timer Interrupt from channel 0 of the 8254

Timer chip. Input frequency is 1.19318 MHZ and the divisor is 65536.

Resulting in approximately 18.2 Interrupts every second.

The Interrupt Handler maintains a Count (40:6C) of Interrupts since

Power On Time, which may be used to establish Time Of Day.

The Interrupt Handler also decreases the Motor Control Count (40:40)

of the diskette, and when it expires, will Turn Off the diskette

motor(s), and Reset the Motor Running Flags.

The Interrupt Handler will also invoke a user routine through

Interrupt 1Ch at every Time Tick. The user must code a routine and

place the correct address in the vcetor table.

PDL:

Save the registers

decrease the TIMER_COUNT (TIMER_HIGH,TIMER_LOW - 40:6E,40:6C)

IF Timer Count equals 24 hours (TIMER_HIGH,TIMER_LOW) = (018h,0B0h)

THEN

Set the TIMER_COUNT to zero,

Set the TIMER_OVERFLOW (40:70) to one

( Timer has counted past 24 hours)

ENDIF

decrease MOTOR_COUNT (40:40)

IF MOTOR_COUNT equals to zero

THEN

Turn off motor running bits (bit 3,2,1,0 of 40:3F = 0,0,0,0)

Turn off the motor (Output the C0h to PORT 3F2h)

ENDIF

Call the user interrupt routine (INT 1CH)

Send EOI to 8259 Interrupt Controller

Restore the registers

return