INT 11H -  EQUIPMENT_I

EQUIPMENT DETERMINATION

This routine attempts to determine what optional devices are attached

to the system.

INPUT -

No registers

The @EQUIPMENT variable is set during the power on diagnostics using

the following hardware assumptions :

Port 03FA = Interrupt ID Register of 8250 (Primary)

02FA = Interrupt ID Register of 8250 (Secondary)

Bits 7-3 are always 0

Port 0378 = Output Port of Printer (Primary)

0278 = Output Port of Printer (Secondary)

03BC = Output Port of Printer (Monochrome-Printer)

OUTPUT -

(AX) is set, Bit Significant, to indicate attached I/O

Bit 15,14 = Number of Printer Attached

Bit 13 = Internal Modem Installed

Bit 12 = Not Used

Bit 11,10,9 = Number of RS232 cards Attached

Bit 8 = Not Used

Bit 7,6 = Number of Diskette Drives

00 = 1, 01 = 2 Only if Bit 0 = 1

Bit 5,4 = Initial Video Mode

0 0 - Unused

0 1 - 40x25 BW using Color Card

1 0 - 80x25 BW using Color Card

1 1 - 80x25 BW using BW Card

Bit 3 = Not used

Bit 2 = Not used

Bit 1 = Math Coprocessor

Bit 0 = 1 (IPL Diskette Installed)

No other register affected

PDL:

Read btye of @EQUIP_FLAG (40:10) to AX register

return