INT 12 - MEMORY_SIZE_DET_I

MEMORY SIZE DETERMINE

This routine returns the amount of memory in the system as determined

by the POST routine. (Up to 640K)

Note that the system may not be able to use I/O memory unless there

is a full complement of 512K bytes on the planar.

INPUT -

No registers

The @MEMORY_SIZE variable is set during power in diagnostics

according to the following assumptions :

1. Configuration record in non-volatile memory equals the actual

memory size installed.

2. All installed memory is functional. If the memory test during

POST indicates less, then this value becomes the default.

If non-volatile memory is not valid (not initialized or battery

failure) then actual memory determined becomes the default.

3. All memory from 0 to 640K must be contiguous.