INT 70H (IRQ 8) - ALARM INTERRUPT HANDLER (RTC) hardware interrupt

This Routine handles the Periodic and Alarm Interrupts from the CMOS

TIMER. Input Frequency is 1.024 KHZ or approximately 1024 interrupts

every second for the Periodic Interrupt. For the Alarm Function,

the interrupt will occur at the designated time.

Interrupts are Enabled when the EVENT or ALARM Function is Activated.

For the EVENT INTERRUPT, the Handler will decrease the WAIT COUNTER

and when it expires will SET the designated Location to 80H. For the

ALARM INTERRUPT, the user must provide a routine to intercept the

correct address from the VECTOR TABLE invoked by INTERRUPT 4AH

prior to SETTING the REAL TIMER CLOCK ALARM (INT 1AH, AH = 06H)

PDL:

Save the registers

RESTART:

Read the PIE and AIE bits from the index 0Bh of COMS (bit 5,4)

IF PIE and AIE is not Enabled (= 0,0)

THEN

jump EXIT:

ELSE

Read the PF and AF bits from the index 0Ch of CMOS (bit 5,4)

IF PIE is Enabled (=1) and Periodic Interrupt is occured (PF=1)

THEN

Decrease the User Wait Count (0,976) (High, Low - 40:9E,40:9C)

IF User Wait Count < 0

THEN

Turn Off Periodic Interrupt Enable

Set the Wait Active Flag Off (40:A0 = 0)

Set User Wait Complete Flag ON (= 80H)

(Offset Address to User Wait Complete Flag - 40:98,

Segment Address to User Wait Complete Flag - 40:9A)

ENDIF

ENDIF

IF AIE is Enabled (=1) and Alarm Interrupt is occured (AF=1)

THEN

Enable NMI from the CMOS

Call the User interrupt routine (INT 4AH)

jump RESTART:

ENDIF

ENDIF

EXIT:

Enable NMI from the CMOS

Send EOI to 8259 Interrupt Controller

restore the registers

return