POST - INITIAL RELIABILITY TESTS

TEST.01

80286 PROCESSOR TEST (REAL MODE)

Description

Verify Flags, Registers, and Conditional Jumps.

Check for Processor ShutDown (0 to A)

SHUT0:

TEST.02

ROM CHECKSUM TEST 1

Description

A Checksum is done for the 32K ROM modules (two)

containing POST, BASIC and BIOS.

TEST.03

VERIFY CMOS SHUTDOWN BYTE

Description

Rolling Bit Written and Verified at Shutdown address.

TEST.04

8254 CHECK TIMER 1 ALL BITS ON

Description

Set Timer Count

Check that Timer 1 all bits on

TEST.05

8254 CHECK TIMER 1 ALL BITS OFF

Description

Set Timer Count

Check that Timer 1 all bits off.

TEST.06

8237 DMA 0 INITIALIZATION

CHANNEL REGISTER TEST

Description

Disable the 8237 DMA Controller 0.

Write/Read the current DMA 0 address and Word Count Redisters

for all channels.

TEST.07

8237 DMA 1 INITIALIZATION

CHANNEL REGISTER TEST

Description

Disable the 8237 DMA Controller 1.

Write/Read the current DMA 1 address and Word Count Redisters

for all channels.

TSET.08

DMA PAGE REGISTER TEST

Description

Write/Read all Page Registers.

TSET.09

STORAGE REFRESH TEST

Description

Verify Refresh is occurring.

TEST.10

8042 INTERFACE TEST

READ CONFIGURATION JUMPERS

Description

Isuue a self test to the 8042.

Insure a 55H is received.

Read Manufacturing and Display Jumpers and Save in MFG_TEST.

TEST.11

BASE 64K READ/WRITE MEMORY TEST

Description

Write/Read/Verify data patterns AA, 55, FF, 01, and 00

to 1st 64K of storage. Verify storage Addressability.

TEST.11A

VERIFY 286 LGDT/SGDT/LIDT/SIDT INSTRUCTIONS

Description

Load GDT and IDT Registers with AA, 55, 00 and Verify correct.

TEST.12

VERIFY CMOS CHECKSUM/BATTERY OK

Description

Determine if Config Record can be used for initialization.

TEST.13

PROTECTED MODE TEST AND MEMORY SIZE DETERMINE (0 --> 640K)

Description

This Routine runs in Protected Mode in order to address all of

storage. It checks the Machine Status Word (MSW) for Protected Mode

and the Base Memory Size is determined and saved. Bit 4 of the CMOS

Diagnostic Status Byte is set if 512K --> 640K memory is installed.

During a Power Up sequence the Memory Size determine is done with

Plannar and I/O Parity Checks Disabled. During a Soft RESET the

Memory Size Determine will check for Parity Errors.

SHUT8:

TEST.13A

PROTECTED MODE TEST AND MEMORY SIZE DETERMINE (ABOVE 1024K)

Description

This routine runs in Protected Mode to address above 1M.

The Memory Size is Determined and Saved in CMOS.

During a Power Up sequence the Memory Size determine is done with

Plannar and I/O Parity Checks Disabled. During a Soft RESET the

Memory Size Determine will check for Parity Errors.

SHUT1:

TEST.14

INITIALIZE AND START CRT CONTROLLER (6845)

TEST VIDEO READ/WRITE STORAGE

Description

Reset the Video Enable Signal.

Select Alphanumeric Mode, 40 * 25, B & W.

Read/Write Data Patterns to memory. Check Storage Addressability.

ERROR = 1 Long and 2 Short Beeps

TEST.15

SETUP VIDEO DATA ON SCREEN FOR VIDEO LINE TEST

Description

Enable Video Signal and Set Mode, Display a horizontal bar on screen.

TEST.16

CRT INTERFACE LINES TEST

Description

Sense ON/OFF Transition of the Video Enable and

Horizontal SYNC Lines.

TEST.17

8259 INTERRUPT CONTROLLER TEST

Description

Read/Write the Interrupt Mask Register (IMR) with all ones and zeros.

Enable System Interrupt. Mask Device Interrupts Off.

Check for Hot Interrupts (Unexpected).

TEST.18

8254 TIMER CHECKOUT

Description

Verify that the System Timer (0) doesn't count too fast or too slow.

TEST.19

ADDITIONAL READ/WRITE STORAGE TEST (MUST RUN IN PROTECTED MODE)

Description

Write/Read Data Patterns to any Read/Write Storage after the 1st 64K.

Storage Addressability is checked.

SHUT3: Memory Error Reporting (R/W Memroy or Parity Errors)

TEST.20

ADDITIONAL PROTECTED (VIRTUAL MODE) TEST

Description

The Processor is put in Protected Mode and the following functions

are verified

1.Verify Protected Mode

The Machine Status is check for Virtual Mode.

2.Programmed Interrupt Test

An Programmed Interrupt 32 is issued and verified.

3.Exception Interrupt 13 Test

A Descriptor Segment Limit is set to zero and a write to that

Segment is attempted and Exception 13 is expected and verified.

4.LDT/SDT LTR/STR Test

Load LDT Register and Verify correct.

Load TASK Register and Verify correct.

They are verified via the STORE Instruction.

5.The Control Flags of the 286 for Direction are verified via the

STD and CLD Commands in Protected Mode.

6.BOUND Instruction Test (Exception INT 5)

Create a Signed Array Index within and outside the limits.

Check that no EXC INT if within limit and that and EXC INT 5 occurs

if outside the limits.

7.PUSH ALL POP ALL Test

Set General Purpose Registers to different values, issue a PUSH ALL,

Clear the Rigisters then issue a POP ALL and verify correct.

8.Check the VERR/VERW Instructions

The Access Byte is set to Read Only then to a Write Only and the

VERR/VERW Instructions are verified.

9.Cause an Interrupt 13 via a Write to a Read Only Segment

10.Verify the ARPL Instruction Functions

Set the RPL Field of a Selector RPL and verify that current Selector

RPL is set correctly.

11.Verify the LAR Instruction Functions

12.Verify the LSL Instruction Functions

13.Low MEG Chip Select Test

SHUT2:

SHUT7:

SHUT6:

TEST.21

KEYBOARD TEST

Description

Reset the Keyboard and Check that Scan Code "AA" is Returned to the

Processor. Check for Stuck Keys.

TSET.22

CHECK FOR OPTIONAL ROM FROM C800->E000 in 2K BLOCKS

Description

A Valid Module has "55AA" in the first 2 locations, Length Indicator

(Length/512) in the 3rd location, and Test/Initial Code Starting in

the 4th location.

<NOTE:> this routine is after the TEST.23.

TEST.23

DISKETTE ATTACHMENT TEST

Description

Check if IPL Diskette Drive is attached to System. If attached, verify

Status of NEC FDC and check Status. Complete System Initialization

then Pass Control to the Boot Loader Program.

BOOTSTRAP - INT 19H

BOOT STRAP LOADER

Description

Track 0, Sector 1 is Read into the Boot Location (0:7C00) and

Controll is Transferred there.

If there is a Hardware Error, Control is Transferred to the

ROM BASIC entry point.

FLOW:

1.Reset the Disk Parameter Table Vector (INT 1EH)

2.Clear @BOOT_LOCN (0:7C00)

3.Load System From Diskette -- CX has Retry Count 4

4.If read successful and Boot Record is valid,

then go to @BOOT_LOCN (0:7C00),

else loop 3.

5.Attempt Bootstrap from Fixed Disk -- CX has Retry Count 3

6.If read successful and Boot Record is valid,

then go to @BOOT_LOCN (0:7C00),

else loop 5.

7.INT 18H -- Go to RESIDENT BASIC