System Timer and Real Time Clock Services

-----------------------------------------

1. Introduction

The ROM BIOS contains five time-related services, INT 08h, INT 1Ah,

INT 1Ch, INT 4Ah and INT 70h. The system has an Intel 8254 (or 8253)

timer chip to generate a timer tick 18.2 times per second and issue

INT 8 and INT 1Ch. The system has a MC146818A real time clock also.

It generates 1024 timer tick per second, and contains 64 bytes of

CMOS. INT 1Ah are used to access the first 12 bytes of CMOS that

contain time related data. INT 70h is the RTC ISR and INT 4Ah is the

user Alarm  interrupt that is invoked by INT 70h.

2. Diagram of Service Routines

�����������������������������������������������������Ŀ

�  MC146818A   � CMOS data 00-0C,32 � CMOS data 0D-3F �

�              �   Time data        �    BIOS using   �

�������������������������������������������������������

�                      #

���Ŀ                  |

#                  |      INT 1Ah

INT 70                |  ��������������Ŀ

���������������������Ŀ       C  � Fun 00,01    �

�Periodic  � Alarm    �       |  � Read/Set     �--|

�Interrupt � Interrupt�#�Ŀ   |  � System timer �  |

�����������������������   �   |  ��������������Ĵ  |

|          |�        �   |  � Fun 02-05    �  |

|          |�        �   |--� Read/Set RTC �  |

F          EE        �   |  � time and day �  |

|          |�        �   D  ��������������Ĵ  |

|----------|�        �   |--� Fun 06,07,09 �  B

|           �        ������ĴSet/Reset/Read�  |

|           #               � RTC alarm    �  |

|      ��������Ŀ           ����������������  |

|      � INT 4A �                             |

|      ����������                             |      ��������Ŀ

|                                             |      �  8254  �

|                        BIOS data            |      ����������

|                    ��������������Ŀ         |           �

|                    � system timer �  <------|           #

|                    �40:6c - 40:70 �  <-|           ��������Ŀ

|                    ��������������Ĵ    |-----A----Ĵ INT 8  �

|                    �  RTC data    �                ����������

|---------------->   �40:98 - 40:a6 �                     A

����������������                     #

��������Ŀ

� INT 1C �

����������

3. Programming

INT 8 -- A timer tick is generated by 8254 18.2 times per second.

Increase the timer count to reach the value of one day and

then reset timer counter and set timer-overflow flag.

Decrement motor off couter and when the count reaches 0,

turns the diskette drive motor off, and resets the flag of

motor running.

Finally, issue interrupt 1CH which may be provided by user.

------  A

INT 1A -- Subfunction 0 and 1 : Read/Set system-time timer counter

from BIOS data.

------  B

Subfunction 2-5 : Read/Set real-time clock time and date.

Before read CMOS data must be wait in RTC time available

(status register A bit 7). Before write CMOS data must be

clear status register B bit 7 (in set mode), and when

finish operation to set this bit again.

Error condition : RTC error and time update in progress

for a long time. (loop 1000h times)

-----  C

Subfunction 6 : Set RTC alarm.

check RTC status.

set alarm interrupt enable.

set alarm time.

enable interrupt of 8259.

Subfunction 7 : Reset RTC alarm.

disable alarm interrupt of register B.

Subfunction 9 : Read alarm time of RTC and status.

-----  D

INT 70 -- RTC interrupt service routine

test interrupt occur or not (if not then send EOI only)

a. alarm interrupt : send EOI and issue INT 4A

-----  E

b. periodic interrupt : decrease wait_time (40:a2,a4)

1000 if no more counter then set

user periodic flag (address of

40:98) and disable PIE.

-----  F