Chapter  5

VGA

-------

****************************************

*                                      *

******       INTRODUCTION       ********

*                                      *

****************************************

A. VIDEO MODE :

1. Text mode

2. Graphics mode

B. DISPLAY MEMORY ORGANIZATION

C. CHARACTER GENERATOR

D. COLOR PALETTE AND COLOR REGISTER

E. GRAPHICS READ AND WRITE MODES

1. Write modes : 0-3

2. Read modes  : 0-1

F. VGA REGISTER

1. General or External registers

2. Sequencer registers

3. CRTC registers

4. Graphics registers

5. Attribute registers

G. VGA BIOS DATA AREA

H. VIDEO POST SEQUENCE

I. VIDEO BIOS SERVICE ROUTINES

************************************

*                                  *

*******     TEXT  MODES      *******

*                                  *

************************************

��������������������������������������������������������������������Ŀ

� Mode       #chars            Display   page   Resolution    Buffer �

�           (X/Y)   Char Box   Format    no.      in Pixels          �

��������������������������������������������������������������������ĳ

� 0,1       40/25     8 x 8    16/256K   8        320 x 200   B8000  �

��������������������������������������������������������������������ĳ

� 0*,1*     40/25     8 x 14   16/256K   8        320 x 350   B8000  �

��������������������������������������������������������������������ĳ

� 0+,1+     40/25     8 x 16   16/256K   8        320 x 400   B8000  �

��������������������������������������������������������������������ĳ

� 2,3       80/25     8 x 8    16/256K   8        640 x 200   B8000  �

��������������������������������������������������������������������ĳ

� 2*,3*     80/25     8 x 14   16/256K   8        640 x 350   B8000  �

��������������������������������������������������������������������ĳ

� 2+,3+     80/25     8 x 16   16/256K   8        640 x 400   B8000  �

��������������������������������������������������������������������ĳ

� 7         80/25     9 x 14   bw        8        720 x 350   B0000  �

��������������������������������������������������������������������ĳ

� 7+        80/25     9 x 16   bw        8        720 x 400   B0000  �

����������������������������������������������������������������������

Note:

1. The asterisk (*) in the display mode indicates that VGA is in a text

mode with 350 vertical lines.

2. The plus (+) in the display mode indicates that VGA is in a text mode

with 400 vertical lines.

3. Display modes 0 through 6 emulate the Color Graphics Adapter(CGA).

Mode 7 emulates the IBM Monochrome MDA or Hercules Monochrome graphics.

4. The code "bw" stands for black and white or synonymously monochrome.

************************************

*                                  *

*******    GRAPHICS MODES    *******

*                                  *

************************************

��������������������������������������������������������������������Ŀ

�   Mode    #chars            Display   page     Resolution  Buffer  �

�  (Hex)    (X/Y)   Char Box  Format    no.      in Pixels           �

��������������������������������������������������������������������ĳ

�  4,5      40/25     8 x 8   4/256K    1        320 x 200   B8000   �

��������������������������������������������������������������������ĳ

�  6        80/25     8 x 8   2/256K    1        640 x 350   B8000   �

��������������������������������������������������������������������ĳ

�  D        40/25     8 x 8   16/256K   8        320 x 200   A0000   �

��������������������������������������������������������������������ĳ

�  E        80/25     8 x 8   16/256K   4        640 x 200   A0000   �

��������������������������������������������������������������������ĳ

�  F        80/25     8 x 14  bw        2        640 x 350   A0000   �

��������������������������������������������������������������������ĳ

� 10        80/25     8 x 14  16/256K   2        640 x 350   A0000   �

��������������������������������������������������������������������ĳ

� 11        80/30     8 x 16  2/256K    1        640 x 480   A0000   �

�������������������������������������������������������������������� �

� 12        80/30     8 x 16  16/256K   1        640 x 480   A0000   �

��������������������������������������������������������������������Ĵ

� 13        40/25     8 x 8   256/256K  1        320 x 200   A0000   �

����������������������������������������������������������������������

������������������������������������������������������������������������Ŀ

�  MODE                                                                  �

�  (Hex)     # of Colors    # Bits per Plane  Bit Planes Affected  Notes �

������������������������������������������������������������������������Ĵ

�   4,5          4           2                 0,1                  1    �

������������������������������������������������������������������������ĳ

�   6            2           1                 0                    2    �

������������������������������������������������������������������������ĳ

�   D           16           1                 0,1,2,3              3    �

������������������������������������������������������������������������ĳ

�   E           16           1                 0,1,2,3              3    �

������������������������������������������������������������������������ĳ

�   F            4           1                 0,2                  4    �

������������������������������������������������������������������������ĳ

�   10          16           1                 0,1,2,3              3    �

������������������������������������������������������������������������ĳ

�   11           2           1                 0                    2    �

������������������������������������������������������������������������ĳ

�   12          16           1                 0,1,2,3              3    �

������������������������������������������������������������������������Ĵ

�   13         256           8                 0,1,2 or 3           5    �

��������������������������������������������������������������������������

Notes:

1.In modes 4 and 5, alternate bytes of data, each containing four

pixels, are stored in bit plane 0 and bit plane 1. Bit plane 0

contributes the low-order bit, and bit plane 1 contributes the

high-order bit of the two-bit pixel.

2.In modes 6 and 11 hex, each pixel consists of one bit in bit plane 0.

3.In mode D,E,10,12 hex, each pixel corresponds to one bit in each of

the four bit planes. Bit plane 0 contributes the low-order bit,

followed by bit plane 1, and bit plane 2. Bit plane 3 contributes the

high-order bit of the four bit pixel.

4.In mode F hex, bit plane 0 contributes the low-order bit and bit plane

2 contributes the high-order bit of the two bit pixel.

5.In mode 13 hex, all four bit planes are chained together and each

pixel consists of eight bits. The plane in which a pixel resides

depends on the position of the pixel on the display. Each plane holds

one-quarter of the pixels.

************************************

*                                  *

*******   CHARACTER  FONTS   *******

*                                  *

************************************

There are 5 character fonts set supported:

8 by 8 ,8 by 14, 8 by 16, 9 by 14, and 9 by 16.

ASCII_8X8       db    00H,00H,00H,00H,00H,00H,00H,00H         ;ASCII 0

db    07EH,081H,0A5H,081H,0BDH,099H,081H,7EH  ;ASCII 1

db    07CH,0FEH,0D6H,0FEH,0C6H,0FEH,07CH,00H  ;ASCII 2

db    06CH,0FEH,0FEH,0FEH,07CH,038H,010H,00H  ;ASCII 3

db    010H,038H,07CH,0FEH,07CH,038H,010H,00H  ;ASCII 4

db    010H,038H,054H,0FEH,054H,010H,07CH,00H  ;ASCII 5

.

.

.

ASCII_8X8_1     db    07CH,0C0H,0C6H,07CH,018H,0CH,078H,00H   ;ASCII 128

db    06CH,00H,0CCH,0CCH,0CCH,0CCH,076H,00H   ;ASCII 129

db    0CH,018H,03CH,066H,07EH,060H,03EH,00H   ;ASCII 131

db    038H,044H,078H,0CH,07CH,0CCH,076H,00H   ;ASCII 132

db    06CH,00H,078H,0CH,07CH,0CCH,076H,00H    ;ASCII 133

db    030H,018H,078H,0CH,07CH,0CCH,076H,00H   ;ASCII 134

db    038H,044H,078H,0CH,07CH,0CCH,076H,00H   ;ASCII 135

.

.

.

ASCII_8X14      db    00H,00H,00H,00H,00H,00H,00H,00H,00H

db    00H,00H,00H,00H,00H                     ;ASCII 0

db    00H,00H,07EH,081H,0A5H,081H,081H,0BDH

db    099H,081H,07EH,00H,00H,00H              ;ASCII 1

db    0H,00H,07EH,0FFH,0DBH,0FFH,0FFH,0C3H

db    0E7H,0FFH,07EH,00H,00H,00H              ;ASCII 2

db    00H,00H,00H,06CH,0FEH,0FEH,0FEH,0FEH

db    07CH,038H,010H,00H,00H,00H              ;ASCII 3

db    00H,00H,00H,010H,038H,07CH,0FEH,07CH

db    038H,010H,00H,00H,00H,00H               ;ASCII 4

.

.

.

ASCII_8X16      db    00H,00H,00H,00H,00H,00H,00H

db    00H,00H,00H,00H,00H,00H,00H,00H,00H     ;ASCII 0

db    00H,00H,07EH,081H,0A5H

db    081H,081H,0BDH,099H,081H,081H

db    07EH,00H,00H,00H,00H                    ;ASCII 1

db    00H,00H,07EH,0FFH,0FFH

db    0DBH,0FFH,0FFH,0C3H,0E7H

db    0FFH,07EH,00H,00H,00H,00H               ;ASCII 2

db    00H,00H,00H,00H,06CH,0FEH

db    0FEH,0FEH,0FEH,07CH,038H

db    010H,00H,00H,00H,00H                    ;ASCII 3

.

.

.

;

;       12 additional character of 8X14

;

ASCII_9X14   db      1dh,0H,00H,00H,00H,024H,066H,0FFH

db      066H,024H,00H,00H,00H,00H,00H           ;code 1dh

db      4dh,00H,00H,0C3H,0E7H,0FFH,0DBH

db      0C3H,0C3H,0C3H,0C3H,0C3H,00H,00H,00H    ;code 4dh

db      54h,00H,00H,0FFH,0DBH,099H,018H,018H

db      018H,018H,018H,03CH,00H,00H,00H         ;code 54h

db      56h,00H,00H,0C3H,0C3H,0C3H,0C3H

.

.

.

ASCII_9X16   db      1dh,00H,00H,00H,00H,00H,024H,066H

db      0FFH,066H,024H,00H,00H,00H,00H,00H,00H   ;code 1dh

db      4dh,00H,00H,0C3H,0E7H,0FFH,0DBH,0C3H

db      0C3H,0C3H,0C3H,0C3H,0C3H,00H,00H,00H,00H ;code 4dh

db      54h,00H,00H,0FFH,0DBH,099H,018H,018H,018H

db      018H,018H,018H,03CH,00H,00H,00H,00H      ;code 54h

db      56h,00H,00H,0C3H,0C3H,0C3H,0C3H,0C3H,0C3H

db      0C3H,066H,03CH,018H,00H,00H,00H,00H      ;code 56h

db      57h,00H,00H,0C3H,0C3H,0C3H,0C3H,0C3H,0DBH

db      0DBH,0DBH,0FFH,066H,00H,00H,00H,00H      ;code 57h

.

.

.

The 9 by 14 and 9 by 16 character font are referred to as the supplement

character set. It is called supplemental because not all of the 256

characters are provided. Typical characters that are included in the

supplement are quotation marks ("),the plus sign (+),the minus sign (-),

M,T,V,W,X,Y,Z,m,v,w,and line-drawing symbols.

************************************

*                                  *

****     CHARACTER ATTRIBUTE    ****

*                                  *

************************************

A. MONOCHROME MODE

Mode 7: Monochrome Attributes

����������������������������������������������������������������������

Background            Forground              Attribute

(bit3 6-4)            (bits 2-0)

����������������������������������������������������������������������

0                     0                      Black

0                     1                      Underline

0                     7                      Normal(white on black)

7                     0                      Reversed(black on white)

7                     7                      White

Mode 7: Examples of Monochrome Attributes

������������������������������������������������������������������������

Blink    Background   Intens   Forground    Code

(Bit 7)  (bits 6-4)   (Bit 3)  (Bit 2-0)    (Hex)   Attribute

������������������������������������������������������������������������

0        0            0        1            1       Underline

0        0            1        1            9       Intense Underline

0        7            0        0            70      Reversed

0        7            1        0            74      Intense Reversed

1        7            0        0            F0      Blinking Reversed

1        7            1        0            F1      Intense Blinking

Reserved

Mode 7: Palette Registers

�����������������������������������������������������������������������

Register #      Register Value       Register #       Register Value

�����������������������������������������������������������������������

0               00                   8                10

1               08                   9                18

2               08                   A                18

3               08                   B                18

4               08                   C                18

5               08                   D                18

6               08                   E                18

7               08                   F                18

B. SIXTEEN-COLOR MODE

Default Colors for a 16-color Palette

������������������������������������������������

Palette Register (Hex)               Color

������������������������������������������������

0                              Black

1                              Blue

2                              Green

3                              Cyan

4                              Red

5                              Magenta

6                              Brown

7                              White

8                              Gray

9                              Light Blue

A                              Light Green

B                              Light cyan

C                              Light red

D                              Light Magenta

E                              Light Brown (yellow)

F                              Bright White

C. ATTRIBUTE BYTE IN THE 16-COLOR MODE

7     6     5     4     3     2     1     0

���������������������������������������������Ŀ

�                      �                      �

�����������������������������������������������

Background             Forground

7     6     5     4     3     2     1     0

���������������������������������������������Ŀ

�                      �    �                 �

�����������������������������������������������

Background           �    Forground

Font #

7     6     5     4     3     2     1     0

���������������������������������������������Ŀ

�   �                  �                      �

�����������������������������������������������

�    Background           Forground

Blinking

7     6     5     4     3     2     1     0

���������������������������������������������Ŀ

�   �                  �   �                  �

�����������������������������������������������

�     Background       �      Forground

Blinking                Font #

Note:

1.  The character attribute byte is used to determine the character color,

to determine the blinking status, and to select from one of the two

character sets.

Bit 3 of the character attribute byte is used either to select one of

16 forground colors for the associated character or to choose from one

of two resident character sets.

Bit 7 is selected to have dual function of either being part of the

background color code or being a switch for the blinking feature.

2.  The default alphanumeric characters are defined by the bit patterns

in the first table in MAP 2, regardless of the value of bit 3 of the

attribute bytes of the characters displayed.

Changing the value in the Character Map Select register, changes the

character definition tables associated with bit 3 of attribute bytes.

If two different values appear in the bit fields in the Character Map

Select register, the value of bit 3 designates one of two different

character definition tables.

************************************

*                                  *

*******    VGA REGISTERS      ******

*                                  *

************************************

General or External Registers

��������������������������������������������������������������������������

Name                 Write (Hex)     Read (Hex)

Misc. Output         3C2             3CC

Feature Control      3DA/3BA         3CA

Input Status #0                      3C2

Input Status #1                      3DA/3BA

��������������������������������������������������������������������������

Sequencer Registers

�������������������������������������������������������������������������

Name               Index        R/W Port(Hex)

Address                x        3C4

Reset                  0        3C5

Clocking Mode          1        3C5

Map Mask               2        3C5

Character Map Select   3        3C5

Memory Mode            4        3C5

�������������������������������������������������������������������������

Attribute Controller Registers

�������������������������������������������������������������������������

Name                  Index        R/W Port(Hex)

Address                   x        3C0

Palette                 0-F        3C1

Mode Control             10        3C1

Overscan Color           11        3C1

Color Plane Enable       12        3C1

Horizontal Pixel Panning 13        3C1

Color Select             14        3C1

�������������������������������������������������������������������������

Graphics Control Registers:

�������������������������������������������������������������������������

Name               Index        R/W Port(Hex)

Graphics Address       x           3CE

Set/Reset              0           3CF

Enable Set/Reset       1           3CF

Color Compare          2           3CF

Data Rotate            3           3CF

Read Map Select        4           3CF

Mode                   5           3CF

Miscellaneous          6           3CF

Color Don't Care       7           3CF

Bit Mask               8           3CF

������������������������������������������������������������������������

CRT Controller Registers

������������������������������������������������������������������������

Name               Index        R/W Port(Hex)

Address                       x        3D4/3B4

Horizontal Total              0        3D5/3B5

Horizontal Display End        1        3D5/3B5

Start Horizontal Blank        2        3D5/3B5

End Horizontal Blank          3        3D5/3B5

Start Horizontal Retrace      4        3D5/3B5

End Horizontal Retrace        5        3D5/3B5

Vertical Total                6        3D5/3B5

Overflow                      7        3D5/3B5

Preset Row Scan               8        3D5/3B5

Max Scan Line                 9        3D5/3B5

Cursor Start                  A        3D5/3B5

Cursor End                    B        3D5/3B5

Start Address High            C        3D5/3B5

Start Address Low             D        3D5/3B5

Cursor Location High          E        3D5/3B5

Cursor Location Low           F        3D5/3B5

Vertical Retrace Start       10        3D5/3B5

Vertical Retrace Low         11        3D5/3B5

Vertical Display End         12        3D5/3B5

Offset                       13        3D5/3B5

Underline Location           14        3D5/3B5

Start Vertical Blank         15        3D5/3B5

End Vertical Blank           16        3D5/3B5

Mode Control                 17        3D5/3B5

Line Compare                 18        3D5/3B5

������������������������������������������������������������������������

************************************

*                                  *

*******    MEMORY MAPPING     ******

*                                  *

************************************

The video buffer can be configured as the following :

1). A000:0 -------  128K for mode D,E,F,10,11,12 and 13

2). A000:0 -------   64K for character generator for text modes

3). B000:0 -------   32k for mode 7 and 7+

4). B800:0 -------   32K for text mode 0,1,2,3 and graphics modes 4,5,6

************************************

*                                  *

*******  CHARACTER GENERATOR  ******

*                                  *

************************************

The character generator consists of the dot patterns that represent

each character in the character set.

The character dot patterns are stored in character boxes, each of

which is 32 bytes long.

Bits set to 1 in the character font will be displayed in the

forground color, and bits reset to 0 will be displayed in the

background color.

The display memory consists of four display planes called bit planes,

each being 64K bytes long.

VGA supports eight font tables. Each table consists of 256 32-byte bit

patterns, so the character box height is 32 scan lines. When the displayed

character box contains fewer than 32 lines, the character generator

ignored the extra bytes in each character definition.

The character generator has 8 blocks illustrated in the following:

A000:0   ���������ͻ            ��������������������Ŀ

� Font 0 ����������� 0 �         0          �

2000   ���������ĺ            ��������������������ĳ

� Font 4  �          1 �         1          �

4000   ���������͹            ��������������������ĳ   character 0

� Font 1  �            �         .          �

6000   ���������ĺ            �         .          �

� Font 5  �            �         .          �

8000   ���������͹            ��������������������ĳ

� Font 2  �         31 �        31          �

A000   ���������ĺ       ������������������������������������

� Font 6  �         32 �         0          �

C000   ���������͹            ��������������������ĳ

� Font 3  �         33 �         1          �

E000   ���������ĺ            ��������������������ĳ

� Font 7  �            �         .          �

���������ͼ            �         .          �    character 1

�         .          �

��������������������ĳ

63 �        31          �

������������������������������������

64 �         .          �        .

�         .          �        .

�         .          �        .

������������������������������������

�         0          �

��������������������ĳ

�                    �

�                    �     character 255

��������������������Ĵ

8191 �        31          �

����������������������

***************************************************

*                                                 *

*******   GRAPHICS MODES PIXEL PROGRAMMING   ******

*                                                 *

***************************************************

A. Relative Registers Accessed:

Graphics Control Registers:

Name               Index        R/W Port(Hex)

Graphics Address       x           3CE

Set/Reset              0           3CF

Enable Set/Reset       1           3CF

Color Compare          2           3CF

Data Rotate            3           3CF

Read Map Select        4           3CF

Mode                   5           3CF

Miscellaneous          6           3CF

Color Don't Care       7           3CF

Bit Mask               8           3CF

B. Read/Write Mode

1. Write Mode 0 (EGA/VGA)

2. Write Mode 1 (EGA/VGA)

3. Write Mode 2 (EGA/VGA)

4. Write Mode 3 (VGA Only)

5. Read Mode 0 (EGA/VGA)

6. Read Mode 1 (EGA/VGA)

***************************************

*                                     *

*******  PALETTE/COLOR REGISTER  ******

*                                     *

***************************************

Palette Utilization

�����������������������������������������������������������������������

Display Mode (hex)     Number of Colors     Palette Register Affected

0,1                      16                     0-15

2,3                      16                     0-15

4,5                       4                     0,1,2,3

6                         2                     0,1

7                         4                     0,1,4,5

D                        16                     0-15

E                        16                     0-15

F                         4                     0,1,4,5

10                       16                     0-15

11                        2                     0,1

12                       16                     0-15

13                      256                     0-15

Mode 7 Palette Defult Values

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  8  8  8  8  8  8  8  10 18 18 18 18 18 18 18

�����������������������������������������������������������

Mode F Palette Default Values

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  8  -  -  18 18 -  -  -  -  -  -  -  -  -  -

�����������������������������������������������������������

Mode 0,1,2,3 Palette Default Values ( 200 scan line)

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  1  2  3  4  5  14 7  38 39 3A 3B 3C 3D 3E 3F

�����������������������������������������������������������

Mode 0*,1*,2*,3*  Palette Default Values ( 350, 400 )

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  1  2  3  4  5  6  7  10 11 12 13 14 15 16 17

�����������������������������������������������������������

CGA-compatible Color Selection

����������������������������������������������������������������

ATR 1    ATR 0   Set 1         Set 0        VGA Palette Register

0        0       Background    Background    0

0        1       Cyan          Green         1

1        0       Magenta       Red           2

1        1       White         Brown         3

Mode 6 Palette Default Values

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  17 17 17 17 17 17 17 17 17 17 17 17 17 17 17

�����������������������������������������������������������

Mode 11 Palette Default Values

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  37 37 37 37 37 37 37 37 37 37 37 37 37 37 37

�����������������������������������������������������������

Mode D,E,10 and 12 Palette Default Values

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  1  2  3  4  5  14 7  38 39 3A 3B 3C 3D 3E 3F

�����������������������������������������������������������

Mode 13 Palette Default Values

����������������������������������������������������������

Register  0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

Value     0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F

�����������������������������������������������������������

. COLOR REGISTERS

The VGA is equipped with 256 Color Registers. A pixel or character in

display memory consists of from one to eight bits of data. This data

is used to address the 16 palette registers. The output of these

palette registers is used in conjunction with the data in display

memory to address the color registers. The color registers contain

the color codes that are output to the digital-to-analog converters

(DAC). The output of the DAC are sent to the monitor. Each of the

color registers consists of a red,green,and blue component. Each of

these color components is six bits wide.

. COLOR REGISTERS ADDRESSING

. COLOR REGISTER ORGANIZATION

1. Four banks of 64 color registers

2. Sixteen banks of 16 color registers

. ACCESSING THE COLOR REGISTERS

1. Modifying color registers through BIOS

2. Modifying color registers directly

Relative registers:

����������������������������������������������������

Address       Read/Write    Function

3C7       Write only    PEL address during read

3C7       Read only     DAC state

3C8       Read/Write    PEL address during write

3C9       Read/Write    PEL data register

3C6       Read/Write    PEL mask

����������������������������������������������������

. COLOR CONVERSION

1. Gray Scale

Gray shades are composed of equal amounts of red, green, and blue.

White is the maximum gray scale value, and black is the minimum

value.

The VGA allocates six bits per color which provides 64 possible

gray scales. These 64 levels of gray scale are achived using the

six-bit color fields within the color registers.

Gray scale using the color registers

����������������������������������������������������

Color Register Value            Gray Scale

������������������������

Red   Green   Blue

0        0       0             0% saturation

1        1       1           1.6% saturation

2        2       2           3.2% saturation

.        .       .           .

.        .       .           .

.        .       .           .

63       63      63           100% saturation

2. Summing Colors to Gray Scale

GRAY SCALE = (0.30 X RED) + (0.59 X GREEN) + (0.11 X BLUE)

3. Relative BIOS Calls

��������������������������������������������������������������������

AH      Subfunction (Hex)                 Function

10       AL = 1B          Sum color values immediately to gray scale

12       BL = 33          Enable/Disable summing to gray scale at

mode set

************************************

*                                  *

*******    VGA BIOS CALLS     ******

*                                  *

************************************

CONTROL

�����������������������������������������������������������

Mode Set

Select Active Display Page

Select Alternate Print Screen Routine

Video Enable/Disable

Display Switch

Video Screen On/Off

RETURN STATE

�����������������������������������������������������������

Return Curren Video State

Return Video Information

Read/Write Display Combination Code

Return Functionality/State Information

Save/Restore Video State

INTERACTIVE

�����������������������������������������������������������

Set Cursor Type

Set Cursor Position

Read Cursor Position

Cursor Emulation

TEXT

�����������������������������������������������������������

Scroll Active Page Up

Scroll Active Page Down

Read Character/Attribute

Write Character/Attribute

Write Character Only

Write Teletypewriter to Active Page

Select Scan Lines for Text Modes

Write String

GRAPHICS

�����������������������������������������������������������

Write Dot

Read Dot

PALETTE

�����������������������������������������������������������

Set Color Palette

Set Individual Palette Register

Set Overscan (Boarder) Register

Set All Palette Registers and Overscan

Toggle Intensity/Blinking Bit

Read Individual Palette Register

Read Overscan Resister

Read all Palette Registers and Overscan

Default Palette Loading During Mode Set

COLOR REGISTERS

�����������������������������������������������������������

Set Individual Color Register

Set block of Color Registers

Select Color Page

Read Individual Color Register

Read Block of Color Registers

Read Current Color Page Number

Sum Color Values to Gray Scale

Default Palette Loading During Mode Set

Summing to Gray Scales

CHARACTER GENERATOR

�����������������������������������������������������������

User text Load with Reset

ROM Monochrome Set with Reset

ROM 8x8 Character Set Load with Reset

Set Block Specifier

ROM 8x16 Character Set Load with Reset

User Alpha Load

ROM Monochrome Set

ROM 8x8 Character Set Load

ROM 8x16 Character Set Load

User Graphics Characters INT 1FH (8x8)

User Graphics Characters

Graphics Mode ROM 8x14 Character Set Load

Graphics Mode ROM 8x8 Character Set Load

Graphics Mode ROM 8x16 Character Set Load

Return Character Generator Information

******************************************

*                                        *

***   VGA POST (DIAGNOSTIC) SEQUENCE   ***

*                                        *

******************************************

���������������������Ŀ

�      Disable        �

�        VGA     1*   �

�����������������������     ����������������Ŀ

�                 �                �

�                 �                �

���������������������Ŀ     �      ���������������������Ŀ

�   Check Other       �     �      �     Dual Monitor    �

�  Display Card Exit? �     �      �        Test    7*   �

�                2*   �     �      �����������������������

�����������������������     �                �

�                 �                �

�                 �      ���������������������Ŀ

���������������������Ŀ     �      �     Set Mode        �

�       Enable        �     �      �                8*   �

�        VGA     3*   �     �      �����������������������

�����������������������     �                �

�                 �                �

�                 �      ���������������������Ŀ

���������������������Ŀ     �      �   Test Vertical     �

�   Set Corresponding �     �      �   Retrace Signal 9* �

�   Video Interrupt   �     �      �����������������������

�   Vectors      4*   �     �                �

�����������������������     �                �

�                 �      ���������������������Ŀ

�                 �      �      Show ACER      �

���������������������Ŀ     �      �      Copyright      �

�     Test Video      �     �      �����������������������

�       Buffer   5*   �     �

�����������������������     �

�                 �

�                 �

���������������������Ŀ     �

�     Sense Monitor   �     �

�        Type    6*   �     �

�����������������������     �

�                 �

�                 �

�������������������

Note 1:

Wake up VGA chip by programming port 46E8 hex bit 4 to 1.

Program port 102 hex bit 0 to 1.

Program port 46E8 hex bit 3 to 0 (disable the accessing of VGA

memory and I/O) for the sake of testing other display cards' presence.

Note 2:

By writing 4 words to VRAM (B000h & B800h)

read back

If VRAM R/W O.K then set the correct equip_flag (40:10) value.

Note 3:

Program port 46E8 hex bit 3 to 1 (enable the accessing of VGA memory

and I/O).

Note 4:

INT 10h  : service routine entry

INT 1Fh  : 2nd 128 8x8 ascii code

INT 43h  : 1st 128 8x8 ascii code

INT 1Dh  : point to the CGA compatible table

INT 6dh  : keep the same value as INT 10h vector

INT 42h  : IBM (CGA,MDA,MGA) original INT 10 vector (F000:F065)

Note 5:

By writing pattern to VRAM then read back.

If error occurs, sound 1 long 2 short error beep.

Note 6:

R G B -- 12h,12h,12h  If port 3C2h bit 4 on, color monitor

R G B --   4,12h,4    If port 3C2h bit 4 on, mono monitor

else, no monitor attached.

Note 7:

Set corresponding values to BIOS data area.

Note 8:

Set CRT mode according to the current flags setting.

Note 9:

Read input status register (3DA/3BA), if error occures, sound

1 long 3 short error beep.