*THE COMPENDIUM*

published with the latest

Unlimited Access Dialer

Version 3.0 by:

Czisar

Agent Orange

Dan

CCITT: -4-

HANGUP:   2040 & 2040 HZ

FREQ1 (2040 HZ): 0

FREQ2 (2400 HZ): 1

PREFIX:        80 MS +/- 20 MS

SHORT SIGNAL:  40 MS +/- 10 MS

LONG- SIGNAL: 200 MS +/- 40 MS

Prefix Signal:

P  150ms +/- 30ms

(0 and 1 Compound!)

Control Signal:

X  100ms +/- 20ms (0 short single signal!)

Y  100ms +/- 20ms (1 short single signal!)

XX 350ms +/- 70ms (0 long single signal!)

YY 350ms +/- 70ms (1 long single signal!)

________   _________   ______________

/ Tone:  \ / -Pulse- \ /Short Comment:\

\________/ \_________/ \______________/

/  -1-   \ / 1-1-1-0 \ /  Digit #-1-  \

|   -2-    |  1-1-0-1  |   Digit #-2-   |

|   -3-    |  1-1-0-0  |   Digit #-3-   |

|   -4-    |  1-0-1-1  |   Digit #-4-   |

|   -5-    |  1-0-1-0  |   Digit #-5-   |

|   -6-    |  1-0-0-1  |   Digit #-6-   |

|   -7-    |  1-0-0-0  |   Digit #-7-   |

|   -8-    |  0-1-1-1  |   Digit #-8-   |

|   -9-    |  0-1-1-0  |   Digit #-9-   |

|   -0-    |  0-1-0-1  |   Digit #-0-   |

| Code 11  |  0-1-0-0  |  Call Operator |

| Code 12  |  0-0-1-1  |  Call Operator |

| SP.C. I  |  0-0-1-0  |  Space Code    |

|  IHES    |  0-0-0-1  |  Half Echo Sup |

| SP.C. II |  1-1-1-1  |  Space Code    |

|  E.O.P.  |  0-0-0-0  |  End of Pulsing|

|   KP1    |    PX     |  Local Call    |

|   KP2    |    PY     |  Global Call   |

| ClrFwd   |    PXX    |  Clear Forward |

| XFer     |    PYY    |  X-Fer Forward |

\________/ \_________/ \______________/

-CCITT #5-

___________   ____________   _____________   ____________   ____________

/   Tone:   \ /   Freq1:   \ /    Freq2:   \ /   Length:  \ /   Delay:   \

\___________/ \____________/ \_____________/ \____________/ \____________/

/    -1-    \ /    700Hz   \ /     900Hz   \ /     55ms   \ /     55ms   \

|     -2-     |     700Hz    |     1100Hz    |      55ms    |      55ms    |

|     -3-     |     900Hz    |     1100Hz    |      55ms    |      55ms    |

|     -4-     |     700Hz    |     1300Hz    |      55ms    |      55ms    |

|     -5-     |     900Hz    |     1300Hz    |      55ms    |      55ms    |

|     -6-     |    1100Hz    |     1300Hz    |      55ms    |      55ms    |

|     -7-     |     700Hz    |     1500Hz    |      55ms    |      55ms    |

|     -8-     |     900Hz    |     1500Hz    |      55ms    |      55ms    |

|     -9-     |    1100Hz    |     1500Hz    |      55ms    |      55ms    |

|     -0-     |    1300Hz    |     1500Hz    |      55ms    |      55ms    |

|     C11     |     700Hz    |     1700Hz    |     100ms    |      55ms    |

|     C12     |     900Hz    |     1700Hz    |     100ms    |      55ms    |

|     KP1     |    1100Hz    |     1700Hz    |     100ms    |      55ms    |

|     KP2     |    1300Hz    |     1700Hz    |     100ms    |      55ms    |

|      ST     |    1500Hz    |     1700Hz    |     100ms    |      55ms    |

\____________|______________|_______________|______________|_____________/

/            |              |               |              |             \

|   SERV_O    |     135Hz    |      135Hz    |     120ms    |      55ms    |

|    C_INT    |     440Hz    |      440Hz    |     120ms    |      55ms    |

|   L_TONE    |     480Hz    |      620Hz    |     120ms    |      55ms    |

|   H_TONE    |     480Hz    |      480Hz    |     120ms    |      55ms    |

|   R_BACK    |     440Hz    |      480Hz    |     120ms    |      55ms    |

|   M_WATT    |    1004Hz    |     1004Hz    |     120ms    |      55ms    |

|   D_TONE    |     350Hz    |      440Hz    |     120ms    |      55ms    |

|     TASI    |    1850Hz    |     1850Hz    |     120ms    |      55ms    |

|     CCIS    |    2010Hz    |     2010Hz    |     120ms    |      55ms    |

|   Q-TONE    |    1400Hz    |     1400Hz    |     120ms    |      55ms    |

|   ClrFwd    |    2600Hz    |     2400Hz    |     120ms    |      55ms    |

|       EO    |    2100Hz    |     2100Hz    |     500ms    |      55ms    |

|    Seize    |    2400Hz    |     2400Hz    |     200ms    |     500ms    |

\___________/ \____________/ \_____________/ \____________/ \____________/

-CCITT R1-

Just as CCITT #5, but ClearForward is 2600/2600.

-CCITT R2-

___________   ____________   _____________   ____________   ____________

/   Tone:   \ /   Freq1:   \ /   Freq2:    \ /   Length:  \ /   Delay:   \

\___________/ \____________/ \_____________/ \____________/ \____________/

/    -1-    \ /   1380Hz   \ /   1500Hz    \ /    55ms    \ /    55ms    \

|     -2-     |    1380Hz    |    1620Hz     |     55ms     |     55ms     |

|     -3-     |    1500Hz    |    1620Hz     |     55ms     |     55ms     |

|     -4-     |    1380Hz    |    1740Hz     |     55ms     |     55ms     |

|     -5-     |    1980Hz    |    1740Hz     |     55ms     |     55ms     |

|     -6-     |    1620Hz    |    1740Hz     |     55ms     |     55ms     |

|     -7-     |    1380Hz    |    1860Hz     |     55ms     |     55ms     |

|     -8-     |    1500Hz    |    1860Hz     |     55ms     |     55ms     |

|     -9-     |    1620Hz    |    1860Hz     |     55ms     |     55ms     |

|     -0-     |    1740Hz    |    1860Hz     |     55ms     |     55ms     |

|     -*-     |    1620Hz    |    1980Hz     |    100ms     |     55ms     |

|     -#-     |    1740Hz    |    1980Hz     |    100ms     |     55ms     |

|     KP2     |    1500Hz    |    1980Hz     |    100ms     |     55ms     |

|     KP2E    |    1380Hz    |    1980Hz     |    100ms     |     55ms     |

|     ST      |    1860Hz    |    1980Hz     |    100ms     |     55ms     |

|   HangUp    |    3825Hz    |    3825Hz     |    640ms     |     55ms     |

\___________/ \____________/ \_____________/ \____________/ \____________/

-CCITT SOCOTEL-

___________   ____________   _____________   ____________   ____________

/   Tone:   \ /   Freq1:   \ /    Freq2:   \ /   Length:  \ /   Delay:   \

\___________/ \____________/ \_____________/ \____________/ \____________/

/    -1-    \ /    700Hz   \ /     900Hz   \ /     55ms   \ /     55ms   \

|     -2-     |     700Hz    |     1100Hz    |      55ms    |      55ms    |

|     -3-     |     900Hz    |     1100Hz    |      55ms    |      55ms    |

|     -4-     |     700Hz    |     1300Hz    |      55ms    |      55ms    |

|     -5-     |     900Hz    |     1300Hz    |      55ms    |      55ms    |

|     -6-     |    1100Hz    |     1300Hz    |      55ms    |      55ms    |

|     -7-     |     700Hz    |     1500Hz    |      55ms    |      55ms    |

|     -8-     |     900Hz    |     1500Hz    |      55ms    |      55ms    |

|     -9-     |    1100Hz    |     1500Hz    |      55ms    |      55ms    |

|     -0-     |    1300Hz    |     1500Hz    |      55ms    |      55ms    |

|     C11     |     700Hz    |     1700Hz    |     100ms    |      55ms    |

|     C12     |     900Hz    |     1700Hz    |     100ms    |      55ms    |

|     KP1     |    1100Hz    |     1700Hz    |     100ms    |      55ms    |

|     KP2     |    1300Hz    |     1700Hz    |     100ms    |      55ms    |

|      ST     |    1500Hz    |     1700Hz    |     100ms    |      55ms    |

|  HangUp     |    3850Hz    |     3850Hz    |      35ms    |              |

\___________/ \____________/ \_____________/ \____________/ \____________/

-DTMF-

___________   ____________   _____________   ____________   ____________

/   Tone:   \ /   Freq1:   \ /   Freq2:    \ /   Length:  \ /   Delay:   \

\___________/ \____________/ \_____________/ \____________/ \____________/

/    -1-    \ /    697Hz   \ /    1209Hz   \ /     55ms   \ /     55ms   \

|     -2-     |     697Hz    |     1336Hz    |      55ms    |      55ms    |

|     -3-     |     697Hz    |     1477Hz    |      55ms    |      55ms    |

|     -4-     |     770Hz    |     1209Hz    |      55ms    |      55ms    |

|     -5-     |     770Hz    |     1336Hz    |      55ms    |      55ms    |

|     -6-     |     770Hz    |     1477Hz    |      55ms    |      55ms    |

|     -7-     |     852Hz    |     1209Hz    |      55ms    |      55ms    |

|     -8-     |     852Hz    |     1336Hz    |      55ms    |      55ms    |

|     -9-     |     852Hz    |     1477Hz    |      55ms    |      55ms    |

|     -0-     |     941Hz    |     1336Hz    |      55ms    |      55ms    |

|     -A-     |     697Hz    |     1633Hz    |     120ms    |      55ms    |

|     -B-     |     770Hz    |     1633Hz    |     120ms    |      55ms    |

|     -C-     |     852Hz    |     1633Hz    |     120ms    |      55ms    |

|     -D-     |     941Hz    |     1633Hz    |     120ms    |      55ms    |

|     -*-     |     941Hz    |     1209Hz    |     120ms    |      55ms    |

|     -#-     |     941Hz    |     1477Hz    |     120ms    |      55ms    |

\___________/ \____________/ \_____________/ \____________/ \____________/

Taken from: Better Homes and Blueboxing Part I, by Mark Tabas.

The Little Operator V0.9�&V1.0(PC), By Urmel/ZZC.

WhiteBox Final, By Quasar/Liberty.

Unlimited Axxess v3.0 by Czisar and Agent Orange.

This text was written as there were many wrong lists around

so here comes my one, should be working just fine...

Greetings:      Scrap      Ragman     Enemy

Guru Josh  Pellhuhn   Agent Orange

ViNix      Zycro      Blackstar