Ok rich!Right,you can use it with modem or normal voice.

Voice method:Run the amigabasic program "ac9" and get the speakers

hooked up.Type in the number you want the phreak to dial

without a 0 on the front.Dial and 0800 number and when you

get a ringing or engaged tone press return and put the phone

mouthpiece in front of the speaker.This sends the 2280mhz

tone down and clears the line.Then the line goes

dead,leaving the phone next to the speakers,press return

again,this will now take control of the bt system and dial

the number for you.

e.g. If you wanted to phone MAX 5 on 090558030 you would

type in 90558030 at the "enter number:"prompt and not press

return.Then dial say,0800800800 and when you hear ringing or

engaged tone hold phone mouthpiece(mic)next to speaker and

press return,when you got a dead line youd press return

again and it would dial MAX 5 for you.

Using with a modem is slightly more difficult.Use a small

comms program like a-term or such and have amigabasic

running at the same time as well as a cli(easiest way is

control d on startup sequence)in shrunk windows dotted

around the screen.RUN both programs not just type in name.Do

the same as voice phreak and type ato(if youve got a hayes

modem)in the open comms prog window.When you have phreaked

the number reclick in the comms prog window and when you get

a carrier tone press return.

Thats It!

For : Rich Energy by Gary Parslow.

Docs for amiga phone phreaker.