THE ELECTRONIC SERIAL NUMBER: A CELLULAR 'SIEVE'?

'SPOOFERS' CAN DEFRAUD USERS AND CARRIERS

by Geoffrey S. Goodfellow, Robert N. Jesse, and Andrew H. Lamothe, Jr.

What's the greatest security problem with cellular phones? Is it privacy of

communications?  No.

Although privacy is a concern, it will pale beside an even greater problem:

spoofing.

'Spoofing' is the process through which an agent (the 'spoofer') pretends to

be somebody he isn't by proffering false identification, usually with intent

to defraud.  This deception, which cannot be protected against using the

current U.S. cellular standards, has the potential to create a serious

problem--unless the industry takes steps to correct some loopholes in the

present cellular standards.

Compared to spoofing, the common security concern of privacy is not so severe.

Most cellular subscribers would, at worst, be irked by having their

conversational privacy violated.  A smaller number of users might actually

suffer business or personal harm if their confidential exchanges were

compromised.  For them, voice encryption equipment is becoming increasingly

available if they are willing to pay the price for it.

Thus, even though technology is available now to prevent an interloper from

overhearing sensitive conversations, cellular systems cannot--at any

cost--prevent pirates from charging calls to any account. This predicament is

not new to the industry.  Even though cellular provides a modern,

sophisticated quality mobile communications service, it is not fundamentally

much safer than older forms of mobile telephony.

History of Spoofing Vulnerability

The earliest form of mobile telephony, unsquelched manual Mobile Telephone

Service (MTS), was vulnerable to interception and eavesdropping.  To place a

call, the user listened for a free channel.  When he found one, he would key

his microphone to ask for service: 'Operator, this is Mobile 1234; may I

please have 555-7890.'  The operator knew to submit a billing ticket for

account number 1234 to pay for the call.  So did anybody else listening to the

channel--hence the potential for spoofing and fraud.

Squelched channel MTS hid the problem only slightly because users ordinarily

didn't overhear channels being used by other parties.  Fraud was still easy

for those who turned off the squelch long enough to overhear account numbers.

Direct-dial mobile telephone services such as Improved Mobile Telephone

Service (IMTS) obscured the problem a bit more because subscriber

identification was made automatically rather than by spoken exchange between

caller and operator.  Each time a user originated a call, the mobile telephone

transmitted its identification number to the serving base station using some

form of Audio Frequency Shift Keying (AFSK), which was not so easy for

eavesdroppers to understand.

Committing fraud under IMTS required modification of the mobile--restrapping

of jumpers in the radio unit, or operating magic keyboard combinations in

later units--to reprogram the unit to transmit an unauthorized identification

number. Some mobile control heads even had convenient thumb wheel switches

installed on them to facilitate easy and frequent ANI (Automatic Number

Identification) changes.

Cellular Evolution

Cellular has evolved considerably from these previous systems.  Signaling

between mobile and base stations uses high-speed digital techniques and

involves many different types of digital messages.  As before, the cellular

phone contains its own Mobile Identification Number (MIN), which is programmed

by the seller or service shop and can be changed when, for example, the phones

sold to a new user.  In addition, the U.S. cellular standard incorporates a

second number, the 'Electronic Serial Number' (ESN), which is intended to

uniquely and permanently identify the mobile unit.

According to the Electronic Industries Association (EIA) Interim Standard

IS-3-B, Cellular System Mobile Station--Land Station Compatibility

Specification (July 1984), 'The serial number is a 32-bit binary number that

uniquely identifies a mobile station to any cellular system.  It must be

factory-set and not readily alterable in the field.  The circuitry that

provides the serial number must be isolated from fraudulent contact and

tampering.  Attempts to change the serial number circuitry should render the

mobile station inoperative.'

The ESN was intended to solve two problems the industry observed with its

older systems.

First, the number of subscribers that older systems could support fell far

short of the demand in some areas, leading groups of users to share a single

mobile number (fraudulently) by setting several phones to send the same

identification.  Carriers lost individual user accountability and their means

of predicting and controlling traffic on their systems.

Second, systems had no way of automatically detecting use of stolen equipment

because thieves could easily change the transmitted identification.

In theory, the required properties of the ESN allow cellular systems to check

to ensure that only the correctly registered unit uses a particular MIN, and

the ESNs of stolen units can be permanently denied service ('hot-listed').

This measure is an improvement over the older systems, but vulnerabilities

remain.

Ease of ESN Tampering

Although the concept of the unalterable ESN is laudable in theory, weaknesses

are apparent in practice.  Many cellular phones are not constructed so that

'attempts to change the serial number circuitry renders the mobile station

inoperative.'  We have personally witnessed the trivial swapping of one ESN

chip for another in a unit that functioned flawlessly after the switch was

made.

Where can ESN chips be obtained to perform such a swap?  We know of one recent

case in the Washington, D.C. area in which an ESN was 'bought' from a local

service shop employee in exchange for one-half gram of cocaine.  Making the

matter simpler, most manufacturers are using industry standard Read-Only

Memory (ROM) chips for their ESNs, which are easily bought and programmed or

copied.

Similarly, in the spirit of research, a west coast cellular carrier copied the

ESN from one manufacturer's unit to another one of the same type and

model--thus creating two units with the exact same identity.

The ESN Bulletin Board

For many phones, ESN chips are easy to obtain, program, and install.  How does

a potential bootlegger know which numbers to use?  Remember that to obtain

service from a system, a cellular unit must transmit a valid MIN (telephone

number) and (usually) the corresponding serial number stored in the cellular

switch's database.

With the right equipment, the ESN/MIN pair can be read right off the air

because the mobile transmits it each time it originates a call.  Service shops

can capture this information using test gear that automatically receives and

decodes the reverse, or mobile-to-base, channels.

Service shops keep ESN/MIN records on file for units they have sold or

serviced, and the carriers also have these data on all of their subscribers.

Unscrupulous employees could compromise the security of their customers'

telephones.

In many ways, we predict that 'trade' in compromised ESN/MIN pairs will

resemble what currently transpires in the long distance telephone business

with AT&T credit card numbers and alternate long-distance carrier (such as

MCI, Sprint and Alltel) account codes.  Code numbers are swapped among

friends, published on computer 'bulletin boards' and trafficked by career

criminal enterprises.

Users whose accounts are being defrauded might--or might not--eventually

notice higher-than-expected bills and be reassigned new numbers when they

complain to the carrier.  Just as in the long distance business, however, this

number 'turnover' (deactivation) won't happen quickly enough to make abuse

unprofitable.  Catching pirates in the act will be even tougher than it is in

the wireline telephone industry because of the inherent mobility of mobile

radio.

Automating Fraud

Computer hobbyists and electronics enthusiasts are clever people.  Why should

a cellular service thief 'burn ROMs' and muck with hardware just to install

new IDs in his radio?  No Herculean technology is required to 'hack' a phone

to allow ESN/MIN programming from a keyboard, much like the IMTS phone thumb

wheel switches described above.

Those not so technically inclined may be able to turn to mail-order

entrepreneurs who will offer modification kits for cellular fraud, much as

some now sell telephone toll fraud equipment and pay-TV decoders.

At least one manufacturer is already offering units with keyboard-programmable

MINs.  While intended only for the convenience of dealers and service shops,

and thus not described in customer documentation, knowledgeable and/or

determined end users will likely learn the incantations required to operate

the feature.  Of course this does not permit ESN modification, but easy MIN

reprogrammability alone creates a tremendous liability in today's roaming

environment.

The Rolls Royce of this iniquitous pastime might be a 'Cellular Cache-Box.' It

would monitor reverse setup channels and snarf ESN/MIN pairs off the air,

keeping a list in memory.  Its owner could place calls as on any other

cellphone.  The Cache-Box would automatically select an ESN/MIN pair from its

catalog, use it once and then discard it, thus distributing its fraud over

many accounts.  Neither customer nor service provider is likely to detect the

abuse, much less catch the perpetrator.

As the history of the computer industry shows, it is not far-fetched to

predict explosive growth in telecommunications and cellular that will bring

equipment prices within reach of many experimenters.  Already we have seen the

appearance of first-generation cellular phones on the used market, and new

units can be purchased for well under $1000 in many markets.

How High The Loss?

Subscribers who incur fraudulent charges on their bills certainly can't be

expected to pay them.  How much will fraud cost the carrier?  If the charge is

for home-system airtime only, the marginal cost to the carrier of providing

that service is not as high as if toll charges are involved.  In the case of

toll charges, the carrier suffers a direct cash loss.  The situation is at its

worst when the spoofer pretends to be a roaming user.  Most inter-carrier

roaming agreements to date make the user's home carrier (real or spoofed)

responsible for charges, who would then be out hard cash for toll and airtime

charges.

We have not attempted to predict the dollar losses this chicanery might

generate because there isn't enough factual information information for anyone

to guess responsibly.  Examination of current estimates of long-distance-toll

fraud should convince the skeptic.

Solutions

The problems we have described are basically of two types.  First, the ESN

circuitry in most current mobiles is not tamper-resistant, much less

tamper-proof.  Second and more importantly, the determined perpetrator has

complete access to all information necessary for spoofing by listening to the

radio emissions from valid mobiles because the identification information

(ESN/MIN) is not encrypted and remains the same with each transmission.

Manufacturers can mitigate the first problem by constructing mobiles that more

realistically conform to the EIA requirements quoted above.  The second

problem is not beyond solution with current technology, either.  Well-known

encryption techniques would allow mobiles to identify themselves to the

serving cellular system without transmitting the same digital bit stream each

time.  Under this arrangement, an interloper receiving one transmission could

not just retransmit the same pattern and have it work a second time.

An ancillary benefit of encryption is that it would reasonably protect

communications intelligence--the digital portion of each transaction that

identifies who is calling whom when.

The drawback to any such solution is that it requires some re-engineering in

the Mobile-Land Station Compatibility Specification, and thus new software or

hardware for both mobiles and base stations.  The complex logistics of

establishing a new standard, implementing it, and retrofitting as much of the

current hardware as possible certainly presents a tough obstacle, complicated

by the need to continue supporting the non-encrypted protocol during a

transition period, possibly forever.

The necessity of solving the problem will, however, become apparent.  While we

presently know of no documented cases of cellular fraud, the vulnerability of

the current standards and experience with similar technologies lead us to

conclude that it is inevitable.  Failure to take decisive steps promptly will

expose the industry to a far more expensive dilemma.  XXX

Geoffrey S. Goodfellow is a member of the senior research staff in the

Computer Science Laboratory at SRI International, 333 Ravenswood Ave., Menlo

Park, CA 94025, 415/859-3098.  He is a specialist in computer security and

networking technology and is an active participant in cellular industry

standardization activities.  He has provided Congressional testimony on

telecommunications security and privacy issues and has co-authored a book on

the computer 'hacking' culture.

Robert N. Jesse (2221 Saint Paul St., Baltimore, MD 21218, 301/243-8133) is an

independent consultant with expertise in security and privacy, computer

operating systems, telecommunications and technology management.  He is an

active participant in cellular standardization efforts.  He was previously a

member of the senior staff at The Johns Hopkins University, after he obtained

his BES/EE from Johns Hopkins.

Andrew H. Lamothe, Jr. is executive vice-president of engineering at Cellular

Radio Corporation, 8619 Westwood Center Dr., Vienna, VA 22180, 703/893-2680.

He has played a leading role internationally in cellular technology

development.  He was with Motorola for 10 years prior to joining American

TeleServices, where he designed and engineered the Baltimore/Washington market

trial system now operated by Cellular One.

--------

A later note indicates that one carrier may be losing something like $180K

per month....

The Wanderjahr

Distributed in part by:

Skeleton Crue  415-376-8060  located out of Moraga, California.

!!Get on the band wagon befor it RUNS YOU DOWN!!

Headquarters for Computer Hackers and Anarchists to Overthrow the State

(CH&AOS)