----------------------------

- Short-Wave Radio Hacking -

----------------------------

by

The Seker

Every day, tons of information is exchanged over the air waves.  I have

found news agencies, military computers, businesses, and even hacks.

The standard method of exchange is called RTTY (Radio Teletype).  It

usually is used at 66/7 words per min.  Instead of using ASCII, Baudot, a 5

bit character set is more widely used.  There are many variations of it in

use also.

There are many other types of transmission standards besides RTTY that are

commonly used.  A few of the known:

FAX (Facsimile)

Helshcrieber- it's used to transmit pictogram-type alphabets (i.e.

Chinese, Jap, etc.) instead of the American letters.

SSTV- is similiar to Viewdata.  Used for transmitting high-resolution

pictures mixed with text.

To start, you'll need to buy (card) a receiver (with a coverage of no less

than 500 kHz-30 MHz and a resolution greater than 100 Hz) and a high quality

antenna.  These can usually be found at electronics stores.  You will also need

to get an interface and some RTTY software for your particuliar computer.  Look

in magazines like 'Amatuer Radio' or 'Ham Radio Today' for more information on

that shit.  Another good place to check is a CB store.

NEWS AGENCIES-

From these you can find all sorts of crap.  You may even intercept a story

being sent to the presses.  They tend to operate at 66/7 words a minute (50

baud).  A few of the more common 'fixed' bands they transmit over are:

at kHz:

3155-3400      3950-4063

9040-9500      12050-12330

13800-14000     15600-16360

19800-19990     25210-25550

An easy way to tell if you have located a news agency is by some lame

transmission being continuosly repeated.

i.e. 'RYRYRYRYRYRYRYRYRYRYRY' etc.

This is done so they can keep their channels opened for reception.

CONFERENCES-

Another thing I found interesting was the channels that the amatuers

congregated around.  I frequently ran into people from foreign countries that

couldn't even speak English.  I even ran into other hackers from all over!

A few of the more popular spots that amatuers hang out are:

at kHz:

3590  14090  21090  28090

at MHz:

432.600  433.300

at VHF/UHF:

144.600  145.300

PACKET RADIO-

A new development in radio transmission is the packet radio.  From what

I've seen, it's just like digital packet switching networks, i.e. Compuserve,

Telenet, Tymnet, etc.; except slower.

In fact, Compuserve has been researching a way to transmit its services

cheaply.

The Wanderjahr

Distributed in part by:

Skeleton Crue  415-376-8060  located out of Moraga, California.

!!Get on the band wagon befor it RUNS YOU DOWN!!

Headquarters for Computer Hackers and Anarchists to Overthrow the State

(CH&AOS)