--------------------------------------------------------------------

BELLCORE Information          by The Mad Phone-man

--------------------------------------------------------------------

So, you've broken into the big phone box on the wall, and are looking at a

bunch of tags with numbers and letters on them.  Which one is the modem

line?  Which one is the 1-800 WATTS line?  Which one is the Alarm Line?

Bell has a specific set of codes that enable you to identify what you're

looking at.  These are the same codes the installer gets from the wire

center to enable him to setup the line, test it, and make sure it matches

the customer's order.  Here are some extracts from the Bellcore book.

First let's take a Hypothetical Line number I'm familiar with:

64FDDV 123456

--------------------------------------------------------------

The serial number format:

Prefix +  service code + modifier +   serial number +

digits:  1,2         3,4           5,6       7,8,9,10,11,12    continued

-------------------------------------------------------------------------

Suffix    +   CO assigning circut number +   segmet

digits:  13,14,15            16,17,18,19              20,21,22

-------------------------------------------------------------------------

The Important shit is in the 3d thru 6th digit.

SERVICE CODES    Intra or Inter LATA Block 1-26

-----------------

AA- Packet Analog Access line

AB- Packet switch trunk

AD- Attendant

AF- Commercial audio fulltime

AI- Automatic Identified outward dialing

AL- Alternate services

AM- Packet, off-network access line

AN- Announcement service

AO- International/overseas audio (full time)

AP- Commercial audio (part time)

AT- International/overseas audio (part time)

AU- Autoscript

BA- Protective alarm (CD)

BL- Bell & Lights

BS- Siren control

CA- SSN Access

CB- OCC Audio facilitys

CC- OCC Digital facility-medium speed

CE- SSN Station Line

CF- OCC Special facility

CG- OCC Telegraph facility

CH- OCC Digital facility High-speed

CI- Concentrator Identifier trunk

CJ- OCC Control facility

CK- OCC Overseas connecting facility wide-band

CL- Centrex CO line

CM- OCC Video facility

CN- SSN Network trunk

CO- OCC Overseas connecting facility

CP- Concentrator identifier signaling link

CR- OCC Backup facility

CS- Channel service

CT- SSN Tie trunk

CV- OCC Voice grade facility

CW- OCC Wire pair facility

CZ- OCC Access facility

DA- Digital data off-net extention

DB- HSSDS 1.5 mb/s access line

DF- HSSDS 1.5 mb/s hub to hub

DG- HSSDS 1.5 mb/s hub to earth station

DH- Digital service

DI- Direct-in dial

DJ- Digit trunk

DK- Data Link

DL- Dictation line

DO- Direct-out dial

DP- Digital data-2 4 kb/s

DQ- Digital data-4 8 kb/s

DR- Digital data-9.6 kb/s

DW- Digital data-56  kb/s

DY- Digital service (under 1 mb/s)

EA- Switched acess

EB- ENFIA II end office trunk

EC- ENFIA II tandem trunk

EE- Combined access

EF- Entrance facility-voice grade

EG- Type #2 Telegraph

EL- Emergency reporting line

EM- Emergency reporting center trunk

EN- Exchange network acess facility

EP- Entrance facility-program grade

EQ- Equipment only-(network only) assignment

ES- Extension service-voice grade

ET- Entrance facility-telegraph grade

EU- Extension service-telegrasph grade

EV- Enhanced Emergency reporting trunk

EW- Off network MTS/WATS equivalent service

FD- Private line-data

FG- Group-supergroup spectrum

FR- Fire dispatch

FT- Foreign exchange trunk

FW- Wideband channel

FV- Voice grade facility

FX- Foreign exchange

HP- Non-DDS Digital data 2.4 kb/s

HQ- Non-DDS Digital data 4.8 kb/s

HR- Non-DDs Digital data 9.6 kb/s

HW- Non-DDS Digital data 56  kb/s

IT- Intertandem tie trunk

LA- Local area data channel

LL- Long distance terminal line

LS- Local service

LT- Long distance terminal trunk

MA- Cellular access trunk 2-way

MT- Wired music

NA- CSACC Link (EPSCS)

NC- CNCC link (EPSCS)

ND- Network data line

OI- Off premises intercommunication station line

ON- Off network access line

OP- Off premises extension

OS- Off premises PBX station line

PA- Protective alarm (AC)

PC- Switched digital-access line

PG- paging

PL- Private line-voice

PM- Protective monitoring

PR- Protective relaying-voice grade

PS- MSC constructed spare facility

PV- Protective relaying-telegraph grade

PW- Protective relaying-signal grade

PX- PBX station line

PZ- MSC constructed circut

QU- Packet asyncronous access line

QS- Packet synchronous access line

RA- Remote attendant

RT- Radio landline

SA- sattelite trunk

SG- control/remote metering signal grade

SL- secretarial line

SM- sampling

SN- Special access termination

SQ- equipment only-customer premises

SS- dataphone select-a-station

TA- Tandem tie-trunk

TC- Control/remote metering-telegraph grade

TF- Telephoto/facsimile

TK- Local PBX trunk

TL- Non-tandem tie trunk

TR- turret or automatic call distributor (ACD) trunk

TT- Teletypewriter channel

TU- Turret or automatic call distributor (ACD) line

TX- Dedicated facility

VF- Commercial television (full time)

VH- Commercial television (part time)

VM- Control/remote metering-voice grade

VO- International overseas television

VR- Non-commercial Television (7003,7004)

WC- Special 800 surface trunk

WD- Special WATS trunk (OUT)

WI- 800 surface trunk

WO- WATS line (OUT)

WS- WATS trunk (OUT)

WX- 800 service line

WY- WATS trunk (2-way)

WZ- WATS line (2-way)

ZA- Alarm circuts

ZC- Call and talk circuts

ZE- Emergency patching circuts

ZF- Order circuts, facility

ZM- Measurement and recording circuts

ZP- Test circut, Plant service center

ZQ- Quality and management circuts

ZS- Switching, control and transfer circuts

ZT- Test circuts, central office

ZV- Order circuts, service

SERVICE CODES FOR LATA ACCESS

---------------------------------------------------

HC- High capacity 1.544 mb/ps

HD- High capacity 3.152 mb/ps

HE- High capacity 6.312 mb/ps

HF- High capacity 6.312

HG- High capacity 274.176 mb/s

HS- High capacity subrate

LB- Voice-non switched line

LC- Voice-switched line

LD- Voice Switched trunk

LE- Voice and tone-radio landline

LF- Data low-speed

LG- Basic data

LH- Voice and data-PSN access trunk

LJ- Voice and data SSN access

LK- Voice and data-SSN-intermachine trunk

LN- Data extension, voice grade data facility

LP- Telephoto/facsimile

LQ- Voice grade customized

LR- Protection relay-voice grade

LZ- dedicated facility

MQ- Metalic customized

NQ- Telegraph customized

NT- Protection alarm-metalic

NU- Protection alarm

NV- Protective relaying/telegraph grade

NW- Telegraph grade facility-75 baud

NY- Telegraph grade facility- 150 baud

PE- Program audio, 200-3500 hz

PF- Program audio, 100-5000 hz

PJ- Program audio, 50-8000 hz

PK- Program audio, 50-15000 hz

PQ- Program grade customized

SB- Switched access-standard

SD- Switched access-improved

SE- Special access WATS-access-std

SF- Special access- WATS access line improved

SJ- Limited switched access line

TQ- Television grade customized

TV- TV Channel one way 15khz audio

TW- TV Channel one way 5khz audio

WB- Wideband digital, 19.2 kb/s

WE- Wideband digital, 50 kb/s

WF- Wideband digital, 230.4 kb/s

WH- Wideband digital, 56 kb/s

WJ- Wideband analog, 60-108 khz

WL- Wideband analog 312-552 khz

WN- Wideband analog 10hz-20 khz

WP- Wideband analog, 29-44 khz

WR- Wideband analog 564-3064 khz

XA- Dedicated digital, 2.4 kb/s

XB- Dedicated digital, 4.8 kb/s

XG- Dedicated digital, 9.6 kb/s

XH- Dedecated digital 56. kb/s

Now the last two positions of real importance, 5 & 6 translate thusly:

Modifier Character Position 5

-----------------------------

INTRASTARE            INTERSTATE

-------------------------------------

A                      B           Alternate data & non data

-------------------------------------

C                                  Customer controlled Service

-------------------------------------

D                      E           Data

-------------------------------------

N                      L           Non-data operation

-------------------------------------

P                                  Only offered under intra Restructured

private line (RPL) tariff

-------------------------------------

S                      T           Simultanious data & non-data

-------------------------------------

F                                  Interexchange Carriers is less than 50%

-------------------------------------

G           Interstate carrier is more than 50% usage

===============================================================================

MODIFIER CHARACTER POSITION 6

--------------------------------------------------------------

TYPE OF SERVICE        Intra LATA

--------------------------------------

ALL EXCEPT US GOVT       US GOVERNMT

--------------------------------------

T                   M          Circut is BOC customer to BOC customer

all facilitys are TELCO provided

--------------------------------------

C                   P          Circut is BOC/BOC and part of facilitys

or equipment is telco provided

--------------------------------------

A                   J          Circut is BOC/BOC all electricaly

connected equip is customer provided

--------------------------------------

L                   F          Circut terminates at Interexchange

carrier customers location

--------------------------------------

Z                              Offical company service

--------------------------------------

Interlata

S                   S          Circut terminates at Interexchange

carriers point of term (POT)

--------------------------------------

V                   V          Circut terminates at an interface of a

Radio common carrier (RCC)

--------------------------------------

Z                              Official company service

--------------------------------------

Corridor

Y                   X          Corridor circut

--------------------------------------

International

K                   H          Circut has at least 2 terminations in

different countries

--------------------------------------

interexchange carrier

Y                   X          Transport circut between Interexchange

carrier terminals.

--------------------------------------

So 64FDDV would be a private line data circut terminating at a radio common

carrier.  Other examples can be decoded likewise.

Enjoy this information as much as I've had finding it.

-= The Mad Phone-man =-

The Wanderjahr

Distributed in part by:

Skeleton Crue  415-376-8060  located out of Moraga, California.

!!Get on the band wagon befor it RUNS YOU DOWN!!

Headquarters for Computer Hackers and Anarchists to Overthrow the State

(CH&AOS)