���۰��۰��۰��۰��۰��۰��۰��۰��۰��۰����۰��۰��۰���

� Monochrome Monitor Emulator for EGA/VGA. Version 1.00. �

�		(C)Copyright Oleg Kibirev, 1992.	    �

��������������������������������������������������������Ĳ

�			  Document file 		    �

��۲��۲��۲��۲��۲��۲��۲��۲��۲��۲��۲��۲��۲��۲��

MONO.COM is a tiny (about 200 byte) TSR that emulates monochrome modes 07 and

0F on an EGA or VGA machine with color monitor. Original IBM EGA BIOS doesn't

support this. Niether do many clones - mine, when put to mode 7 (without MONO

loaded), blanks display & makes very strange sounds. It's interesting that this

is NOT because hardware can't do this - just a flaw of BIOS.

The only major difference between true MDA mode 7 & this emulated by MONO is

that the last supports more attribute combinations - for example you can have

bright foreground on normal intensity background. But, I guess, this shouldn't

be a problem.

You may wonder why would one want to use monochrome modes with color monitor.

Well, for example to test if your program can run in these modes. Also, if a

program uses some odd color scheme, you may be able to get more readable display

by running it in mode 7. Finally, there are still some older programs around

that can run ONLY in monochrome mode ...

VMODE.COM sets a mode to a specified hex. number (say, "VMODE F" to set 640X350

monochrome graphics). Perfect for playing with MONO (by the way, don't use DOS

MODE command to set video mode when MONO is loaded - it'll go wrong).

These programs are freeware. You are encouraged to distribute them either

separately or as a part of your own programs. (of course - just look at their

sizes...)

��������������

���������������������������������Oleg Kibirev۱���������������������������������

�				��������������				       �

� Current address :		       �  E-MAIL : Try both:		       �

� 277 Babcock Street P.O. Box 1869     �  kibirev@BUCSF.BU.EDU	  &	       �

� Boston MA 02215 USA		       �  kibirev@CSA.BU.EDU		       �

� Phone: (617)352-5563		       �				       �

��������������������������������������������������������������������������������

�	 Home address : Flat 42 Ilycha street 7, Novosibirsk 630090, USSR      �

��������������������������������������������������������������������������������