***  COMPUTER MATH CALCULATOR  ***

***      FOR THE HP 95LX       ***

Copyright (c) 1991 by Eugene Dorr

All rights reserved.

CMCALC is distributed in an archive called CMCALC.ZIP.  When "unzipped", you

should have three files:

CMCALC.DOC -- this documentation file

CMCALC.EXM -- executable file

APNAME.LST -- system manager application list

CMCALC is a system-manager compliant application which provides a rich set

of computer-science-oriented integer manipulations.  It features variable

word size (from 1 to 32 bits), selectable complement modes (unsigned, 1's

complement, and 2's complement), four number bases (binary, octal, decimal

and hexadecimal), and visible carry and overflow indicators.  Only RPN

entry is supported.

Because CMCALC is "system-manager compliant" it can be started and used along

with all other system-manager compliant applications, including the built-in

applications.  You can switch between these applications without quitting any

of them, and when you return to an application, you are returned right where

you were when you left.  Of course, this also means that CMCALC will operate

ONLY on HP 95LX computers.

Installing CMCALC

-----------------

Basic installation of CMCALC is easy:

(1) Copy CMCALC.EXM to the root directory on the HP 95's drive C:

(2) Copy APNAME.LST to the _DAT subdirectory on the HP 95's drive C:

(3) Quit all active applications and reboot the HP 95 by pressing

CTRL + ALT + DEL.

(4) Start CMCALC by holding down the ALT key and pressing the blue

HP CALC key.

If you already have an APNAME.LST file on your drive C:, then just use the

built-in MEMO application to add the following line to your existing

APNAME.LST file:

C:CMCALC.EXM,C300,CompMathCalc

If you want to install CMCALC on a RAM card, then the APNAME.LST file must

be copied to the root directory on drive A:.

Using CMCALC

------------

To start CMCALC once it has been installed, hold down the ALT key and press

the blue HP CALC key.  To quit CMCALC, press MENU, Q.  CMCALC uses the HP 95

calculator paradigm, so if you are familiar with using the built-in HP 95

calculator, operation of CMCALC should be straight-forward.

CMCALC uses the RPN mode of data entry.  This documentation assumes that you

are familiar with RPN; if you are not, it is discussed in Appendix D of the

HP 95LX User's Guide.  The CMCALC calculator operates on integers.  If you

need to operate on floating-point numbers, you can use the HP 95 built-in

calculator.

Controlling How CMCALC Works

----------------------------

You can control how CMCALC operates in several ways:  you can specify how many

bits will be used to store numbers (the WORD SIZE -- from 1 to 32 bits); you

can specify what complement mode is to be used (the complement mode affects the

results of arithmetic functions, and the display of numbers in decimal mode),

you can choose the number base you wish numbers to be displayed in (base 2 --

binary, base 8 -- octal, base 10 -- decimal, or base 16 -- hexadecimal); and

whether you want leading zeros to be displayed.  All of these features are

controlled using MENUs.

For each of the following procedures, the operation may be aborted before it

has been completed by pressing ESC once (if at the first level menu) or

twice.

WORDSIZE

To set the wordsize, press MENU W(ordsize).  Type in the number of bits per

word (in decimal) and press ENTER.  You may specify any wordsize between 1

and 32, inclusive.  Except in unsigned mode, the high bit of a word indicates

the sign of a number.

COMPLEMENT MODE

To set the complement mode, press MENU C(omplement).  A secondary menu appears,

which gives you the option of Unsigned mode, 1's complement mode, or 2's

complement mode.  Press U to select unsigned mode, 1 to select 1's complement

mode, or 2 to select 2's complement mode.  In unsigned mode all numbers are

considered to be positive.

BASE

To set the base used for displaying numbers, press MENU B(ase).  A secondary

menu appears, which gives you the option of Binary, Octal, Decimal, or

Hexadecimal.  When numbers are displayed in decimal mode, a minus sign (-)

is used to indicate negative numbers, and leading zeros are never displayed,

even if that option is selected.

LEADING ZEROS

You may prefer to see numbers displayed with leading zeros.  This option can

be toggled on and off by pressing MENU O(ptions) L(eadingZeros).  In decimal

mode, leading zeros are never displayed.

STATUS

To see the current number base, word size, and complement mode, press MENU

S(tatus).  A window will appear which show the current settings.  Press ESC

to remove the window.

ERASE

MENU E(rase) causes a secondary menu to be displayed, which gives you the

option of zeroing the calcline (the X register), the storage registers

(registers 0..9 and A..F), the stack (registers X, Y, Z and T), or the

flags (Carry and Overflow).

Functions

---------

In the following descriptions, upper case X, Y, Z and T refer to the stack

registers, lower case x, y, z and t refer to the values contained in the

corresponding stack registers.  No stack movement occurs unless explicitly

stated.  The Carry and Overflow flags are unaffected unless stated otherwise.

The Carry flag is usually used to indicate the value of a bit in the result

which could not be accomodated in the current size of the register, for

example, a carry during addition, a borrow during subtraction, or a bit

shifted out of a register during a shift function.  The Overflow flag is

used to indicate that the result could not be expressed within the current

register size and complement mode.  For example, if the wordsize is 8 and the

complement mode is 2's complement, then the largest positive number which can

be expressed is 127, and the largest negative number which can be expressed

is -128.  If the CHS (change sign) function is applied to -128, then the

Overflow flag will be set, since +128 cannot be expressed in 8 bits.

Some functions (the ones which refer to bit positions) use the absolute

(positive) value of x.  This is indicated by the notation |x|.

As with the HP 95 built-in calculator, many functions are assigned to F-keys,

which are labelled on the last two lines of the display.  Four groups of

functions are available, and the different groups are accessed by pressing

F10 (labelled 'More').  In the following descriptions, functions which are

assigned to Fkeys carry the notation '(Fkey)'.

ARITHMETIC FUNCTIONS

+        Add -- (+)

x is added to y, result returned in X.  The stack drops.  Carry and

Overflow are set as appropriate for the complement mode.

-        Subtract -- (-)

x is subtracted from y, result returned in X.  The stack drops.  Carry

and Overflow are set as appropriate for the complement mode.

*        Multiply -- (*)

x is multiplied by y, result returned to X, the stack drops.  Overflow

is sometimes set if the result cannot be represented in the current

wordsize and complement mode.  The overflow flag is not a reliable

indication of whether an overflow occured with this function.

/        Divide -- (/)

The quotient of y divided by x, result returned in X.  The stack

drops.  The Carry flag is set if there is a remainder, otherwise it is

cleared.  Overflow is set only in 2's complement mode when the largest

possible negative number is divided by -1.  In all other cases it is

cleared.  The function is inhibited and an error message is displayed

if x is 0 or -0.

ABS      Absolute value -- (Fkey)

If x is negative (this can only be true if the complement mode is not

unsigned) then it is made positive, if possible, the result is

returned to X, and Overflow is reset.  If it is not possible to make x

positive (i.e. the complement mode is 2's complement, and x is the

largest possible negative number), then X remains unchanged and the

Overflow flag is set.

RMD      Remainder -- (Fkey)

y is divided by x, the quotient is discarded, and the remainder is

returned in X.  The stack drops.  The function is inhibited and an

error message is displayed if x is 0 or -0.

CHS      Change Sign -- (assigned to the 'L' key)

Complements x, result returned in X.  Carry and Overflow are set as

appropriate for the complement mode.  The function is inhibited and an

error message is displayed in unsigned mode.

BOOLEAN LOGIC FUNCTIONS

AND      Bitwise AND -- (Fkey)

Bitwise logical AND of x and y, result returned in X.  The stack drops.

OR       Bitwise OR -- (Fkey)

Bitwise logical OR of x and y, result returned in X.  The stack drops.

XOR      Bitwise Exclusive OR -- (Fkey)

Bitwise logical exclusive OR of x and y, result returned in X.  The

stack drops.

NOT      Bitwise NOT -- (Fkey)

Bitwise logical NOT of x, result returned in X.

BIT-MANIPULATION FUNCTIONS

MASKL    Mask left -- (Fkey)

A bit mask of the size specified by |x| is created and placed in the

leftmost positions of X, replacing x.

MASKR    Mask right -- (Fkey)

A bit mask of the size specified by |x| is created and placed in the

rightmost positions of X, replacing x.

SB       Set bit -- (Fkey)

The bit specified by |x| (x=0 specifies the rightmost bit) is set in y,

the result is returned in X.  The stack drops.

CB       Clear bit -- (Fkey)

The bit specified by |x| (x=0 specifies the rightmost bit) is cleared

in y, the result is returned in X.  The stack drops.

INV      Invert bit order -- (Fkey)

The order of the bits in x is reversed, the result is returned in X.

#BIT     Bit count -- (Fkey)

The number of 1-bits in x is returned in X.

SHIFT AND ROTATE FUNCTIONS

SL       Shift left -- (Fkey)

x is shifted to the left by one position, a zero bit is introduced

into the rightmost bit position, and the result is returned in X.

The Carry flag is set to the value of the leftmost bit of x.

+---+     +----------------+

|   | <-- |     <-----     | <-- 0

+---+     +----------------+

Carry

SR       Shift right -- (Fkey)

x is shifted to the right by one position, a zero bit is introduced

into the leftmost bit position, and the result is returned in X.

The Carry flag is set to the value of the rightmost bit of x.

+----------------+     +---+

0 --> |     ----->     | --> |   |

+----------------+     +---+

Carry

SLn      Shift left multiple -- (Fkey)

y is shifted to the left by the number of positions specified by |x|,

zero bits are introduced into the vacated rightmost bit positions,

the result is returned in X.  The stack drops.  The Carry flag is set

to the value of the last bit to be shifted from the leftmost position

of y.

SRn      Shift right multiple -- (Fkey)

y is shifted to the right by the number of positions specified by |x|,

zero bits are introduced into the vacated leftmost bit positions,

the result is returned in X.  The stack drops.  The Carry flag is

set to the value of the last bit to be shifted from the rightmost

position of y.

ASR      Arithmetic shift right -- (Fkey)

x is shifted to the right by one position, the leftmost bit retains

its original value, the result is returned in X.  The Carry flag is

set to the value of the leftmost bit of x.

+------+

|      |

|    +----------------+     +---+

+--> |     ----->     | --> |   |

+----------------+     +---+

Carry

LJ       Left justify -- (Fkey)

If x<0, then x is shifted left until the leftmost bit contains a one

bit.  The result is returned to Y.  The number of shifts performed is

returned in X.  The stack is lifted.  If x=0, the stack is lifted and

X and Y are set to 0.

RL       Rotate left -- (Fkey)

x is shifted left by one position, the value of the leftmost bit is

returned in the rightmost bit position, and also in the Carry flag.

The result is returned in X.

+--------------------------+

+---+     |    +----------------+    |

|   | <---+--- |     <-----     | <--+

+---+          +----------------+

Carry

RR       Rotate right -- (Fkey)

x is shifted right by one position, the value of the rightmost bit is

returned in the leftmost bit position, and also in the Carry flag.

The result is returned in X.

+--------------------------+

|    +----------------+    |     +---+

+--> |     ----->     | ---+---> |   |

+----------------+          +---+

Carry

RLn      Rotate left multiple -- (Fkey)

y is shifted left by |x| positions, and as each bit is shifted out of

the leftmost bit position, it is introduced into the rightmost bit

position, and into the Carry flag.  The result is returned in X.  The

stack drops.

RRn      Rotate right multiple -- (Fkey)

y is shifted right by |x| positions, and as each bit is shifted out of

the rightmost bit position, it is introduced into the leftmost bit

position, and into the Carry flag.  The result is returned in X.  The

stack drops.

RLC      Rotate left through carry -- (Fkey)

x is shifted left one position, the value of the Carry flag is

introduced into the vacated rightmost bit position, and the Carry

flag is set to the value of the bit shifted out of the leftmost

position.

+------------------------------------+

|    +---+     +----------------+    |

+--- |   | <-- |     <-----     | <--+

+---+     +----------------+

Carry

RRC      Rotate right through carry -- (Fkey)

x is shifted right one position, the value of the Carry flag is

introduced into the vacated leftmost bit position, and the Carry flag

is set to the value of the bit shifted out of the rightmost position.

+------------------------------------+

|    +---+     +----------------+    |

+--> |   | --> |     ----->     | ---+

+---+     +----------------+

Carry

RLCn     Rotate Left through Carry multiple -- (Fkey)

y is shifted left by |x| positions, and at each shift, the rightmost

bit position receives the value of the Carry bit, and the Carry bit

receives the value of the bit shifted out of the leftmost position.

The result is placed in X.  The stack drops.

RRCn     Rotate Right through Carry multiple -- (Fkey)

y is shifted right by |x| positions, and at each shift, the leftmost

bit position receives the value of the Carry bit, and the Carry bit

receives the value of the bit shifted out of the rightmost position.

The result is placed in X.  The stack drops.

STACK CONTROL OPERATIONS

Rv       Roll Down -- (assigned to the 'v' key)

The contents of each stack register is shifted into the next lower

register, and x is shifted into T.

R^       Roll Up -- (assigned to the 'V' key)

The contents of each stack register is shifted into the next higher

register, and t is shifted into X.

x<>y     Exchange X and Y -- (assigned to the 'X' key)

The contents of the X and Y registers are exchanged.

NUMBER STORAGE AND RETRIEVAL

STO      Store -- (assigned to the 'N' key)

The calculator waits for a register specifier - a digit key (0..9)

or A..F; when a register has been specified, x is stored into that

register.  The value may later be recalled to X by pressing RCL

and the same register specifier.  x remains in X.  The STO operation

may be cancelled before specifying a register, by pressing ESC.

RCL      Recall -- (assigned to the 'M' key)

As with STO, RCL must be followed by a register specifier.  The

stack is lifted and the value last stored into the specified register

is placed into X.  The value remains in the specified register.  The

RCL operation may be cancelled before specifying a register by

pressing ESC.

RCL L    Last X -- (Press the 'M' (RCL) key, followed by 'L')

Returns to X the value which was in X prior to the last function.

MISCELLANEOUS

SHOW     Show X in all bases -- (Fkey)

The value in X is displayed simultaneously in binary, octal, decimal

and hexdecimal.  The complement and leading-zeros settings are

observed for each base displayed.  This display remains until a key

is pressed.  If ESC is pressed, then the SHOW display is erased, and

no action is taken.  If any other key is pressed, the SHOW display

is erased, and the action of that key is processed.  LastX is not set.

RCL V    Display version -- (Press the 'M' key (RCL), followed by 'V')

A window is displayed, which shows the version of CMCALC.

NUMBER ENTRY

0..9     Digits -- Used to enter numbers.  The letters A, B, C, D, E and F

A..F     are used for digit entry in hexadecimal mode.

ENTER    Enter -- (ENTER)

Copies x into Y, and disables stack lift.  If entry mode is active,

it is terminated.

<--      Backspace -- (assigned to the backarrow key)

When entry mode is active (the cursor is displayed on the entry

line) the last digit entered is deleted.  If no digits remain,

entry mode is terminated.  When entry mode is not active, the X

register is set to zero, and stack lift is disabled.

Error Messages

--------------

Divide by zero

A divide (/) or RMD function was attempted when the value in X was

0 or (in 1's complement mode) -0.  Division by 0 is undefined.

Exceeds current wordsize

A mask (MASKL or MASKR), bit-addressing (SB or CB), shift multiple

(SLn or SRn), or rotate multiple (RLn or RRn) function was attempted

and |x| was greater than the currently selected word size; or a rotate

through carry multiple (RLCn or RRCn) function was attempted and |x|

was greater than the currently selected word size + 1.  You can display

the currently selected word size by pressing MENU S.  You can change

the currently selected wordsize by pressing MENU W, entering the

desired wordsize (in decimal), and pressing ENTER.

Current mode is unsigned

You attempted to change the sign (using CHS) of a number, but the

currently selected complement mode is unsigned.  A negative number

cannot be expressed in unsigned mode.  You can display the currently

selected complement mode by pressing MENU S.  You can change the

currently selected complement mode by pressing MENU C, and then

pressing the first character of the desired complement mode.

The HP 95 Calculator Paradigm

-----------------------------

I have tried to make CMCALC adhere to the HP 95 calculator paradigm, but

there are some exceptions:

* Algebraic entry mode is not supported

* STO @ and RCL @ are not supported

* Shift-V performs roll-up

* Square root and inverse are omitted

Since the "B" and "C" keys are used for entering hexadecimal numbers, they are

unavailable for the standard LASTx and x<>y functions.  Therefore...

* LASTx is obtained by RCL L

* x<>y is assigned to the X key

CMCALC Compared to the HP-16C

-----------------------------

For those familiar with the HP-16C, CMCALC does not provide programmability,

and omits functions directy related to programmability (there is no indirect

register, no x?y or x?0 comparisons, no flag manipulaton functions, and no

bit test (B?) function).  CMCALC also omits floating-point mode, and the

square root function, since these are available in the built-in HP 95

calculator.  The three DBL functions have been omitted.  Word size is

limited to a maximum of 32 bits.  Whereas the HP-16C had a variable number

of registers, CMCALC has a fixed set of 16 registers: 0..9 and A..F.

Problems, Bugs, Comments

------------------------

If you have problems with CMCALC, or have found a bug, or if you wish

to contact me with suggestions for change or improvement (or just a

friendly "hello"), you can contact me on CompuServe (75156.3132).

CMCALC may be freely copied and distributed ONLY in unmodified form, and

you may not charge any fee for it.  I neither make nor express any warranty

that CMCALC operates correctly (though I believe it to do so), or that it

is suitable for your purposes (though I hope it is).

Gene Dorr