Box Plot For Windows 3.0

By Rick Carlson

For Sparta Sound Co.

COMPUSERVE 71041,3015

SHAREWARE VERSION 1.0

Box Plot is a Microsoft Windows based program that shows

the frequency response and maximum Sound pressure level

of a given Loudspeaker/Box Combination.

This Version of Box Plot requires Microsoft Windows 3.0

or later to run. A math coprocessor is not required but

really helps speed things up if you want do alot of

iterations on the box parameters.

(Turn off Auto Plot on slow machines).

If Terms like Qes, Fs, Vas are unfamiliar to you, then

this program will be difficult for you to use. It is intended

for the audio hobbyist or professional interested in building

there own speaker enclosures, or modifying their present

system.

If you are interested in building your own speakers, I would

suggest that you subscribe to Speaker Builder Magazine. It

is a good source of information on this subject.

(Unsolicited advertisement)

============================================================================

History:

Box  Plot  is based on a program called Boxresponse,  which

was published in Speaker Builder magazine a while back.   Box

Plot   adds  graphical  output,   MS  Windows  compatability,

printing, file storage, and a couple of other new features to

the original BASIC version of Boxresponse.

Operation:

This  Program  was  designed  to  allow 'almost'  real time

update of the response plot as box parameters are  changed.

This  is usefull when trying to shoehorn a speaker into a non

optimum box.  Many different combinations of  box  parameters

can  be  tried  in  a  short period of time.  If Auto Plot is

turned on,  then  clicking  on  the  OK  button  of  the  BOX

Parameters dialog box will replot the response curves.

Speaker Parameters:

Speaker parameters are entered into  a  dialog  box.   When

done,  click  on  the  OK button to save the parameters.  The

Parameter names are the Thiele Small Parameters specified  on

most good spec sheets.

Box Parameters:

Box Parameters are entered in a dialog box.   When  the  OK

button  is clicked the Box Parameters dialog box stays on the

screen.  To close the dialog box click on the Close button or

use the system drop down menu (upper left of dialog  box)  to

close  the box.  The reason the dialog box stays on screen is

to make it easier to edit parameters and see  the  effect  on

frequency response.

There are 6 types of boxes that can be modeled with Box Plot.

Closed Box:

Simple sealed Enclosure.

Closed Box with first order equalizer:

Sealed box with first order filter. The first order

filter usually provides subsonic attenuation.

Closed Box with second order filter:

The second order filter is usually an active filter

placed before the power amp. This filter can be used

to extend the low end, or modify the response of an

existing system.

Vented Box:

Box with tuned port.

Vented Box with first order eq:

First order Eq is more usefull here than is a sealed box.

Vented Box with second order eq:

Same as sealed box.

Comments:

The comments field is a text string,  255 chars  long  that

can be used to annotate the graph.

Vent Design:

This  box  will  show  the vent length based on the present

box/ speaker combination.

Auto Plot:

When the Auto Plot menu item is checked,  anytime a speaker

or  box parameter is changed,  the graph will be regenerated.

This keeps the graph up to date with the  latest  parameters.

This  also  may  be  annoying on slower machines so it can be

turned off.

File:

New:

This will reset all parameters to some predefined settings.

The settings are based on nothing at all, just some numbers

that wont do real wierd things. (The speaker parameters are

from a well known manufacturer.)

Save:

Save as:

This will save the speaker, box, and comments in the specified

file. (Registered Versions)

Open:

Read a file on the disk. The default file extension is .SPK

but has no real significance.

Print:

This will print the response curves as shown on the screen

along with the speaker and box parameters. (Registered versions)

In the Unregistered shareware version, files cannot be saved

and the response curves cannot be printed. Please register

to recieve the complete version, as well as updates and

bug fixes, as they become available.

To register Please send $25.00 to

Rick Carlson

62 Fox Trail Rd

Sparta, NJ. 07871

Compuserve 71041,3015

I will send out a disk with the complete version, and some

more on line documentation.

If you have suggestions, comments, bug reports or just feel

like writing please do so.

Let me know what other kinds of programs would be usefull to

the audiophile.