#include "dwu.h"

#define KEY_BUFFER          0x60

#define KEY_CONTROL         0x61

#define INT_CONTROL         0x20

#define KEYBOARD_INT        0x09

#include "conio.h"

// ASCII in SCAN order

// this copy buffers all

static byte kbd_to_ascii[] = {

0x00,    // NULL     00

0x1B,    // ESC      01

0x31,    // 1        02

0x32,    // 2        03

0x33,    // 3        04

0x34,    // 4        05

0x35,    // 5        06

0x36,    // 6        07

0x37,    // 7        08

0x38,    // 8        09

0x39,    // 9        0A

0x30,    // 0        0B

0x2d,    // -        0C

0x3d,    // =        0D

0x08,    // BKSP     0E

0x09,    // TAB      0F

0x71,    // q        10

0x77,    // w        11

0x65,    // e        12

0x72,    // r        13

0x74,    // t        14

0x79,    // y        15

0x75,    // u        16

0x69,    // i        17

0x6F,    // o        18

0x70,    // p        19

0x5b,    // [        1A

0x5d,    // ]        1B

0x0d,    // ENTER    1C

0xe0,    // CTRL     1D

0x61,    // a        1E

0x73,    // s        1F

0x64,    // d        20

0x66,    // f        21

0x67,    // g        22

0x68,    // ,        23

0x6a,    // j        24

0x6b,    // k        25

0x6c,    // l        26

0x3b,    // ;        27

0x27,    // '        28

0x60,    // `        29

0xe2,    // L_SHIFT  2A

0x5c,    // \        2B

0x7a,    // z        2C

0x78,    // x        2D

0x63,    // c        2E

0x76,    // v        2F

0x62,    // b        30

0x6e,    // n        31

0x6d,    // m        32

0x2c,    // ,        33

0x2e,    // .        34

0x2f,    // /        35

0xe3,    // R_SHIFT  36

0x2a,    // *        37

0xe4,    // ALT      38

0x20,    // SPC      39

0xe6,    // CAPS     3A

0xf1,    // F1       3B

0xf2,    // F2       3C

0xf3,    // F3       3D

0xf4,    // F4       3E

0xf5,    // F5       3F

0xf6,    // F6       40

0xf7,    // F7       41

0xf8,    // F8       42

0xf9,    // F9       43

0xfa,    // F10      44

0xe7,    // NUMLCK   45

0xe8,    // SCRLOCK  46

0xd7,    // k_7    47

0xd8,    // k_8    48

0xd9,    // k_9    49

0xb1,    // k_-    4A

0xd4,    // k_4    4B

0xd5,    // k_5    4C

0xd6,    // k_6    4D

0x2b,    // +      4E

0xd1,    // k_1      4F

0xd2,    // k_2     50

0xd3,    // k_3    51

0xd0,    // k_0      52

0xb0,    // k_dot      53

0x00,    // NOTHING  54

0x00,    // NOTHING  55

0x00,    // NOTHING  56

0xfb,    // F11      57

0xfc,    // F12      58

0xb2,    // R ENTER  1C 59

0xe1,    // R CTRL   1D 5A --

0xb3,    // R /      35 5B

0xe5,    // R ALT    38 5C --

0xb4,    // R HOME   47 5D

0xb5,    // R UP     48 5E

0xb9,    // R PGUP     49 5F

0xb7, 	 // R LEFT     4B 60

0xb8,	 // R RIGHT    4D 61

0xbb,    // R END      4F 62

0xb6,	 // R DOWN     50 63

0xba,    // R PGDN     51 64

0xbc,    // R INS         52 65

0xbd,    // R DEL         53 66

0xff,    // print screen  54 67

0xbf,    // ctrl-break  0x68

0xbe,    // pause       0x69

};

// this copy buffers what is appropriate

/*

static byte buffered_keys[256];

*/

static byte extended_codes[0x55-0x1c];

int kbd_mode=0;

void (__interrupt __far *Old_Isr)();  // holds old keyboard interrupt handler

volatile char kbd_flags[256];  // table of key presses low 128 is current

int kbd_misc;   // keyboad interrupt vars

int kbd_key_in;

int kbd_extend=0;

/*

byte kbd_buf[KBD_Q_SZ];

volatile byte * kbd_put = kbd_buf;

byte * kbd_get = kbd_buf;

*/

#define KBD_Q_SZ 32

volatile char * kbd_put;

char kbd_buf[KBD_Q_SZ];

char * kbd_get;

#pragma off (check_stack)

void _interrupt _far pkbd_int()

{

_enable();                         // sti

kbd_key_in=inp(KEY_BUFFER);      // get the key

kbd_misc=inp(KEY_CONTROL);     // clear and reset the 8259

kbd_misc|=0x82;

outp(KEY_CONTROL,kbd_misc);

outp(KEY_CONTROL,kbd_misc&0x7f);

outp(INT_CONTROL,0x20);

if(kbd_key_in>=0xe0)  // extended

{

kbd_extend = kbd_key_in-0xe0+1;

return;

}

if(kbd_key_in>128)         // check for a release

{

kbd_key_in-=128;

if(kbd_extend)

{

kbd_extend--;

if(kbd_extend) return;

kbd_key_in = extended_codes[kbd_key_in-0x1c];

}

kbd_flags[kbd_to_ascii[kbd_key_in]] &= ~1; // if release clear the table entry

return;

}

if(kbd_extend)

{

kbd_extend--;

if(kbd_extend) return;  // that damn print screen

kbd_key_in = extended_codes[kbd_key_in-0x1c];

}

//  not a release

kbd_flags[kbd_to_ascii[kbd_key_in]] = 3;

}

void _interrupt _far pkbd_int_buf()

{

_enable();                         // sti

kbd_key_in=inp(KEY_BUFFER);      // get the key

kbd_misc=inp(KEY_CONTROL);     // clear and reset the 8259

kbd_misc|=0x82;

outp(KEY_CONTROL,kbd_misc);

outp(KEY_CONTROL,kbd_misc&0x7f);

outp(INT_CONTROL,0x20);

if(kbd_key_in>=0xe0)  // extended

{

kbd_extend = kbd_key_in-0xe0+1;

return;

}

if(kbd_key_in>128)         // check for a release

{

return;

}

if(kbd_extend)

{

kbd_extend--;

if(kbd_extend) return;  // that damn print screen

kbd_key_in = extended_codes[kbd_key_in-0x1c];

}

*kbd_put++ = kbd_to_ascii[kbd_key_in];

if(kbd_put>= kbd_buf+KBD_Q_SZ)

kbd_put = kbd_buf;

}

#pragma on (check_stack)

void set_kbd_mode(int mode)   // stick vec

{

if(mode == kbd_mode)

return;

kbd_mode = mode;

switch(mode)

{

case 0: _dos_setvect(KEYBOARD_INT, Old_Isr); break;

case 1: _dos_setvect(KEYBOARD_INT, pkbd_int); break;

case 2: memset((void *)kbd_flags,0,256);

kbd_flags[1] = 3;

kbd_put = kbd_buf;

kbd_get = (char*)kbd_put;

_dos_setvect(KEYBOARD_INT, pkbd_int_buf);

break;

}

}

#define PUTEXT(KEY,NEW) extended_codes[KEY-0x1C] =  NEW;

void init_kbd()   // stick vec

{

// damn that num lock keyf

//  *((char *)0x417) = *((char *)0x417)&(~(1<<5));

memset((void *)kbd_flags,0,256);

kbd_flags[1] = 3;

Old_Isr = _dos_getvect(KEYBOARD_INT);

PUTEXT(0x1c,0x59)

PUTEXT(0x1D,0x5A)

PUTEXT(0x35,0x5B)

PUTEXT(0x38,0x5C)

PUTEXT(0x47,0x5D)

PUTEXT(0x48,0x5E)

PUTEXT(0x49,0x5F)

PUTEXT(0x4B,0x60)

PUTEXT(0x4D,0x61)

PUTEXT(0x4F,0x62)

PUTEXT(0x50,0x63)

PUTEXT(0x51,0x64)

PUTEXT(0x52,0x65)

PUTEXT(0x53,0x66)

// that damn print screen

// plus that damn num lock

PUTEXT(0x2a,0x00)

PUTEXT(0x37,0x67)

// that damn pause

// ctrl break

PUTEXT(0x46,0x68)

// just pause

PUTEXT(0x45,0x69)

//pause break sends

// damn that keyboard!

// 0x29 (ctrl), 0x??, release 0x61 (grey right)

// buffer_kbd();

}

#undef  PUTEXT

/*

void buffer_kbd()

{

memset(buffered_keys+1,0,255);

buffered_keys[0] = 0;

}

void buffer_keys(byte key1, byte key2)

{

if(key1 > key2)

{

byte a = key1; key1 = key2; key2 = a;

}

int lg = key2-key1+1;

memset(buffered_keys+key1,1,lg);

}

void buffer_key(byte key)

{

buffered_keys[key] = 1;

}

void no_buffered_keys()

{

memset(buffered_keys,0,256);

}

void no_buffer_key(byte key)

{

buffered_keys[key] = 0;

}

void no_buffer_keys(byte key1, byte key2)

{

if(key1 > key2)

{

byte a = key1; key1 = key2; key2 = a;

}

int lg = key2-key1+1;

memset(buffered_keys+key1,0,lg);

}

*/