RELEASE NOTES

NT 3.51 Driver Version 1.33.08

8/22/1997

1.0 Purpose

This Trio family release is a bug fix driver for HP Saxo (KV2) system to address SPR 16103, 16104, and 16375.  This is a tip build using the latest S3MINI.SYS and S3DSPLY.DLL source code.

2.0 Products Supported

S3 Trio64V2, S3 Trio64V+ and Trio64V+ forward compatible chipsets (in forward compatibility mode only, if not explicitly listed here).  Also older S3 chipsets as follows:  86C911, 86C924, 86C928, 801/805, 805I, 928PCI, Vision864, Vision964, Trio32, Trio64, Vision866, Vision868, and Vision968.

3.0 Release Contents

The installation disk for this release will include the following files:

S3DSPLY.DLL

S3MINI.SYS

OEMSETUP.INF

VERSION

NTMODES.EXE

README.TXT

## 4.0 Resolutions and color depths

## Across the product lines, the following modes are supported in this release. The driver supports these modes, provided these modes are supported by particular OEM BIOS.

## This package does not contain a .CFG file. But, if this driver is used along with a mode table .CFG file, available modes are determined by the content of that file. To check the modes supported by the OEM, run NTMODES.EXE and open the .CFG file, which is generally supplied by an OEM.

| Pixel Resolution | Colors | Memory Required |
| --- | --- | --- |
| 640x480 | 256 | 1MB |
| 640x480 | 65536 | 1MB |
| 640x480 | 16.7M | 2MB |
| 800x600 | 256 | 1MB |
| 800x600 | 65536 | 1MB |
| 800x600 | 16.7M | 2MB |
| 1024x768 | 256 | 1MB |
| 1024x768 | 65536 | 2MB |
| 1024x768 | 16.7M | 4MB |
| 1152x864 | 256 | 1MB |
| 1280x1024 | 256 | 2MB |
| 1280x1024 | 65536 | 4MB |
| 1600x1200 | 256 | 2MB |

5.0 New Features

None.

6.0 Bug Fixes

SPR 16103 - shutdown garbage on Saxo system only.

SPR 16104 - When moving the icon with the mouse the arrow will jump back to former position

to get the pixel. (Partial fix. No more stray pixel appearing anymore).

SPR 16375 - Quicktime sample.mov file application hang with 32-bit quicktime. (Installing SP5

solves problem).

7.0 Known issues

SPR 16104 - cursor still jumps back. Must somehow change the hardware sprite to the location of the current software cursor.

8.0 Source Files Modified

The following highlights changes in the source.

Relevant files listed only for:

Changes in Miniport Driver

S3COMMON.H	Changed version number  to 1.33.08.

S3.C	Must clear out video memory and ZeroMemAndDac().

Installation:

OEMSETUP.INF	Changed the version no. to 1.33.08.

VERSION	Changed the version no. to 1.33.08.

README.TXT	Eliminate versioning info for now on in this file.

TRIO351.FL	Added S3DEF.H and S3IOCTL.H for newer miniport.

DISPLAY DRIVER:

POINTER.C - add macro WAIT_FOR_VBLANK.

DRIVER.H - set HW_POINTER_HIDE equal 62 rather than 63. For some hardware reason, interlaced modes are effected.

9.0 PVCS identification

The source files included in this release are version stamped as “NT3.51_1.33.08” under PVCS control at Q:\ SOFTDEV\SRC\NT\MINIPORT (miniport) and  Q:\SOFTDEV\SRC\NT\351\S3 (display driver and install files).

10.0 Test Recommendations

1.	Test (3) SPRS on HP Saxo machine under various resolutions and color depths.

- Run regression test versus 1.33.07.