This document was last modified on  16-Feb-96.

| Product Name | S3 Refresh Property Sheet |
| --- | --- |
| Description | The S3 Refresh Windows 95 property sheet allows the user to modify the display refresh rate. |

# Requirements

## Operating system(s)

Windows 95

## Hardware

IBM Compatible Computer with Intel 386 or higher processor

S3 Graphics Chips - All

## Software

S3 Video BIOS - All

S3  Windows 95 Display Driver

## Functionality

- Provide a way for the user to set the display’s refresh rate in Windows 95 by adding a property sheet to the Windows 95 built-in “Display Properties” utility.

- Show the current refresh rate setting for all possible display resolutions.

- Provide an easy way for the user to simultaneously set the lowest (safest) refresh rates for all possible display resolutions.

- Provide an easy way for the user to simultaneously set the highest refresh rates for all possible display resolutions.

- Provide a way for the user to individually set the refresh rate for each possible display resolution.

- Allow the refresh rate for the current display resolution to be changed on-the-fly.

- Prevent the user from setting a refresh rate that doesn’t work by requiring verification within 10 seconds of changing the refresh rate on-the-fly. If there’s no reply before the time-out then restore the previous refresh rate.

- Automatically install the refresh rate property page when the driver is installed.

- Limit the refresh rates the user is allowed to choose from by the capabilities of the:

- video controller

- video BIOS

- monitor

## Deliverables & Dates

S3 Refresh is currently in SQA.

# Human Interface

## Overview

An additional property sheet in Windows 95 display properties allows the user to modify the display refresh rate.

## Detailed Description

2.2.1   The user needs to activate “Display Properties”  window  using one of the following methods:

A -    Open "My Computer" Folder.

Open the "Control Panel" Folder

Select “Display” folder.

B -   Click the right mouse bottom on the desktop.

Select “Properties” from the popup menu.

C -   Access the “Display Properties”  window  via one of the short cut buttons in Window 95’s “Help”.

- Select “S3 Refresh”  from the top menu. (1.4.1)

The currently selected  configuration and settings are visible but disabled. The user can modify the Refresh Rate by selecting  “Change Configuration”. (1.4.2)

- Once the user select  “Change Configuration”, the user can view the “Refresh Rate”  window and modify the Refresh Rate Configuration and Settings.

The user can select one of the three available configurations:

1. Low    	The lowest Refresh Rate for each display resolution is automatically             selected. (1.4.3)

2. High       The highest Refresh Rate for each display resolution is automatically selected. (1.4.4)

3. Custom  Allows the user to individually select a  refresh rate for each display resolution. (1.4.5)

Note:  Anytime  the user tries to select a Refresh Rate in the resolution list box,  the “Custom” configuration is selected and all Refresh Rates change to their custom values.

If the Refresh Rate for the current display resolution was modified, selecting the “Ok” button will change the refresh rate on the-fly through a test & verify process. Otherwise the selected settings and  configuration are stored in the Registry. (1.4.6)

2.2.4   The “S3 Refresh” utility allows the user to test  and verify the selected refresh rate for the current display resolution, starting with the following warning message. (1.4.7)

Once the user selects “OK”,  the S3 Refresh utility changes the refresh rate and displays the following message for 10 seconds.

By selecting “Yes”, the new setting will be stored in the registry and the original “S3 Refresh” property page will be displayed.

If the user selects “No”  or does not make a selection within 10 seconds, the original Refresh Rate will be restored, and the following message will be displayed.

Note:  “S3 Refresh” allows the user to test and verify the refresh rate for the current display resolution, all other resolutions can be tested via display mode change in the  “Settings” Property Sheet.

# System Interface

Available refresh rates for each resolution is stored in “S3_REFRE.INF”,   S3 Refresh Rate installation file. (1.4.8.1, 1.4.8.2)

HKLM,SOFTWARE\S3\Refresh,ref_640x480,1,3c,48,4b,55

HKLM,SOFTWARE\S3\Refresh,ref_800x600,1,38,3c,48,4b,55

HKLM,SOFTWARE\S3\Refresh,ref_1024x768,1,38,3c,46,4b,55

HKLM,SOFTWARE\S3\Refresh,ref_1280x1024,1,38,3c,48,4b

Note: The refresh rates are stored as Hex values

During Install,  the available refresh rates are written to “HKLM,SOFTWARE\S3\Refresh” in  the Windows 95 registry.

Once the user changes the Refresh Rates  and select  “OK”:

- The current  configuration (Low, High, or Custom) and the custom refresh rates  are  stored in  “HKLM,SOFTWARE\S3\Refresh” in the registry.

Note: The configuration is stored as 4 bytes Hex value, in this case 00 00 03 f0.

- The selected Refresh Rate for the current resolution is stored  in “HKLM\System\CurrentControlSet\Services\Class\Display\000x\DEFAULT” in the registry.

Note:  000x Specify the display node (0000, 0001, …), currently all nodes are updated.

- “ChangeDisplaySettings” is then called to force a mode change and to set the new Refresh Rate.

Display  Mode Change via Settings Property Sheet:

Prior to a mode change, “S3 Refresh” Property Sheet receives a “WM_ENABLE”  message from  Display Properties. This allows “S3 Refresh” to set the proper refresh rate in  “HKLM\System\CurrentControlSet\Services\Class\Display\000x\DEFAULT” in the Windows 95 registry. Also , “WM_CHANGEDISPLAY“ is monitored to detect a cancel of a mode	     change to restore the original refresh rate for the original resolution.

# Installation

- Open either "My Computer"  or  “Windows Explorer”  Folder.

- Open the Folder that contains "s3_refre.inf" and "s3_refre.dll".

- Click the right mouse bottom on "s3_refre" and select "Install".

# Release Note

“S3 Refresh  Version 1.3“ allows the user to test  and verify the refresh rate for the current display resolution.

Note: All other resolutions can be tested via display mode change in the  “Settings Property Sheet” in Display Properties.