Colorado Backup for Windows Troubleshooting

Version 1.10

Contents  Revision 1.10b

1.0  Additional Information.

1.1  Sound Boards.

1.2  Fax Modems and Voice Mail Systems

1.3  Third Party Windows Backup Software.

2.0  Trouble Shooting.

2.1  General Protection Fault.

3.0  Performance Tips.

3.1  Video Drivers.

3.2  Operating CBW in the background.

3.3  Memory and Resources.

4.0  Considerations for Network Users

5.0  General Trouble Shooting Hints

1.0  Additional Information.

1.1  Sound Boards

Using a sound board with CBW may require special consideration

to avoid interrupt and DMA conflicts.  This is particularly

true when dealing with the Trakker tape drive unit.  Most

sound boards use an interrupt that corresponds with a parallel

port.  If a Trakker unit is connected to one of these parallel

ports, you may have poor sound quality.  To correct this,

choose an interrupt for the sound board that is different from

your parallel port.  See your sound board's documentation for

instructions.

Some sound boards install device drivers that conflict with

CBW's device driver, CMSWTAPE.386.  These sound board device

drivers install MIDI support.  If you are experiencing

problems with CBW, you might have a conflict with these MIDI

drivers.  To correct this, you can remove the MIDI support

from the Drivers entry in the Control Panel in windows.  See

your sound board's documentation for instructions.

1.2  Fax Modems and Voice Mail Systems.

Some fax modems and voice mail systems install device drivers

under windows that may conflict with CBW.  Symptoms of this

problem include having the tape backup software produce an

error message when a fax or voice mail is received.  To

correct this, disable the fax or voice mail reception either

through the control panel or through the control program for

your board.

1.3  Third Party Windows Backup Software.

Most third party backup programs for windows install a device

driver that conflicts with the CBW driver.  If any other

backup software has been installed through windows, you may

need to remove these drivers before CBW will operate.  To do

this, you will need to edit the SYSTEM.INI file in your

windows directory.  Under the section [386enh] you should see

several lines that read 'DEVICE='.  If any of these lines

reference 'VFINTD.386',  'CPBVXD.386', or 'FASTBACK.386', you

will need to add a semicolon to the beginning of these lines.

2.0  Troubleshooting.

2.1  General Protection Fault.

In some situations, CBW may fail with a message indicating

that a General Protection Fault was detected.  In situations

like this, the most common cause is having a parameter of the

EMM386.EXE device driver not properly set.  To check for this

situation, edit the CONFIG.SYS file and place a remark, REM

before the DEVICE=EMM386.EXE line.  Next, reboot the computer

and retry the CBW operation.  If the error no longer occurs,

it is an indication that one of the parameters of the

EMM386.EXE line is conflicting.  Try removing parameters one

at a time until the conflicting paramter is located.  The Dos

manual contains an explanation of the EMM386.EXE and its

parameters.

If the error persists, or if you do not have a line with

EMM386.EXE, then we need to check for a device driver conflict

as stated in section 1.3.  Occasionally device driver

conflicts can cause a General Protection Fault.

3.0  Performance Tips.

3.1  Video Drivers

Some systems may experience slow backup speeds.  On these

systems, improvements may be gained by changing from the

default VGA driver to the driver supplied with the video

board.  To determine if this is the problem, start a backup,

then minimize the CBW window.  It is normal for backup speeds

to improve somewhat with CBW minimized, however, if the

results are extreme it may be necessary to change the video

driver.  See the video board manual for instructions.

3.2  Operating CBW in the background.

While CBW is capable of running on a system in the background,

performance may decline with each additional application

running.  Memory is used by each application which could cause

CBW to fail with an out of memory error.

When running a backup operation in the background, some open

files may not back up.

For fastest performance and to assure a complete backup, you

may wish to operate CBW as the only application open on the

system.

3.3  Memory and Resources.

If you do not have sufficient memory or resources to execute

CBW you will receive an error message.  However, having less

than 2.5M of  available memory or less than 70% resources will

result in slow performance.  If this is the case, you can

improve performance by making more memory or more resources

available to CBW.

To check your available memory, choose Help from the Program

Manager, and then choose About Program Manager.  This screen

will provide you with the amount of memory and system

resources available.

To make more memory and resources available, you may need to

close other applications and minimize Program Manager.

4.0  Considerations for Network Users.

Generally CBW works transparently with Windows for Workgroups,

NetWare Lite and Lantastic, however, with the current version

of CBW, specific file attributes, bindery and privilege

information may not be saved.  Please refer to the Networks

Applications section of the manual for more information.

In the Windows For Workgroups environment, if a "Sharing

Buffers Exceeded" message occurs try the following changes:

In the [386enh] section of the SYSTEM.INI increase

NetHeapSize= to 48 and DMABufferSize= to 064.  If the error

still occurs you may need to de-select the enable Share Option

in Network under Control Panel.

5.0  General Trouble-Shooting Hints.

SOFTWARE.  If you are not able to resolve an error after

following the suggestions given on the error screen, it is

recommended to do the following:

1.   Re-try the operation, look for consistent results.

2.   Try another tape.  Make sure your tapes are on the

Approved Tapes List in the manual or use a Colorado

Memory Systems brand tape for your drive.

3.   Check the hard drive for lost clusters, bad sectors, and

fragmentation.  If errors occur, correct by using 'CHKDSK

/F'.  See your Dos manual for more information.

4.   Close down all other applications, remove items from your

startup group and remark items out of the load= and run=

lines in the WIN.INI file.

5.   Re-boot with a clean AUTOEXEC.BAT and CONFIG.SYS.  It is

recommended that this be done from a bootable floppy

diskette.  To receive a fax with instructions for this

process, dial our fax-back service at 1-800-368-9673 and

request document #2900.

If you have tried the above steps and the software still fails it

may be necessary to load a basic SYSTEM.INI and WIN.INI for testing

purposes.  Your Windows manual explains the process needed to

modify these system files.  Be sure to make backup copies of these

files before making changes.

HARDWARE.  If the drive does not respond when you try to access it

with the software, do the following:

1.   Remove the tape from the drive and examine it.  If the tape is

unspooled, refer to the drive cleaning instructions in the

manual.

2.   Close down all programs and turn the power off on the

computer.  If your tape drive plugs directly into the wall

outlet, disconnect it from the wall outlet, wait a few seconds

and then re-connect it.

3.   Turn the power back on.

4.   Place a tape in the drive and look to see if the LED is

functioning normally.  The tape drive LED should not be on

after the tape stops spinning.