D'Bridge EMAIL System

Single-line 1.58 / 5 April 1994

------------------------------------------------------------------------------

Installing D'Bridge from the release ZIP file is quick and easy.

First, make sure that you mave a copy of both PKZIP.EXE and PKUNZIP.EXE in

the DOS PATH.   Version 2.04c or later is REQUIRED!

PKZIP is a compression/decompression system available from:

PKWARE, Inc.

7545 N. Port Washington Rd.

Glendale, WI 53217

This program is available as a SHAREWARE product and is available for file-

request from most of our dealers in the DBNET nodelist.  It is REQUIRED for

D'Bridge operation.

Next, unZIP the release file, DB-158.ZIP into a subdirectory of your choice.

Assuming that the ZIP file is on a floppy in drive B: type the following DOS

commands:

ie.

C>MD \DB

C>CD \DB

C>PKUNZIP B:DB-158

At this point, you should have three files in the \DB directory that came

from the release ZIP file:

DB-158.EXE

DB-158.DOC (this file)

READ.ME

To install or upgrade D'Bridge, simply type the command DB-158 in the \DB

directory:

C>DB-158

The installation program will walk you through the installation process.

Once the installation is complete, the file DB-158.EXE will be erased.

------------------------------------------------------------------------------

D'Bridge EMAIL System

Single-line 1.58 / 5 April 1994

------------------------------------------------------------------------------

Major changes since 1.54

------------------------

- Add file-sharing support for multiple DBEDIT executions on a LAN

- Fixed problem where DB would answer the phone, then process mail, start

an event or other such nonsense.

- Fixed the bug where sometimes DB wouldn't load at all unless a parameter

(even blank) was specified.  Please let me know if this is not the case.

- As a side-effect of beta 2101, my system was locked-up all night because

of a modem timeout while answering the phone.  "Oops."  This solves that...

- If you want DB to leave your file and directory pathnames alone and not

expand them, set the environment variable DB-NOEXPAND to YES.  This will

allow invalid DOS paths (such as are used by many network OS's...)

- A couple of modem control issues have been resolved.  The environment

variables DB-MODEM-ANSWER and DB-MODEM-OFFHOOK have been eliminated.

They have been replaced with function within the MCF file to accomplish

the same thing.  The DOCS have been updated, but since they don't come

with beta releases, here's the new keywords in the MCF file:

OFFHOOK <modem string>

This defines the command to take the modem offhook.

- Default ATH1M0S0=0

ANSWER <modem string>

This defines the command to answer the phone if Hardware or Software

answer modes are selected.

- Default ATA

- A new "unsecured" inbound files directory is now supported for systems which

want to review all unsecured mail and files before processing.  To specify

that all data from unpassworded systems should be placed in a certain inbound

directory, set the following environment variable:

SET DB-UNSECURED-PATH=C:\DB\UNSECUR

If the directory does not exist, it will be created.  Set like this, D'Bridge

will only process mail from secured links or mail picked-up from an outbound

call.  (Outbound sessions are defined as "Secured.")

For D'Bridge to process .PKTs and XMAIL, it must first be moved to the

standard INBOUND FILES DIRECTORY (on the DIRECTORY PATHS screen.)  Setting

this variable causes D'Bridge to treat the normal inbound files directory

as a secured file area and using the path specified in DB-UNSECURED-PATH as

a holding area for unsolicited mail.

- The way in which outbound mail is transmitted has been changed.  Due to

popular request, the way NETmail packets with file-attaches has been re-

designed.  Now, for a given priority, attached files are transmitted BEFORE

the corresponding .PKT file.  This causes the NETmail to not arrive at all

unless the attached files are first sent OK.  If carrier drops, D'Bridge

will try to retransmit ALL queued files, not just the ones that weren't

already sent (as before.)  Now we are counting on the ZMODEM skip feature

to bypass already transmitted files.

- Of course, no beta release involving structural changes is complete without

a bug (or a could in  this case.)  FrontDoor added a couple new flags: TFS

(truncate file when sent) KFS (Kill file when sent.)  These flags were not

honored in Beta 2105 and that problem is now fixed.  Thanks to the people at

PlanetConnect and Arthur Greenberg for pointing out that problem...

- A new modem control file (MCF) keyword has been added.  According to Wilma

Morgel, many new modems are coming out with multi-line responses before the

"CONNECT" string.  This new feature allows D'Bridge to ignore these messages

and (in this beta version ONLY) display them in the log for debugging.

IGNORE <MODEM RESPONSE>

ie. IGNORE DATA

<MODEM RESPONSE> is a unimportant code returned from the modem to be

ignored completely.

- A new activation file has been added:

If DBRIDGE.AMR exists in the D'Bridge root directory, mail will be

unpacked and the "after mail received" commands will be executed.

This was added to facilitate mail processing from external receivers

such as PlanetConnect, etc.  Thanks to Martin Belcke for his suggestion.

- New MCF files have been added.  272 modems are now supported directly.

Thanks to Wilma Morgel for compiling the new list!

Your input will be greatly appreciated.  (1:110/68)