ENERGY.EXE

ELECTRICAL DESIGN SOFTWARE

Energy Cost Version 2.0  Documentation

(c) Copyright 1993-1994 Electrical Design Software

Licensed Material. All Rights Reserved.

����������  ��������       ������

���    ��   ���    �     ��    ��

���     �   ���     �   ��      �

���         ���     ��  ��

���  �      ���     ��  ���

���  �      ���     ��   ���

������      ���     ��     ���

���  �      ���     ��       ���

���  �      ���     ��        ���

���         ���     ��         ��

���     �   ���     �   �      ��

���    ��   ���    �    ��    ��

����������  ��������      ������

ELECTRICAL DESIGN SOFTWARE

12 Vassar Drive Pelham, NH 03076

Phone or Fax: (603) 893-9008

Copyright (c) 1993-1994  All Rights Reserved

Energy Cost 2.0  From Electrical Design Software

TABLE OF CONTENTS                                          PAGE

~~~~~~~~~~~~~~~~~                                          ~~~~

Introduction ............................................... 1

Definition of shareware .................................... 1

Disclaimer - Agreement ..................................... 2

What is Energy Cost 2.0 .................................... 3

Ordering Energy Cost 2.0 ................................... 3

System requirements ........................................ 3

Files ...................................................... 4

Command line options ....................................... 4

Energy Cost Tutorial........................................ 5-13

Special Keys ............................................... 14

Quick access keys from the main screen ..................... 15

Command buttons main work and names screen ................. 15

Command buttons calculate kilowatt hour screen ............. 16

Command buttons ampere averaging screen .................... 16

Help system ................................................ 16-17

General overview of the main menu selections................ 17-19

Pop up calculator........................................... 19

Each data entry field explained............................. 19-24

The electric bill information screen........................ 25-28

Data base subjects ......................................... 29-30

Ampere averaging utility ................................... 30

Reporting - print or file .................................. 31

Lighting schedules ......................................... 31-33

Motor schedules ............................................ 33-34

Power factor schedules ..................................... 34-35

Formulas ................................................... 36

When you Register........................................... 37

Why you should register .................................... 37

Support..................................................... 38

Copyright (c) 1993-1994  All Rights Reserved

Energy Cost 2.0  From Electrical Design Software

I N T R O D U C T I O N

~~~~~~~~~~~~~~~~~~~~~~~

Electrical Design Software is a New Hampshire based company

that was established in 1992.  Our organization's primary objective

is to produce quality software, at a reasonable price for the

electrical industry.

ENERGY COST 2.0 requires no special computer knowledge, and

is designed to accommodate both the amateur and professional user.

Context Sensitive Help Screens have been programmed into this

application in order for you, the user, to continue working

quickly and efficiently.

Most importantly, here at Electrical Design Software, YOUR

comments are considered essential, for the development of future

software products.  With your input we can design programs that

will accommodate the Electrical Industry and other specialized

fields.

DEFINITION OF SHAREWARE

~~~~~~~~~~~~~~~~~~~~~~~

Shareware distribution gives users a chance to try software

before buying it.  If you try a Shareware program and continue

using it, you are required to register. Individual programs differ

on details -- some request registration while others require it,

some specify a maximum trial period.  With registration, you get

anything from the simple right to continue using the software to

an updated program.

Copyright laws apply to both shareware and commercial

software, and the copyright holder retains all rights, with a

few specific exceptions as stated below. Shareware authors are

accomplished programmers, just like commercial authors, and the

programs are of comparable quality. (In both cases, there are

good programs and bad ones!) The main difference is in the method

of distribution. The author specifically grants the right to copy

and distribute the software, either to all or to a specific group.

For example, some authors require written permission before a

commercial disk vendor may copy their Shareware.

Shareware is a distribution method, not a type of software.

You should find software that suits your needs and pocketbook,

whether it's Commercial or Shareware. The Shareware system makes

fitting your needs easier, because you can try it before you buy.

And because the overhead is low, prices are low also. Shareware

has the ultimate money-back guarantee -- if you don't use the

product, you don't pay for it.

Copyright (c) 1993-1994  All Rights Reserved               Page 1

Energy Cost 2.0  From Electrical Design Software

DISCLAIMER - AGREEMENT

~~~~~~~~~~~~~~~~~~~~~~

Users of ENERGY COST 2.0 must accept this disclaimer of

warranty: ENERGY COST 2.0 is supplied as is.  Electrical Design

Software disclaims all warranties, expressed or implied, including,

without limitation, the warranties of merchantability and of fitness

for any purpose. The author assumes no liability for damages, direct

or consequential, which may result from the use of ENERGY COST 2.0.

ENERGY COST 2.0 is a "Shareware Program" and is provided at

no charge to the user for EVALUATION.  Feel free to share it with

your friends, but please do not give it away altered or as part of

another system.  The essence of "user-supported" software is to

provide personal computer users with quality software without high

prices, and yet to provide incentive for programmers to continue to

develop new products.

If you use ENERGY COST 2.0 after a 30 day trial period, you MUST

make a registration of $64.00 to Electrical Design Software.  The

*$64.00 registration fee will license one copy for use on any ONE

computer at any one time.

You must treat this software just like a book. An example is

that this software may be used by any number of people and may be

freely moved from one computer location to another, so long as

there is no possibility of it being used at one location while it's

being used at another. Just as a book cannot be read by two

different persons at the same time.

Commercial users of ENERGY COST 2.0 must register and pay for

their copies of ENERGY COST 2.0 within 30 days of first use or their

license is withdrawn.  Site or distribution license arrangements

may be made by contacting Electrical Design Software.

Anyone distributing ENERGY COST 2.0 for any kind of

remuneration must first contact Electrical Design Software at the

address listed for authorization. This authorization will be

automatically granted to distributors recognized by the (ASP) as

adhering to its guidelines for shareware distributors, and such

distributors may begin offering ENERGY COST 2.0 immediately

(However Electrical Design Software must still be advised so that

the distributor can be kept up-to-date with the latest version of

ENERGY COST 2.0).

You are encouraged to pass a copy of ENERGY COST 2.0 along to

your friends for evaluation.  Please encourage them to register

their copy if they find that they can use it.  All registered users

will receive a copy of the latest version of ENERGY COST

Copyright (c) 1993-1994  All Rights Reserved               Page 2

Energy Cost 2.0  From Electrical Design Software

WHAT IS ENERGY COST - VERSION 2.0

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Energy 2.0 will produce TOTAL or COMPARISON reports, in cost

kilowatt hours or demand charges.

With ENERGY 2.0 you can quickly identify the cost of one lighting system

to another, determine the savings on changing to energy efficient equipment

to calculate a pay back period, analyze the savings by improving the power

factor, compare usage at peak & off-peak times, etc.  Also, the electric

bill information screen helps in calculating the cost per peak and off-peak

kilowatt hours. Built-in calculator.  Ampere averaging screen to calculate

for rapidly changing loads. Complete data base system allows you to save and

recall valuable energy usage records.

New England Business Service, Inc offers compatible forms for all Electrical

Design Software programs .  The advantage of forms is to enhance the

appearance of the printed reports.

To order forms phone.............(800) 388-3810

Multipurpose Form ...............9331

Mail code number ................11028

ORDERING  ENERGY COST - VERSION 2.0

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

You can order directly from Electrical Design Software by filling in

the registration form and enclosing a check, OR

Credit card orders ONLY are available from Public (software) Library

with your MC, Visa, AmEx, or Discover card by calling

Voice phone (800)-242-4775              By fax (713)-524-6398

From overseas (713)-524-6394         By CompuServe to 71355.470

These numbers are for ORDERING only.  Electrical Design Software can

NOT be reached at those numbers. To contact Electrical Design Software

for information about dealer pricing, volume discounts, site or

distribution licensing, the status of a product shipment, the latest

version number, for technical information, or to discuss returns,

call (603)-893-9008 or write Electrical Design Software, 12 Vassar

Drive, Pelham, NH 03076.

SYSTEM REQUIREMENTS

~~~~~~~~~~~~~~~~~~~

ENERGY COST 2.0 requires the following hardware and/or software

to operate:

1.  640K memory - 286 or higher CPU.

2.  IBM(R) PC/XT/AT or true "compatible" running MS-DOS or PC-DOS

Operating System version 3.1 or higher.

3.  One high-density floppy-disk drive or a hard disk drive (hard

disk recommended)

4.  Expanded memory suggested, but not required.

5.  Use of mouse is optional.

Copyright (c) 1993-1994  All Rights Reserved               Page 3

Energy Cost 2.0  From Electrical Design Software

FILES

~~~~~

You have the following files for the ENERGY COST 2.0 program:

1.  ENERGY.EXE      Main program

2.  ENERGY.MDB      Data base file will be created on first start

3.  ENERGY.HLP      Help screens

4.  ENERGY.DOC      Documentation

5.  ENERGY.INI      Initialization file

6.  FEEDBACK.DOC    Program feedback

7.  ISAMPACK.EXE    Utility to pack data file. (IN REG. VERSION)

8.  ISAMREPR.EXE    Utility to repair a damaged data file.(IN

REG.VERSION)

COMMAND LINE OPTIONS

~~~~~~~~~~~~~~~~~~~~

When starting ENERGY you can set the monitor to 43 or, 50

line mode from the command line with the following command.

For 43 line mode type ENERGY 43 then press ENTER

For 50 line mode type ENERGY 50 then press ENTER

On 43 line mode the main work screen will be at the top leaving

a lower section to pull down the other screens. On 50 line mode

the main work screen will be at the top and the Names screen

will setup at the bottom. The main work screen is stationary

while all other screens are moveable and some are re-sizeable.

Moving windows

��������������

With the keyboard use Ctrl + F7 to activate the window move

feature. Use the arrow keys to place the window at the

desired location. Press ENTER to set the window in the new

location. The cursor must be on the window you want to move

when using the keyboard method.

With the mouse click the top bar and drag to the desired

location.

Copyright (c) 1993-1994  All Rights Reserved               Page 4

Energy Cost 2.0  From Electrical Design Software

ENERGY COST TUTORIAL

~~~~~~~~~~~~~~~~~~~~

Welcome to Energy Cost - a professional software program

that is especially designed for Engineers, Energy Consultants,

Electrical Contractors, Electrical Power Companies and students.

Gain the competitive edge with advanced PC technology and

individual know-how.

This tutorial is designed to familiarize you with the major

features that Energy Cost has been widely known for.  You

will feel at ease with these step-by-step instructions.  So

lets begin!

1.  First, start Energy Cost by going to the directory containing

the Energy Cost files. This may be the default directory

ENERGY  or any other directory that you may have specified

during installation. From that directory type ENERGY, then

press ENTER.  This will load the program and present the

opening screen.

2.  You can begin this tutorial by pressing the ENTER key or SPACE

BAR. This will access the main work screen.

Remember to display a help message about the task you are

performing, press F1.  To exit help press ESC.

3.  We will continue  by entering EXISTING LIGHTING in the tag

field.  The tag field is used to characterize the information

on the main work screen. As we gather various records the tag

field is one of five indexes we can use to find a specific

record.  Pressing ENTER will advance to the next field.

We must now enter the cost per kilowatt hour at peak time.  We

can calculate this using the electric bill information screen.

4.  To calculate the cost per kilowatt hour access the electric bill

information screen by selecting (calculate kilowatt hour cost)

from the menu under utility. Or for quick access from the main

work screen press the Ctrl + K keys.

5.  We will continue by entering the information on our electric

bill. This screen is not only a utility but this information

will be saved in the data file ENERGY.MDB along with the main

work screen and the names screen a one record. Lets go over

each field.

Copyright (c) 1993-1994  All Rights Reserved               Page 5

Energy Cost 2.0  From Electrical Design Software

6.  The meter number and service period are for two purposes.

. When printing a report explaining the cost per kilowatt hour.

. For future reference

Type in the sample information below.

Example:

Meter #:  A12345678

Service period:  Nov 30 to Dec 30 1993    30 days

-----------------------------------------------------------------

7.  Continue with the Peak Kilowatt hours. This is the total

kilowatt hours used during peak time. You can find this on

the electric bill under the heading of KWH Usage. If your

electric bill has no off-peak categories use the peak rate

fields to insert the information.

Example electric bill:

Meter readings            KWH

RATE             Present  Previous         USAGE

~~~~~~~  ~~~~~~~~         ~~~~~

Peak              20523     19085          86280  <--- Here

Off-Peak          16879     15604          76500

Enter the value into the Peak KWH: field.  Press ENTER.

-----------------------------------------------------------------

8.  Now we want to enter the cost peak KWH usage $.  This is the

usage charge we pay for peak time kilowatt hours.

Example electric bill:

Usage Charge:        86280  KWH  X  .03784  =  3264.84  < --- Here

Enter the value into the cost peak KWH usage $: field.  Press ENTER.

As soon as we press enter,  ENERGY will calculate the base cost per

peak kilowatt hour. Please note two things at this point.

1. Some electric bills show the cost per kilowatt hour in dollar

format (.03784) = 3.784 cents. ENERGY shows the cost per kilowatt

hour in cents format (3.7840) = 3.784 cents.

2. ENERGY carries the calculation one additional decimal space to

the right.  This will become more significant for large loads

on for long periods of time.

Copyright (c) 1993-1994  All Rights Reserved               Page 6

Energy Cost 2.0  From Electrical Design Software

9.  Continue with the Off-Peak Kilowatt hours. This is the total

kilowatt hours used during off-peak time. You can find this

on the electric bill under the heading of KWH Usage.

Example electric bill:

Meter readings             KWH

RATE            Present  Previous          USAGE

~~~~~~~  ~~~~~~~~          ~~~~~

Peak              20523    19085           86280

Off-Peak          16879    15604           76500  <--- Here

Enter the value into the Off-Peak KWH: field.  Press ENTER.

-----------------------------------------------------------------

10.  Now we want to enter the cost off-peak KWH usage $. This is

the usage charge we pay for off-peak time kilowatt hours.

Example electric bill:

Usage Charge:        86280  KWH  X  .03784  =  3264.84

76500  KWH  X  .02346  =  1794.69  <--- Here

Enter the value into the cost off-peak KWH usage $: field. Press ENTER.

As soon as we press enter ENERGY will calculate the base cost per

off-peak kilowatt hour.

-----------------------------------------------------------------

11. To get a true value of our cost per kilowatt hour we must add

any other charges that may be on the electric bill. Lets start

with the fuel charge. What ENERGY will do is take the total

cost for the fuel charge and distribute it among the peak and

off-peak kilowatt hours.

12. Enter the cost for fuel charge,  3838.35  then press ENTER.

. We can now see that the cost per kilowatt hour has

adjusted upward.

13. Again we may have additional charges, However this time we can

put in any heading we want such as:

. Type in (Oca charge $)     Press ENTER.

. Put in the cost.  188.82   Press ENTER.

14. Again we may have additional charges,

. Type in (Esc charge $)     Press ENTER.

Put in the cost.     .33   Press ENTER.

The cost per kilowatt adjusted upward. Lets put one more.

Copyright (c) 1993-1994  All Rights Reserved               Page 7

Energy Cost 2.0  From Electrical Design Software

15. Type in (Rate settlement credit $)  then press ENTER.

Put in the cost .  -133.48  of the credit then press ENTER.

Yes ENERGY will take negative numbers and adjust the per

kilowatt hour rate accordingly.  We now have the true cost

per kilowatt hour based on our actual electric bill.

-----------------------------------------------------------------

16. We have a demand charge on our electric bill so lets continue

otherwise we would jump ahead to the section on sending the

rates to the main screen. If we did not have a demand charge

on the electric bill, leaving the Demand $ field of the main

screen empty will automatically take demand reporting out of

the reports.

-----------------------------------------------------------------

17. Bring the cursor to the Per KVA cost $. In this area we can do

some very important calculating to determine the true cost of

demand charges.  Most power companies charge the higher of 90%

of the actual KVA or 100% of the actual KW .  We need to know

the demand charge information to properly set the demand

adjustment factor and calculate the demand charges. Lets start

by entering the cost per KVA as listed on our electric bill.

Example electric bill:

Demand Charge:   461.1 KVA  X  8.490  =  3914.74

^

|

Here

Enter the value into the Per KVA cost $: field. Press ENTER.

-----------------------------------------------------------------

18. Enter the total cost for demand charges at Demand $.

Example electric bill:

Demand Charge:   461.1 KVA  X  8.490  =  3914.74

^

|

Here

Enter the value into the Demand $: field.  Press ENTER.

As soon as we press ENTER we can confirm our billed KVA to be

461.1 KVA. What ENERGY did was to divide $3914.74 by $8.49

Copyright (c) 1993-1994  All Rights Reserved               Page 8

Energy Cost 2.0  From Electrical Design Software

19. Enter the Actual KVA

Example electric bill:

Actual

Demand

~~~~~

512.4  KVA <--- Here

417.0  KW

Enter the value into the Actual KVA: field.  Press ENTER.

As soon as we press ENTER we can confirm that we are getting billed

for 90% of our actual demand. 461.1 billed KVA is 90% of the 512.4

actual KVA. This also confirms that our power factor is 90% or less.

For example if our actual KVA is 512.4 and our power factor is 92%

then our billed KVA would have been 92% of 512.4 or 471.4 billed KVA.

-----------------------------------------------------------------

20. The billed KVA is already entered into the billed KVA field.

If a change is made in the billed KVA field, the Demand $ field

and the Adjustment Factor will adjust accordingly. Press ENTER

to continue to Actual KW.

-----------------------------------------------------------------

21. Enter the Actual KW

Example electric bill:

Actual

Demand

~~~~~

512.4  KVA

417.0  KW   <--- Here

Enter the value into the Actual KW: field.  Press ENTER.

This value is used to calculate a power factor for your service based on

the actual KW divided by the actual KVA times 100 = Service Power Factor.

This information will print on the Print cost per KWH report.

Example: 417KW  \  512.4KVA  X  100  =  81.38% Power Factor

Copyright (c) 1993-1994  All Rights Reserved               Page 9

Energy Cost 2.0  From Electrical Design Software

22. The adjustment factor is already entered into the adjustment

factor field. This value may be changed if a different billing

format is being used.

The adjustment factor is used in calculating demand charges. For

example if we have a 10 KVA load and the cost per KVA is $8.490

we would expect the charges for our 10 KVA load to be $84.90.

However since we are paying for only 90% of the actual KVA load

our true cost for the 10 KVA load is.

10 KVA  X  8.490 per KVA  X  .90 adjustment factor  =  $76.41

With all this information in place press the Send Rates command button by

pressing the Alt + S keys or click it with the mouse.  This will place the

rates in their respective locations and we can enter the loads.

-----------------------------------------------------------------

23. At this point we have defined four cost parameters.

1. Cost per peak kilowatt hour.

2. Cost per off-peak kilowatt hour.

3. Cost per KVA of demand.

4. The adjustment factor.

24. Lets continue by entering some loads. Our first study will be to

analyze the lighting in the office area of the all American flag

manufacturing company. Bob the facilities engineer has a proposal

to convert some lighting to a more energy efficient system.

Bob must determine EXACTLY what the new lighting will save each

month before making any decisions.

25. What we have in the office area is a total of 155 - 2X4 4 lamp

recessed fluorescent fixtures. Our first objective is to list the

loads and the times they are on. The office personal work from

7:30 am to 4:30 pm Monday through Friday with the lighting on a half

hour before and a half hour after each work day.

26. Enter the statistics for the existing lighting system as shown below.

Load               Volts   Amps  Phase  P-F   KVA   P-H   O-H   P-D  Op-D

~~~~               ~~~~~   ~~~~  ~~~~~  ~~~   ~~~   ~~~   ~~~  ~~~~  ~~~~

150 - 2x4 4 lamp    120   240.0    1     98  28.8   9.0   1.0    23    23

5 - 2x4 4 lamp    120     8.0    1     98   1.0  13.0  11.0    23    23

5 - 2x4 4 lamp    120     8.0    1     98   1.0        24.0           8

Copyright (c) 1993-1994  All Rights Reserved               Page 10

Energy Cost 2.0  From Electrical Design Software

27.     Line 1: Our first load is the 150 fixtures that are working at 120

volts.  The total Amps for the 150 fixtures is .8 amps per

ballast times 2 ballast per fixture times 150 fixtures =

240 amps. Since the amps are based on single phase we have

a 1 in the phase field.

Please note that we could base the amps on 3 phase as long as the

KVA remains at 28.8

Example: A load of 240   amps at 120 volts 1 phase = 28.8 KVA

A load of  79.9 amps at 208 volts 3 phase = 28.8 KVA

The power factor of the existing ballast is 98% therefore put 98

in the PF% field. Most high power factor ballast operate from

10 % to 2% of unity or a power factor of 90% to 98%.

We listed our load description for the 150 fixtures that are on

from 7:00 am to 5:00 pm Monday through Friday. Since the peak time

for this utility company is from 8 am to 9 pm Monday through Friday.

The 150 fixtures are on for 1 hour during off-peak time and 9 hours

at peak time. This occurs for 23 peak days during this particular month.

28.     Line 2: We also have 5 fixtures that are night lights and never

shut off. These fixtures are on during PEAK AND OFF-PEAK

DAYS. Lets first consider usage on peak days. The 5

fixtures are on for 24 hours a day OR 13 hours per day at

peak time and 11 hours per day at off-peak time for 23

peak days.

Unlike the 150 fixtures on line 1 that operate on peak days

only.  We need to define the 5 fixtures two times once for

peak days (line 2) and once for off-peak days (line 3).

(Line 2 covers Monday through Friday)

29.     Line 3: The same 5 fixtures are also on for 24 hours a day at

off-peak time for 8 off-peak days.

(Line 3 covers Saturday & Sunday)

30. We have just established the criteria for the existing lighting

system. To complete this record we need to put a name on this record

by accessing the names screen.  Press the Alt + S keys or click the

screen command button with the mouse.

31. Fill in the prepared for and prepared by information.  In the

header #1 field type the following.

Existing lighting 155 - 2x4  4 lamp  std ballast, std lamps.

Copyright (c) 1993-1994  All Rights Reserved               Page 11

Energy Cost 2.0  From Electrical Design Software

32. Lets return to the main work screen by pressing the Alt + S

keys or click the screen command button with the mouse or the easy

way by pressing the Esc key once.

33. From here we will enter the proposed lighting into a new record.

We can do this two ways.  By selecting New Record and entering the

information for the proposed lighting system into a new record.  Or

copy the existing lighting record into a new record and change the

relevant fields.

34. Lets copy the existing lighting record by selecting (copy this

record to a new record) from the menu. When ever we leave a record

for any reason ENERGY will check if the record has been created or

changed in any way and verify that we want to update this record.

35. Select yes.  Energy will update the existing lighting record and

copy this record to a new record.  We now have two copies of the

same record, we will modify the 2nd record to become our proposed

lighting.

36. Start by entering (Proposed lighting) in the TAG field. Since the

Voltage, Phase, Power Factor & Times are the same we need only change

the KVA or Amps field. If we change the Amps field energy will

automatically update the KVA. If we change the KVA field energy will

automatically update the Amps field.

37. Our proposed fixture is a 2 lamp fluorescent fixture with a

reflective housing, T8 lamps and an electronic ballast. The amp rating

on the ballast is .52 amps. 150 fixtures times .52 amps = 78 amps.

Enter 78 in the Amps field.

38. The Amps for the 5 fixtures is .52 times 5 fixtures = 2.6 amps.

Enter 2.6 in the Amps field.

39. Enter 2.6 amps for line 2 and 3. We now have the information for

the existing and proposed lighting systems.

40. Go to the names screen and enter the following in header #1.

Proposed lighting 155 - 2x4  2 lamp T8, Electronic ballast.

41. Lets print a report for the existing & proposed lighting systems. But

first a few notes about going from record to record.

Copyright (c) 1993-1994  All Rights Reserved               Page 12

Energy Cost 2.0  From Electrical Design Software

42. Records are displayed in an ascending order based on the current

index. The default index when ENERGY is started is the TAG field.

Therefore we can go from record to record by:

1. Select the Next command button to go to the next record.

2. Select the Prev command button to go to the previous record.

3. Select the Find command button to bring up a dialogue box.

Enter the search criteria. EXAMPLE: Type  Existing  then press

ENTER. This will go to the record with the word Existing in the

tag field or the record with the next closest match.

4. When the cursor is on the TAG field press Page Up to go to the

previous record and Page Down to go to the next record.

43. Go to the EXISTING LIGHTING record & print a COST TO OPERATE WITH

TOTALS report then do the same for the PROPOSED LIGHTING record.

44. TO PRINT A REPORT

Press ALT to activate the main menu.

Select Reports, press ENTER.

Highlight the  Cost to Operate with Totals  press ENTER.

Accept the default or select your specific printer port, press ENTER.

Your printer is now printing a detailed report.

45. As we examine the report we can see the total cost for the existing

and proposed lighting systems to be.

Existing  $656.52 per month

Proposed  $213.49 per month

=======

Monthly Savings  $443.03 per month

Therefore $443.03 per month X 12 months = $5316.36 per year

With this solid information in hand Bob can make a positive decision

if his company should invest in the new lighting system

Congratulations!  You have just completed the ENERGY COST 2.0 tutorial.

Copyright (c) 1993-1994  All Rights Reserved               Page 13

Energy Cost 2.0  From Electrical Design Software

SPECIAL KEYS

~~~~~~~~~~~~

ENERGY COST 2.0 uses the following operating functions.

1.   ENTER         -  Accept input & advance to the next field.

2.   TAB           -  Advance to the next field. When in a help

window pressing tab will jump from one hyper-

link to the next.

3.   SHIFT + TAB   -  Move to the previous field.

4.   ALT           -  Activate the top Menu.

5.   ALT + HOT KEY -  Select menu item or command button.  Hot keys

are highlighted in white.

6.   ESC           -  Move towards the main screen & out of the program.

7.   NAVIGATION    -  Up or down arrows keys.

8.   PAGE UP       -  When at the prepared for name, page up will

move to the next record.

9.   PAGE DOWN     -  When at the prepared for name, page down will

move to the previous record.

10.  MOUSE         -  Select any field or button.

11.  F1            -  Context help.

12.  Ctrl + F4     -  Close a window.

13.  Ctrl + F6     -  Activate next window.

14.  Ctrl + F7     -  Move a window.

Copyright (c) 1993-1994  All Rights Reserved               Page 14

Energy Cost 2.0  From Electrical Design Software

QUICK ACCESS KEYS FROM THE MAIN SCREEN

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

15.  Ctrl + X      -  Exit program

16.  Ctrl + T      -  Set index to tag.

17.  Ctrl + P      -  Move to previous record.

18.  Ctrl + N      -  Move to next record.

19.  Ctrl + F      -  Find record screen.

20.  Ctrl + A      -  Quick access to ampere averaging screen.

21.  Ctrl + K      -  Quick access to Calculate KWH Cost screen.

22.  Ctrl + C      -  Copy the contents of this record to

a new record.

23.  Ctrl + H      -  Quick access to the contents help window.

24.  Ctrl + S      -  Quick access to the search help window.

COMMAND BUTTONS MAIN WORK & NAMES SCREEN

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

New Rec....... Create a new record.

Update........ Update the current record. When a record is changed

in any way, a message will ask if the program can

update the record for you before you exit.

Delete........ Delete the current record.

Previous...... Move to the previous record in the indexing sequence.

Next.......... Move to the next record in the indexing sequence.

Find.......... Find a specific record. The search will take place

on the field that is the current index.

Calc.......... Bring up a pop up calculator.

Screen........ Toggle the Name screen.

Exit.......... Exit the program.

Esc........... The Esc button will move out of the program.

Copyright (c) 1993-1994  All Rights Reserved               Page 15

Energy Cost 2.0  From Electrical Design Software

COMMAND BUTTONS CALCULATE KILOWATT HOUR RATE & AMPERE AVERAGING

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Send Rates............ Calculate the information on the electric

billing information screen and send this

information to the main work screen.

Clear Fields.......... Clear the current screen.

Calculator............ Bring up a pop up calculator.

Main Screen........... Exit this screen & access the main work screen.

HELP SYSTEM

~~~~~~~~~~~

The help system is a context help system and hyper-text Help

system.  Some help screens have hyper links that relate to the

subject in discussion.

F1  =  Pressing the F1 key on any field will bring up context help.

SELECTING A HELP LINK

~~~~~~~~~~~~~~~~~~~~~

With the keyboard Press TAB or Shift-TAB to select a link.

Press ENTER to accept and go to that subject.

With a MOUSE     - Click on phrases outlined by two triangles.

SCROLLING THE HELP WINDOW

~~~~~~~~~~~~~~~~~~~~~~~~~

With KEYBOARD    - press the cursor navigation keys.

With a MOUSE     - click on the horizontal or vertical scroll bars.

CLOSING THE HELP WINDOW

~~~~~~~~~~~~~~~~~~~~~~~

With KEYBOARD    - Press Esc or Ctrl-F4.

With a MOUSE     - Double click on the window's control box icon,

top left.

Contents         - Displays a Table of Contents for the application's

HELP. Press C to select Contents.

Copyright (c) 1993-1994  All Rights Reserved               Page 16

Energy Cost 2.0  From Electrical Design Software

Search           - Opens a dialogue box that lists all available HELP

topics.  Select a topic from the list box and press

ENTER. Press S to select Search.

Back             - Jumps to the last HELP topic viewed. Press B to

select Back.

History          - Lists the last 20 HELP topics viewed. Jump to a

topic by selecting from the list box and pressing

ENTER. Press T to select History.

Copy             - Copies the contents of the current HELP topic into

an edit area. Select the text you want copied to

the clipboard and choose copy. Choosing copy

without selecting any text in the edit area copies

the entire HELP topic to the clipboard. Press P to

select copy.

GENERAL OVERVIEW OF THE MAIN MENU SELECTIONS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

REPORTS

~~~~~~~

All reports can be sent to the printer or an ASCII file.

Kilowatt Hour, Cost        Daily kilowatt hour usage and cost,

& Demand ...............   monthly kilowatt hour usage, cost and

demand charge. Each line item is broken

down into a detailed report.

Kilowatt Hour Usage        Kilowatt hour usage with line item totals

with Totals .............  and group totals.

Kilowatt Hour Usage        Kilowatt hour usage with line item totals

comparison ..............  only.

Cost to Operate            Cost to operate with line item totals

with Totals .............  and group totals.

Cost to Operate            Cost to operate with line item totals

comparison ..............  only.

Demand Charges ..........  Demand charge summary, user can select

from the reports menu to report on peak

time loads only or peak & off-peak loads.

Copyright (c) 1993-1994  All Rights Reserved               Page 17

Energy Cost 2.0  From Electrical Design Software

Print Field Form ........  Print a form that can be used to collect

information in the field.

Calculating Demand

on Peak Only ............  Toggle if you want the reporting for the

demand charges to be based on the default

peak time only or report the demand charges

for the loads on off-peak also.

Exit.....................  Exit ENERGY.

DATA BASE

~~~~~~~~~

Set Index to Tag.........  Index records on the Tag field.

First Record.............  Make the first record current.

Last Record..............  Make the Last record current.

Create New Record........  Create a new record.

Update...................  Update the current record.

Delete...................  Delete the current record.

Previous.................  Move to the previous record in the

indexing sequence.

Next.....................  Move to the next record in the indexing

sequence.

Find.....................  Find a specific record. The search will

take place on the field that is the

current index.

UTILITY

~~~~~~~

Calculate Kilowatt         Bring up the electric bill information

Hour Cost................  screen  Use this work screen to calculate

peak, off-peak and demand charges.

Ampere Averaging.........  Bring up the ampere averaging screen. Use

this work screen to calculate the average

amperes for rapidly changing loads.

Copy this record to

a new record.............  This will take the current record copy the

contents and create a new duplicate record.

Copyright (c) 1993-1994  All Rights Reserved               Page 18

Energy Cost 2.0  From Electrical Design Software

HELP

~~~~

Contents ................  Various help hyper-link topics.

Search ..................  Sort the help links in alphabetical order.

Special Keys ............  Screen navigation and functions.

Using Help ..............  Explains about the help system.

About ...................  Information about this program.

POP-UP CALCULATOR

~~~~~~~~~~~~~~~~~

Bring up a pop-up calculator that can add, subtract, multiply,

divide and do percents. With the mouse you can move the calculator

around. Put the mouse cursor on the top of the calculator, press

the left mouse button and drag to a new screen location.

%  -  percent.

/  -  divide.

*  -  multiply.

-  -  subtract.

+  -  addition.

c  -  clear.

ce  -  clear last entry.

To remove the calculator press one of the functions listed below:

1.    Press ESC,

2.    Click the exit button on the calculator,

3.    Click the mouse on the background screen.

-----------------------------------------------------------------

EACH DATA ENTRY FIELD EXPLAINED

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

TAG

~~~

Use the tag field to identify information on the main screen.

The tag field is the only indexing field for the main screen.

When the cursor is on the tag field you can move to the

previous record by pressing Page UP or move to the next

record by pressing page Down.

Information in the tag field will not print on reports.

The data file will store up to 25 characters for this field.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 19

Energy Cost 2.0  From Electrical Design Software

PEAK RATE

~~~~~~~~~

The cost per peak kilowatt hour.  A peak kilowatt hour is the

use of 1000 watts for one hour during peak hours.

This field will accept a value up to 50 �.

Perform the following to calculate the value for the peak rate:

1. select calculate kilowatt hour cost from the menu.

2. Enter the information from the electric bill

3. Press the send rates command button. This will place the

calculated rate into the main work screen.

-----------------------------------------------------------------

OFF-PEAK RATE

~~~~~~~~~~~~~

The cost per off-peak kilowatt hour. An off-peak kilowatt hour

is the use of 1000 watts for one hour during off-peak hours.

This field will accept a value up to 50 �.

Perform the following to calculate the value for the peak rate:

1. select calculate kilowatt hour cost from the menu.

2. Enter the information from the electric bill

3. Press the send rates command button. This will place the

calculated rate into the main work screen.

-----------------------------------------------------------------

DEMAND CHARGE

~~~~~~~~~~~~~

The Demand Charge is the cost per KVA of demand. This charge is

generally for commercial & industrial users.

This field will accept a value up to $50.000

Example: 461.1 KVA X Demand Charge of $8.490 = $3914.74

An example of how the demand is measured by some utility Companies.

The Peak Hours demand for each month under ordinary Load conditions

shall be the greatest of the following:

A. The greatest fifteen-minute peak occurring during the

peak hours period within such a month as measured in

kilowatts (KW).

B. 90% of the greatest fifteen-minute peak occurring during

the peak hours period of such a month as measured in

kilovolt-amperes (KVA).

Copyright (c) 1993-1994  All Rights Reserved               Page 20

Energy Cost 2.0  From Electrical Design Software

LOAD

~~~~

Identify the load being analyzed.

Example: 25 hp air compressor

The data file will store up to 25 characters for this field.

-----------------------------------------------------------------

AMPS

~~~~

Amperes of the load.

If you make an entry into the Amps field with the Volts &

Phase fields greater than 0, a value will automatically be

calculated & inserted into the KVA field.

This field will accept a value up to 999 amps.

-----------------------------------------------------------------

VOLTS

~~~~~

Enter the voltage.

This field will accept a value up to 15000 volts.

�������������������� Standard AC Voltages  ������������������͸

�                                                             �

�    110 - 120 - 208 - 220 - 240 - 440 - 480 - 550 - 600      �

�                                                             �

�    2,200 - 2,300 - 2,400 - 4,000 - 4,160 - 6,600 - 6,900    �

�                                                             �

�    11,000 - 11,500 - 13,200 - 13,800                        �

�������������������������������������������������������������;

-----------------------------------------------------------------

PHASE

~~~~~

Phase of the load. This field will accept 1 or 3.

When you are at the Phase field, and a value is in the Amps & Volts

fields, a non entry in the Phase field will cause it to default to 3.

-----------------------------------------------------------------

POWER FACTOR

~~~~~~~~~~~~

Power Factor of the load as expressed in percent.

Power factor is the ratio of the true power or watts to the

apparent power or voltamperes. The power factor is expressed

as a decimal or in percentage. Thus power factors of 0.8 and

of 80% are the same.

This field will accept a value from 1 to 100.

Copyright (c) 1993-1994  All Rights Reserved               Page 21

Energy Cost 2.0  From Electrical Design Software

KVA

~~~

KVA or apparent power of the load. When the Amps, Volts & Phase

fields are greater than 0 a change in the KVA field will

automatically calculate & adjust the Amps.

This field will accept a value up to 25,000 KVA.

-----------------------------------------------------------------

PEAK HOURS

~~~~~~~~~~

Enter the peak hours per day.

This field will accept a value up to 24 hours.

Check with the customer representative for your area to confirm

the peak hours as applied to your electric bill.

The following is a typical schedule of peak hours.

Peak hours will be from 8:00 am TO 9:00 pm daily Monday through

Friday, excluding holidays.

The utility company reserves the right to change these peak and

off-peak hours, but in no case will the off-peak hours be less

than eleven hours a day.

-----------------------------------------------------------------

OFF-PEAK HOURS

~~~~~~~~~~~~~~

Enter the off-peak hours per day.

This field will accept a value up to 24 hours.

Check with the customer representative for your area to confirm

the off-peak hours as applied for your electric bill.

The following is typical schedule of off-peak hours.

Off-peak hours will be from 9:00 pm TO 8:00 am daily Monday

through Friday, and all day on Saturdays, Sundays and holidays.

The utility company reserves the right to change these peak and

off-peak hours, but in no case will the off-peak hours be

less than eleven hours a day.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 22

Energy Cost 2.0  From Electrical Design Software

PEAK DAYS

~~~~~~~~~

Enter the number of days that the Load is using power during

the peak hours period.

This field will accept a value up to 9,999 Days.

The following is typical schedule for peak days.

8:00 am TO 9:00 pm daily Monday through Friday,

excluding holidays.

-----------------------------------------------------------------

OFF-PEAK DAYS

~~~~~~~~~~~~~

Enter the number of days that the Load is using power during

the off-peak hours period.

This field will accept a value up to 9,999 Days.

The following is typical schedule of off-peak days.

9:00 pm TO 8:00 am daily Monday through Friday, and all day on

Saturdays, Sundays and holidays.

-----------------------------------------------------------------

PEAK KWH

~~~~~~~~

The value that reflects usage during peak time.

The Peak, Off-Peak and Total Kilowatt Hours will always be

displayed on the KILOWATT HOUR BAR of the main screen.

The Kilowatt Hours will automatically adjust when enter is pressed.

-----------------------------------------------------------------

OFF-PEAK KWH

~~~~~~~~~~~~

The value that reflects usage during off-peak time.

The Peak, Off-Peak And Total Kilowatt Hours will always be

displayed on the KILOWATT HOUR BAR of the main screen.

The Kilowatt Hours will automatically adjust when enter is pressed.

-----------------------------------------------------------------

LOAD DESCRIPTION FOR AMP AVERAGING SCREEN

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Enter a description for this category. This is used when

printing an ampere averaging report.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 23

Energy Cost 2.0  From Electrical Design Software

LOAD AMPS FOR AMP AVERAGING SCREEN

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Enter the amps that reflects the load in the description

and seconds fields.

This field will accept a value up to 999,999 Amps.

-----------------------------------------------------------------

LOAD SECONDS FOR AMP AVERAGING SCREEN

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Enter the time in seconds that the amps is at the value

as shown in the Amps field.

This field will accept a value up to 86,400 Seconds.

-----------------------------------------------------------------

CUSTOMER INFORMATION

~~~~~~~~~~~~~~~~~~~~

The Name, City, State & Zip fields are indexing fields.

To change the index to the prepared for name, select set

index to Name from the top menu.

When the cursor is on the prepared for name field you can move

to the previous record by pressing Page Up or move to the next

record by pressing Page Down.

The default index is the Tag field of the main work screen.

-----------------------------------------------------------------

PREPARED BY INFORMATION

~~~~~~~~~~~~~~~~~~~~~~~

This information will print on all reports under the prepared

by heading.  If the Prepared by NAME field is empty no prepared

by information or heading will print on the report.

-----------------------------------------------------------------

HEADERS AND FOOTERS

~~~~~~~~~~~~~~~~~~~

If information is entered into the header field it will print at

the top of all reports. The header is an excellent location to list

a description or purpose of the report.

If information is entered into the footer field it will print near

the bottom of all reports.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 24

Energy Cost 2.0  From Electrical Design Software

THE ELECTRIC BILL INFORMATION SCREEN

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

METER NUMBER

~~~~~~~~~~~~

Enter the meter number indicated on the electric bill. This is useful

when printing a report or for future reference.

-----------------------------------------------------------------

SERVICE PERIOD

~~~~~~~~~~~~~~

Enter the service period indicated on the electric bill. This is useful

when printing a report or for future reference.

-----------------------------------------------------------------

PEAK COST

~~~~~~~~~

Enter the usage charge on the electric bill for kilowatt hour usage

during peak time.

-----------------------------------------------------------------

OFF-PEAK COST

~~~~~~~~~~~~~

Enter the usage charge on the electric bill for kilowatt hour

usage during off-peak time.

-----------------------------------------------------------------

FUEL CHARGE

~~~~~~~~~~~

Enter the fuel charge on the electric bill.

This dollar amount is distributed among the total peak and

off-peak kilowatt hours to calculate the actual cost per

kilowatt hour.

This field will accept a value up to 9,999,999.00 Dollars.

-----------------------------------------------------------------

LIST CHARGE

~~~~~~~~~~~

Use this field to enter the description of a charge or credit.

You can customize any heading you want, and ENERGY will save that

information in the data base file and use it for the printed reports.

Example: Oca charge

Example: Rate settlement credit

Example: Sales tax

Copyright (c) 1993-1994  All Rights Reserved               Page 25

Energy Cost 2.0  From Electrical Design Software

FIXED COST

~~~~~~~~~~

Enter any charge or credit on the electric bill.

This dollar amount is distributed among the total peak and

off-peak kilowatt hours to calculate the actual cost per

kilowatt hour.

This field will accept a value up to 9,999,999.00 Dollars.

-----------------------------------------------------------------

ADDITIONAL CHARGE

~~~~~~~~~~~~~~~~~

Enter any charge or credit recorded from the electric bill.

This dollar amount is distributed among the total peak and

off-peak kilowatt hours to calculate the actual cost per

kilowatt hour.

This field will accept a value up to 9,999,999.00 Dollars.

-----------------------------------------------------------------

PER KVA COST

~~~~~~~~~~~~

Enter the cost per KVA for the Demand Charge.

This field will accept a value up to 50.00 Dollars.

���������������������������������������������������

When you enter the information from the electric bill the

per KVA cost, Demand$ & the Billed KVA must all reflect the

figures on the electric bill. This will assure ENERGY that the

rates that are sent to the main screen (via the Send Rates)

command button are correct.

To Verify your input ENERGY will:

. Take (Demand$) � (Per KVA cost) to verify the (Billed KVA).

. Take (Per KVA cost) X (Billed KVA) to verify the (Demand$)

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 26

Energy Cost 2.0  From Electrical Design Software

DEMAND COST

~~~~~~~~~~~

Enter the Total Demand Cost.

This field will accept a value up to 9,999,999.00 Dollars.

When you enter the information from the electric bill the

Per KVA cost, Demand$ & the Billed KVA must all reflect the

figures on the electric bill. This will assure ENERGY that the

rates that are sent to the main screen (via the Send Rates)

command button are correct.

To Verify your input ENERGY will:

. Take (Demand$) � (Per KVA cost) to verify the (Billed KVA).

. Take (Demand$) � (Billed KVA) to verify the (Per KVA cost)

-----------------------------------------------------------------

BILLED KVA

~~~~~~~~~~

Enter the Total billed KVA.

This field will accept a value up to 25,000 KVA.

When you enter the information from the electric bill the

Per KVA cost, Demand$ & the Billed KVA must all reflect the

figures on the electric bill. This will assure ENERGY that the

rates that are sent to the main screen (via the Send Rates)

command button are correct.

To Verify your input ENERGY will:

. Take (Per KVA cost) X (Billed KVA) to verify the (Demand$)

. Take (Demand$) � (Billed KVA) to verify the (Per KVA cost)

-----------------------------------------------------------------

ACTUAL KVA

~~~~~~~~~~

Enter the actual KVA as shown on the electric bill.

This field will accept a value up to 25,000 KVA.

This value is used to:

. Calculate the adjustment factor.

. Calculate the Power Factor based on the electric bill information.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 27

Energy Cost 2.0  From Electrical Design Software

ACTUAL KW

~~~~~~~~~

Enter the actual KW as shown on the electric bill.

This field will accept a value up to 25,000 KW.

This value is used to:

. Calculate the Power Factor.

-----------------------------------------------------------------

ADJUSTMENT FACTOR

~~~~~~~~~~~~~~~~~

The adjustment is automatically entered for you when you

enter the Actual KVA & the Billed KVA. This value is used

to adjust the actual cost burden for Demand Charge.

This field will accept a value from 1 to 100%

The reason for the adjustment factor is that most electric

utility companies will allow for a Power Factor of 90%.

Therefore if you have a 10 KVA Load and the power company is

billing for 90% of the total KVA then the billable KVA on the

10 KVA load is 90% of the 10 KVA or 9 KVA. The adjustment factor

will assure that the reports reflect the

true cost that you incurred on the electric bill.

Example: Assume a per KVA cost of $10.00. When we look at the

electric bill we see that the actual KVA is 100 but the billed

KVA is 90.

This would cause the electric bill to look like this:

90 KVA X $10.00 per KVA = a Demand Cost of $900.00

Now lets calculate the demand cost of a motor load with a KVA

demand of 10 KVA. If we took the 10 KVA X $10.00 per KVA the

demand cost would be $100.00 This would be $10.00 higher than

we were actually charged on the electric bill for this motor.

Since we are paying 90% of the actual KVA the formula for the

motor must be

10 KVA  X  $10.00 per KVA  X  .9 adjustment factor = $90.00

This value is used to adjust the demand cost on the reports.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 28

Energy Cost 2.0  From Electrical Design Software

DATA BASE SUBJECTS

~~~~~~~~~~~~~~~~~~

INDEXING

ENERGY has 5 Indexing fields.

Tag

Prepared for Name

Prepared for State

Prepared for City

Prepared for Zip

Records are sorted in an ascending order based on the current

indexing field. When ENERGY is started is the default index is

the Tag field of the main work screen. To change the current

index select the desired index from the Names screen menu.

Four of the five indexes also have multiple sub indexes.

Prepared for Name  > City > State

Prepared for State > Name > City

Prepared for City  > Name > State

Prepared for Zip   > Name > City

Example: The name index will sort based on the name field

first, then as sub-index it will sort on city and

lastly on state.

-----------------------------------------------------------------

DATA BASE

~~~~~~~~~

All records can be saved to disk in a data base file named

ENERGY.MDB. The data base will first show records on an

ascending order based on the Tag field of the main screen.

-----------------------------------------------------------------

DATABASE PACKING UTILITY

~~~~~~~~~~~~~~~~~~~~~~~~

ISAMPACK purges records marked for deletion and recovers disk

space formerly used by deleted records in 32K blocks.

When a record is deleted it is removed from the indexing order.

However the information remains in the data file. Using ISAMPACK

reconstructs the data file with active records only.

Syntax     ISAMPACK  ENERGY.MDB

-----------------------------------------------------------------

DATABASE REPAIR UTILITY

~~~~~~~~~~~~~~~~~~~~~~~

ISAMREPR recovers databases that become corrupted; it recreates

them by systematically evaluating every table and index and using

every piece of internally consistent information in the file.

Syntax     ISAMREPR  ENERGY.MDB

Copyright (c) 1993-1994  All Rights Reserved               Page 29

Energy Cost 2.0  From Electrical Design Software

MAXIMUM FIELD LENGTH

~~~~~~~~~~~~~~~~~~~~

The Maximum Field Lengths for various fields is the data that

will be saved in the data base record.

TAG ...................25

LOAD DESCRIPTION ......20

PREPARED FOR NAME .....25

PREPARED FOR ADDRESS ..25

PREPARED FOR CITY .....20

PREPARED FOR STATE ....20

PREPARED FOR ZIP ......10

PREPARED BY NAME ......25

PREPARED BY ADDRESS ...25

PREPARED BY CITY ......20

PREPARED BY STATE .....20

PREPARED BY ZIP .......10

HEADER ONE ............72

HEADER TWO ............72

FOOTER ................72

-----------------------------------------------------------------

AMPERE AVERAGING UTILITY

~~~~~~~~~~~~~~~~~~~~~~~~

The ampere averaging utility is a convenient way to analyze rapidly

changing loads and calculate an average amp value over a defined

period of time.

For example you may have an injection molding machine with electric

band heaters that heat the product a different parts of the machine.

As you take amp readings you find that the heater load changes every

2 to 30 seconds.

However as you further examine the ampere readings you discover a

repetitive cycle over a 7 minute period. This repetitive cycle

continues for the entire eight hour shift. If we calculate the

average amps for that 3 minute period we can list that value in

the main work screen. With that information we can easily report

the exact cost.

Copyright (c) 1993-1994  All Rights Reserved               Page 30

Energy Cost 2.0  From Electrical Design Software

REPORTING - PRINT OR FILE

~~~~~~~~~~~~~~~~~~~~~~~~~

All reports can be sent to

1.  Printer ports LPT1, LPT2 or LPT3

2.  An ASCII file via a user supplied name.

When sending reports to the Printer you  have the option to

produce multiple copies from 1 - 10 with the copies field.

When sending a report to file ENERGY produces an ASCII file

that you can edit with any word processor. If the default

left and right margins of your word processor are less than

the width of the report it may be necessary to set the

margins to accept the width of the report.

. With replace ENERGY will first check if the file name you

supplied already exist. If so a message box will confirm

if you want to continue and overwrite the existing file

or cancel the operation.

. With append if the file already exist ENERGY will add the

report to the end of the existing file otherwise a new file

will be created. A convenient way to use append is to

produce multiple reports all into the same file.

Energy will produce the following types of reports:

. Cost to Operate, Kilowatt Hour Usage & Demand charges.

. Kilowatt Hour Usage with Totals.

. Kilowatt Hour Usage Comparison.

. Cost to Operate with Totals.

. Cost to Operate Comparison.

. Demand Charges.

-----------------------------------------------------------------

Copyright (c) 1993-1994  All Rights Reserved               Page 31

Energy Cost 2.0  From Electrical Design Software

LIGHTING SCHEDULES

~~~~~~~~~~~~~~~~~~

Below is a general list of the rated WATTAGE for various types of

fixtures. Most ballast today have high power factor ballast that

bring the power factor to within 2% to 10% of unity.

The figures below represent the wattage or true power consumed by

the various fixtures and not the volt-amps or apparent power. Please

consider this if you are using these figures and reporting on the

demand charges.

Example:  A 2 Lamp 4' Standard lamps and Standard ballast fixture is

rated at 94 watts therefore 94 watts / 120 volts = .783 amps.

However one manufacture has the ballast listed at .8 amps. The

volt amps at .8 amps is 120 volts X .8 amps = 96 va

With this information we can list the fixture at .8 amps instead of

the  .783 but in the power factor column we can put 98 instead of 100.

(94 watts / 96volt amps = .979% power factor -> rounded to 98%). Using

this method we would have a report that has a 2% better accuracy on

the demand charges.

STD - Standard ballast or lamp  EEMAG - Energy efficient magnetic ballast

EE  - Energy efficient lamp      ELIG - Electronic or hybird ballast

FOUR-FOOT FLUORESCENT SYSTEMS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Description    Rated Wattage        Description    Rated Wattage

F40T12

1L4'STD/STD         57              1L4'STD/EEMAG       50

2L4'STD/STD         94              2L4'STD/EEMAG       86

3L4'STD/STD        151              3L4'STD/EEMAG      136

4L4'STD/STD        188              4L4'STD/EEMAG      172

1L4'STD/ELIG        46              1L4'EE/STD          50

2L4'STD/ELIG        72              2L4'EE/STD          86

3L4'STD/ELIG       110              3L4'EE/STD         136

4L4'STD/ELIG       144              4L4'EE/STD         172

1L4'EE/EEMAG        40              1L4'EE/ELIG         38

2L4'EE/EEMAG        70              2L4'EE/ELIG         60

3L4'EE/EEMAG       110              3L4'EE/ELIG         90

4L4'EE/EEMAG       140              4L4'EE/ELIG        120

F32T8

1L4'T8/EEMAG        37              1L4'T8/ELIG         30

2L4'T8/EEMAG        70              2L4'T8/ELIG         60

3L4'T8/EEMAG       107              3L4'T8/ELIG         90

4L4'T8/EEMAG       140              4L4'T8/ELIG        120

Copyright (c) 1993-1994  All Rights Reserved               Page 32

Energy Cost 2.0  From Electrical Design Software

F48T12

1L4'F48T12/STD      60              1L4'F48T12EE/STD    50

2L4'F48T12/STD     102              2L4'F48T12EE/STD    82

3L4'F48T12/STD     162              3L4'F48T12EE/STD   132

4L4'F48T12/STD     204              4L4'F48T12EE/STD   164

F48T12HO

1L4'HO/STD/STD      85              1L4'HO/EE/STD       80

2L4'HO/STD/STD     145              2L4'HO/EE/STD      135

3L4'HO/STD/STD     230              3L4'HO/EE/STD      215

4L4'HO/STD/STD     290              4L4'HO/EE/STD      270

FIVE-FOOT FLUORESCENT SYSTEMS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

2L5'STD/STD        135

SIX-FOOT FLUORESCENT SYSTEMS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1L6'STD/STD         95              2L6'STD/STD        173

1L6'VHO/STD/STD    180              2L6'VHO/STD/STD    300

1L6'T8/EEMAG        53              2L6'HO/STD/STD     206

3L6'STD/STD        213

EIGHT-FOOT FLUORESCENT SYSTEMS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

F96T12

1L8'STD/STD        100

2L8'STD/STD        173              2L8'STD/EEMAG      158

3L8'STD/STD        273

4L8'STD/STD        346              4L8'STD/EEMAG      316

1L8'STD/ELIG        70              1L8'EE/STD          83

2L8'STD/ELIG       134              2L8'EE/STD         138

3L8'EE/STD         221

4L8'STD/ELIG       268              4L8'EE/STD         276

1L8'EE/ELIG         60

2L8'EE/EEMAG       123              2L8'EE/ELIG        109

4L8'EE/EEMAG       246              4L8'EE/ELIG        218

HIGH INTENSITY DISCHARGE SYSTEMS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

MERCURY VAPOR

~~~~~~~~~~~~~

40 W Mercury Vapor     50      50 W Mercury Vapor     75

75 W Mercury Vapor     95     100 W Mercury Vapor    120

175 W Mercury Vapor    205     250 W Mercury Vapor    290

400 W Mercury Vapor    455    1000 W Mercury Vapor   1075

Copyright (c) 1993-1994  All Rights Reserved               Page 33

Energy Cost 2.0  From Electrical Design Software

METAL HALIDE

~~~~~~~~~~~~

50 W Metal Halide      85      70 W Metal Halide      95

100 W Metal Halide     130     150 W Metal Halide     195

175 W Metal Halide     205     250 W Metal Halide     300

400 W Metal Halide     455     750 W Metal Halide     825

1000 W Metal Halide    1080    1500 W Metal Halide    1620

HIGH PRESSURE SODIUM

~~~~~~~~~~~~~~~~~~~~

35 W - HPS             45      50 W - HPS             65

70 W - HPS             95     100 W - HPS            130

150 W - HPS            190     200 W - HPS            245

250 W - HPS            295     310 W - HPS            365

400 W - HPS            460    1000 W - HPS           1100

LOW PRESSURE SODIUM

~~~~~~~~~~~~~~~~~~~

35 W - LPS             60      55 W - LPS             85

90 W - HPS            130     135 W - HPS            180

180 W - HPS            230

MOTOR AMPERE SCHEDULES

~~~~~~~~~~~~~~~~~~~~~~

Table 430-148 Full-Load Currents in Amperes

Single-Phase Alternating-Current Motors

��������������������������������������������������������

HP         115v         200v         208v         230v

��������������������������������������������������������

1/6        4.4          2.5          2.4          2.2

1/4        5.8          3.3          3.2          2.9

1/3        7.2          4.1          4.0          3.6

1/2        9.8          5.6          5.4          4.9

3/4       13.8          7.9          7.6          6.9

��������������������������������������������������������

1          16            9.2          8.8          8

1�         20           11.5         11           10

2          24           13.8         13.2         12

3          34           19.6         18.7         17

��������������������������������������������������������

5          56           32.2         30.8         28

7�         80           46           44           40

10         100           57.5         55           50

��������������������������������������������������������

Copyright (c) 1993-1994  All Rights Reserved               Page 34

Energy Cost 2.0  From Electrical Design Software

Table 430-150 Full-Load Currents*

Three-Phase Alternating-Current Motors

��������������������������������������������������������

HP       200v      208v      230v      460v      575v

��������������������������������������������������������

1/2       2.3       2.2       2.0       1.0       0.8

3/4       3.2       3.1       2.8       1.4       1.1

1         4.1       4.0       3.6       1.8       1.4

��������������������������������������������������������

1�        6.0       5.7       5.2       2.6       2.1

2         7.8       7.5       6.8       3.4       2.7

3        11.0      10.6       9.6       4.8       3.9

��������������������������������������������������������

5        17.5      16.7      15.2       7.6       6.1

7�       25.3      24.2      22.0      11.0       9.0

10        32.2      30.8      28.0      14.0      11.0

��������������������������������������������������������

15        48.3      46.2      42.0      21.0      17.0

20        62.1      59.4      54.0      27.0      22.0

25        78.2      74.8      68.0      34.0      27.0

��������������������������������������������������������

30        92.0      88.0      80.0      40.0      32.0

40       119.6     114.4     104.0      52.0      41.0

50       149.5     143.0     130.0      65.0      52.0

��������������������������������������������������������

60       177.1     169.4     154.0      77.0      62.0

75       220.8     211.2     192.0      96.0      77.0

100       285.2     272.8     248.0     124.0      99.0

��������������������������������������������������������

125       358.8     343.2     312.0     156.0     125.0

150       414.0     396.0     360.0     180.0     144.0

200       552.0     528.0     480.0     240.0     192.0

��������������������������������������������������������

POWER FACTOR SCHEDULES

~~~~~~~~~~~~~~~~~~~~~~

Many of the newer energy efficient motors have a

power factor rating listed on the name plate. This

value would represent the power factor at rated load.

The most accurate method for determining power factor would

be to measure the specific circuit with a power factor meter.

Below is a table showing average power factors for various

3 phase 1800 RPM motors.

These values will vary from manufacture to manufacture as

well as for different RPM values. Lower RPM motors have

lower power factor ratings and higher RPM motors have higher

power factor ratings.

Copyright (c) 1993-1994  All Rights Reserved               Page 35

Energy Cost 2.0  From Electrical Design Software

Average Power Factors for Standard Industrial Design

3 phase Squirrel-Cage Induction Motors.

�����������������������������������������

Hp     1/2 load   3/4 load   Full load

���������������������������������������������

1/2   �    45    �    56    �    65

3/4   �    48    �    58    �    65

1    �    57    �    69    �    76

1�   �    64    �    76    �    81

2    �    68    �    79    �    84

3    �    70    �    80    �    84

5    �    76    �    83    �    86

7�   �    77    �    84    �    87

10    �    77    �    86    �    88

15    �    81    �    85    �    87

20    �    82    �    86    �    87

25    �    82    �    86    �    87

30    �    83    �    86.5  �    87

40    �    84    �    87    �    88

50    �    84    �    87    �    88

60    �    84    �    87    �    88

75    �    84    �    87    �    88

100    �    84    �    88    �    88

125    �    84    �    88    �    89

150    �    84    �    88    �    89

200    �    85    �    89    �    90

250    �    84    �    89    �    90

300    �    84    �    89    �    90

���������������������������������������������

Typical Power Factors

���������������������

1 phase induction motors  1/20 to 1hp ....... 55 to 75

average at rated load ....... 68

1 phase induction motors 1hp  to 10hp ........75 to 86

average at rated load ........82

Incandescent lighting ........100

Electric resistance heaters ........100

Fluorescent lighting ........50 - 95

H.I.D. lighting ........50 - 95

Copyright (c) 1993-1994  All Rights Reserved               Page 36

Energy Cost 2.0  From Electrical Design Software

FORMULAS

~~~~~~~~

OHM'S LAW 1 PHASE

�����������������

Watts   =   E  x I    =   Volts  X  Amps

Watts   =   E� � R    =   (Volts squared) � Ohm's

Watts   =   I� x R    =   (Amps squared) X  Ohm's

���������������������������������������������������

Volts   =   I X R     =   Amps X  Ohm's

Volts   =   P � I     =   Watts   �  Amps

Volts   =   �(PxR)    =   Square of (Watts X Ohm's)

���������������������������������������������������

Amps    =   P � E     =   Watts   �  Volts

Amps    =   E � R     =   Volts   �  Ohm's

Amps    =   �(P � R)  =   Square of (Watts � Ohm's)

���������������������������������������������������

Ohm's   =   E � I     =   Volts �  Amps

Ohm's   =   E� � P    =   (Volts squared) � Watts

Ohm's   =   P � I�    =   Watts �  (Amps squared)

���������������������������������������������������

OHM'S LAW FOR A 3 PHASE LOAD WITH A POWER FACTOR OF 100%

���������������������������������������������������������

Watts   =   E  x I x 1.732   =   Volts  X  Amps  X  1.732

���������������������������������������������������������

Volts   =   P � (I X 1.732)  =   Watts  � (Amps  X  1.732)

���������������������������������������������������������

Amps    =   P � (E X 1.732)  =   Watts  � (Volts X  1.732)

���������������������������������������������������������

OHM'S LAW FOR A 3 PHASE LOAD WITH ANY POWER FACTOR

��������������������������������������������������

Watts   =   Volts  X  Amps  X  1.732  X  POWER FACTOR

���������������������������������������������������������

Volts   =   Watts  � (Amps  X  1.732  X  POWER FACTOR)

���������������������������������������������������������

Amps    =   Watts  � (Volts X  1.732  X  POWER FACTOR)

���������������������������������������������������������

Power Factor = Watts  �  (Amps  X  Volts  X  1.732)

���������������������������������������������������������

Copyright (c) 1993-1994  All Rights Reserved               Page 37

Energy Cost 2.0  From Electrical Design Software

WHEN YOU REGISTER

~~~~~~~~~~~~~~~~~

Registration is $64.00  Upon registration you will receive the

following

1.  A copy of the latest version of ENERGY COST.

2.  Unlock the 3 record limit.

3.  Unlock the limitation on voltage and KVA input.

4.  Printing of NON REGISTERED VERSION is eliminated from the reports.

5.  Ability to send reports to an ASCII file that can be edited with

any word processor.

6.  A database compaction utility.

7. A database file repair utility.

8.  Information on updates & other reporting configurations.

9.  Your name or company displayed in the program.

10. All documentation is on disk.

WHY YOU SHOULD REGISTER

~~~~~~~~~~~~~~~~~~~~~~~

1. We will put your name on our mailing list to keep you updated on

new and revised products. We listen to you.  When a registered user

request a specific program or feature we put in our proposed projects

list. The most requested program or feature becomes our next job.

2. We are committed to producing specialized quality software for the

electrical industry, at a reasonable price. The professionals that

use our products will enhance their ability to serve their clients

and will have a decisive advantage over those that do not.

3. We at Electrical Design Software sincerely thank you for evaluating

ENERGY COST 2.0.  It is our hope that our knowledge and skill will

become a part of your effort to further your professional success.

Thank YOU.

4. Every time you reduce the usage of electrical energy you help save

the environment.  For example, did you know that every kilowatt

hour of electricity you avoid using saves over two pounds of carbon

dioxide that would otherwise be pumped into the atmosphere?.

Copyright (c) 1993-1994  All Rights Reserved               Page 38

Energy Cost 2.0  From Electrical Design Software

SUPPORT

~~~~~~~

Support for ENERGY COST 2.0 is FREE for REGISTERED users for the

first year please use any method you want.

When you call for support have available:

1. Your name or company as displayed on the opening screen.

2. Program name and version.

BY PHONE

~~~~~~~~

Monday through Friday 9:00 am to 5:00 pm Eastern Standard time phone

(603) 893-9008.  If we are unavailable at the time of your call,

please leave your name, phone number & the best time to call and

we will call back COLLECT.

-----------------------------------------------------------------

BY FAX

~~~~~~

24 hours a day 7 days a week call 603-893-9008

-----------------------------------------------------------------

BY MAIL

~~~~~~~

Send to:    Electrical Design Software

12 Vassar Drive

Pelham, NH 03076

Attn: Customer Support

Copyright (c) 1993-1994  All Rights Reserved               Page 39